| NPRR Number | 281 | NPRR Title | Replace 7-Day Forecast Requirement for QSEs Representing WGRs |
| --- | --- | --- | --- |
| Date |  | October 20, 2010 |  |
| Submitter’s Information |  |  |  |
| Name |  | Jennifer Bevill |  |
| E-mail Address |  | jlbevill@aep.com |  |
| Company |  | AEP Energy Partners |  |
| Phone Number |  |  |  |
| Cell Number |  | (512) 934-4255 |  |
| Market Segment |  | IOU |  |

Comments

| Overall Market Benefit | More reliable and accurate 7-day forecast for Weekly Reliability Unit Commitment (WRUC). |
| --- | --- |
| Overall Market Impact | More reliable and accurate 7-day forecast for WRUC. |
| Consumer Impact | N/A |

Due to impacts to Nodal Systems, the ERCOT CEO Review Process has determined that this revision is not necessary for go-live.  In order to eliminate the system impacts, AEP would like to offer the following language revisions.

Revised Proposed Protocol Language

## DEFINITIONS

Long Term Wind Powered Profile (LTWPP)

An ERCOT provided profile for each Wind-powered Generation Resource (WGR) having an aggregated Rating larger than 10 MW at its point of interconnection with the transmission system.  The profiles must forecast the daily generation shape by hourly production of wind power Renewable Production Potential (RPP) based upon 100% turbine availability for each month of the year.

3.9.1	Current Operating Plan (COP) Criteria

(1)	Each QSE that represents a Resource must submit a COP to ERCOT that reflects expected operating conditions for each Resource for each hour in the next seven Operating Days.

(2)	Each QSE that represents a Resource shall update its COP reflecting changes in availability of any Resource as soon as reasonably practicable, but in no event later than 60 minutes after the event that caused the change.

(3)	The Resource capacity in a QSE’s COP must be sufficient to supply the Ancillary Service Supply Responsibility of that QSE.

(4)	A COP must include the following for each Resource represented by the QSE:

(a)	The name of the Resource;

(b)	The expected Resource Status:

(i)	Select one of the following for Generation Resources synchronized to the ERCOT System that best describes the Resource’s status:

(A)	ONRUC – On-Line and the hour is a RUC-Committed Interval;

(B)	ONREG – On-Line Resource with Energy Offer Curve providing Regulation Service;

(C)	ON – On-Line Resource with Energy Offer Curve;

(D)	ONDSR – On-Line Dynamically Scheduled Resource (DSR);

(E)	ONOS – On-Line Resource with Output Schedule;

(F)	ONOSREG – On-Line Resource with Output Schedule providing Regulation Service;

(G)	ONDSRREG – On-Line DSR providing Regulation Service;

(H)	ONTEST – On-Line Test with Output Schedule;

(I)	ONEMR – On-Line EMR (available for commitment or dispatch only for ERCOT-declared Emergency Conditions; the QSE may appropriately set LSL and High Sustained Limit (HSL) to reflect operating limits); and

(J)	ONRR – On-Line as a synchronous condenser (hydro) providing Responsive Reserve (RRS) but unavailable for dispatch by Security-Constrained Economic Dispatch (SCED) and available for commitment by RUC.

(ii)	Select one of the following for Off-Line Generation Resources not synchronized to the ERCOT System that best describes the Resource status:

(A)	OUT – Off-Line and unavailable;

(B)	OFFNS – Off-Line but reserved for Non-Spinning Reserve (Non-Spin);

(C)	OFF – Off-Line but available for commitment by Day-Ahead Market (DAM) and RUC; and

(D)	EMR – Available for commitment only for ERCOT-declared Emergency Condition events; the QSE may appropriately set LSL and HSL to reflect operating limits; and

(iii)	Select one of the following for Load Resources:

(A)	ONRGL – Available for dispatch of Regulation Service;

(B)	ONRRCLR – Available for dispatch of Responsive Reserve Service as a Controllable Load Resource;

(C)	ONRL – Available for dispatch of RRS Service or Non-Spin, excluding Controllable Load Resources; and

(D)	OUTL – Not available;

(c)	The HSL;

(d)	The LSL;

(e)	The High Emergency Limit (HEL);

(f)	The Low Emergency Limit (LEL); and

(g)	Ancillary Service Resource Responsibility capacity in MW for:

(i)	Regulation Up (Reg-Up);

(ii)	Regulation Down (Reg-Down);

(iii)	RRS Service; and

(iv)	Non-Spin

(5)	For combined-cycle Resources, the above items are required for each operating configuration.

(6)	ERCOT may accept COPs only from QSEs.

(7)	For the first 48 hours of the COP a QSE representing a Wind-powered Generation Resource (WGR) must enter an HSL value that is less than or equal to the amount for that Resource from the most recent Short-Term Wind Power Forecast (STWPF) provided by ERCOT.

(8)	For hours 49 to 168 of the COP a QSE representing a WGR must enter an HSL value equal to the Long Term Wind Power Profile (LTWPP) provided for each WGR by ERCOT.  ERCOT shall post  the LTWPP forecasts on the MIS Secured Area.

(9)	A QSE representing a Resource that has a Resource Status of ONTEST must self-commit the Resource and must submit an Output Schedule for the Resource.