| NPRR Number | 275 | NPRR Title | Clarify QSE’s Ability to Make Changes to Ancillary Service Resource Responsibility In Real Time |
| --- | --- | --- | --- |
| Date Posted |  | September 16, 2010 |  |
| Nodal Protocol Sections Requiring Revision |  | 6.3.1, Activities for the Adjustment Period 6.3.2, Activities for Real-Time Operations 6.4.6, QSE-Requested Decommitment of Resources 6.4.6.1, QSE Request to Decommit Resources in the Operating Period |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) clarifies the responsibility of Qualified Scheduling Entities (QSEs) to telemeter the same Ancillary Service Resource Responsibility in Real-Time as the QSE had previously submitted in the Current Operating Plan (COP) for the effective Operating Hour. |  |
| Reason for Revision |  | ERCOT performs studies on the deliverability of Ancillary Services as part of its responsibilities in the Operating Period described in Section 6.4.8.1, Evaluation and Maintenance of Ancillary Service Capacity Sufficiency. Just as the status of Resources is studied for having sufficient capacity based on QSE provided information in the COP, a similar study to validate the deliverability of Ancillary Services from those Resources is also performed on the Ancillary Service Resource Responsibility information provided from the COP. Any changes to Ancillary Service Resource Responsibility subsequent to the study must be approved by ERCOT to assure continued deliverability of Ancillary Services. As such, this NPRR clarifies that QSEs may not change in Real-Time the Ancillary Service Resource Responsibility that was previously presented in the COP unless the QSE experiences a failure of the QSE’s control systems or equipment on the Resource providing the Ancillary Services. |  |
| Overall Market Benefit |  | Assures that the reliability studies performed by ERCOT validating the deliverability of Ancillary Services use data expected to be provided by QSEs in Real-Time. |  |
| Overall Market Impact |  | Providers of Ancillary Services who desire to co-optimize their delivery of Ancillary Services with any energy within the QSE’s generation portfolio must do so prior to establishing the Ancillary Services quantities for each Resource in the first hour of the COP before use of these quantities by ERCOT in deliverability studies. |  |
| Consumer Impact |  | Deliverability studies of Ancillary Services providers assures those services can be provided if needed during Real-Time operations. |  |
| Credit Implications |  | No. |  |
| Submitter Justification for Necessity Prior to the Texas Nodal Market Implementation Date |  | NPRR275 does not impact Nodal systems, budget or schedule, so there is no reason not to allow the NPRR to proceed in the stakeholder review process. |  |
| CEO Determination |  | The ERCOT CEO has determined that, as written, NPRR275 is necessary prior to the Texas Nodal Market Implementation Date. |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | Providers of Ancillary Services who desire to co-optimize their delivery of Ancillary Services with any energy within the QSE’s generation portfolio must do so prior to establishing the Ancillary Services quantities for each Resource in the first hour of the COP before use of these quantities by ERCOT in deliverability studies. | Unknown. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Assures that the reliability studies performed by ERCOT validating the deliverability of Ancillary Services use data expected to be provided by QSEs in Real-Time. | Unknown. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | John Adams |
| E-mail Address | jadams@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3130 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

Proposed Protocol Language Revision

6.3.1 	Activities for the Adjustment Period

(1)	The following table summarizes the timeline for the Adjustment Period and the activities of Qualified Scheduling Entities (QSEs) and ERCOT.  The table is intended to be only a general guide and not controlling language, and any conflict between this table and another section of the Protocols is controlled by the other section:

| Adjustment Period | QSE Activities | ERCOT Activities |
| --- | --- | --- |
| Time = From 1800 in the Day-Ahead up to one hour before the start of the Operating Hour | Submit and update Energy Trades, Capacity Trades, Self-Schedules, and Ancillary Service Trades Submit and update Output Schedules Submit and update Incremental and Decremental Energy Offer Curves for Dynamically Scheduled Resources (DSRs) Submit and update Energy Offer Curves Update Current Operating Plan (COP) Request Resource decommitments Submit Three-Part Supply Offers for Off-Line Generation Resources Submit offers for any Supplemental Ancillary Service Markets Communicate Resource Forced Outages | Post shift schedules on the Market Information System (MIS) Secure Area. Validate Energy Trades, Capacity Trades, Self-Schedules, and Ancillary Service Trades and identify invalid or mismatched trades. Validate Output Schedules Validate Incremental and Decremental Energy Offer Curves Validate Energy Offer Curves Validate Current Operating Plan (COP) including validation of the deliverability of Ancillary Services from Resources Review and approve or reject Resource decommitments Validate Three-Part Supply Offers Publish Notice of Need to Procure Additional Ancillary Service capacity if required Validate Ancillary Service Offers At the end of the Adjustment Period snap-shot the net capacity credits for Hourly Reliability Unit Commitment (HRUC) Settlement Update Short-Term Wind Power Forecast (STWPF) Execute the Hour-Ahead Sequence Notify the QSE via the MIS Certified Area that an Energy Offer Curve or Output Schedule has not yet been submitted for a Resource as a reminder that one of the two must be submitted by the end of the Adjustment Period |

6.3.2	Activities for Real-Time Operations

(1)	Activities for Real-Time operations begin at the end of the Adjustment Period and conclude at the close of the Operating Hour.

(2)	The following table summarizes the timeline for the Operating Period and the activities of QSEs and ERCOT during Real-Time operations where “T” represents any instant within the Operating Hour.  The table is intended to be only a general guide and not controlling language, and any conflict between this table and another section of the Protocols is controlled by the other section:

| Operating Period | QSE Activities | ERCOT Activities |
| --- | --- | --- |
| During the first hour of the Operating Period |  | Execute the Hour-Ahead Sequence, including HRUC, beginning with the second hour of the Operating Period Review and communicate HRUC commitments Snapshot the Scheduled Power Consumption for Controllable Load Resources |
| Before the start of each Security-Constrained Economic Dispatch (SCED) run | Update Output Schedules for DSRs | Validate Output Schedules for DSRs Execute Real-Time Sequence |
| SCED run |  | Execute SCED |
| During the Operating Hour | Telemeter the Ancillary Service Resource Responsibility for each Resource as it was previously reported to ERCOT in the COP for the Operating Hour Acknowledge receipt of Dispatch Instructions Comply with Dispatch Instruction Review Resource Status to assure current state of the Resources is properly telemetered Update COP with actual Resource Status and limits and Ancillary Service Schedules Communicate Resource Forced Outages to ERCOT Communicate to ERCOT required changes to Ancillary Service Resource Responsibility required due to failure of control equipment | Communicate all Base Points, Dispatch Instructions and LMPs for energy and Ancillary Services using Inter-Control Center Communications Protocol (ICCP) or Verbal Dispatch Instructions (VDIs) Monitor Resource Status and identify discrepancies between COP and telemetered Resource Status Restart Real-Time Sequence on major change of Resource or Transmission Element Status Monitor ERCOT total system capacity providing Ancillary Services Validate COP information Monitor ERCOT control performance Distribute by ICCP, and post to the MIS Public Area, the LMPs created by each SCED process for each Resource Node. These prices shall be posted immediately subsequent to deployment of Base Points from SCED with the time stamp the prices are effective Post SCED Shadow Prices, Hub LMP, Load Zone LMP, and LMPs for each Electrical Bus via the MIS Public Area. These prices shall be posted immediately subsequent to deployment of Base Points from SCED with the time stamp the prices are effective Post on the MIS Public Area active binding transmission constraints by Transmission Element name (contingency /overloaded element pairs) via the MIS Public Area Post the Settlement Point Prices for each Settlement Point immediately following the end of each Settlement Interval Post parameters as required by Section 6.4.8, Ancillary Services Capacity During the Adjustment Period and in Real-Time, to the MIS Secure Area |

(3)	At the beginning of each hour, ERCOT shall post on the MIS Secure Area the following information:

(a)	Changes in ERCOT System conditions that could affect the security and dynamic transmission limits of the ERCOT System, including:

(i)	Changes or expected changes, in the status of Transmission Facilities for the remaining hours of the current Operating Day and all hours of the next Operating Day, supplementing those that are already in the Outage Scheduler; and

(ii)	Any conditions such as adverse weather conditions as determined from the ERCOT-designated weather service;

(b)	Updated system-wide Load forecasts;

(c)	The quantities of Reliability Must-Run (RMR) Services deployed by ERCOT for each previous hour of the current Operating Day;

(d)	Total ERCOT System Demand, from Real-Time operations, integrated over each Settlement Interval; and

(e)	Updated Electrical Bus Load distribution factors and other information necessary to forecast Electrical Bus Loads for each hour of the current Operating Day and all hours of the next Operating Day.

6.4.6	QSE-Requested Decommitment of Resources and Changes to Ancillary Services Resource Responsibility of Resources

(1)	A Resource must remain committed during any Reliability Unit Commitment (RUC)-Committed Interval unless the Resource has a Forced Outage.

(2)	In the Operating Period, a QSE may request to decommit a Resource for any interval that is not a RUC-Committed Interval by verbally requesting ERCOT to consider its request.

(3)	In the Adjustment Period, a QSE may request to decommit a Resource for any interval that is not a RUC-Committed Interval by indicating a change in unit status in the QSE’s COP.

(4)	A Resource cannot be decommitted for just a portion of a Day-Ahead Market (DAM)-Committed Interval, which is a one-hour interval.  If a Resource that is decommitted for a DAM-Committed Interval, that one-hour DAM-Committed Interval is excluded from the calculation of any Day-Ahead Make-Whole Payment for that Resource.

(5)	In the Adjustment Period, a QSE may request ERCOT approval for moving an Ancillary Service Resource Responsibility from one Resource to another like Resource by changing its COP.  ERCOT shall use the Hourly Reliability Unit Commitment (HRUC) and other processes to study the move and if Ancillary Services becomes undeliverable in accordance with Section 6.4.8.1.2, Replacement of Undeliverable Ancillary Service Due to Transmission Constraints, ERCOT shall follow the provisions of Section 6.4.8.1.2.  The phrase “like Resource” means that Ancillary Service Resource Responsibility moves may only be from a Generation Resource to a Generation Resource or from a Load Resource to a Load Resource.

(6)	In the Operating Period, a QSE may change the Ancillary Service Resource Responsibility of a Resource from that which was reported to ERCOT in COP for the effective Operating Hour only if the Resource becomes incapable of supplying the Ancillary Service and subject to the verbal approval of ERCOT.  ERCOT shall consider the nature of the equipment failure and the need for continuing compliance by the QSE to its Ancillary Services Supply Responsibility.  Ancillary Service Resource Responsibility moves within the Operation Period may only be to another like Resource qualified to provide that Ancillary Service within the QSE’s portfolio.

6.4.6.1	QSE Request to Decommit Resources in the Operating Period

(1)	For a request made during the Operating Period to decommit a Resource, ERCOT may perform a study using Real-Time conditions to determine if ERCOT will remain n-1 secure with that Resource Off-Line.  ERCOT may grant the request provided the Resource is not providing any Ancillary Service Resource Responsibility and if analysis indicates the Resource Outage contingency results in no additional active constraints for SCED.  ERCOT may only approve requests that do not have a reliability impact.

(2)	If more units are requesting decommitment than can be accommodated, ERCOT shall review the requests in order of receipt.