### 2.3.1 SCED Inputs and Outputs

This section provides a summary of inputs and outputs to the Real-Time process (SCED). Table 2-1 lists the required input data for SCED. The Market Infrastructure (MI) listed in the table is a sub-system of the Market Management System. MI provides various functions to support QSEs data submission and validation. All the validated QSE market data are stored in the MI market database as input to SCED.

The “Source” column of the input table identifies the source where the data is provided to the SCED. The “Destination” column of the output table identifies the destination target where the data generated by SCED is sent.

Table 2-1. SCED Inputs

| Source | Data |
| --- | --- |
| Registration | - Generation Resource Parameters: (Used for Energy Offer Curve and Output Schedule validation) - Type of Resource – Need to look at the definition - QF Status – Not applicable - Non Modeled Generation flag – Not applicable - Block Load Transfer flag – Needs further discussion - Status (Active, Mothballed, or Decommissioned) -- Needed - Generation Resource, Controllable Load Resource and Load Resource Parameters: (Used for COP validation) - Resource name -- Needed - High Reasonability limit -- Needed - Low Reasonability limit -- Needed - Resource and QSE- mapping (Used for Energy Offer Curve and Output Schedule validation) -- Needed - Resource Commercial Operation Date (Used for Mitigated Energy Offer Cap Curve creation) -- Needed - Resources qualified as a synchronous condenser capable Resource (Used for COP validation) – Not applicable - Flag to indicate whether a QSE is qualified to have DSR – Needs further discussion - Flag to indicate whether the Load Resource or Controllable Load Resource is part of DSR Load – Needs little further discussion |
| Network Model Management System (NMMS) | - Hub definition (for Settlement Point Price calculation) – Not applicable - Load zone definition (for Settlement Point Price calculation) -- Needs further discussion |
| Market Infrastructure (MI) | - COP (from QSE) ( Used for Energy Offer Curve and Output Schedule validation) -- Needed - Resource Name -- Needed - Resource Status -- Needed - High Sustained Limit (HSL) and Low Sustained Limit (LSL) – Needed; review HSL and LSL definitions (Two options to explore with economic and time constraints) - Ancillary Service Schedule (Reg-up, Reg-Down, Responsive Reserve Service and Non-Spin) – Needs further discussion - Energy Offer Curve (from QSE) - QSE ID -- Needed - Resource ID -- Needed - Energy Offer Curve (price vs. quantity) -- Needed - Percentage of FIP – Not applicable - Percentage of FOP – Not applicable - Incremental and Decremental Offer Curve (from QSE) – Not applicable (for all sub-bullets) - QSE ID - Resource ID - Energy Offer Curve (price vs. quantity) - Percentage of FIP - Percentage of FOP - Output Schedules (from QSE) – Not applicable (for all sub-bullets) - QSE ID - Resource ID - Desired MW output level - DSR Energy Trades (used for DSR Output Schedule validation) – Not applicable (for all sub-bullets) - Buying QSE - Selling QSE - MW Quantity - Purchase or Sale Flag - RMR Energy Offer Curve (from ERCOT operators): – Not applicable (for all sub-bullets) - QSE ID - Resource ID - Energy Offer Curve (price vs. quantity) - Percentage of FIP - Percentage of FOP - Mitigated Offer Cap curve and Floor (calculated in MI) – Needed (for all sub bullets) - Resource ID - Mitigated Offer Cap curve (cap vs. quantity) - Mitigated Offer Floor |
| Settlement System | - (for Mitigated Energy Offer Cap Curve creation) - Verifiable incremental heat rate curve – Needs further discussion - Verifiable Variable O&M – Needs further discussion - Capacity Factor – Needs further discussion |
| Platts – Do not need to address | - Fuel Index Price (FIP) - Fuel Oil Price (FOP) |
| Energy Management System (EMS) | - State Estimator solution (for adjusting constraint limits if needed) – Do not need to address - Time stamp - Generator MW - Load MW (for calculating Load Zone Weighting Factor) - State Estimator Status - The State Estimated Flow on transmissions elements where there is a Current Transformer (CT) associated with an EPS meter installed for a Net Metering facility. [These State Estimated Flows will be used to calculate the 15 minute Settlement Price at the EPS meter location, for those EPS meters with multiple CT’s associated with the EPS meter. The 15 minute Settlement Price will be used in the Settlement for a Net Metering facility. ] - Network Constraints to be enforced in SCED from Transmission Constraint Manager (TCM) – Do not need to address - Constraint ID - Contingency name - Overloaded Element name - From/To Station name and kV level - Constraint Limit - Constraint value (the Base Case flow for a Base Case constraint, the post Contingency flow MW for a Contingency constraint) - Shift Factors - Maximum Shadow Price corresponding to each constraint - Generation Sub-system - SCADA Data - Generation MW – Not applicable - Resource Status – Not applicable - Ancillary Service Schedule including Reg-Up, Reg-Down, RRS, Non-Spin – Not applicable - Ancillary Service Responsibility including Reg-Up, Reg-Down, RRS, Non-Spin – Not applicable - Generation Resource normal and emergency limits including HSL, LSL, HEL, LEL, normal and emergency ramp rate (for information for ERCOT operator displays) – Not applicable - Combined Cycle configuration – Not applicable - DSR Telemetered Output Schedule – Not applicable - All Load Resource telemetered data (Input to a process separate from SCED for Non-Spin and RRS deployment and recall from Load Resources in Real Time) – Needed (all sub bullets) - Net real power consumption (in MW) - Breaker status - Low Power Consumption (LPC) - Maximum Power Consumption (MPC) - Responsive Reserve Resource Responsibility - Responsive Reserve Resource Schedule - Non-Spin Resource Schedule - Non-Spin Resource Responsibility - Status of the high-set under-frequency relay if applicable - Resource Status - Controllable Load Resource Scheduled Power Consumption - Resource Limit Calculator – Needed (all sub bullets) - HASL, LASL, HDL, LDL, SURAMP and SDRAMP - Generation Resource normal and emergency ramp rate (calculated based on Resource’s telemetered MW) (for information) - LFC – Not applicable (all sub bullets) - Sum of DSR loads per QSE (used for DSR Output Schedule validation) - Triggering of SCED - Other Generation Sub-System process – Not applicable (all sub bullets) - Generation Requirement - RRS deployment amount (for determining Base Point of Resources with ONRR telemetered Resource Status) - A flag to indicate Energy Emergency Alert (EEA) activation (for flagging LMP and SPP during EEA) - 15 minute data – Needs further discussion; dependent on outcome of settlement discussion - The energy at the bus associated with the settlement meter integrated for the 15 minute interval. |
| Constraint Competitiveness Test (CCT) | - List of Competitive Constraints for that operating day – Needs further discussion (Do QSEs representing LR in SCED need to be added to the CCT calculations to determine competitiveness?) |
| NMMS (Default) / MMS (Updated) | Resource Parameters for Generation (Protocol 3.7.1.1): - Normal Ramp Rate Curve -- Needed - Emergency Ramp Rate Curve -- Needed - Minimum On-line Time – Not applicable - Minimum Off-line Time – Not applicable - Hot start time – Not applicable - Intermediate Start Time – Not applicable - Cold start time – Not applicable - Max Weekly Starts – Not applicable - Max On-Line Time – Not applicable - Max Daily Starts – Not applicable - Max weekly Energy – Not applicable - Hot-to-intermediate Time – Not applicable - Intermediate-to-cold Time – Not applicable Resource Parameters for Load Resource (not Controllable) (Protocol 3.7.1.2 (1)): - Minimum Interruption Time – Needs further discussion - Minimum Restoration Time – Needs further discussion - Maximum Weekly Deployments – Not applicable - Maximum Interruption Time - Not applicable - Maximum Daily Deployments - Not applicable - Maximum Weekly Energy - Not applicable - - Minimum Notice Time - Not applicable Resource Parameters for Controllable Load Resources (Protocol 3.7.1.2 (2)) - Normal Ramp Rate Curve - Needed - Emergency Ramp Rate Curve - Needed - Maximum Deployment Time - Not applicable - Maximum Weekly Energy - Not applicable |