| NPRR Number | 320 | NPRR Title | Minimum PTP Option Bids and CRR Auction Fees |
| --- | --- | --- | --- |
| Date |  | March 1, 2011 |  |
| Submitter’s Information |  |  |  |
| Name |  | Arleen Spangler and Tamila Nikazm on behalf of the Credit Work Group (Credit WG) |  |
| E-mail Address |  | Arleen.Spangler@nrgenergy.com / tamila.nikazm@austinenergy.com |  |
| Company |  | NRG / Austin Energy |  |
| Phone Number |  | (609) 524-4846 / (512) 322-6550 |  |
| Market Segment |  | N/A |  |

Comments

The credit constraints in Section 7.5.5.3, Auction Process, of the Nodal Protocols do not include the Point-to-Point (PTP) Option Bidding Fee within the calculation of exposure.

In the event ERCOT rejects any Congestion Revenue Right (CRR) Account Holder’s PTP Option bids, CWG recommends that ERCOT informs CRR Account Holders as soon as possible and preferably before the CRR Auction submission window ends.

Revised Proposed Protocol Language

2.1	DEFINITIONS

Minimum Point-to-Point (PTP) Option Bid Price

A value recommended by the Technical Advisory Committee (TAC) and approved by the ERCOT Board for the minimum price that can be submitted into the CRR Auction for a PTP Option bid.

Point-to-Point Option Bidding Fee

A value recommended by the Technical Advisory Committee (TAC) and approved by the ERCOT Board of Directors; a fee placed on all bids for PTP Options.

7.5.2	CRR Auction Offers and Bids

(1)	To submit bids or offers into a CRR Auction, an Entity must become a CRR Account Holder and satisfy financial assurance criteria required to participate, under Section 16.8, Registration and Qualification of Congestion Revenue Rights Account Holders.

(2)	In order to enforce a volume limitation on the number of market transactions (bids and offers) submitted into the CRR Auction, ERCOT shall evaluate the maximum number transactions which are available prior to the auction, and evenly divide the limit across the CRR Account Holders.  This limit shall be designated as the allocated CRR transaction limit.  The allocated CRR transaction limitation for all CRR Account Holders will be communicated as part of the CRR Auction Notice prior to each auction.

(a)	No CRR Account Holder shall submit more than 10,000 transactions in any CRR Auction.

(b)	If the total number of transactions submitted by all Market Participants into the CRR Auction does not exceed the maximum number of transactions available prior to the auction, then the allocated CRR transaction limit will not apply and all transactions will be accepted.

(c)	If the total number of transactions submitted by CRR Account Holders exceeds the maximum number of transactions available prior to the auction, ERCOT shall notify all CRR Account Holders within one hour of the close of each CRR Auction that the maximum number of transactions has been exceeded.  Each CRR Account Holder shall then adjust their transactions to be less than or equal to the allocated CRR transaction limitation for the affected CRR Auction within one Business Day.  If the Market Participant fails to reduce its transactions to the allocated CRR transaction limitation within one Business Day, ERCOT shall reject all transactions submitted by that CRR Account Holder into the affected CRR Auction.  ERCOT will then execute the CRR Auction using the updated set of transactions as revised by Market Participants.

(d) 	Each Counter-Party is limited to a total of three CRR Account Holders.

(e)	ERCOT shall charge a Point-to-Point Option Bidding Fee on each PTP Option bid and a fee per PTP Option bid awarded  in each CRR Auction as described in Section 7.7, ERCOT Point-to-Point (PTP) Option Bid Fee,

7.5.2.3	CRR Auction Bid Criteria

(1)	A CRR Auction Bid indicates a willingness to buy CRRs at the auction clearing price, if it is equal to or less than the Not-to-Exceed Price. It must be submitted by a CRR Account Holder and must include the following:

(a)	The name of the CRR Account Holder;

(b)	The single type of CRR being bid;

(c)	The source Settlement Point and the sink Settlement Point or name of flowgate for the block of CRRs being bid;

(d)	The month for which the block of CRRs is being bid, including block designation;

(e)	The quantity of CRRs in MW, which must be the same for each hour within the block,  for which the Not-to-Exceed Price is effective; and

(f)	A dollars per CRR (i.e. dollars per MW per hour) for the Not-to-Exceed Price.

(2)	The CRR Account Holder may submit a self-imposed auction-wide credit limit, if desired.

(3)	A bid to buy a PTP Option or FGR cannot specify a Not-to-Exceed Price less than the ERCOT Board-approved Minimum PTP Option Bid Price.

(4)	A bid to buy a PTP Obligation can specify a negative Not-to-Exceed Price.

(5)	A bid to buy an FGR must specify the name of a flowgate defined in Section 7.3.1, Flowgates.

(6)	A CRR bid for a specified MW quantity of CRRs constitutes a bid to buy a quantity of CRRs equal to or less than the specified quantity.  A CRR bid may not specify a minimum quantity of MW that the CRR Account Holder wishes to buy.

7.5.5.3	Auction Process

(1)	The auction must be a single-round, simultaneous auction for selling the CRRs available for all auction products, with the following steps:

(a)	ERCOT shall enter into the CRR Auction engine model a credit constraint for each Counter-Party.  A Counter-Party’s CRR Auction credit limit is equal to the lesser of the credit limit as determined in Section 16.11.4.6.1, Credit Requirements for CRR Auction Participation, or, if provided, the Counter-Party’s self-imposed CRR Auction credit limit.  A Counter-Party’s CRR Auction credit limit will be reduced by the amount of its assessed PTP Option Bidding Fees as described in Section 7.7, ERCOT Point-to-Point (PTP) Option Bid Fees.  CRR Auction bids and offers of all CRR Account Holders of the Counter-Party are accepted in the order submitted while ensuring the PTP Option Bidding Fees from accepted bids do not exceed the Counter-Party’s CRR Auction credit limit.  ERCOT shall reject the CRR Account Holder’s PTP Option bids if the PTP Option Bidding Fees exceed the CRR Account Holder’s credit limit, and shall notify the CRR Account Holder as soon as practicable.  The credit constraint for each Counter-Party ensures that the following sum for all of the Counter-Party’s CRR Account Holders is less than or equal to the Counter-Party’s CRR Auction credit limit, net of the amount on its assessed PTP Option Bidding Fees as described in Section 7.7:

(i)	All awarded CRR Auction Bids multiplied by the absolute value of the corresponding bid price; plus

(ii)	All awarded CRR Auction Offers with negative offer prices multiplied by the absolute value of their corresponding offer price; plus

(iii)	The additional credit requirement for all awarded PTP Obligations, which is $A per MW per hour, plus the absolute value of the PTP Obligation bid price multiplied by M.  The values of A and M Shall be posted on the MIS Public Area.  TAC shall review these values at least annually and may recommend, to the ERCOT Board, changes to these values that become effective at least 30 days prior to a monthly CRR Auction and 60 days prior to an annual CRR Auction.  Any changes to these values shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval.

(b)	ERCOT shall award CRRs in quantities truncated to the nearest tenth MW (0.1 MW).

(c)	The CRR clearing price is equal to the corresponding Shadow Price for that CRR product.  If the CRR clearing price for a PTP Option is less than the ERCOT Board-approved Minimum PTP Option Bid Price, then ERCOT shall charge a fee equal to the positive difference between the Minimum PTP Option Bid Price and the CRR clearing price for the quantity of awarded PTP Option bids.

(d)	When a CRR Account Holder is awarded CRRs as a result of a CRR Auction, the CRRs do not become the property of the winning CRR Account Holder, and the CRRs may not be placed in their CRR accounts, until the CRR Invoices have been paid in full.

(e)	When a CRR Account Holder sells PTP Obligations as a result of an auction at a negative price, the CRR Account Holder is not relieved of the PTP Obligations until the CRR Invoices have been paid in full.

(2)	ERCOT shall use a linear programming auction engine model for each CRR Auction that evaluates all CRR Auction Bids and CRR Auction Offers submitted, and selects a combination of CRR Auction Bids and CRR Auction Offers that:

(a)	Makes the solution simultaneously feasible within the limits of the ERCOT network capability over the auction term; and

(b)	Maximizes the objective function, which is equal to the total economic value (as expressed in the CRR Auction Bids) of the awarded CRR Auction Bids, less the total economic cost (as expressed in CRR Auction Offers) of the awarded CRR Auction Offers, while observing all applicable constraints.

(3)	The CRR Network Model must, to the extent practicable, reflect the continuous and post-contingency system operating limits and operational procedures (i.e., Special Protection Systems and Remedial Action Plans) in the Network Operations Model used by ERCOT during Real-Time Operations, as discussed below in Section 7.5.5.4, Simultaneous Feasibility Test.

(4)	Once a CRR Auction is complete, ERCOT shall archive and keep the CRR Auction system and all models used to finalize the CRR Auction results under ERCOT’s data retention policy as that policy applies to data that may be needed to resolve requests for billing adjustments under applicable billing adjustment procedures.

7.5.7	Method for Distributing CRR Auction Revenues

(1)	ERCOT shall determine, for each month, the CRR Monthly Revenues (CMR). The CMR is the sum of:

(a)	Monthly CRR revenue for that month; and

(b)	PCRR revenues.

(2)	For the first three years after the TNT Market Implementation Date, ERCOT shall credit the net CRR Auction revenue (including PCRR revenue) produced from CRRs cleared in each CRR Auction that source from a Settlement Point located within a 2003 ERCOT Congestion Management Zone (CMZ) and sink at a Settlement Point located within the same 2003 ERCOT CMZ to QSEs in the 2003 ERCOT CMZ on a zonal Load Ratio Share basis.  All other net CRR Auction revenues must be allocated to QSEs on an ERCOT-wide Load Ratio Share basis.  For these allocation purposes, any NOIE Load Zone is considered to be located entirely within the 2003 ERCOT CMZ that represented the largest Load for that NOIE or group of NOIEs in 2003.  Before the end of the three-year period described above, the ERCOT Board shall consider whether to extend the policy or ratify some other alternative.

(3)	For Initial distribution of CRR Monthly Revenues, revenues shall be paid to each QSE based on that QSE’s Load Ratio Share in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the month.

(4)	ERCOT shall true up the distribution of CRR Monthly Revenues based on that QSE’s Load Ratio Share in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the month.

(5)	The net CRR Auction Revenue produced from CRRs cleared and paid for in each CRR Auction that source from a Settlement Point within a 2003 ERCOT CMZ and sink at a Settlement Point located within the same 2003 ERCOT CMZ shall be distributed on a zonal Load Ratio Share basis.  The portion of the net monthly CRR Auction Revenue to be distributed to each QSE with load in that zone for a given month is calculated as follows:

LACMRZAMT z, q=	(-1) * Σa(CRRZREV z, a + PCRRZREV z, a) * MLRSZ z, q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LACMRZAMT z, q | $ | Load-Allocated CRR Monthly Revenue Zonal Amount per zone per QSE—The payment to QSE q of the revenues resulted from the CRRs that source and sink in CMZ z, for the month. |
| CRRZREV z, a | $ | CRR Zonal Revenue per zone per CRR Auction—The revenue resulted from the CRRs that source and sink in CMZ z, cleared through CRR Auction Offers and CRR Auction Bids in CRR Auction a, for the month. |
| PCRRZREV z, a | $ | PCRR Zonal Revenue per zone per CRR Auction—The revenue resulted from the PCRRs that source and sink in CMZ z, pertaining to CRR Auction a, for the month. |
| MLRSZ q, z | none | Monthly Load Ratio Share Zonal per QSE per zone—The LRS of QSE q for its Load in CMZ z, for the peak-Load 15-minute Settlement Interval in the month. |
| q | none | A QSE. |
| z | none | A 2003 ERCOT CMZ. |
| a | none | A CRR Auction. |

(6)	The net CRR Auction Revenue produced from CRRs cleared and paid for in each CRR Auction that do not source from a Settlement Point within a 2003 ERCOT CMZ and sink at a Settlement Point located within the same 2003 ERCOT CMZ shall be distributed on an ERCOT-wide LRS basis.  The portion of the net monthly CRR Auction Revenue Amount (from CRRs with paths that cross the 2003 ERCOT CMZ boundaries) to be distributed for a given month is calculated as follows:

LACMRNZAMT q	=	(-1) * Σa(CRRNZREV a + PCRRNZREV a) * MLRS q

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LACMRNZAMT q | $ | Load-Allocated CRR Monthly Revenue Non-Zonal Amount per QSE—The payment to QSE q of the revenues resulted from the CRRs that source and sink in different CMZs, for the month. |
| CRRNZREV a | $ | CRR Zonal Revenue per CRR Auction—The revenue resulted from the CRRs that source and sink in different CMZs, cleared through CRR Auction Offers and CRR Auction Bids in CRR Auction a, for the month. |
| PCRRNZREV a | $ | PCRR Zonal Revenue per CRR Auction—The revenue resulted from the PCRRs that source and sink in different CMZs, pertaining to CRR Auction a, for the month. |
| MLRS q | none | Monthly Load Ratio Share per QSE—The LRS calculated for QSE q for the peak-Load 15-minute Settlement Interval in the month. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |
| a | none | A CRR Auction. |

Σa

7.6	CRR Balancing Account

(1)	In the DAM, if the Congestion Rent is equal to or greater than the net amounts due to all CRR Owners for any Settlement Interval, then ERCOT shall pay the net amounts due to the CRR Owners and put any excess amount into the CRR Balancing Account.

(2)	In the DAM, if the Congestion Rent is less than the net amounts due to all CRR Owners for any Settlement Interval, then ERCOT shall short-pay each CRR Owner on a prorated basis and shall keep track of how much each CRR Owner has been short-paid. The proration must be calculated using only the amounts due to the CRR Owner for CRRs settled in both the DAM and Real-Time and not using amounts due to ERCOT for PTP Obligations owned by the CRR Owner.

(3)	ERCOT shall put any revenue collected from ERCOT Point-to-Point (PTP) Option Bids and charges  into the CRR Balancing Account after the revenue is received from CRR Account Holders who bid in each CRR Auction.

(4)	ERCOT shall pay any positive balance in the CRR Balancing Account to each short-paid CRR Owner, with the amount paid to each CRR Owner being the lesser of (a) a prorated amount based on the short-paid amount for that CRR Owner compared to the total short-paid amount, and (b) the short-paid amount for that CRR Owner. Any remaining positive balance in the CRR Balancing Account must be allocated to all QSEs on the QSE’s Load Ratio Share in the interval coincident with the ERCOT-wide peak 15-minute Settlement Interval for the month.

7.7	ERCOT Point-to-Point (PTP) Option Bid Fees

After the close of each Congestion Revenue Right (CRR) Auction, ERCOT shall charge each CRR Account Holder a fee for any PTP Option bids; as approved by the ERCOT Board of Directors.

In addition, ERCOT will charge each CRR Account Holder for each PTP Option bid awarded where the clearing price for the PTP Option bid awarded is less than the Minimum PTP Option Bid Price.  The charge to be assessed to each CRR Account Holder will be determined by calculating the sum of the product of the quantity of each PTP Option bidsawarded to that CRR Account Holder and the positive difference between the Minimum PTP Option Bid Price and the clearing price for the CTP Option for that auction.

(a)	The Minimum PTP Option Bid Price shall be recommended by the Technical Advisory Committee (TAC) and approved by the ERCOT Board.  TAC and the ERCOT Board shall review the current minimum bid price at least every January and may recommend to the ERCOT Board changes to these values.  Changes to the minimum bid price are effective upon ERCOT Board approval, and such revised values shall be posted on the MIS Public Area a minimum of seven days prior to the CRR Auction lockdown timeline .

(b)	ERCOT shall charge each CRR Account Holder for its PTP Option bids awarded in each CRR Auction.  The charge for each source and sink pair for a given Operating Hour is calculated as follows:

OPTPCHGAMT crrh, (j, k), a	= Σa(TOTOPTBIDS crrh, a * OPTFEE + Max[ , OPTMBP - OPTPR (j, k), a ]
* PTP crrh, (j, k), a ) cr

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| OPTPCHGAMT crrh, (j, k), a | $ | PTP Option Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids with the source j and the sink k awarded in CRR Auction a, for the hour plus the total number of PTP Options the CRR Account Holder bid on times the bid fee. |
| OPTFEE | $/MW/ per hour | Point-to-Point (PTP) Option Fee – The ERCOT Board approved Point-to-Point Option Bidding Fee assessed on all CRR Option Bids. |
| OPTPR (j, k), a | $/MW per hour | PTP Option Price per source and sink pair per CRR Auction—The clearing price of a PTP Option with the source j and the sink k in CRR Auction a, for the hour. |
| OPTP crrh, (j, k), a | MW | PTP Option Purchase per CRR Account Holder per source and sink pair per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Option bids associated with the source j and the sink k awarded in CRR Auction a, for the hour. |
| OPTMBP | $/MW per hour | Minimum Point-to-Point (PTP) Option Bid Price—As defined in Section 2.1, Definitions. |
| TOTOPTBIDS crrh, a | CRR | The total number of PTP Option CRRs the CRR Account Holder bid on in the auction (not the total number of bids). |
| Crrh | None | A CRR Account Holder. |
| J | None | A source Settlement Point. |
| k | None | A sink Settlement Point. |
| a | None | A CRR Auction. |

7.9.3.2	Credit to CRR Balancing Account

If the Day-Ahead Congestion Rent is greater than the total payment to all CRR Owners for the CRRs settled in the DAM for any Operating Hour, a credit is put into the CRR Balancing Account for that Operating Hour.  The credit to the CRR Balancing Account for a given Operating Hour is calculated as follows:

CRRBACR	=	Max (0, (DACONGRENT + DACRRCRTOT + DACRRCHTOT + OPTPAMTTOT))

Where:

DACRRCRTOT	=	DAOBLCRTOT + DAOBLRCRTOT + DAOPTAMTTOT + DAOPTRAMTTOT + DAFGRAMTTOT

DACRRCHTOT	=	DAOBLCHTOT + DAOBLRCHTOT

DAOBLCRTOT	=	ΣoDAOBLCROTOT o

DAOBLCHTOT	=	ΣoDAOBLCHOTOT o

DAOBLRCRTOT	=	ΣoDAOBLRCROTOT o

DAOBLRCHTOT	=	ΣoDAOBLRCHOTOT o

DAOPTAMTTOT	=	ΣoDAOPTAMTOTOT o

DAOPTRAMTTOT	=	ΣoDAOPTRAMTOTOT o

DAFGRAMTTOT	=	ΣoDAFGRAMTOTOT o

OPTPCHGAMTTOT	=	ΣcrrhOPTPCHGAMTcrr, a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| CRRBACR | $ | CRR Balancing Account Credit—The credit to the CRR Balancing Account for the hour. |
| DACONGRENT | $ | Day-Ahead Congestion Rent—The congestion rent collected in the DAM for the hour. See Section 7.9.3.1, DAM Congestion Rent. |
| DACRRCRTOT | $ | Day-Ahead CRR Credit Total—The total payment to all CRR Owners of all CRRs settled in the DAM, for the hour. |
| DACRRCHTOT | $ | Day-Ahead CRR Charge Total—The total charge to all CRR Owners of all CRRs settled in the DAM, for the hour. |
| DAOBLCRTOT | $ | Day-Ahead Obligation Credit Total—The total payment of all PTP Obligations settled in the DAM, for the hour. |
| DAOBLCHTOT | $ | Day-Ahead Obligation Charge Total—The total charge of all PTP Obligations settled in the DAM, for the hour. |
| DAOBLRCRTOT | $ | Day-Ahead Obligation with Refund Credit Total—The total payment of all PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOBLRCHTOT | $ | Day-Ahead Obligation with Refund Charge Total—The total charge of all PTP Obligations with Refund settled in the DAM, for the hour. |
| DAOPTAMTTOT | $ | Day-Ahead Option Amount Total—The total payment of all PTP Options settled in the DAM, for the hour. |
| DAOPTRAMTTOT | $ | Day-Ahead Option with Refund Amount Total—The total payment of all PTP Options with Refund settled in the DAM, for the hour. |
| DAFGRAMTTOT | $ | Day-Ahead FGR Amount Total—The total payment of all FGRs settled in the DAM, for the hour. |
| DAOBLCROTOT o | $ | Day-Ahead Obligation Credit Owner Total per owner—The total payment to CRR Owner o of PTP Obligations settled in the DAM, for the hour. See Section 7.9.1.1, Payments and Charges for PTP Obligations Settled in DAM. |
| DAOBLCHOTOT o | $ | Day-Ahead Obligation Charge Owner Total per owner—The total charge to CRR Owner o of PTP Obligations settled in the DAM, for the hour. See Section 7.9.1.1. |
| DAOBLRCROTOT o | $ | Day-Ahead Obligation with Refund Credit Owner Total per owner—The total payment to the CRR Owner o of PTP Obligations with Refund settled in the DAM, for the hour. See Section 7.9.1.5, Payments and Charges for PTP Obligations with Refund Settled in DAM. |
| DAOBLRCHOTOT o | $ | Day-Ahead Obligation with Refund Charge Owner Total per owner—The total charge to CRR Owner o of PTP Obligations with Refund settled in the DAM, for the hour. See Section 7.9.1.5. |
| DAOPTAMTOTOT o | $ | Day-Ahead Option Amount Owner Total per owner—The total payment to the CRR Owner o of PTP Options settled in the DAM, for the hour. See Section 7.9.1.2, Payments for PTP Options Settled in DAM. |
| DAOPTRAMTOTOT o | $ | Day-Ahead Option with Refund Amount Owner Total per owner—The total payment to the CRR Owner o of PTP Options with Refund settled in the DAM, for the hour. See Section 7.9.1.6, Payments for PTP Options with Refund Settled in DAM. |
| DAFGRAMTOTOT o | $ | Day-Ahead FGR Amount Owner Total per owner—The total payment to the CRR Owner o of FGRs settled in the DAM, for the hour. See Section 7.9.1.4, Payments for FGRs Settled in DAM. |
| OPTPCHGAMTTOT | $ | The sum of the PTP Option Fees and charges charged to all CRR Account Holder in an Auction, as described in Section 7.7 |
| OPTPCHGAMT | $ | The sum of all PTP Option Fees and charges charged to one CRR Account Holder in an Auction, as described in Section 7.7 |
| o | none | A CRR Owner. |

9.8	CRR Auction Award Invoices

(1)	ERCOT shall prepare invoices for each CRR Auction (CRR Auction Invoice) on a net basis.  Invoices must be issued on the first Business Day following the completion of a CRR Auction on the date specified in the Settlement Calendar.  For each CRR Auction Invoice, the CRR Account Holder to whom the Invoice is addressed (“Invoice Recipient”) is either a net payee or net payor.  The Invoice Recipient is responsible for accessing the CRR Auction Invoice on the MIS Certified Area once posted by ERCOT.

(2)	Each Invoice Recipient shall pay any net debit and be entitled to receive any net credit shown on the CRR Auction Invoice on the payment due date.  Payments for CRR Auction Invoices are due on the applicable payment due date, whether or not there is any Settlement and Billing dispute regarding the amount of the payment.

(3)	ERCOT shall post on the MIS Certified Area for each Invoice Recipient a CRR Auction Invoice based on CRR Auction charges and payments as set forth in:

(a)	Section 7.5.6.1, Payment of an Awarded CRR Auction Offer;

(b)	Section 7.5.6.2, Charge of an Awarded CRR Auction Bid;

(c)	Section 7.5.6.3, Charge of PCRRs Pertaining to a CRR Auction; and

(d)	Section 7.7, ERCOT Point-to-Point (PTP) Option Bid Fees.

(4)	CRR Auction Invoices must contain the following information:

(a)	The Invoice Recipient’s name;

(b)	The ERCOT identifier (Settlement identification number issued by ERCOT);

(c)	Net Amount Due/Payable – the aggregate summary of all charges owed to or due from the Invoice Recipient summarized by CRR Auction;

(d)	Time Period – the CRR Auction for which the Invoice is generated;

(e)	Run Date – the date on which ERCOT created and published the Invoice;

(f)	Invoice Reference Number – a unique number generated by ERCOT for payment tracking purposes;

(g)	Product Description – a description of each product awarded in, sold in, or allocated before the CRR Auction;

(h)	Payment Date – the date and time that Invoice amounts are to be paid or received; and

(i)	Remittance Information Details – details including the account number, bank name and electronic transfer instructions of the ERCOT account to which any amounts owed by the Invoice Recipient are to be paid or of the Invoice Recipient’s account from which ERCOT may draw payments due.