2705 West Lake Drive

Taylor, Texas 76574

(512) 248-6800

HTTP://WWW.ERCOT.COM

Methodologies for Determining Actual Fuel Prices Incurred by QSEs for RUC Commitments

V0.01

March 3, 2011

Table of Contents

Section 1 – Introduction	3

Section 2 – Statement of Purpose	3

Section 3 – Proposed Methodology for Determining the SUPR and MEPR	3

# Section 1 – Introduction

Per Nodal Protocol Section 9.14.7, Disputes for RUC Make-Whole Payment for Exceptional Fuel Costs, QSEs may recover the actual price of fuel paid for providing a Reliability Unit Commitment (RUC) if the actual fuel price is higher than FIP (or FOP) plus VOX (value of X).  This white paper describes a methodology to determine the Startup Price (SUPR) and Minimum Energy Price (MEPR) when a QSE submits a dispute to recoup actual fuel prices.

# Section 2 – Statement of Purpose

As amended by NPRR174, FIP Modifications in Verifiable Startup and Minimum Energy Cost and Recovery of Exceptional Fuel Costs During RUC Intervals, and NPRR212, Disputing Fuel Oil Price (FOP) Costs, Section 9.14.7 allows the recovery of the actual fuel price paid whenever a Resource receives a RUC commitment.  In order to compensate the QSE for the incremental fuel cost, ERCOT will recalculate the RUC Make-Whole Payments using the actual fuel price paid.  Specifically, ERCOT will recalculate the RUC Guarantee with the higher fuel price.  However, given that the QSE may submit Startup and Minimum Energy Offers that are below the Offer caps, ERCOT must make some assumptions on what the QSE “actually” offered since the Offers are composed of the fuel rate, fuel price and O&M values.  Section 3 below describes two scenarios that may occur that require market input.

# Section 3 – Proposed Methodology for Determining the SUPR and MEPR

There are four possible scenarios that may arise when QSEs dispute additional fuel compensation.  This white paper only addresses two of the scenarios that require market approval.

- No approved Verifiable Costs and approved Offers

- Approved Verifiable Costs and approved Offers

Scenario 1: No approved Verifiable Costs and Approved Offers

MEPR ($/MWh) = HROffer * AFPrice

Where,

MEPR = Minimum Energy Price

HROffer = Minimum-Energy Offer / (FIP/FOP)

AFPrice = Actual price of fuel paid

Note:  The calculated Offered Heat Rate (HROffer) is used instead of the Generic Heat Rate otherwise the Resource could be paid more than what it Offered.  The maximum (Cap) Heat Rate value the QSE can submit with an Offer is the Generic Heat Rate.  Since generic startup costs do not include fuel, the SUPR is not calculated and additional compensation is only based on the MEPR.

Scenario 2: Approved Verifiable Costs and Approved Offers

If Offers < Cap

When offers are less than the Cap, it is assumed that the QSE has used a low O&M (even negative) value to construct its Offer since it is unlikely a lower fuel rate or price was used.

Therefore,

SUO = VC Fuel * FIP/FOP + O&MNew

Where,

SUO = Startup Offer

VC Fuel = approved fuel rate, and

O&MNew = SUO - VC Fuel * FIP/FOP

Note:  the O&MNew could be negative when the SUO is    below the product of VC Fuel * FIP/FOP

Or

SUPR ($)  = VC Fuel * AFPrice + O&MNew

Where

SUPR = Startup Price

And

MEO = HRVC * FIP/FOP + O&MNew-LSL

Where,

HRVC = approved heat rate, and

O&MNew-LSL = MEO – HRVC * FIP/FOP

Note:  the O&MNew-LSL could be negative when the MEO is below the product of HRVC * FIP/FOP

Then

MEPR ($/MWh) = HRVC * AFPrice + O&MNew-LSL

If the Resource does not have approved O&M at Minimum Energy, then HRVC is replaced with HROffer.

Or

MEPR = HROffer * AFPrice

Where,

HROffer = MEO / (FIP/FOP)