Texas SET Change Control Request Form Change Control Number: 2011 -778 Implementation Version: 4.0

This Section Is Completed by Submitter of Change Control Request Only:

| Submitter Name: Ed Echols | Submitting Company Name: Oncor | Phone Number: 469-261-2483 |
| --- | --- | --- |
| Date of Submission: 3/11/11 | Affected TX SET Transaction(s): 814_20 | Submitter’s E-Mail Address: Ed.echols@oncor.com |
| Texas SET Issue cross-reference number: |  | Protocol Impact (Y/N): N |
| Detailed Description and Reason for Proposed Change(s): In order to provide REPs information on whether an ESI ID has a non-Advanced Meter installed at the premise, a new REF segment will be added to the 814_20. The scenario of a TDSP in certain instances may need to be shown as non-AMS meter. The CR gaining the PRMS will be aware that the meter is and AMSM or AMSR with those two (2) codes in the registration related transactions. If the registration related transactions do not contain the AMSM or AMSR code then the default is understood to be “R” (ie AMSM or AMSR is being removed). |  |  |
| NOTE: Requester must complete above fields and include a redlined example of modifications to each impacted implementation guide. This must be included at the time the request form is submitted. Please submit this completed form via e-mail to txsetchangecontrol@ercot.com and RMS Chair. |  |  |

This Section Is Used to Request a Revision of an Approved Change Control Only:

For ERCOT Change Control Manager Use Only:

| Texas SET Recommendation: Approval | Recommendation for Emergency (Y/N): N | Date of TX SET Recommendation: 03/21/11 |
| --- | --- | --- |
| Detailed Description and Reason for Revision: Recommend Approval with changes made on 03/21/11 |  |  |
| RMS Decision: Approved | Emergency (Y/N): N | Date of RMS Decision: 03/23/2011 |
| Summary of RMS Discussion: |  |  |

814_20 Transaction TDSP to ERCOT and ERCOT to CR

Segment:	REF Reference Identification (AMS Indicator)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The AMS Indicator is used to identify a meter that has a Load Profile of AMS and is provisioned at ERCOT. Conversion/Create ESI ID Request: Optional Change ESI ID Information Request: Required if changing this item Retire ESI ID Request: Not Used |
| --- | --- | --- |
|  |  | REF~MR~AMSM |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | MR |  | Merchandise Type Code |  |  |  |  |  |
|  |  |  |  |  |  | Indication that an AMS meter has been installed and provisioned at the premise for this ESI ID or is a non-AMS meter |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | AMSM |  | AMS Manual |  |  |  |  |  |
|  |  |  |  |  |  | ESI ID has provisioned AMS meter without remote connect and disconnect capability |  |  |  |  |  |
|  |  |  |  | AMSR |  | AMS Remote |  |  |  |  |  |
|  |  |  |  |  |  | ESI ID has provisioned AMS meter with remote connect and disconnect capability |  |  |  |  |  |
|  |  |  |  | R |  | Remove |  |  |  |  |  |
|  |  |  |  |  |  | Remove the AMSM or AMSR indicator. |  |  |  |  |  |

Segment 	Segment:	REF Reference Identification (Reason for Change)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | This transaction allows multiple change request on a single 814. If any single change request is invalid, the transaction is rejected in its entirety with appropriate reject reasons. More than one REF~TD may be present. |
| --- | --- | --- |
|  |  | Conversion/Create ESI ID Request: Not Used Change ESI ID Information Request: Required change is at an ESI ID level (Header or LIN Loop), otherwise not used. Retire ESI ID Request: Not Used |
|  |  | REF~TD~N18R |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |
|  |  |  |  | TD |  | Reason for Change |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |
|  |  |  |  | DTM307 |  | Change Pilot Eligibility Date |  |  |  |  |
|  |  |  |  | N18R |  | Change Customer Name/Service Address |  |  |  |  |
|  |  |  |  | REFAQ |  | Change Distribution Loss Factor Code |  |  |  |  |
|  |  |  |  | REFMR |  | Change AMS Indicator |  |  |  |  |
|  |  |  |  | REFPTC |  | Change Premise Type |  |  |  |  |
|  |  |  |  | REFSPL |  | Change Station ID |  |  |  |  |
|  |  |  |  | REFSR |  | Change Power Region |  |  |  |  |
|  |  |  |  | REFSU |  | Change Special Needs Indicator |  |  |  |  |