Texas

Standard

Electronic

Transaction

814_22:

##### CSA CR Move In Request

Electronic Data Interchange

ANSI ASC X12 Ver/Rel 004010

Transaction Set 814

Texas 814_22:

CSA

CR Move In Request

This transaction set, from ERCOT to CSA CR, is used to start CSA service of the ESI ID.

Document Flow:

- ERCOT to CSA CR

The Functional Acknowledgement (997) transaction set from the receiver of the originating transaction to the sender of the originating transaction, is used to acknowledge the receipt of the originating transaction and indicate whether the transaction passed ANSI X12 validation.  This acknowledgement does not imply that the originating transaction passed Texas SET validation. “CR, TDSP, or ERCOT shall respond with a 997 within 24 hours of receipt of an inbound transaction.”

|  |  | # Summary of Changes |
| --- | --- | --- |
| July 3, 2000 Version 1.0 |  | Initial Release |
| August 23, 2000 Version 1.1 |  | The following changes were made: |
|  |  | - Changed Life Support (REF~SU) to remove “Required” and add the following note, “We are not sure of the requirements of this information at this time. When the rules are finalized we will revisit whether the life support indicator will be sent.” |
|  |  | - Corrected Scenarios |
| December 12, 2000 Version 1.2 |  | The following changes were made: |
|  |  | - Change the term Utility to Transmission Distribution Service Provider (TDSP) to match ERCOT Protocols. |
|  |  | - Change the term Retail Electric Provider (REP) to Competitive Retailer (CR) to match ERCOT Protocols. |
|  |  | - Change description page transaction purpose to read the same as ERCOT Protocols. |
|  |  | - Insert the “How to use this Implementation Guide” page. |
|  |  | - Remove “.” between X and 12 in all references to ANSI ASC X12. |
|  |  | - Add BGN08 Element to indicate the TX SET Transaction Number per Change Control #2000-015. |
|  |  | - Add examples to end of transaction. |
|  |  | - Add Time of Use code 71-- Summer Super On-Peak to REF04 of REF~4P and REF~IX Segments in the NM1 loop per Change Control #2000-18. |
| March 9, 2001 Version 1.3 |  | The following changes were made: |
|  |  | - Removed Scenario Names from Transaction Description page |
|  |  | - Corrected the How to Use this Implementation Guide page |
|  |  | - Changed RA/Clearinghouse to ERCOT |
|  |  | - Corrected BGN06 to “Must Use” |
|  |  | - Removed second gray box example on N1~AY (ERCOT) |
|  |  | - Added decimal gray box example to REF~4P Meter Multiplier |
| August 3, 2001 Version 1.4 |  | The following changes were made: |
|  |  | - Added REF~PRT Unmetered Service Type. Ref. 2001-041 & 2001-129 |
|  |  | - Added REF~SPL Unmetered Service Type. Ref. 2001-041 & 2001-129 |
|  |  | - Added REF~AQ Unmetered Service Type. Ref. 2001-041 & 2001-129 |
|  |  | - Modify gray box in REF~4P. Ref. 2001-118 |
|  |  | - Modify gray box in REF~LO. Ref. 2001-116 |
|  |  | - Modify examples to show real load profiles and station id’s. Ref. 2001-116 |
|  |  | - Modify gray box on REF01 of REF~LO. Ref. 2001-116 |
|  |  | - Modified example in REF~IX. Changed from K1MON to KHMON |
|  |  | - Fixed example in gray box of REF~MT from “REF~IX~6.0~TU^51” to “REF~IX~6.0~KHMON~TU^51” |
| April 1, 2002 Version 1.5 |  | The following changes were made: |
|  |  | - Add REF~PMT for premise type code Ref. 2001-161 |
|  |  | - Change the language in the REF~PRT gray box Unmetered Ref. 2001-215 |
|  |  | - Change the language in definitions for the values of Premise Type Code Ref. 2001-236 |
|  |  | - Change the gray box for REF Meter Read cycle / date Ref. 2002-284 |
|  |  | - Change Response to Required for REF~SPL and REF~AQ. Ref. 2002-289 |
| May 27th, 2003 Version 1.6 |  | The following changes were made: |
| 5/27/03 |  | - Change Control 2002-386 – Removed D-U-N-S+4 from the N104 gray box in the N1~AY loop |
| July 21st, 2003 Version 2.0 |  | The following changes were made: |
| 8/15/03 |  | - Change Control 2003-547 – Please include the Move-In/Move-Out Task Force (MIMO) High Level Requirement Specification as part of the Baseline for Version 2.0. |
| October 8, 2004 Version 2.0A |  | Change Control 2004-634: - As per discussions at the June 04 TX SET meeting, additional language should be added to each Transaction Set to identify the requirements and required response to the 997 Functional acknowledgement – The Functional Acknowledgement (997) transaction set from the receiver of the originating transaction to the sender of the originating transaction, is used to acknowledge the receipt of the originating transaction and indicate whether the transaction passed ANSI X12 validation. This acknowledgement does not imply that the originating transaction passed Texas SET validation. “CR, TDSP, or ERCOT shall respond with a 997 within 24 hours of receipt of an inbound transaction.” |
| March 1, 2005 Version 2.1 |  | Change Control 2003-490: - Remove QTR from valid values of Meter Type Change Control 2003-569: - Provide gray box clarification to the LIN segment to indicate that the Texas Market supports only 1 LIN loop per EDI Transaction. Change Control 2005-683: - Add clarity to the transaction notes section regarding the Texas Market use of characters in alphanumeric fields |
| September 1, 2006 Version 3.0 |  | Change Control 2004-678: - Update the Example on the 814_22 Implementation Guide to remove the unmetered REF~PRT segment (Unmetered Service Type) since there is already a metered segment in the same NM1 loop. Both Metered and Unmetered Services cannot be sent in the same NM1 Loop. |
| November 30, 2010 Version 3.0A |  | Change Control 2009-729: - Remove all examples from the TX SET Implementation guides and post them into a separate document for quicker correction and addition of new examples without a TX SET release |
| TBD, 2012 Version 4.0 |  | Change Control 2010-738: - New Critical Care status designations are required when Critical Care = Y and additional Contact information is required for certain Critical Care status. Change Control 2010-747: - In order to provide REPs information on whether an ESI ID has a provisioned Advanced Meter installed at the premise, a new REF segment will be added to the 814_04, 814_05, 814_14, 814_22 and 814_20. Change Control 2010-748: - The purpose of this Change Control is to sync the names of the Texas SET Guide transactions, with the names of the transactions in Protocol and the Retail Market Guide. Change Control 2010-760: - Add a new code in the ESIID attributes to specify LED bulb types for Unmetered Service type. Change Control 2010-761: - Add to the existing REF02 of the REF~PRT segment the "WF" code to represent Wi-Fi (Wireless Fidelity). Change Control 2010-764: - Add an expiration date to the Special Needs designation to allow CRs to audit these designations and ask TDSPs for clarification of whether a status has been removed if the expiration date has passed without receiving an update. Change Control 2010-765: - The final rule language for Critical Care adopted the concept that TDSPs are required to provide notice to REPs when a Customer's ESIID is placed on Temporary Status for either Critical Care Residential or Chronic Care Residential. Adding 2 additional codes for the temporary Critical Care status Change Control 2010-773: - Clarifications to bring the TX SET Implementation Guides compliant and resolve disparities between Change Control 2010-738 and PUCT Critical Care Form. |

How to Use this Implementation Guide

This section is used to show the X12 Rules for this segment.  You must look further into the grayboxes below for Texas Rules.

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

This section is used to show the Texas Rules for implementation of this segment.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

One or more examples.

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

These are X12 code descriptions, which often do not relate to the Texas descriptions.

X12 cannot keep up with Texas needs, thus, Texas often changes the meaning of existing codes.  See the corresponding graybox for the Texas definitions.

This column shows the X12 attributes for each data element.

M = Mandatory

O= Optional

X = Conditional

AN = Alphanumeric

N# = Implied Decimal at position #

ID = Identification

R = Real

1/30 = Minimum 1, Maximum 30

This column shows the Texas use of each data element.

814 General Request, Response or Confirmation

ANSI ASC X12 Structure

Functional Group ID=GE

Introduction:

This Draft Standard for Trial Use contains the format and establishes the data contents of the General Request, Response or Confirmation Transaction Set (814) for use within the context of an Electronic Data Interchange (EDI) environment. This standard can be used to request actions to be performed, to respond to a request for actions to be performed or to confirm information related to actions performed.

Heading:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

| M | 010 | ST | Transaction Set Header | M | 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | 020 | BGN | Beginning Segment | M | 1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - N1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 040 | N1 | Name | O | 1 |  | n1 |  |  |  |  |  |  |

Detail:

Pos.	Seg.		Req.		Loop	Notes and

No.	ID	Name	Des.	Max.Use	Repeat	Comments

|  |  |  | LOOP ID - LIN |  |  | >1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 010 | LIN | Item Identification | O | 1 |  |  |  |  |  |  |  |  |
|  | 020 | ASI | Action or Status Indicator | O | 1 |  |  |  |  |  |  |  |  |
|  | 030 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 040 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
|  |  |  | LOOP ID - NM1 |  |  | >1 |  |  |  |  |  |  |  |
|  | 080 | NM1 | Individual or Organizational Name | O | 1 |  | n2 |  |  |  |  |  |  |
|  | 130 | REF | Reference Identification | O | >1 |  |  |  |  |  |  |  |  |
|  | 140 | DTM | Date/Time Reference | O | >1 |  |  |  |  |  |  |  |  |
| M | 150 | SE | Transaction Set Trailer | M | 1 |  |  |  |  |  |  |  |  |

Transaction Set Notes

1.	The N1 loop is used to identify the transaction sender and receiver.

2.	The NM1 loop is used to identify the parties associated with the individual line item (LIN), such as an individual consumer in a consolidated third party Consumer Service Provider transaction.

For use on an alphanumeric field, TX SET recognizes all characters within the Basic Character Set.  Within the Extended Character Set, TX SET recognizes all character sets except all Select Language Characters found in Section (4) of X-12 Standards Application.’  Segment/Data Element gray box guidelines for alphanumeric fields will continue to take priority over any ANSI Standards.

Segment:	ST Transaction Set Header

Position:	010

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the start of a transaction set and to assign a control number

Syntax Notes:

Semantic Notes:	1	The transaction set identifier (ST01) is used by the translation routines of the interchange partners to select the appropriate transaction set definition (e.g., 810 selects the Invoice Transaction Set).

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | ST~814~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ST01 | 143 | Transaction Set Identifier Code |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code uniquely identifying a Transaction Set |  |  |  |  |  |  |  |
|  |  |  |  | 814 |  | General Request, Response or Confirmation |  |  |  |  |
| Must Use | ST02 | 329 | Transaction Set Control Number |  |  |  | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |  |  |  |

Segment:	BGN Beginning Segment

Position:	020

Loop:

Level:	Heading

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the beginning of a transaction set

Syntax Notes:	1	If BGN05 is present, then BGN04 is required.

Semantic Notes:	1	BGN02 is the transaction set reference number.

2	BGN03 is the transaction set date.

3	BGN04 is the transaction set time.

4	BGN05 is the transaction set time qualifier.

5	BGN06 is the transaction set reference number of a previously sent transaction affected by the current transaction.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | BGN~13~200104021200719~20010402~~~200104011956531~~22 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | BGN01 | 353 | Transaction Set Purpose Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying purpose of transaction set |  |  |  |  |  |  |  |  |
|  |  |  |  | 13 |  | Request |  |  |  |  |  |
| Must Use | BGN02 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | A unique transaction identification number assigned by the originator of this transaction. This number must be unique over time. Transaction Reference numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |  |
| Must Use | BGN03 | 373 | Date |  |  |  | M |  | DT 8/8 |  |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |  |
|  |  |  | The transaction creation date - the date that the data was processed by the sender's application system. |  |  |  |  |  |  |  |  |
| Must Use | BGN06 | 127 | Reference Identification |  |  |  | O |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Refers to the BGN02 of the Move Out Request (814_24). This number will be tracked in the BGN06 through the lifecycle of the Move Out Process. |  |  |  |  |  |  |  |  |
| Must Use | BGN08 | 306 | Action Code |  |  |  | O |  | ID 1/2 |  |  |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |  |
|  |  |  | This segment is used to initially identify the type of 814 that is being sent or received. Ignore the ANSI X12 definition of the code. |  |  |  |  |  |  |  |  |
|  |  |  |  | 22 |  | Dissolve |  |  |  |  |  |
|  |  |  |  |  |  | Indicates Texas SET Transaction 814_22 |  |  |  |  |  |

Segment:	N1 Name (Transmission Distribution Service Provider)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~8S~TDSP COMPANY~1~007909411 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 8S |  | Consumer Service Provider (CSP) |  |  |  |  |  |
|  |  |  |  |  |  | Transmission Distribution Service Provider (TDSP) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | TDSP D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |

Segment:	N1 Name (ERCOT)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~AY~ERCOT~1~183529049~~41 (ERCOT to CSA CR) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | AY |  | Clearinghouse |  |  |  |  |  |
|  |  |  |  |  |  | ERCOT |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | ERCOT D-U-N-S Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Submitter |  |  |  |  |  |

Segment:	N1 Name (Competitive Retailer)

Position:	040

Loop:	N1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To identify a party by type of organization, name, and code

Syntax Notes:	1	At least one of N102 or N103 is required.

2	If either N103 or N104 is present, then the other is required.

Semantic Notes:

Comments:	1	This segment, used alone, provides the most efficient method of providing organizational identification. To obtain this efficiency the "ID Code" (N104) must provide a key to the table maintained by the transaction processing party.

2	N105 and N106 further define the type of entity in N101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | N1~SJ~CSA CR NAME~1~007909422~~40 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | SJ |  | Service Provider |  |  |  |  |  |
|  |  |  |  |  |  | Continuous Service Agreement (CSA) Competitive Retailer (CR) |  |  |  |  |  |
| Must Use | N102 | 93 | Name |  |  |  | X |  | AN 1/60 |  |  |
|  |  |  | Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | CSA CR Name |  |  |  |  |  |  |  |  |
| Must Use | N103 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 1 |  | D-U-N-S Number, Dun & Bradstreet |  |  |  |  |  |
|  |  |  |  | 9 |  | D-U-N-S+4, D-U-N-S Number with Four Character Suffix |  |  |  |  |  |
| Must Use | N104 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | CSA CR D-U-N-S Number or D-U-N-S + 4 Number |  |  |  |  |  |  |  |  |
| Must Use | N106 | 98 | Entity Identifier Code |  |  |  | O |  | ID 2/3 |  |  |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | 40 |  | Receiver |  |  |  |  |  |

Segment:	LIN Item Identification

Position:	010

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To specify basic item identification data

Syntax Notes:	1	If either LIN04 or LIN05 is present, then the other is required.

2	If either LIN06 or LIN07 is present, then the other is required.

3	If either LIN08 or LIN09 is present, then the other is required.

4	If either LIN10 or LIN11 is present, then the other is required.

5	If either LIN12 or LIN13 is present, then the other is required.

6	If either LIN14 or LIN15 is present, then the other is required.

7	If either LIN16 or LIN17 is present, then the other is required.

8	If either LIN18 or LIN19 is present, then the other is required.

9	If either LIN20 or LIN21 is present, then the other is required.

10	If either LIN22 or LIN23 is present, then the other is required.

11	If either LIN24 or LIN25 is present, then the other is required.

12	If either LIN26 or LIN27 is present, then the other is required.

13	If either LIN28 or LIN29 is present, then the other is required.

14	If either LIN30 or LIN31 is present, then the other is required.

Semantic Notes:	1	LIN01 is the line item identification

Comments:	1	See the Data Dictionary for a complete list of IDs.

2	LIN02 through LIN31 provide for fifteen different product/service IDs for each item. For example: Case, Color, Drawing No., U.P.C. No., ISBN No., Model No., or SKU.

| Notes: |  | Required Only 1 LIN Loop per EDI transaction is accepted in the Texas Market |
| --- | --- | --- |
|  |  | LIN~1~SH~EL~SH~CE~SH~MVI (Move In for CSA CR) |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | LIN01 | 350 | Assigned Identification |  |  |  | O |  | AN 1/20 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Alphanumeric characters assigned for differentiation within a transaction set |  |  |  |  |  |  |  |  |
| Must Use | LIN02 | 235 | Product/Service ID Qualifier |  |  |  | M |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN03 | 234 | Product/Service ID |  |  |  | M |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  | Commodity |  |  |  |  |  |  |  |  |
|  |  |  |  | EL |  | Electric Service |  |  |  |  |  |
| Must Use | LIN04 | 235 | Product/Service ID Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN05 | 234 | Product/Service ID |  |  |  | X |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  |  | CE |  | Energy Services |  |  |  |  |  |
| Must Use | LIN06 | 235 | Product/Service ID Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type/source of the descriptive number used in Product/Service ID (234) |  |  |  |  |  |  |  |  |
|  |  |  |  | SH |  | Service Requested |  |  |  |  |  |
| Must Use | LIN07 | 234 | Product/Service ID |  |  |  | X |  | AN 1/48 |  |  |
|  |  |  | Identifying number for a product or service |  |  |  |  |  |  |  |  |
|  |  |  |  | MVI |  | Move In |  |  |  |  |  |
|  |  |  |  |  |  | Request for a customer enrollment when a new customer moves into a premise. In this case the CSA is "moving in" to the premise. |  |  |  |  |  |

Segment:	ASI Action or Status Indicator

Position:	020

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To indicate the action to be taken with the information provided or the status of the entity described

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Identifies the action to be taken or the status of a requested action for the service identified in the LIN segment. |
| --- | --- | --- |
|  |  | Required |
|  |  | ASI~7~021 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | ASI01 | 306 | Action Code |  |  |  | M |  | ID 1/2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code indicating type of action |  |  |  |  |  |  |  |
|  |  |  |  | 7 |  | Request |  |  |  |  |
| Must Use | ASI02 | 875 | Maintenance Type Code |  |  |  | M |  | ID 3/3 |  |
|  |  |  | Code identifying the specific type of item maintenance |  |  |  |  |  |  |  |
|  |  |  |  | 021 |  | Addition |  |  |  |  |

Segment:	REF Reference Identification (AMS Indicator)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required when meter is a provisioned AMS meter at ERCOT. The AMS Indicator is used to identify a meter that has a Load Profile of AMS and is provisioned at ERCOT. |
| --- | --- | --- |
|  |  | REF~MR~AMSM |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | MR |  | Merchandise Type Code |  |  |  |  |  |
|  |  |  |  |  |  | Indication that an AMS meter has been installed and provisioned at the premise for this ESI ID |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | AMSM |  | AMS Manual |  |  |  |  |  |
|  |  |  |  |  |  | ESI ID has provisioned AMS meter without remote connect and disconnect capability |  |  |  |  |  |
|  |  |  |  | AMSR |  | AMS Remote |  |  |  |  |  |
|  |  |  |  |  |  | ESI ID has provisioned AMS meter with remote connect and disconnect capability |  |  |  |  |  |

Segment:	REF Reference Identification (Distribution Loss Factor Code)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The Distribution Loss Factor Code is used to identify the related loss factor value that is used in settlement. The code must be the contained in a loss factor table that the TDSP has provided to ERCOT. It is usually related to the delivery voltage or rate class. It is not the same as the transformer loss factor used when the TDSP meters service at a different point than it delivers service. |
| --- | --- | --- |
|  |  | Required |
|  |  | REF~AQ~T |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | AQ |  | Access Code |  |  |  |  |  |
|  |  |  |  |  |  | Distribution Loss Factor Code |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | With the exception of code "T", each Distribution Company will post what each of the next 5 codes represent and how they are applied. |  |  |  |  |  |  |  |  |
|  |  |  |  | A |  |  |  |  |  |  |  |
|  |  |  |  | B |  |  |  |  |  |  |  |
|  |  |  |  | C |  |  |  |  |  |  |  |
|  |  |  |  | D |  |  |  |  |  |  |  |
|  |  |  |  | E |  |  |  |  |  |  |  |
|  |  |  |  | T |  | Transmission Only |  |  |  |  |  |

Segment:	REF Reference Identification (Premise Type)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Accept Response: Required Reject Response: Not Used |
| --- | --- | --- |
|  |  | REF~PTC~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PTC |  | Patent Type |  |  |  |  |  |
|  |  |  |  |  |  | Premise Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 01 |  | Residential |  |  |  |  |  |
|  |  |  |  |  |  | Designates a premise where a customer will reside or an unmetered service located at such a premise (e.g. Outdoor Lights) provided it is not a master metered, multi-family facility, or a facility metered as a commercial facility. |  |  |  |  |  |
|  |  |  |  | 02 |  | Small Non-residential |  |  |  |  |  |
|  |  |  |  |  |  | Designates a nonresidential premise with peak demand of 1 Megawatt or below (i.e. Where the demand for the previous 12 months is less than or equal to 1 megawatt) or an unmetered service located at such a premise. |  |  |  |  |  |
|  |  |  |  | 03 |  | Large Non-residential |  |  |  |  |  |
|  |  |  |  |  |  | Designates a nonresidential premise with peak demand greater than 1 Megawatt (i.e. Where demand for at least 1 month of the previous 12 months is greater than 1 Megawatt) or an unmetered service located at such a premise. |  |  |  |  |  |

Segment:	REF Reference Identification (ESI ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | REF~Q5~~10111111234567890ABCDEFGHIJKLMNOPQRS |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | Q5 |  | Property Control Number |  |  |  |  |  |
|  |  |  |  |  |  | Electric Service Identifier (ESI ID) |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | ESI ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Station ID)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The value shall correspond to an ERCOT-defined Station ID that is published on the ERCOT website. |
| --- | --- | --- |
|  |  | Required |
|  |  | REF~SPL~~WEST |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | SPL |  | Standard Point Location Code (SPLC) |  |  |  |  |  |
|  |  |  |  |  |  | Substation at which the TDSP's distribution system is connected to the transmission grid for the Service Delivery Point (SDP). The value shall correspond to an ERCOT-defined Station ID that is published on the ERCOT website. |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Station ID |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Special Needs Indicator)

Position:	030

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required A "Y" in this field means that the customer is either: A residential customer who qualifies through the Residential Critical Care Eligibility Determination Form, as issued by the PUCT. Or A customer for which electric service is considered crucial for the protection and maintenance of public safety pursuant to Subst. Rules 25.52 and 25.53. Or An industrial customer for whom an interruption or suspension of service will create a dangerous or life threatening condition at the customers premise. REF~SU~N |
| --- | --- | --- |
|  |  | REF~SU~Y~CLI |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

|  | Must Use | REF01 | 128 |  | Reference Identification Qualifier |  |  |  |  | M |  | ID 2/3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | SU |  |  | Special Processing Code |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Special Needs Indicator |  |  |  |  |  |  |  |
|  | Must Use | REF02 | 127 |  | Reference Identification |  |  |  |  | X |  | AN 1/30 |  |  |  |  |
|  |  |  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | N |  |  | No |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Special Needs are not required |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Y |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Special Needs are required |  |  |  |  |  |  |  |
|  | Must Use | REF03 | 352 |  | Description |  |  |  |  | X |  | AN 1/80 |  |  |  |  |
|  |  |  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Required when REF02 = Y indicating Special Needs are required Used to communicate the Customer’s Special Needs Status as defined by PUCT Subst. Rule 25.497 “Customer Protection Rules Relating to Designation of Critical Care Customers”, otherwise not used. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CCC |  |  | Critical Care Residential Customer |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Secondary Contact Information Required if Special Needs Status is “CCC” |  |  |  |  |  |  |  |
|  |  |  |  | CCCT |  |  |  |  | Critical Care Residential Customer - Temporary |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Temporary status is required when the TDSP deems the registration form incomplete Secondary Contact Information Required if Special Needs Status is “CCCT” |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CLI |  |  | Critical Load Industrial Customer |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CLP |  |  | Critical Load Public Safety Customer |  |  |  |  |  |  |  |
|  |  |  |  |  |  | CRC |  |  | Chronic Condition Residential Customer |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Secondary Contact Information Required if Special Needs Status is “CRC” |  |  |  |  |  |  |  |
|  |  |  |  | CRCT |  |  |  |  | Chronic Condition Residential Customer - Temporary |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Temporary status is required when the TDSP deems the registration form incomplete Secondary Contact Information Required if Special Needs Status is “CRCT” |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Expiration Date)

Position:	040

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required when available in the TDSPs system and is a Residential Premise Type and Special Needs = Y |
| --- | --- | --- |
|  |  | DTM~036~20010415 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |
|  |  |  |  | 036 |  | Expiration |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Service Period Start)

Position:	040

Loop:	LIN        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required |
| --- | --- | --- |
|  |  | DTM~150~20010415 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |
|  |  |  |  | 150 |  | Service Period Start |  |  |  |  |
| Must Use | DTM02 | 373 | Date |  |  |  | X |  | DT 8/8 |  |
|  |  |  | Date expressed as CCYYMMDD |  |  |  |  |  |  |  |

Segment:	NM1 Individual or Organizational Name (Meter Level Information)

Position:	080

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	1

Purpose:	To supply the full name of an individual or organizational entity

Syntax Notes:	1	If either NM108 or NM109 is present, then the other is required.

2	If NM111 is present, then NM110 is required.

Semantic Notes:	1	NM102 qualifies NM103.

Comments:	1	NM110 and NM111 further define the type of entity in NM101.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | NM1~MQ~3~~~~~~32~GE10349811 NM1~MQ~3~~~~~~93~UNMETERED |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | NM101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | MQ |  | Metering Location |  |  |  |  |  |
| Must Use | NM102 | 1065 | Entity Type Qualifier |  |  |  | M |  | ID 1/1 |  |  |
|  |  |  | Code qualifying the type of entity |  |  |  |  |  |  |  |  |
|  |  |  |  | 3 |  | Unknown |  |  |  |  |  |
| Must Use | NM108 | 66 | Identification Code Qualifier |  |  |  | X |  | ID 1/2 |  |  |
|  |  |  | Code designating the system/method of code structure used for Identification Code (67) |  |  |  |  |  |  |  |  |
|  |  |  |  | 32 |  | Assigned by Property Operator |  |  |  |  |  |
|  |  |  |  |  |  | Meter Number |  |  |  |  |  |
|  |  |  |  | 93 |  | Code assigned by the organization originating the transaction set |  |  |  |  |  |
|  |  |  |  |  |  | This code used when a Meter Number is NOT provided in NM109 (i.e., when NM109 contains "UNMETERED"). |  |  |  |  |  |
| Must Use | NM109 | 67 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code |  |  |  |  |  |  |  |  |
|  |  |  | Meter Number or, UNMETERED - for Unmetered Services Meter numbers will only contain uppercase letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded, and leading and trailing zeros that are part of the meter number must be present. |  |  |  |  |  |  |  |  |

Segment:	NM1 Individual or Organizational Name (Special Needs Secondary Contact Name)

Position:	080

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To supply the full name of an individual or organizational entity

Syntax Notes:	1	If either NM108 or NM109 is present, then the other is required.

2	If NM111 is present, then NM110 is required.

Semantic Notes:	NM102 qualifies NM103

Comments:	1	NM110 and NM111 further define the type of entity in NM101.

| Notes: |  | Required when REF~SU=Y and Critical Care Status in the REF03of the Special Needs (REF~SU) segment = “CCC”, “CCCT”, “CRC” or “CRCT”, otherwise this segment will not be provided by the TDSP. |
| --- | --- | --- |
|  |  | NM1~SC~~~~~~~~SNOW, JOE RAY JR NM1~SC~~~~~~~~XYZ COMPANY |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | NM101 | 98 | Entity Identifier Code |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying an organizational entity, a physical location, property or an individual |  |  |  |  |  |  |  |  |
|  |  |  |  | SC |  | Store Class |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs Secondary Contact Name |  |  |  |  |  |
| Must Use | NM102 | 93 | Identification Code |  |  |  | X |  | AN 2/80 |  |  |
|  |  |  | Code identifying a party or other code Free-form name |  |  |  |  |  |  |  |  |
|  |  |  | The Customer Contact Name should be formatted as follows: Last Name, First Name. |  |  |  |  |  |  |  |  |

Segment:	N3 Address Information (Special Needs Secondary Contact Address)

Position:	100

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	2

Purpose:	To specify the location of the named party

Syntax Notes:

Semantic Notes:

Comments:

| Notes: |  | Required when REF~SU=Y and Critical Care Status in the REF03 of the Special Needs (REF~SU) segment = “CCC”, “CCCT”, “CRC” or “CRCT”, otherwise this segment will not be provided by the TDSP |
| --- | --- | --- |
|  |  | N3~123 N MAIN ST~ANY ADDRESS OVERFLOW |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | N301 | 166 | Address Information | M |  | AN 1/55 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Service Address |  |  |  |  |
| Dep | N302 | 166 | Address Information | O |  | AN 1/55 |  |
|  |  |  | Address information |  |  |  |  |
|  |  |  | Customer Service Address Overflow Condition: If there is an overflow, it must be sent. |  |  |  |  |

Segment:	N4 Geographic Location (Special Needs Secondary Contact Mailing Address)

Position:	110

Loop:	NM1        Optional

Level:	Heading

Usage:	Optional

Max Use:	1

Purpose:	To specify the geographic place of the named party

Syntax Notes:	1	If N406 is present, then N405 is required.

Semantic Notes:

Comments:	1	A combination of either N401 through N404, or N405 and N406 may be adequate to specify a location.

2	N402 is required only if city name (N401) is in the U.S. or Canada.

| Notes: |  | Required when REF~SU=Y and Critical Care Status in the REF03 of the Special Needs (REF~SU) segment = “CCC”, “CCCT”, “CRC” or “CRCT”, otherwise this segment will not be provided by the TDSP |
| --- | --- | --- |
|  |  | N4~ANYTOWN~TX~78111 N4~ANYTOWN~TX~781110001 N4~MISSISSAUGA~ON~L4W4E4~CA |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

|  | N401 | 19 | City Name |  |  |  | O |  | AN 2/30 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Free-form text for city name |  |  |  |  |  |  |  |
| Dep | N402 | 156 | State or Province Code |  |  |  | O |  | ID 2/2 |  |
|  |  |  | Code (Standard State/Province) as defined by appropriate government agency |  |  |  |  |  |  |  |
|  |  |  | Required when REF~SU=Y and Critical Care Status in the REF03 of the Special Needs (REF~SU) segment = “CCC”, “CCCT”, “CRC” or “CRCT” to provide Special Needs Secondary Contact Mailing Address, otherwise not used. |  |  |  |  |  |  |  |
| Must Use | N403 | 116 | Postal Code |  |  |  | O |  | ID 3/15 |  |
|  |  |  | Code defining international postal zone code excluding punctuation and blanks (zip code for United States) |  |  |  |  |  |  |  |
|  |  |  | Postal codes will only contain upper case letters (A to Z) and digits (0 to 9). Note that punctuation (spaces, dashes, etc.) must be excluded. |  |  |  |  |  |  |  |
| Dep | N404 | 26 | Country Code |  |  |  | O |  | ID 2/3 |  |
|  |  |  | Code identifying the country |  |  |  |  |  |  |  |
|  |  |  | For country codes not listed, please refer to ISO 3166 |  |  |  |  |  |  |  |
|  |  |  | Required if outside the United States. |  |  |  |  |  |  |  |
|  |  |  |  | CA |  | Canada |  |  |  |  |
|  |  |  |  | CH |  | Switzerland |  |  |  |  |
|  |  |  |  | DE |  | Germany |  |  |  |  |
|  |  |  |  | ES |  | Spain |  |  |  |  |
|  |  |  |  | FR |  | France |  |  |  |  |
|  |  |  |  | GB |  | United Kingdom |  |  |  |  |
|  |  |  |  | IT |  | Italy |  |  |  |  |
|  |  |  |  | JP |  | Japan |  |  |  |  |
|  |  |  |  | MX |  | Mexico |  |  |  |  |
|  |  |  |  | PR |  | Puerto Rico |  |  |  |  |
|  |  |  |  | SE |  | Sweden |  |  |  |  |

Segment:	PER Administrative Communications Contact (Special Needs Secondary Contact Telephone Numbers)

Position:	120

Loop:	NM1        Optional

Level:	Heading

Usage:	Optional

Max Use:	>1

Purpose:	To identify a person or office to whom administrative communications should be directed

Syntax Notes:	1	If either PER03 or PER04 is present, then the other is required.

2	If either PER05 or PER06 is present, then the other is required.

3	If either PER07 or PER08 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required when REF~SU=Y and Critical Care Status in the REF03 of the Special Needs (REF~SU) segment = “CCC”, “CCCT”, “CRC” or “CRCT”, otherwise this segment will not be provided by the TDSP |
| --- | --- | --- |
|  |  | PER~SNC~~TE~8005551212 PER~SNC~~TE~8005551212~OT~8005552121 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | PER01 | 366 | Contact Function Code |  |  |  | M |  | ID 2/2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code identifying the major duty or responsibility of the person or group named |  |  |  |  |  |  |  |  |
|  |  |  |  | SP |  | Special Program Contact |  |  |  |  |  |
|  |  |  |  |  |  | Special Needs Contact Information |  |  |  |  |  |
|  | PER03 | 365 | Communication Number Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type of communication number |  |  |  |  |  |  |  |  |
|  |  |  |  | TE |  | Telephone |  |  |  |  |  |
|  |  |  |  |  |  | Primary Telephone Number |  |  |  |  |  |
| Dep | PER04 | 364 | Communication Number |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | Complete communications number including country or area code when applicable |  |  |  |  |  |  |  |  |
|  |  |  | Punctuation (dashes, symbols etc.) must be excluded. |  |  |  |  |  |  |  |  |
|  | PER05 | 365 | Communication Number Qualifier |  |  |  | X |  | ID 2/2 |  |  |
|  |  |  | Code identifying the type of communication number |  |  |  |  |  |  |  |  |
|  |  |  |  | OT |  | Other Residential Telephone Number |  |  |  |  |  |
|  |  |  |  |  |  | Other Telephone Number if available |  |  |  |  |  |
| Dep | PER06 | 364 | Communication Number |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | Complete communications number including country or area code when applicable |  |  |  |  |  |  |  |  |
|  |  |  | Punctuation (dashes, symbols etc.) must be excluded. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Multiplier)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~4P segment for each Meter Type, creating the potential for more than one REF~4P segment per NM1 Loop. For instance, if the meter has multiple meter types, and therefore the REF~MT segment is REF~MT~COMBO, there will be multiple REF~4P segments, one to indicate the meter multiplier for each type. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~4P~1200~K1015~TU^41 REF~4P~1~KH015~TU^41 Note: Format of the multiplier could be in the form of a fraction (e.g. 48.78432) Also note that since the Meter Multiplier is required for each Time of Use, this segment is also used to determine which Time of Use will be sent on the 867 transaction. |
| --- | --- | --- |
|  |  | Required for each meter type for each meter that is used for billing purposes. This segment will not be provided in the NM1 loop for UNMETERED services. |
|  |  | REF~4P~10~KHMON~TU^41 REF~4P~1~K1MON~TU^41 REF~4P~1.5~K1MON~TU^41 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | 4P |  | Affiliation Number |  |  |  |  |  |
|  |  |  |  |  |  | Meter Multiplier (Ending Reading - Beginning Reading) * Meter Multiplier = Billed Usage |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Meter Multiplier |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note this is a composite data element. Populate C04001 & C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (Number of Dials)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~IX segment for each applicable Meter Type, creating the potential for more than one REF~IX segment per NM1 Loop. For instance, if the meter has multiple types, and therefore the REF~MT segment is REF~MT~COMBO, there may be multiple REF~IX segments, one to indicate the number of dials for each type that has dials. For example, NM1~MQ~3~~~~~~32~1234568MG REF~MT~COMBO REF~IX~6.0~KHMON~TU^51 REF~IX~5.1~KHMON~TU^51 |
| --- | --- | --- |
|  |  | Required for each meter type for each meter that is used for billing purposes that has dials. This segment will not be provided in the NM1 loop for UNMETERED services or for demand. |
|  |  | REF~IX~6.0~KHMON~TU^51 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | IX |  | Item Number |  |  |  |  |  |
|  |  |  |  |  |  | Number of Dials on the Meter displayed as X.Y. The notation X.Y means that the meter has X dials to the left of the decimal point, and Y dials to the right. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Number of Dials |  |  |  |  |  |  |  |  |
| Must Use | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Meter Type (see REF~MT for valid codes). "COMBO" is not a valid code for this element. |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TU |  | Trial Location Code |  |  |  |  |  |
|  |  |  |  |  |  | Time of Use |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  |  | 41 |  | Off Peak |  |  |  |  |  |
|  |  |  |  | 42 |  | On Peak |  |  |  |  |  |
|  |  |  |  | 43 |  | Intermediate Peak |  |  |  |  |  |
|  |  |  |  | 51 |  | Totalizer |  |  |  |  |  |
|  |  |  |  |  |  | Totalizer/Total/Max (Demand) |  |  |  |  |  |
|  |  |  |  | 71 |  | Summer Super On Peak |  |  |  |  |  |

Segment:	REF Reference Identification (Load Profile)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | The load profile is made up of the following codes, with mandatory underscores between each: Profile Group and Segment Code, Weather Zone Code, Meter Data Type Code, Weather Sensitivity Code, TOU Schedule Code Required |
| --- | --- | --- |
|  |  | REF~LO~RESLOWR_WEST_NIDR_NWS_NOTOU |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | LO |  | Load Planning Number |  |  |  |  |  |
|  |  |  |  |  |  | Load profile |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Load Profile |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Type)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will only be one REF~MT segment for each NM1 loop. If there are multiple meter types on this meter, only one REF~MT will be sent. It will contain the code COMBO. The specific meter types will be identified in REF03 for the attribute segments contained within this loop. For example, REF~IX~6.0~KHMON~TU^51. |
| --- | --- | --- |
|  |  | Required for each meter that is used for billing purposes. This segment will not be provided in the NM1 loop for UNMETERED services. |
|  |  | REF~MT~KHMON REF~MT~COMBO |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | MT |  | Meter Ticket Number |  |  |  |  |  |
|  |  |  |  |  |  | Meter Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | When REF01 is MT, the meter type is expressed as a five-character field. The first two characters are the type of consumption, the last three characters are the metering interval reported for billing purposes. "COMBO" is used for a meter that records more than one measurement. Valid values can be a combination of the following values: Type of Consumption K1 Kilowatt Demand (kW) K2 Kilovolt Amperes Reactive Demand (kVAR) K3 Kilovolt Amperes Reactive Hour (kVARH) K4 Kilovolt Amperes (kVA) KH Kilowatt Hour (kWh) Metering Interval Reported for Billing Purposes nnn Number of minutes from 001 to 999 DAY Daily MON Monthly For Example: KHMON Kilowatt Hours Per Month K1015 Kilowatt Demand per 15 minute interval Other Valid Codes COMBO This code is used to indicate that the meter has multiple measurements, e.g., one meter that measures both kWh and Demand. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (TDSP Rate Class)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required for each meter that is used for billing purposes. This segment must also be sent when account has UNMETERED services available. |
| --- | --- | --- |
|  |  | REF~NH~RS1 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | NH |  | Rate Card Number |  |  |  |  |  |
|  |  |  |  |  |  | TDSP Rate Class or Tariff |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Rate Class or Tariff |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (TDSP Rate Subclass)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Required if maintained by TDSP, must be sent for each meter that is used for billing purposes. This segment must also be sent when account has UNMETERED services available. |
| --- | --- | --- |
|  |  | REF~PR~123 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PR |  | Price Quote Number |  |  |  |  |  |
|  |  |  |  |  |  | TDSP Rate Subclass - Used to provide further classification of a rate. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | TDSP Rate Subclass |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Unmetered Service Type)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | There will be one REF~PRT segment for each applicable Unmetered Service Meter Type, creating the potential for more than one REF~PRT segment per NM1 Loop. This segment is used to provide additional information to the CR for the specified Unmetered Service. It describes the type of device which this measurement loop references and can include additional text information which may be useful to the CR (i.e., a specific wattage of a light, additional text information for further clarification, etc.) |
| --- | --- | --- |
|  |  | Accept Response: Required for each unmetered service type used for billing purposes. Reject Response: Not Used |
|  |  | REF~PRT~MV~750~QQ^100 REF~PRT~SD~400 Company Owned~QQ^20 REF~PRT~MV~400 Customer Owned~QQ^15 REF~PRT~MV~Third party maintained facilities~QQ^100 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | PRT |  | Product Type |  |  |  |  |  |
|  |  |  |  |  |  | Defined Unmetered Service Type |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | If a new code is needed, it must be requested via the Change Control process. |  |  |  |  |  |  |  |  |
|  |  |  |  | AN |  | Antenna |  |  |  |  |  |
|  |  |  |  | AR |  | Argon |  |  |  |  |  |
|  |  |  |  | BB |  | Bill Boards |  |  |  |  |  |
|  |  |  |  | BS |  | Bus Shelters |  |  |  |  |  |
|  |  |  |  | CU |  | Cat Unit |  |  |  |  |  |
|  |  |  |  | ED |  | Electronic Device |  |  |  |  |  |
|  |  |  |  | FL |  | Fluorescent |  |  |  |  |  |
|  |  |  |  | HA |  | Historical/Antique |  |  |  |  |  |
|  |  |  |  | IN |  | Incandescent |  |  |  |  |  |
|  |  |  |  | LE |  | Light Emitting Diode |  |  |  |  |  |
|  |  |  |  | LV |  | Levys |  |  |  |  |  |
|  |  |  |  | MH |  | Metal Halide |  |  |  |  |  |
|  |  |  |  | MV |  | Mercury Vapor |  |  |  |  |  |
|  |  |  |  | OT |  | Other Unmetered |  |  |  |  |  |
|  |  |  |  | PA |  | Power Analog node |  |  |  |  |  |
|  |  |  |  | PB |  | Phone Booth |  |  |  |  |  |
|  |  |  |  | PO |  | Phone Outlet |  |  |  |  |  |
|  |  |  |  | PS |  | Pump Station |  |  |  |  |  |
|  |  |  |  | RR |  | Rail Road Crossings |  |  |  |  |  |
|  |  |  |  | SD |  | Sodium |  |  |  |  |  |
|  |  |  |  | TL |  | Traffic Lights |  |  |  |  |  |
|  |  |  |  | TR |  | Transceiver |  |  |  |  |  |
|  |  |  |  | WF |  | Wi-Fi (Wireless Fidelity) |  |  |  |  |  |
|  |  |  |  | WM |  | Wallpacked Mercury Vapor |  |  |  |  |  |
|  |  |  |  | WS |  | Warning Sirens |  |  |  |  |  |
| Dep | REF03 | 352 | Description |  |  |  | X |  | AN 1/80 |  |  |
|  |  |  | A free-form description to clarify the related data elements and their content |  |  |  |  |  |  |  |  |
|  |  |  | Used to provide additional clarification information to the CR for the Unmetered Service when necessary. When applicable, used to provide the specific wattage/lumens for a light. This free-form text can not contain any characters that may be used as element delimiters, sub-element delimiters, segment terminators, or field separators(This includes: asterisk *, pipes ¦, tabs, linefeeds, carets ^, angle brackets < >, and tildes ~). |  |  |  |  |  |  |  |  |
| Must Use | REF04 | C040 | Reference Identifier |  |  |  | O |  |  |  |  |
|  |  |  | To identify one or more reference numbers or identification numbers as specified by the Reference Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Note that this is a composite data element. Populate C04001 and C04002. |  |  |  |  |  |  |  |  |
| Must Use | C04001 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | QQ |  | Unit Number |  |  |  |  |  |
|  |  |  |  |  |  | Number of devices |  |  |  |  |  |
| Must Use | C04002 | 127 | Reference Identification |  |  |  | M |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Number of unmetered devices of this type. |  |  |  |  |  |  |  |  |

Segment:	REF Reference Identification (Meter Reading Cycle)

Position:	130

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify identifying information

Syntax Notes:	1	At least one of REF02 or REF03 is required.

2	If either C04003 or C04004 is present, then the other is required.

3	If either C04005 or C04006 is present, then the other is required.

Semantic Notes:	1	REF04 contains data relating to the value cited in REF02.

Comments:

| Notes: |  | Meter cycles will be provided on the TDSP's website. |
| --- | --- | --- |
|  |  | Required when the Meter Read is done by cycle. Required for un-metered services. One of either the REF~TZ or DTM~313 is required for metered services but not both. |
|  |  | REF~TZ~15 REF~TZ~04 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | REF01 | 128 | Reference Identification Qualifier |  |  |  | M |  | ID 2/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code qualifying the Reference Identification |  |  |  |  |  |  |  |  |
|  |  |  |  | TZ |  | Total Cycle Number |  |  |  |  |  |
|  |  |  |  |  |  | Cycle number when the meter will be read. |  |  |  |  |  |
| Must Use | REF02 | 127 | Reference Identification |  |  |  | X |  | AN 1/30 |  |  |
|  |  |  | Reference information as defined for a particular Transaction Set or as specified by the Reference Identification Qualifier |  |  |  |  |  |  |  |  |
|  |  |  | Metered: Meter Reading Cycle Unmetered: Billing Cycle Must always be numeric and only two digits. |  |  |  |  |  |  |  |  |

Segment:	DTM Date/Time Reference (Meter Cycle by Day of the Month)

Position:	140

Loop:	NM1        Optional

Level:	Detail

Usage:	Optional

Max Use:	>1

Purpose:	To specify pertinent dates and times

Syntax Notes:	1	At least one of DTM02 DTM03 or DTM05 is required.

2	If DTM04 is present, then DTM03 is required.

3	If either DTM05 or DTM06 is present, then the other is required.

Semantic Notes:

Comments:

| Notes: |  | Required on metered services if meter read based on a day of the month. Not used for un-metered services. One of the either the REF~TZ or the DTM~313 is required for metered services but not both. |
| --- | --- | --- |
|  |  | DTM~313~~~~DD~01 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | DTM01 | 374 | Date/Time Qualifier |  |  |  | M |  | ID 3/3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code specifying type of date or time, or both date and time |  |  |  |  |  |  |  |  |
|  |  |  |  | 313 |  | Cycle |  |  |  |  |  |
|  |  |  |  |  |  | Meter Cycle by Day of the Month |  |  |  |  |  |
| Must Use | DTM05 | 1250 | Date Time Period Format Qualifier |  |  |  | X |  | ID 2/3 |  |  |
|  |  |  | Code indicating the date format, time format, or date and time format |  |  |  |  |  |  |  |  |
|  |  |  |  | DD |  | Day of Month in Numeric Format |  |  |  |  |  |
| Must Use | DTM06 | 1251 | Date Time Period |  |  |  | X |  | AN 1/35 |  |  |
|  |  |  | Expression of a date, a time, or range of dates, times or dates and times |  |  |  |  |  |  |  |  |
|  |  |  | Day of month that the meter will be read, in DD format (e.g., 01 - 28). |  |  |  |  |  |  |  |  |

Segment:	SE Transaction Set Trailer

Position:	150

Loop:

Level:	Detail

Usage:	Mandatory

Max Use:	1

Purpose:	To indicate the end of the transaction set and provide the count of the transmitted segments (including the beginning (ST) and ending (SE) segments)

Syntax Notes:

Semantic Notes:

Comments:	1	SE is the last segment of each transaction set.

| Notes: |  | Required |
| --- | --- | --- |
|  |  | SE~10~000000001 |

Data Element Summary

Ref.	Data

Des.	Element	Name	Attributes

| Must Use | SE01 | 96 | Number of Included Segments | M |  | N0 1/10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total number of segments included in a transaction set including ST and SE segments |  |  |  |  |
| Must Use | SE02 | 329 | Transaction Set Control Number | M |  | AN 4/9 |  |
|  |  |  | Identifying control number that must be unique within the transaction set functional group assigned by the originator for a transaction set |  |  |  |  |