| NPRR Number |  | NPRR Title | Multi-Month CRR Auction and Revisions to Collateral Requirements |
| --- | --- | --- | --- |
| Date Posted |  |  |  |
| Nodal Protocol Section(s) Requiring Revision (Include Section No. and Title) |  | 7.5.CRR Auctions; 7.5.5.3 Auction Process; 7.5.6 CRR Auction Settlements; 7.5.6.2 Charge of an Awarded CRR Auction Bid; 7.5.6.3 Charge of PCRRs Pertaining to a CRR Auction; 16.11.4.5 Determination of the Counter-Party Future Credit Exposure |  |
| Requested Resolution (Normal or Urgent, and justification for Urgent status) |  | TBD |  |
| Revision Description |  | The NPRR replaces the annual auction with a multi-month rolling auction for twelve months to provide forward price discovery and hedging opportunities. It delays the second year annual strip until ERCOT has more operating and credit experience. The NPRR will collateralize the prompt month only, create a forward month credit risk based on updated monthly clearing prices, and use initial margin concept to mitigate credit risk for thirty days between each auction. |  |
| Reason for Revision |  | Due to operational, risk-management and collateral concerns around the annual CRR auction, this revision creates a multi-month rolling auction for twelve months. The NPRR is designed for organized market liquidity, which creates basis visibility and hedging opportunities. At the same time, the revision seeks to right-sized credit risk in the pre-auction engine and future credit exposure calculations. By allowing for more accurate cash management and more liquidity of CRRs, market participants can better manage their CRR activities. |  |
| Overall Market Benefit |  | Positive- increases market liquidity and right-sizes credit risk |  |
| Overall Market Impact |  | Positive- increases market liquidity and right-sizes credit risk |  |
| Consumer Impact |  | Positive- increases market liquidity and right-sizes credit risk |  |
| Credit Implications (Yes or No, and summary of impact) |  | Yes- TBD |  |

Quantitative Impacts and Benefits

| Assumptions | 1 | e.g.: Key assumptions used in estimating market cost and/or benefit |  |
| --- | --- | --- | --- |
|  | 2 | Dependencies on other projects or other timing requirements |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | e.g.: Cost per MP to implement | e.g.: $10,000 each for 50 QSEs |
|  | 2 | Add’l staff required per MP | 1.5 FTE each for 6 TDSPs @ $65/hour |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | e.g.: Reduced MP costs | e.g.: 2 FTE reduction for 25 CRs @ $65/hour |
|  | 2 | Enhanced MP efficiency | 2 hour savings per day for 50 generators @$65 |
|  | 3 | Reduced congestion cost | 0.5% reduction in total congestion cost |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | e.g.: Benefits that are difficult to quantify |  |
|  | 2 | Benefits that are not certain but relatively likely |  |
|  | 3 | Customer service impacts, cash flow impacts, transaction speed, etc. |  |
|  | 4 |  |  |
| Other Comments | 1 | e.g.: Thoughts on ERCOT systems impacts |  |
|  | 2 | Potential manual workarounds or delivery options |  |
|  | 3 | Other comments of value to PRS, TAC and the Board of Directors |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Randa Stephenson |
| E-mail Address | Randa.stephenson@luminant.com |
| Company | Luminant |
| Phone Number | 512-349-6491 |
| Cell Number | 214-498-6661 |
| Market Segment | IOU |

| Market Rules Staff Contact |  |
| --- | --- |
| Name |  |
| E-Mail Address |  |
| Phone Number |  |

Proposed Protocol Language Revision

7.5	CRR Auctions

7.5.1	Nature and Timing

(1)	The Congestion Revenue Right (CRR) Auction auctions the available network capacity of the ERCOT transmission system not allocated as described in Section 7.4, Allocation of Preassigned Congestion Revenue Rights, or sold in a previous auction.  The CRR Auction also allows CRR Owners an opportunity to offer for sale CRRs that they hold.  Each annual and monthly CRR Auction allows for the purchase of CRR products as described in paragraph (6) of Section 7.3, Types of Congestion Revenue Rights to Be Auctioned, in one-month strips and allows for the reconfiguration of all CRR blocks that were previously awarded.

(2)	The CRR Network Model must be based on, but is not the same as, the Network Operations Model.  The CRR Network Model must, to the extent practicable, include the same topology, contingencies, and operating procedures as used in the Network Operations Model as reasonably expected to be in place for each month.  The expected network topology used in the CRR Network Model for any month must include all Outages from the Outage Scheduler and identified by ERCOT as expected to have a significant impact upon transfer capability during the month.  These Outages included in the CRR Network Model shall be posted on the Market Information System (MIS) Secure Area consistent with model posting requirements by ERCOT with accompanying cause and duration information, as indicated in the Outage Scheduler.  Transmission system upgrades and changes must be accounted for in the CRR Network Model for CRR Auctions held after the month in which the element is placed into service.    As referenced in the ERCOT Protocols, Operating Guides and Planning Guides, CRRs offers provide visible market indicators of necessary transmission system improvements that may provide consumer benefits.  Therefore, CRR Account Holders understand CRRs  are sold under a “caveat emptor” warning and the transmission modeled for the CRR Auctions may not adequately represent transmission improvements, upgrades, remedial action plans, or other similar activities by ERCOT and/or the Transmission Service Providers.

(a)	ERCOT shall use Dynamic Ratings in the CRR Network Model as required under Section 3.10.8, Dynamic Ratings.

(b)	The CRR Network Model must use the peak Load conditions of the month being modeled.

(c)	ERCOT’s criteria for determining if an Outage should be in the CRR Network Model shall be in accordance with these Protocols and described in the ERCOT Operating Guides.

(3)	ERCOT shall model bids and offers into the CRR Auction as flows based on the MW offer and defined source and sink.  When the Simultaneous Feasibility Test (SFT) is run, the model must weight the Electrical Buses and Hub Buses included in a Hub or Load Zone appropriately to determine the system impacts of the CRRs.

(a)	To distribute injections and withdrawals to buses within a Hub, ERCOT shall use distribution factors specified in Section 3.5.2, Hub Definitions.

(b)	To distribute injections and withdrawals to Electrical Buses in Load Zones, ERCOT shall use the Load-weighted distribution factors for On-Peak Hours in each Load Zone.  For monthly CRR Auctions, ERCOT shall derive CRR Auction Load distribution factors with the set of Load distribution factors constructed in accordance with the ERCOT Load distribution factor methodology specified in paragraph (5) of Section 4.5.1, DAM Clearing Process, for use in the Day-Ahead Market (DAM).  For Annual CRR Auctions, ERCOT shall derive CRR Auction Load distribution factors from the corresponding planning model or with the set of Load distribution factors constructed in accordance with the ERCOT Load distribution factor methodology specified in paragraph (5) of Section 4.5.1, for use in the DAM.  ERCOT shall notify the market as to which method was used for the annual CRR Auction model in the corresponding auction notice.  ERCOT shall post the CRR Auction Load distribution factors as part of the CRR Network Model pre-auction posting.

(4)	ERCOT shall conduct CRR Auctions with the frequency, on the dates, and for the terms specified as follows:

(a)	Point-to-Point (PTP) Options, PTP Obligations, and Flowgate Rights (FGRs) in monthly auctions for one-month terms beginning with the month prior to the Texas Nodal Market Implementation Date.

(b)	ERCOT shall conduct a monthly CRR Auction during the month preceding the month during which the CRRs being auctioned are effective.  ERCOT shall publish a calendar of relevant auction dates each year for the following year’s activities.

(c)	No later than December 2011, ERCOT shall conduct each month a multi-month auction for Point-to-Point (PTP) Options, PTP Obligations, and Flowgate Rights (FGRs)  for one-month strip for each of the succeeding twelve forward months.

(d)	Upon approval by the ERCOT Board, ERCOT may conduct once a year, beginning after October 1, 2012, a second-year CRR Auction offering one-month strips for each of the calendar months of the second year and an annual strip for that second year. After the completion of at least six monthly CRR Auctions ERCOT shall conduct an annual CRR Auction for CRR Options and ERCOT Board-approved PTP Obligations commencing during October for the two-year period that starts on the immediately following January 1.

(5)	ERCOT shall auction the following products:

(a)	In each monthly 12 month CRR Auction:

(i)	PTP Options in one-month strips for one-month terms;

(ii)	PTP Obligations in one-month strips for one-month terms; and

(iii)	FGRs (on any defined flowgates) in one-month strips for one-month terms.

(6)	ERCOT shall offer network capacity for the second year in each annual CRR Auction equal to the difference between:

(a)	For each month, the expected network topology for that month of the first year in the CRR Network Model scaled down to no more than 15% for the second year; and

(b)	All outstanding CRRs that were previously awarded or allocated for the corresponding months in each year scaled to no more than 15% for the second year.

(7)	ERCOT shall offer network capacity for each monthly 12 month CRR Auction equal to the difference between:

(a)	The expected transmission network topology in the CRR Network Model of the month for which the CRRs are effective scaled down to the following percentages for each of the succeeding twelve months; and

| Month | Scaled Capacity % | Month | Scaled Capacity % |
| --- | --- | --- | --- |

| Prompt | 90.00% | 7 | 52.50% |
| --- | --- | --- | --- |
| 2 | 83.75% | 8 | 46.25% |
| 3 | 77.50% | 9 | 40.00% |
| 4 | 71.25% | 10 | 33.75% |
| 5 | 65.00% | 11 | 27.50% |
| 6 | 58.75% | 12 | 21.25% |

(b)	All outstanding CRRs that were previously awarded or allocated for the month.

7.5.5.3	Auction Process

(1)	The auction must be a single-round, simultaneous auction for selling the CRRs available for all auction products, with the following steps:

(a)	ERCOT shall enter into the CRR Auction engine model a credit constraint for each Counter-Party.  A Counter-Party’s CRR Auction credit limit is equal to the lesser of the credit limit as determined in Section 16.11.4.6.1, Credit Requirements for CRR Auction Participation, or, if provided, the Counter-Party’s self-imposed CRR Auction credit limit.  The credit constraint for each Counter-Party ensures that the following calculated exposure for all of the Counter-Party’s CRR Account Holders is less than or equal to the Counter-Party’s CRR Auction credit limit:

CEc,a = Σo [ CEOBLBIDo,a + CEOPTBIDo,a + CEOBLOFFERo,a + CEOPTOFFERo,a ]

Where:

CEOBLBIDo=ΣdΣtΣ(j,k) [[ Max[ 0 , Min[ Pbid, MCEP(j,k),c,t,d,z ]] * ( + A(j,k),t,d] * QOBLB(j,k),o,t,d ] + [+ FA(j,k),t,d] * QFDOBLB(j,k),o,t,d ]

CEOPTBIDo = ΣdΣtΣ(j,k)[Max[ 0 , Min[ Pbid,, MCEP(j,k),c,t,d,z ]] * QOPTB(j,k),o,t,d] + [+ FA(j,k),t,d * QFDOPTB(j,k),o,t,d ]

CEOBLOFFERo=ΣdΣtΣ(j,k)[-1 * Min[ 0 , Min[ Poffer, MCEP(j,k),c,t,d,z ]] * QOBLO(j,k),o,t] + FA(j,k),t,d] * QFDOBLO(j,k),o,t,d ]

CEOPTOFFERo=ΣdΣtΣ(j,k)[-1 * Min[ 0 , Min[ Poffer, MCEP(j,k),c,t,d,z ]] * QOPTO(j,k),o,t]+ FA(j,k),t,d] * QFDOPTO(j,k),o,t,d ]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| CEc,a | $ | Credit Exposure for a Counter-Party for an auction — The calculated credit exposure for all CRRs awarded to a Counter-Party. |
| CEOBLBIDo,a | $ | Credit Exposure for PTP Obligations Bid for a CRR Account Holder for an auction |
| CEOPTBIDo,a | $ | Credit Exposure for PTP Options Bid for a CRR Account Holder for an auction |
| CEOBLOFFERo,a | $ | Credit Exposure for PTP Obligations Offered for a CRR Account Holder for an auction |
| CEOPTOFFERo,a | $ | Credit Exposure for PTP Options Offered for a CRR Account Holder for an auction |
| MCEP(j,k),c,t,d,z | $/MW | Maximum Credit Exposure Price for Hedge Type for a Counter-Party for a time of use block for a delivery month per a source and sink pair – For each Counter-Party ERCOT shall calculate per the unique combination of source, sink, TOU, delivery month, PTP type, a single price that results in the maximum product of the quantity submitted multiplied by the submitted price |
| QOBLB (j,k),o,t,d | MWh | Quantity for PTP Obligations Bid for a CRR Account Holder for a Time Of Use block, for the prompt delivery month per a source and sink pair |
| QOPTB(j,k),o,t,d | MWh | Quantity for PTP Options Bid for a CRR Account Holder for a Time Of Use block, for the prompt delivery month per a source and sink pair |
| QFDOBLB(j,k),o,t,d | MWh | Quantity for forward delivery months for PTP Obligations Bid for a CRR Account Holder for a Time Of Use block, for delivery months exceeding the prompt month per a source and sink pair |
| QFDOPTB(j,k),o,t,d | MWh | Quantity for forward delivery months for PTP Options Bid for a CRR Account Holder for a Time Of Use block, for delivery months exceeding the prompt month per a source and sink pair |
| QOBLO(j,k),o,t,d | MWh | Quantity for PTP Obligations Offered for a CRR Account Holder for a Time Of Use block, for a delivery month per a source and sink pair |
| QOPTO(j,k),o,t,d | MWh | Quantity for PTP Options Offered for a CRR Account Holder for a Time Of Use block, for a delivery month per a source and sink pair |
| A(j,k),t,d | $/MW | Adder per source and sink pair for a time of use block for the prompt delivry month |
| FA(j,k),t,d | $/MW | Adder per source and sink pair for a time of use block per delivery month for all delivery months greater than the prompt month |
| A | None | A CRR Auction |
| D | None | Delivery Month |
| J | None | A source settlement point. |
| K | None | A sink settlement point. |
| T | none | Time Of Use (TOU) block |
| O | none | A CRR Account Holder |
| C | none | Counter-Party |
| Z | none | CRR Type (i.e. Obligation or Option) |

(i)	The values of Aj,k,t,d, Mj,k,t,d, FAj,k,t,d,and FMj,k,t,d Shall be posted on the MIS Public Area.  TAC shall review these values at least annually and may recommend, to the ERCOT Board, changes to these values that become effective at least 30 days prior to a monthly CRR Auction and 60 days prior to an annual CRR Auction.  Any changes to these values shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval.

(ii)	The values of Aj,k,t,d, Mj,k,t,d, FAj,k,t,d,and FMj,k,t,d will be defined and modified using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”).

(b)	ERCOT shall award CRRs in quantities truncated to the nearest tenth MW (0.1 MW).

(c)	The CRR clearing price is equal to the corresponding Shadow Price for that CRR product.

(d)	When a CRR Account Holder is awarded CRRs as a result of a CRR Auction, the CRRs do not become the property of the winning CRR Account Holder, and the CRRs may not be placed in their CRR accounts, until the CRR Invoices have been paid in full.

(e)	When a CRR Account Holder sells PTP Obligations as a result of an auction at a negative price, the CRR Account Holder is not relieved of the PTP Obligations until the CRR Invoices have been paid in full.

7.5.6	CRR Auction Settlements

7.5.6.1	Payment of an Awarded CRR Auction Offer

(1)	Following each CRR Auction, ERCOT shall pay each CRR Account Holder  its PTP Obligation offers previously awarded for the following operating month only.  Payments related to PTP Obligation offers previously awarded for months exceeding the following operating month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion the Monthly Auction that is associated with the delivery month related to the PTP Obligation offers awarded. Within each monthly CRR Auction net Invoice, the payment for each source and sink pair for a given Operating Hour is calculated as follows:

OBLSAMT crrh, (j, k), a	=	(-1) * OBLPR (j, k), a * OBLS crrh, (j, k), a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| OBLSAMT crrh, (j, k), a | $ | PTP Obligation Sale Amount per CRR Account Holder per source and sink pair per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation offers with the source j and the sink k awarded in CRR Auction a, for the hour. |
| OBLPR (j, k), a | $/MW per hour | PTP Obligation Price per source and sink pair per CRR Auction—The clearing price of a PTP Obligation with the source j and the sink k in CRR Auction a, for the hour. |
| OBLS crrh, (j, k), a | MW | PTP Obligation Sale per CRR Account Holder per source and sink pair per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Obligation offers associated with the source j and the sink k awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |

(2)	Following each CRR Auction, ERCOT shall pay each CRR Account Holder  its PTP Option offers previously awarded for the following operating month only.  Payments related to PTP Option offers presviously awarded for months exceeding the following operating month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion the Monthly Auction that is associated with the delivery month related to the PTP Option offers awarded. Within each monthly CRR Auction net Invoice, the payment for each source and sink pair for a given Operating Hour is calculated as follows:

OPTSAMT crrh, (j, k), a	=	(-1) * OPTPR (j, k), a * OPTS crrh, (j, k), a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| OPTSAMT crrh, (j, k), a | $ | PTP Option Sale Amount per CRR Account Holder per source and sink pair per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids with the source j and the sink k awarded in CRR Auction a, for the hour. |
| OPTPR (j, k), a | $/MW per hour | PTP Option Price per source and sink pair per CRR Auction—The clearing price of a PTP Option with the source j and the sink k in CRR Auction a, for the hour. |
| OPTS crrh, (j, k), a | MW | PTP Option Sale per CRR Account Holder per source and sink pair per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Option offers with the source j and the sink k awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |

(3)	Following each CRR Auction, ERCOT shall pay each CRR Account Holder  its FGR offers previously awarded for the following operating month only.  Payments related to FGR offers previously awarded for months exceeding the following operating month will besubsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion the Monthly Auction that is associated with the delivery month related to the FGR offers awarded. Within each monthly CRR Auction net Invoice, the payment for each flowgate for a given Operating Hour is calculated as follows:

FGRSAMT crrh, f, a	=	(-1) * FGRPR f, a * FGRS crrh, f, a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FGRSAMT crrh, f, a | $ | Flowgate Right Sale Amount per CRR Account Holder per flowgate per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR offers associated with FGR f awarded in CRR Auction a, for the hour. |
| FGRPR f, a | $/MW per hour | Flowgate Right Price per flowgate per CRR Auction—The clearing price of FGR f in CRR Auction a, for the hour. |
| FGRS crrh, f, a | MW | Flowgate Right Sale per CRR Account Holder per flowgate per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s FGR offers associated with FGR f awarded in CRR Auction a, for the hour. |
| crrh | None | A CRR Account Holder. |
| f | None | An FGR. |
| a | None | A CRR Auction. |

7.5.6.2	Charge of an Awarded CRR Auction Bid

(1)	Following each CRR Auction, ERCOT shall charge each CRR Account Holder  its PTP Obligation bids previously awarded for the following operating month only.  Charges related to PTP Obligation bids previously awarded for months exceeding the following operating month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion the Monthly Auction that is associated with the delivery month related to the PTP Obligation bid awards. Within each monthly CRR Auction net Invoice, the charge for each source and sink pair for a given Operating Hour is calculated as follows:

OBLPAMT crrh, (j, k), a	=	OBLPR (j, k), a * OBLP crrh, (j, k), a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| OBLPAMT crrh, (j, k), a | $ | PTP Obligation Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation bids with the source j and the sink k awarded in CRR Auction a, for the hour. |
| OBLPR (j, k), a | $/MW per hour | PTP Obligation Price per source and sink pair per CRR Auction—The clearing price of a PTP Obligation with the source j and the sink k in CRR Auction a, for the hour. |
| OBLP crrh, (j, k), a | MW | PTP Obligation Purchase per CRR Account Holder per source and sink pair per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Obligation bids associated with the source j and the sink k awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |

(2)	Following each CRR Auction, ERCOT shall charge each CRR Account Holder its PTP Option bids previously awarded for the following operating month only.  Charges related to PTP Option bids previously awarded for months exceeding the following operating month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion the Monthly Auction that is associated with the delivery month related to the PTP Option bid awards. Within each monthly CRR Auction net Invoice, the charge for each source and sink pair for a given Operating Hour is calculated as follows:

OPTPAMT crrh, (j, k), a	=	OPTPR (j, k), a * OPTP crrh, (j, k), a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| OPTPAMT crrh, (j, k), a | $ | PTP Option Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids with the source j and the sink k awarded in CRR Auction a, for the hour. |
| OPTPR (j, k), a | $/MW per hour | PTP Option Price per source and sink pair per CRR Auction—The clearing price of a PTP Option with the source j and the sink k in CRR Auction a, for the hour. |
| OPTP crrh, (j, k), a | MW | PTP Option Purchase per CRR Account Holder per source and sink pair per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Option bids associated with the source j and the sink k awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |

(3)	Following each CRR Auction, ERCOT shall charge each CRR Account Holder  its flowgate bids previously awarded for the following operating month only.  Charges related to flowgate bids previously awarded for months exceeding the following operating month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion the Monthly Auction that is associated with the delivery month related to the flowgate bid awards. Within each monthly CRR Auction net Invoice, the charge for each flowgate for a given Operating Hour is calculated as follows:

FGRPAMT crrh, f, a	=	FGRPR f, a * FGRP crrh, f, a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FGRPAMT crrh, f, a | $ | Flowgate Right Purchase Amount per CRR Account Holder per flowgate per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR bids associated with FGR f awarded in CRR Auction a, for the hour. |
| FGRPR f, a | $/MW per hour | Flowgate Right Price per flowgate per CRR Auction—The clearing price of FGR f in CRR Auction a, for the hour. |
| FGRP crrh, f, a | MW | Flowgate Right Purchase per CRR Account Holder flowgate per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s FGR bids associated with FGR f awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| f | none | An FGR. |
| a | none | A CRR Auction. |

7.5.6.3	Charge of PCRRs Pertaining to a CRR Auction

(1)	For pre-assigned PTP Obligations allocated before each CRR Auction (annual or monthly auction), following each CRR Auction ERCOT shall charge each CRR Account Holder for the following operating month only.  Charges related to pre-assigned PTP Obligations allocated for months exceeding the following operating month will be invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion the Monthly Auction that is associated with the delivery month related to the pre-assigned PTP Obligations allocated. Within each monthly CRR Auction net Invoice,the charge for each source and sink pair for a given Operating Hour is calculated as follows:

If OBLPR (j, k), a >0

PCRROBLAMT crrh, (j, k), a, tech =	PCRROBLF tech * OBLPR (j, k), a

* PCRROBL crrh, (j, k), a, tech

Otherwise

PCRROBLAMT crrh, (j, k), a, tech =	OBLPR (j, k), a * PCRROBL crrh, (j, k), a, tech

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRROBLAMT crrh, (j, k), a, tech | $ | PCRR PTP Obligation Amount per CRR Account Holder per source and sink pair per CRR Auction by resource technology—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Obligations associated with the source j and the sink k allocated before CRR Auction a based on Resources of the technology tech, for the hour. |
| PCRROBLF tech |  | PCRR PTP Obligation pricing Factor per resource technology—The pricing factor of pre-allocated PTP Obligations based on Resources of the technology tech. See Section 7.4.2, PCRR Allocation Terms and Conditions, item (f)(ii). |
| OBLPR (j, k), a | $/MW per hour | PTP Obligation Price per source and sink pair per CRR Auction—The clearing price of a PTP Obligation with the source j and the sink k in CRR Auction a, for the hour. |
| PCRROBL crrh, (j, k), a, tech | MW | PCRR PTP Obligation per CRR Account Holder per source and sink pair per CRR Auction by resource technology—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Obligations associated with the source j and the sink k allocated before CRR Auction a based on Resources of the technology tech, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |
| tech | none | A Resource technology. See Section 7.4.2, PCRR Allocation Terms and Conditions, item (f). |

(2)	For pre-assigned PTP Options allocated before each CRR Auction (annual or monthly auction), following each CRR Auction ERCOT shall charge each CRR Account Holder for the following operating month only.  Charges related to pre-assigned PTP Options allocated for months exceeding the following operating month will be invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion the Monthly Auction that is associated with the delivery month related to the pre-assigned PTP Options allocated. Within each monthly CRR Auction net Invoice, the charge for each source and sink pair for a given Operating Hour is calculated as follows:

PCRROPTAMT crrh, (j, k), a, tech	=	PCRROPTF tech * OPTPR (j, k), a * 					PCRROPT crrh, (j, k), a, tech

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRROPTAMT crrh, (j, k), a, tech | $ | PCRR PTP Option Amount per CRR Account Holder per source and sink pair per CRR Auction by resource technology—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Options associated with the source j and the sink k allocated before CRR Auction a based on Resources of the technology tech, for the hour. |
| PCRROPTF tech |  | PCRR PTP Option pricing Factor per resource technology—The pricing factor of pre-allocated PTP Options based on Resources of the technology tech. See Section 7.4.2, PCRR Allocation Terms and Conditions, item (f) (i). |
| OPTPR (j, k), a | $/MW per hour | PTP Option Price per source and sink pair per CRR Auction—The clearing price of a PTP Option with the source j and the sink k in CRR Auction a, for the hour. |
| PCRROPT crrh, (j, k), a, tech | MW | PCRR PTP Option per CRR Account Holder per source and sink pair per CRR Auction by resource technology—The MW quantity that represents the total of CRR Account Holder crrh’s PTP Options with the source j and the sink k allocated before CRR Auction a based on Resources of the technology tech, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| A | none | A CRR Auction. |
| Tech | none | A Resource technology. See Section 7.4.2, PCRR Allocation Terms and Conditions, item (f). |

16.11.4.5	Determination of the Counter-Party Future Credit Exposure

(1)	ERCOT shall monitor and calculate the Counter-Party’s FCE for all CRRs held by the Counter-Party as owner of record at ERCOT, for all Operating Days for which a Day Ahead Market has not executed and for CRRs that have not settled, using the formula below.

FCE o  =  FCEOBL o + FCEOPT o + FCRFGR o- Max[ 0, FDAR o ]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCE o | $ | Future Credit Exposure—Counter-Party FCE for all CRRs held by the Counter-Party as owner o of record at ERCOT, for all Operating Days that have not yet occurred and for CRRs that have not settled. |
| FCEOBL o | $ | Future Credit Exposure for PTP Obligations—Counter-Party FCE for all PTP Obligations held by the Counter-Party as owner o of record at ERCOT, for all Operating Days for which a Day Ahead Market has not executed . |
| FCEOPT o | $ | Future Credit Exposure for PTP Options—Counter-Party FCE for all PTP Options held by the Counter-Party as owner o of record at ERCOT, for all Operating Days for which a Day Ahead Market has not executed. |
| FCEFGR o | $ | Future Credit Exposure for FGRs—Counter-Party FCE for all Flowgate Rights (FGRs) held by the Counter-Party as owner o of record at ERCOT, for all Operating Days for which a Day Ahead Market has not executed. |
| FDAR o | $ | Future Discounted Auction Revenue— ERCOT shall determine, for each forward operating month for which CRRs have been auctioned, the expected CRR Monthly Revenues the Counter-Party as owner o is forecasted to realize. |
| o | none | A CRR Owner |

(2)	The Counter-Party’s FCE for all PTP Obligations (FCEOBL) held by the Counter-Party as owner of record at ERCOT for:

(a)	All Operating Days for which a Day Ahead Market has not executed within the current month and for all hours related to PTP Obligations that exceed the current month for which ERCOT and the Counter-Party have settled payment for an awarded CRR, FCEOBL is calculated as follows:

FCEOBL o   	= 	- FMMOBL o + INTMOBL o

Where:

INTMOBL o =Σ(h) Σ(j,k)(INTM h, (j,k) * OBLMW o, h, (j,k))

FMMOBL o = Σ(h)Σ(j,k)[ACP h, (j, k) * OBLMWo, h, (j, k)]

- For all hours related to PTP Obligations that exceed the current month for which ERCOT and the Counter-Party have not settled payment for an awarded CRR, FCEOBL is calculated as follows:

FCEOBL o   = - FMMOBL o + INTMOBL o

Where:

INTMOBL o =Σ(h) Σ(j,k)(INTM h, (j,k) * OBLMW o, h, (j,k))

FMMOBL o =Σ(h)Σ(j,k)[(ACP h, (j, k) - OBLPR (j, k), a)* OBLMWo, h, (j, k)]

Σ(h)Σ(j,k)

Σ(h)Σ(j,k)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCEOBL o | $ | Future Credit Exposure for PTP Obligations—Counter-Party FCE for all PTP Obligations held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a Day Ahead Market has not executed and for CRRs that have not settled. |
| INTMOBL o | $ | Initial Margin Exposure for all PTP Obligations held by the Counter-party as owner o of record at ERCOT for all Operating Days for which a Day Ahead Market has not executed and for CRRs that have not settled. |
| INTm h, (j, k) | $/MW per hour | Initial Margin Exposure for PTP Obligations with the source j and the sink k for hour h - Exposure level calculated using A TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”). The procedures will be used to define and modify INTM h, (j, k) - The procedures to determine INTM and the value of INTM h, (j, k) shall be posted on the MIS Public Area. The Technical Advisory Committee (TAC) shall review the procedures at least annually and may recommend to the ERCOT Board changes to the parameters. If changes to these parameters are approved by the ERCOT Board, such revised procedures shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval. - ERCOT shall generally notify counterparties two Bank Business Days prior to implementing either a change in procedures to determine INTM or the value of INTM h, (j, k) - ERCOT may adjust any INTM h, (j, k) immediately if, in its sole discretion, ERCOT determines that either the parameters or the reported value of INTM h, (j, k) does not adequately match the financial risk created by the PTP Obligation. |
| FMMOBL o | $ | Forward Mark-to-Market for PTP Obligations—Estimate of the forward mark-to-market value of PTP Obligations held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a Day Ahead Market has not executed and for CRRs that have not settled. |
| ACP h, (j, k) | $/MW per hour | Auction Clearing Price—The auction clearing price for a PTP Obligation within the most recent CRR Auction corresponding to the CRR with a delivery month m, with the source j and the sink k for hour h. ERCOT has the discretion to use a price from the bilateral and/or over-the-counter market for a delivery month m, with the source j and the sink k for hour h if the price originated from an executed transaction and has a transaction date in excess of the most recent CRR auction associated with delivery month m. |
| OBLPR (j, k), a | $/MW per hour | PTP Obligation Price per source and sink pair per CRR Auction—The clearing price of the PTP Obligation with the source j and the sink k in CRR Auction a, for the hour. |
| OBLMW o, h, (j, k) | MW | PTP Obligation with the source j and the sink k for hour h owned by the Counter-Party as owner o for all Operating Days for which a Day Ahead Market has not executed and for CRRs that have not settled. |
| A | None | A CRR Auction. |
| J | None | A source Settlement Point |
| K | None | A sink Settlement Point |
| H | None | An Operating Hour. |
| M | none | An Operating Month |
| O | None | A CRR Owner |

(3)		The Counter-Party’s FCE for all PTP Options (FCEOPT) held by the Counter-Party as owner of record at ERCOT for:

(a) 	All Operating Days for which a Day Ahead Market has not executed within the current month and for all hours related to PTP Options that exceed the current month for which ERCOT and the Counter-Party have settled payment for an awarded CRR, FCEOPT is calculated as follows.

FCEOPT o  	=	- FMMOPT o

Where:

FMMOPT o = Σ(h)Σ(j,k)[( ACP h, (j, k)  ) * OPTMW o, h, (j, k)]

(b) 	For all hours related to PTP Options that exceed the current month for which ERCOT and the Counter-Party have not settled payment for an awarded CRR, FCEOPT is calculated as follows.

FCEOPT o   = - FMMOPT o

Where:

FMMOPT o = Σ(h)Σ(j,k)[Max [- OPTPR (j, k), a , ( ACP h, (j, k) - OPTPR (j, k), a - ACPE h, (j,k))] * OPTMW o, h, (j, k)]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCEOPT o | $ | Future Credit Exposure for PTP Options—Counter-Party FCE for all PTP Options held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a Day Ahead Market has not executed and for CRRs that have not settled. |
| FMMOPT o | $ | Forward Mark-to-Market for PTP Options—Estimate of the forward mark-to-market value of PTP Options held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a Day Ahead Market has not executed and for CRRs that have not settled. |
| INTM h, (j, k) | $/MW per hour | Initial Margin Exposure for PTP Options with the source j and the sink k for hour h - Exposure level calculated using A TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”). The procedures will be used to define and modify INTM h, (j, k) - The procedures to determine INTM and the value of INTM h, (j, k) shall be posted on the MIS Public Area. The Technical Advisory Committee (TAC) shall review the procedures at least annually and may recommend to the ERCOT Board changes to the parameters. If changes to these parameters are approved by the ERCOT Board, such revised procedures shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval. - ERCOT shall generally notify counterparties two Bank Business Days prior to implementing either a change in procedures to determine INTM or the value of INTM h, (j, k) - ERCOT may adjust any ACPE h, (j, k) immediately if, in its sole discretion, ERCOT determines that either the parameters or the reported value of INTM h, (j, k) does not adequately match the financial risk created by the PTP Options. |
| ACP h, (j, k) | $/MW per hour | Auction Clearing Price— The auction clearing prices for a PTP Option within the most recent CRR Auction corresponding to the CRR with a delivery month m, with the source j and the sink k for hour h. ERCOT has the discretion to use a price from the bilateral and/or over-the-counter market for a delivery month m, with the source j and the sink k for hour h if the price originated from an executed transaction and has a transaction date in excess of the most recent CRR auction associated with delivery month m.. |
| OPTMW o, h, (j, k) | MW | PTP Option with the source j and the sink k owned by the Counter-Party as owner o for hour h. |
| OPTPR (j, k), a | $/MW per hour | PTP Option Price per source and sink pair per CRR Auction—The clearing price of a PTP Option with the source j and the sink k in CRR Auction a, for the hour. |
| a | None | A CRR Auction. |
| j | none | A source settlement point. |
| k | none | A sink settlement point. |
| h | none | An operating hour. |
| m | none | An Operating Month |
| o | none | A CRR owner |

- The Counter-Party’s FCE for all FGRs (FCEFGR) held by the Counter-Party as owner of record at ERCOT for

(a)	All Operating Days that for which a Day Ahead Market has not executed within the current month and for all hours related to Flow Gate Rights that exceed the current month for which ERCOT and the Counter-Party have settled payment for an awarded CRR, FCEFGR is calculated as follows:

FCEFGR o  	=	- FMMFGR o + INTMFGR o

Where:

INTMFGR o	=Σ(h) Σ(f)(INTM h, f * FGRMW o, h, f)

FMMFGR o   =Σ(h)Σ(f)[ACP h, f  * FGRMW o, h, f]

(b)	For all hours related to PTP Obligations that exceed the current month for which ERCOT and the Counter-Party have not settled payment for an awarded CRR, FCEFGR is calculated as follows:

FCEFGR o  = - FMMFGR o + INTMFGR o

Where:

INTMFGR o =Σ(h) Σ(f)(INTM h, f * FGRMW o, h, f)

FMMFGR o =Σ(h)Σ(f)[(ACP h, f - FGRPR f, a )* FGRMWo, h, f]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCEFGR o | $ | Future Credit Exposure for FGRs—Counter-Party FCE for all FGRs held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a Day Ahead Market has not executed and for CRRs that have not settled. |
| ACPEFGR o | $ | Auction Clearing Price Exposure for all Flow Gate Rights held by the Counter-party as owner o of record at ERCOT for all Operating Days for which a Day Ahead Market has not executed and for CRRs that have not settled. |
| INTM h, f | $/MW per hour | Initial Margin Exposure for Flow Gate Rights on the flowgate f - Exposure level calculated using A TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”). The procedures will be used to define and modify INTM h, f - The procedures to determine INTM and the value of INTM h, f shall be posted on the MIS Public Area. The Technical Advisory Committee (TAC) shall review the procedures at least annually and may recommend to the ERCOT Board changes to the parameters. If changes to these parameters are approved by the ERCOT Board, such revised procedures shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval. - ERCOT shall generally notify counterparties two Bank Business Days prior to implementing either a change in procedures to determine INTM or the value of ACPE h, f - ERCOT may adjust any INTM h, f immediately if, in its sole discretion, ERCOT determines that either the parameters or the reported value of ACPE h, f does not adequately match the financial risk created by the Flow Gate Rights. |
| FMMFGR o | $ | Forward Mark-to-Market for FGRs—Estimate of the forward mark-to-market value of FGRs held by the Counter-Party as owner o of record at ERCOT for all Operating Days that have not yet occurred and for CRRs that have not settled. |
| ACP h, f | $/MW per hour | Auction Clearing Price— The auction clearing prices for a FGR within the most recent CRR Auction corresponding to the CRR with a delivery month m, with the flowgate f for hour h. ERCOT has the discretion to use a price from the bilateral and/or over-the-counter market for a delivery month m, with the flowgate f for hour h if the price originated from an executed transaction and has a transaction date in excess of the most recent CRR auction associated with delivery month m.. |
| FGRMW o, h, f | MW | FGR on the flowgate f owned by the Counter-Party as owner o for hour h . |
| FGRPR f, a | $/MW per hour | Flow Gate Right Price per source and sink pair per CRR Auction—The clearing price of the Flow Gate Rights with the flowgate f in CRR Auction a, for the hour. |
| A | None | A CRR Auction. |
| F | None | A Flowgate Right |
| H | None | An Operating Hour |
| M | none | An Operating Month |
| O | none | A CRR Owner |

- ERCOT shall determine the Counter-Party’s Future Discounted Auction Revenue for each operating month associated with either awarded CRRs or allocated PCRRs that are unrelated to settled CRRs for which ERCOT has distributed Auction Revenues as calculated under Section 7.5.7 and Section 9.10. The Counter-Party’s FDAR o is calculated as follows

FDAR o = ΣqΣz [(-1) * [Σa(FDCRRZREV z, a + FDPCRRZREV z, a) * FDMLRSZ z, q + Σa (FDCRRNZREV z, a + FDPCRRNZREV a) * FDMLRS q]]

Where

FDCRRZREV z, a =Σh(ΣcrrhΣjΣkOBLSAMTcrrh,(j,k),z,a,h + ΣcrrhΣjΣkOPTSAMTcrrh,(j,k),z,a,h + ΣcrrhΣfFGRSAMT crrh,f,z,a,h + ΣcrrhΣjΣkFDOBLPAMT crrh,(j,k),z,a,h + ΣcrrhΣjΣkFDOPTPAMTcrrh,(j,k),z,a,h + ΣcrrhΣfFDFGRPAMT crrh,f,z,a,h)

FDPCRRZREV z, a	=	Σh(ΣcrrhΣjΣkΣtechFDPCRROBLAMT crrh,(j,k),z,a,tech,h + ΣcrrhΣjΣkΣtechFDPCRROPTAMT crrh,(j,k),z,a,tech,h)

FDCRRNZREV a =Σh(ΣcrrhΣjΣkOBLSAMTcrrh,(j,k),a,h + ΣcrrhΣjΣkOPTSAMTcrrh,(j,k),a,h + ΣcrrhΣfFGRSAMT crrh,f,a,h + ΣcrrhΣjΣkFDOBLPAMT crrh,(j,k),a,h + ΣcrrhΣjΣkFDOPTPAMTcrrh,(j,k),a,h + ΣcrrhΣfFDFGRPAMT crrh,f,a,h)

FDPCRRNZREV a	=	Σh(ΣcrrhΣjΣkΣtechFDPCRROBLAMT crrh,(j,k),a,tech,h + ΣcrrhΣjΣkΣtechFDPCRROPTAMT crrh,(j,k),a,tech,h)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FDCRRZREV z, a | $ | Future Discounted CRR Zonal Revenue per zone per CRR Auction— The probabilistic revenue for a given month produced from CRRs that source and sink within the same 2003 ERCOT CMZ z, cleared through CRR Auction Offers and CRR Auction Bids in CRR Auction a. Using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) probabilistic revenue shall be defined. |
| OBLSAMT crrh, (j, k), z, a, h | $ | PTP Obligation Sale Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per hour—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation offers awarded in CRR Auction a with the source j and the sink k, both in CMZ z, for the hour h. |
| OPTSAMT crrh, (j, k), z, a, h | $ | PTP Option Sale Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per hour—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids awarded in CRR Auction a with the source j and the sink k, both in CMZ z, for the hour h. |
| FGRSAMT crrh, f, z, a, h | $ | Flowgate Right Sale Amount per CRR Account Holder per flowgate per zone per CRR Auction per hour—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR offers awarded in CRR Auction a associated with FGR f in CMZ z, for the hour h. |
| FDOBLPAMT crrh, (j, k), z, a, h | $ | Future Discounted PTP Obligation Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation bids awarded in CRR Auction a with the source j and the sink k, both in CMZ z, for the hour h. |
| FDOPTPAMT crrh, (j, k), z, a, h | $ | Future Discounted PTP Option Purchase Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids awarded in CRR Auction a with the source j and the sink k, both in CMZ z, for the hour h. |
| FDFGRPAMT crrh, f, z, a, h | $ | Future Discounted Flowgate Right Purchase Amount per CRR Account Holder per flowgate per zone per CRR Auction per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR bids awarded in CRR Auction a associated with FGR f in CMZ z, for the hour h. |
| FDMLRS q | none | Future Discounted Monthly Load Ratio Share (LRS) per QSE—The LRS calculated for QSE q for the peak-Load 15-minute Settlement Interval in the month. See the TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) for defining probable Load Ratio Share values. |
| FDMLRSZ q, z | none | Future Discounted Monthly Load Ratio Share Zonal per QSE per zone—The Load Ratio Share of QSE q for its Load in CMZ z, for the peak-Load 15-minute Settlement Interval in the month. See the TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) for defining probable Load Ratio Share values. |
| FDCRRNZREV a | $ | Future Discounted CRR Non-Zonal Revenue—The probabilistic revenue for a given month produced from CRRs that source and sink in different 2003 ERCOT CMZs, cleared through CRR Auction Offers and CRR Auction Bids in CRR Auction a. Using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) probabilistic revenue shall be defined. |
| OBLSAMT crrh, (j, k), a, h | $ | PTP Obligation Sale Amount per CRR Account Holder per source and sink pair per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation offers awarded in CRR Auction a with the source j and the sink k in different CMZs, for the hour h. |
| OPTSAMT crrh, (j, k), a, h | $ | PTP Option Sale Amount per CRR Account Holder per source and sink pair per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option offers awarded in CRR Auction a with the source j and the sink k in different CMZs, for the hour h. |
| FGRSAMT crrh, f, a, h | $ | Flowgate Right Sale Amount per CRR Account Holder per flowgate per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR offers awarded in CRR Auction a associated with FGR f across CMZs, for the hour h. |
| FDOBLPAMT crrh, (j, k), a, h | $ | Future Discounted PTP Obligation Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation bids awarded in CRR Auction a with the source j and the sink k in different CMZs, for the hour h. |
| FDOPTPAMT crrh, (j, k), a, h | $ | Future Discounted PTP Option Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids awarded in CRR Auction a with the source j and the sink k in different CMZs, for the hour h. |
| FDFGRPAMT crrh, f, a, h | $ | Future Discounted Flowgate Right Purchase Amount per CRR Account Holder per flowgate per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR bids awarded in CRR Auction a associated with FGR f across CMZs, for the hour h. |
| FDPCRRZREV z, a | $ | Future Discounted PCRR Zonal Revenue per zone per CRR Auction—The probabilistic revenue for a given month produced from PCRRs that source and sink within the same 2003 ERCOT CMZ, pertaining to each CRR Auction. The revenue resulted from the PCRRs that source and sink in CMZ z, pertaining to CRR Auction a. Using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) probabilistic revenue shall be defined. |
| FDPCRROBLAMT crrh, (j, k), z, a, tech, h | $ | Future Discounted PCRR PTP Obligation Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per resource technology per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Obligations pertaining to CRR Auction a with the source j and the sink k in CMZ z, based on Resources of the technology tech, for the hour h. |
| FDPCRROPTAMT crrh, (j, k), z, a, tech, h | $ | Future Discounted PCRR PTP Option Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per resource technology per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Options pertaining to CRR Auction a with the source j and the sink k in CMZ z, based on Resources of the technology tech, for the hour h. |
| FDPCRRNZREV a | $ | Future Discounted PCRR Non-Zonal Revenue per CRR Auction—The probabilistic revenue for a given month produced from PCRRs that source and sink in different 2003 ERCOT CMZs, pertaining to each CRR Auction. The revenue resulted from the PCRRs that source and sink in different CMZs, pertaining to CRR Auction a. Using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) probabilistic revenue shall be defined. |
| FDPCRROBLAMT crrh, (j, k), a, tech, h | $ | Future Discounted PCRR PTP Obligation Amount per CRR Account Holder per source and sink pair per CRR Auction per resource technology per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Obligations pertaining to CRR Auction a with the source j and the sink k in different CMZs, based on Resources of the technology tech, for the hour h. |
| FDPCRROPTAMT crrh, (j, k), a, tech, h | $ | Future Discounted PCRR PTP Option Amount per CRR Account Holder per source and sink pair per CRR Auction per resource technology per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Options pertaining to CRR Auction a with the source j and the sink k in different CMZs, based on Resources of the technology tech, for the hour h. |
| a | none | A CRR Auction. |
| q | none | A QSE. |
| z | none | A 2003 ERCOT CMZ. |
| crrh | none | A CRR Account Holder that paid the invoice in full. |
| tech | none | A Resource technology. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| f | none | An FGR. |
| h | none | An hour in the month. |