| NPRR Number | 326 | NPRR Title | Adjust the Calculation of the Real-Time Settlement Point Price for a Resource Node |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | April 21, 2011 |  |  |
| Proposed Effective Date |  | Upon system implementation. |  |  |
| Priority and Rank Assigned |  | Priority – High; Rank – 12.9 |  |  |
| Nodal Protocol Section Requiring Revision |  | 6.6.1.1, Real-Time Settlement Point Price for a Resource Node |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) adjusts the calculation of the Real-Time Settlement Point Price for a Resource Node. The adjustment is to remove the Base Point weighting of the Real-Time Locational Marginal Prices (LMPs). |  |  |
| Reason for Revision |  | The current method of calculating payments and charges of Point-to-Point (PTP) Obligations and PTP Options settled in Real-Time based on Base Point weighted Real-Time Settlement Point Prices can lead to payments and charges when there is no congestion, which are not supported by congestion rent and can lead to Real-Time Revenue Neutrality Allocation uplift. With the implementation of this NPRR, payments and charges for PTP Obligations and PTP Options settled in Real-Time will be more accurate and should only occur when there is Real-Time congestion. |  |  |
| Overall Market Benefit |  | Dollar amounts for PTP Obligations and PTP Options settled in Real-Time will be more accurate. |  |  |
| Overall Market Impact |  | The Real-Time Settlement Point Price for a Resource Node is used in many other Settlement calculations in addition to the calculations for the payments and charges of PTP Obligations and PTP Options settled in Real-Time. These other calculations include calculations for Real-Time Energy Imbalance at a Resource Node, Reliability Unit Commitment (RUC), Real-Time congestion payment or charge for Self-Schedules, Base Point Deviation, Reliability Must-Run (RMR), voltage support and payment for emergency power increase. Changing the methodology on calculating the Real-Time Settlement Point Price for a Resource Node should improve the accuracy of the Real-Time Energy Imbalance at a Resource Node. The change should not have a significant negative impact on the accuracy of the other calculations. |  |  |
| Consumer Impact |  | Unknown. |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - On 2/10/11, NPRR326 and an Impact Analysis were posted. - On 2/11/11, Reliant Energy Retail Services comments were posted. - On 2/16/11, the motion to grant NPRR326 Urgent status failed via PRS e-mail vote. - On 3/15/11, a Cost Benefit Analysis was posted. - On 3/24/11, PRS considered NPRR326. - On 4/20/11, a revised Cost Benefit Analysis was posted. - On 4/21/11, PRS considered the 3/24/11 PRS Report and Impact Analysis for NPRR326. |  |  |
| PRS Decision |  | On 3/24/11, PRS voted to recommend approval of NPRR326 as submitted. There were two abstentions from the Consumer and Independent Power Marketer (IPM) Market Segments. All Market Segments were present for the vote. On 4/21/11, PRS voted to endorse and forward the 3/24/11 PRS Report and Impact Analysis for NPRR326 to TAC and to recommend a priority of High and a rank of 12.9. There were two abstentions from the Consumer and Independent Generator Market Segments. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 3/24/11, ERCOT Staff stated that its primary concerns in submitting NPRR326 were (1) performance of the Day-Ahead Market (DAM) in light of unexpected intra-station Congestion Revenue Right (CRR) activity, and (2) the possibility of payments and charges occurring in the absence of congestion and thus without matching congestion rent, causing Real-Time Revenue Neutrality Allocation uplift. It was noted that Reliant Energy Retail Services no longer recommended referring NPRR326 to WMS as stated in its 2/11/11 comments given the discussion of this NPRR at the 3/22/11 ERCOT Board meeting. Concern was expressed that while NPRR326 proposes a solution for Resource Node Real-Time Settlement Point Prices, it does not address Load Zones. Concern was also expressed that NPRR326 could have unintended consequences related to the other calculations that use the Real-Time Settlement Point Price. It was emphasized that no Resource would be impacted by NPRR326 as the Settlement Point Price is not used to settle generation. On 4/21/11, PRS reviewed the revised Cost Benefit Analysis for NPRR326. ERCOT Staff noted that an extremely conservative approach was used in the analysis of market benefits. It was recommended that the statement of market benefits be revised to more realistically capture the actual benefits. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 |  |  |
| --- | --- | --- | --- |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | See Overall Market Impact above. | Unknown. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Dollar amounts for PTP Obligations and PTP Options settled in Real-Time will be more accurate. | Unknown. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Steve Reedy |
| E-mail Address | sreedy@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-4515 |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Reliant Energy Retail Services 021111 | Recommended that NPRR326 be referred to WMS or tabled. |

Proposed Protocol Language Revision

6.6.1.1	Real-Time Settlement Point Price for a Resource Node

(1)	Except for a logical Resource Node for a Combined Cycle Train, the Real-Time Settlement Point Price for a Resource Node Settlement Point is the time-weighted average of the Real-Time Locational Marginal Prices (LMPs).  The Real-Time Settlement Point Price for a 15-minute Settlement Interval is calculated as follows:

RTSPP	=	Σy(RNWFy * RTLMPy)

Where the Resource Node weighting factor is:

RNWFy	=	ΣrTLMPy  / ΣyΣr TLMPy

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point PriceThe Real-Time Settlement Point Price at the Settlement Point for the 15-minute Settlement Interval. |
| RTLMPy | $/MWh | Real-Time Locational Marginal Price per intervalThe Real-Time LMP at the Settlement Point for the Security-Constrained Economic Dispatch (SCED) interval y. |
| RNWFy | none | Resource Node Weighting Factor per intervalThe weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| TLMPy | second | Duration of SCED interval per intervalThe duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |

(2)	The Real-Time Settlement Point Price at the logical Resource Node for the On-Line Combined Cycle Generation Resource shall be determined in accordance with paragraph (1) above using a Real-Time LMP calculated for the logical Resource Node in each SCED Interval as follows:

(a)	The Real-Time LMP for the logical Resource Node for each SCED interval shall be the sum of the Real-Time LMP in each SCED interval at each of the Resource Nodes of the generation units registered in the On-Line (as determined by Real-Time telemetry) Combined Cycle Generation Resource times a weight factor determined as set forth in paragraph (b) below.

Where:

RTLMP = ∑CCGR_PhyR RTLMPCCGR_PhyR * RTCCGRWFCCGR_PhyR

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTLMP | $/MWh | Real-Time Locational Marginal Price at a logical Resource Node for a Combined Cycle TrainThe Real-Time LMP at the Combined Cycle Generation Resource logical Resource Node for a SCED Interval. |
| RTLMPCCGR_PhyR | $/MWh | Real-Time Locational Marginal Price at a generation unit Resource Node registered to the Combined Cycle Generation ResourceThe Real-Time LMP at the Resource Node of an On-Line generation unit designated in a Combined Cycle Train registration for the Combine Cycle Generation Resource for each SCED interval. |
| RTCCGRWFCCGR_PhyR | none | Real-Time Combined Cycle Generation Resource Weighting FactorThe Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the Combined Cycle Generation Resource. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train registration for a Combined Cycle Generation Resource. |

(b)	The weight factor for each generation unit registered in a Combined Cycle Generation Resource shall be the Real-Time net power output telemetry in each SCED interval for each generation unit registered in the Combined Cycle Generation Resource divided by the total Real-Time net power output telemetry for all of the generation units registered in the Combined Cycle Generation Resource.

Where:

RTCCGRWFCCGR_PhyR = TGCCGR_PhyR / ∑CCGR_PhyR TGCCGR_PhyR

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTCCGRWFCCGR_PhyR | none | Real-Time Combined Cycle Generation Resource Weighting FactorThe Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the Combined Cycle Generation Resource. |
| TGCCGR_PhyR | MW | Telemetered Generation for a Combined Cycle Generation Resource generation unitThe telemetered generation of a generation unit designated in a Combined Cycle Train registration for the Combined Cycle Generation Resource at the time of each SCED run. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train registration for a Combined Cycle Generation Resource. |