| NPRR Number | 309 | NPRR Title | Providing STWPF to QSEs Contractually Affiliated with WGR |
| --- | --- | --- | --- |
| Date |  | May 12, 2011 |  |
| Submitter’s Information |  |  |  |
| Name |  | Eric Goff on behalf of the Wholesale Market Subcommittee (WMS) |  |
| E-mail Address |  | egoff@reliant.com |  |
| Company |  | Reliant Energy Retail Services, LLC |  |
| Cell Number |  | 512-284-4908 |  |
| Market Segment |  | Independent Retail Electric Provider (IREP) |  |

Comments

At its May 11, 2011 meeting, WMS reviewed Nodal Protocol Revision Request (NPRR) 309.  After discussion, WMS unanimously voted to endorse NPRR309 as submitted.  After an Impact Analysis has been prepared, WMS recommends that the cost of implementing NPRR309 be compared with the cost of creating a separate sub-QSE to represent the relevant WGR(s), which may be an alternative solution.

Revised Proposed Protocol Language

4.2.2	Wind-Powered Generation Resource Production Potential

(1)	ERCOT shall produce and update hourly a Short-Term Wind Power Forecast (STWPF) that provides a rolling 48-hour hourly forecast of wind production potential for each Wind-powered Generation Resource (WGR).  ERCOT shall produce and update an hourly Total ERCOT Wind Power Forecast (TEWPF) providing a probability distribution of the hourly production potential from all wind-power in ERCOT for each of the next 48 hours.  Each Generation Entity that owns a WGR shall install and telemeter to ERCOT the site-specific meteorological information that ERCOT determines is necessary to produce the STWPF and TEWPF forecasts.  ERCOT shall establish procedures specifying the accuracy requirements of WGR meteorological information telemetry.

(2)	ERCOT shall use the probabilistic TEWPF and select the forecast that the actual total ERCOT WGR production is expected to exceed 50% of the time (50% probability of exceedance forecast).  To produce the STWPF, ERCOT will allocate the TEWPF 50% probability of exceedance forecast to each WGR such that the sum of the individual STWPF forecasts equal the TEWPF forecast.  The updated STWPF forecasts for each hour for each WGR are to be used as input into each Reliability Unit Commitment (RUC) process as per Section 5, Transmission Security Analysis and Reliability Unit Commitment .

(3)	ERCOT shall produce the Wind-powered Generation Resource Production Potential (WGRPP) forecasts using the information provided by WGR owners including WGR availability, meteorological information, and Supervisory Control and Data Acquisition (SCADA).

(4)	Each hour, ERCOT shall provide, through the Messaging System, the STWPF forecasts for each WGR to the QSE that represents that WGR and any QSE contractually affiliated with that WGR.  ERCOT shall also post each STWPF forecast on the MIS Certified Area for those associated QSEs.

(5)	Each hour, ERCOT shall post the STWPF 50% probability of exceedance forecast on the MIS Secure Area.  ERCOT shall retain the STWPF for each hour.

(6)	ERCOT shall post to the MIS on a regional basis a rolling 48 hour actual wind power production and the forecasted amounts from the STWPF and the TEWPF.