| NPRR Number | 354 | NPRR Title | Revisions to Non-Spin Performance Criteria Language and Provision for ICCP Telemetry of Non-Spin Deployment |
| --- | --- | --- | --- |
| Date Posted |  | April 15, 2011 |  |
| Nodal Protocol Sections Requiring Revision |  | 6.5.7.6.2.3, Non-Spinning Reserve Service Deployment 8.1.1.4.3, Non-Spinning Reserve Service Energy Deployment Criteria |  |
| Requested Resolution |  | Normal |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) clarifies the performance criteria for Non-Spinning Reserve (Non-Spin) Service deployments, adds language for provision of a Non-Spin deployment flag to Qualified Scheduling Entities (QSEs) via Inter-Control Center Communications Protocol (ICCP), and adds a requirement for Extensible Markup Language (XML) notification to all QSEs when Non-Spin is deployed. Specifically, a clarification for the requirement that a Resource be “released to SCED” is added to Sections 6.5.7.6.2.3 and 8.1.1.4.3 to clarify the expectation of ERCOT that at 25 minutes after receipt of the deployment instruction, the Resource must be participating in Security-Constrained Economic Dispatch (SCED) at a net MW output at least equal to its Low Sustained Limit (LSL). The requirements for the three Resource types providing Non-Spin have been divided into separate paragraphs, one each for Load, On-Line and Off-Line Generation Resources. In addition, Generation Resources that are providing Non-Spin from Off-Line Generation Resources would be entitled to receive a compliance exemption for documented start-up loading failures. This language is adopted from the zonal Protocols and is included to recognize the difficulty inherent in unit start-up from a cold state for immediate deployment. |  |
| Reason for Revision |  | This NPRR is necessary to address issues realized during Nodal stabilization. The Non-Spin performance criteria in Section 6.5.7.6.2.3, paragraph (4) requires revision to make them consistent with Section 8.1.1.4.3, paragraph (3). The full deployment of Non-Spin capacity must be released and available for SCED within 25 minutes. The current XML notification of Non-Spin deployment is not adequate for such a crucial operating parameter due to the high volume of XML notifications that normally occur. By relying on an XML notification, the QSE’s awareness of the need to start Resources for system reliability in response to a market notification can be delayed. The Non-Spin deployment flag communicated via ICCP is necessary to immediately notify the QSEs of Non-Spin deployments when timely deployment of Off-Line Resources is critical to provide capacity for system reliability. |  |
| Overall Market Benefit |  | The ICCP notification of Non-Spin deployment will enhance reliability to ensure that QSEs receive timely notification of their Non-Spin deployments without delay. Additionally, this NPRR will provide clarity for operating parameters for QSEs and Resource Entities |  |
| Overall Market Impact |  | To be determined. |  |
| Consumer Impact |  | Improves reliability and potentially lowers energy prices due to more efficient deployment of Non-Spin Resources. |  |
| Credit Implications |  | No. |  |

| Quantitative Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | To be determined. | To be determined. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Increased reliability due to clear performance objectives. | To be determined. |
|  | 2 | Increased reliability due to timely notification of Non-Spin deployments. | To be determined. |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | The addition of XML notification to all QSEs of Non-Spin deployment allows for more efficient market operation, which should result in lower energy prices to consumers and avoid inefficient dispatch of Resources. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Eric Goff on behalf of the Wholesale Market Subcommittee (WMS) |
| E-mail Address | EGoff@reliant.com |
| Company | Reliant Energy Retail Services, LLC |
| Cell Number | 512-284-4908 |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

Proposed Protocol Language Revision

6.5.7.6.2.3	Non-Spinning Reserve Service Deployment

(1)	ERCOT shall deploy Non-Spin Service by operator Dispatch Instruction for On-Line Generation Resources with Energy Offer Curves, Resources with Output Schedules, Off-Line Generation Resources and Load Resources.  ERCOT shall develop a procedure approved by TAC to deploy Resources providing Non-Spin Service.  ERCOT Operators shall implement the deployment procedure when a specified threshold(s) in MW of capability available to SCED to increase generation is reached.  ERCOT Operators may implement the deployment procedure to recover deployed RRS or when other Emergency Conditions exist.  The deployment of Non-Spin must always be 100% of that scheduled on an individual Resource.

(2)	Once Non-Spin capacity from Off-Line Generation Resources providing Non-Spin is deployed and the Generation Resources are On-Line, ERCOT shall use SCED to determine the amount of energy to be dispatched from those Resources.

(3)	Off-Line Generation Resources providing Non-Spin (OFFNS Resource Status) are required to provide an Energy Offer Curve for use by SCED.

(4)	On receipt of a deployment instruction, Load Resources providing Non-Spin must, at a minimum, provide the requested deployment energy within 30 minutes of the Dispatch Instruction.  Within 20 minutes after receipt of the deployment instruction, the QSE shall reduce the Non-Spin Ancillary Service Schedule by the amount of the deployment to make the capacity available for SCED to dispatch.

(5)	For On-Line Generation Resources with Energy Offer Curves, on deployment of Non-Spin capacity, the QSE shall reduce the Non-Spin Ancillary Service Schedule by the amount of the deployment.  As described in Section 6.5.7.2, Resource Limit Calculator, ERCOT shall adjust the HASL and LASL based on the QSE’s telemetered Non-Spin Ancillary Service Schedule to account for such deployment.  These Resources should be able to dispatch to their Non-Spin Resource Responsibility within 30 minutes of the deployment instruction.

(6)	Off-Line Generation Resources providing Non-Spin, while Off-Line and before the receipt of any deployment instruction, shall be capable of being dispatched to their Non-Spin Resource Responsibility within 30 minutes of a deployment instruction.  Upon receipt of a deployment instruction, an Off-Line Generation Resource providing Non-Spin must be On-Line and Released to SCED within 25 minutes after receipt of the deployment instruction and be capable of being dispatched by SCED to subsequent Base Points using the Resource’s Normal Ramp Rate curve.  A Generation Resource has been released to SCED if it has telemetered one of the Resource Status codes listed in paragraph (4)(b)(i) of Section 3.9.1, Current Operating Plan (COP) Criteria, excluding ONTEST, and has a net generation output greater than or equal to P1 multiplied by LSL where P1 is defined in the “ERCOT and QSE Operations Business Practices During the Operating Hour.”

(7)	For DSRs providing Non-Spin, on deployment of Non-Spin, the DSR’s QSE shall adjust its Resource Output Schedule to reflect the amount of deployment.  For non-DSRs with Output Schedules providing Non-Spin, on deployment of Non-Spin, ERCOT shall adjust the Resource Output Schedule for the remainder of the Operating Period to reflect the amount of deployment.  ERCOT shall notify the QSEs representing the non-DSR of the adjustment through the MIS Certified Area.

(8)	For On-Line Generation Resources with Energy Offer Curves, Base Points include Non-Spin energy as well as any other energy dispatched as a result of SCED.

(9)	Each QSE providing Non-Spin from a Resource shall inform ERCOT of the Non-Spin Resource availability using the Resource Status and Non-Spin Ancillary Service Resource Responsibility indications for the Operating Hour using telemetry and shall use the COP to inform ERCOT of Non-Spin Resource Status and Non-Spin Ancillary Service Resource Responsibility for hours in the Adjustment Period through the end of the Operating Day.

(10)	ERCOT may deploy Non-Spin at any time in a Settlement Interval.

(11)	ERCOT’s Non-Spin deployment Dispatch Instructions must include:

(a)	The Resource name;

(b)	A MW level of capacity deployment for Generation Resources with Energy Offer Curve, a MW level of energy for Generation Resources with Output Schedules, and interrupted amount for Load Resources; and

(c)	The anticipated duration of deployment.

(12)	ERCOT shall provide a signal via ICCP to the QSE of a deployed Generation or Load Resource indicating that its Non-Spin capacity has been deployed.

(13)	ERCOT shall, as part of its TAC-approved Non-Spin deployment procedure, provide for the recall of Non-Spin energy including descriptions of changes to Output Schedules and release of energy obligations from On-Line Resources with Output Schedules and from On-Line Resources that were previously Off-Line Resources providing Non-Spin capacity.

(14)	Non-Spin procured from a Load Resource block offer must be deployed as a block.

(15)	ERCOT shall provide a notification to all QSEs via the MIS Public Area when any Non-Spin capacity is deployed on the ERCOT System showing the time, MW quantity and duration of the deployment.

8.1.1.4.3	Non-Spinning Reserve Service Energy Deployment Criteria

(1)	ERCOT shall, as part of its Ancillary Service deployment procedure under Section 6.5.7.6.2.3, Non-Spinning Reserve Service Deployment, include all performance metrics for a Resource receiving a Non-Spin recall instruction from ERCOT.

(2)	A Non-Spin Dispatch Instruction from ERCOT must respect the minimum runtime of a Generation Resource.  After the recall of a Non-Spin Dispatch Instruction, any Generation Resource previously Off-Line providing Non-Spin is allowed to remain On-Line for 30 minutes following the recall.  During that time period, the On-Line Generation Resource is treated as if the Non-Spin is being provided.

(3)	Control performance during periods in which ERCOT has deployed Non-Spin shall be based on the requirements below and failure to meet any one of these requirements for the greater of one or 5% of Non-Spin deployments during a month shall be reported to TRE as non-compliance:

(a)	Within 20 minutes following a deployment instruction, the QSE must update the telemetered Ancillary Service Schedule for Non-Spin for Generation Resources and Load Resources to reflect the deployment amount.

(b)	Off-Line Generation Resources, within 25 minutes following a deployment instruction,  must be On-Line and Released to SCED for Dispatch.  A Generation Resource has been released to SCED if it has telemetered one of the Resource Status codes listed in paragraph (4)(b)(i) of Section 3.9.1, Current Operating Plan (COP) Criteria, excluding ONTEST, and has a net generation output greater than or equal to P1 multiplied by LSL where P1 is defined in the “ERCOT and QSE Operations Business Practices During the Operating Hour.”

(c)	If an Off-Line Generation Resource experiences a Startup Loading Failure (excluding those caused by operator error), the Resource may be considered for exclusion from performance non-compliance if the QSE provides to ERCOT the following documentation regarding the incident:

(i)	Its generation log documenting the Startup Loading Failure; and

(ii)	Equipment failure documentation such as, but not limited to, GADS reports, plant operator logs, work orders, or other applicable information.

(d)	For Load Resources, excluding Controllable Load Resources, 30 minutes following a deployment instruction, the Load Resource response must not be less than 95%, nor more than 150% of the requested MW deployment and be maintained until recalled or the Resource obligation to provide Non-Spin expires.

(e)	A Load Resource providing Non-Spin excluding Controllable Load Resources must return to at least 95% of its committed obligation for Non-Spin no more than three hours following a recall instruction from ERCOT.  Each Load Resource that is not a Controllable Load Resource and unable to return within three hours to its committed obligation for Non-Spin or pre-deployment capability that was specified in the COP at the time of the deployment, may be replaced by the QSE providing Non-Spin on other Generation Resources or other Load Resources not previously committed.

(f)	During periods when the MW Load level of a Load Resource (excluding Controllable Load Resources) has been affected by a Dispatch Instruction from ERCOT, the performance of a Load Resource in response to a Dispatch Instruction will be determined by subtracting the Load Resource’s actual Load response from its Baseline.  “Baseline” capacity is calculated by measuring the average of the real power consumption for five minutes before the Dispatch Instruction if the Load level of the Resource had not been affected by a Dispatch Instruction from ERCOT.  The actual Load response is the average of the real power consumption data being telemetered to ERCOT during the Settlement Interval indicated in the Dispatch Instruction.