ERCOT Planning Guide

Section 8:  Planning Reserve Margin

May 1, 2011

8	PLANNING RESERVE MARGIN	8-1

8.1	ERCOT Planning Reserve Margin	8-1

8.2	Minimum ERCOT Planning Reserve Margin Criterion	8-1

8.3	ERCOT Planning Reserve Margin Calculation Methodology	8-1

8.3.1	Peak Load Estimate	8-1

8.3.2	Total Capacity Estimate	8-2

# planning RESERVE MARGIN

8.1	ERCOT Planning Reserve Margin

ERCOT shall calculate the Planning Reserve Margin (PRM) for each Peak Load Season (summer months are June, July August and September; winter months are December, January and February) using the following equation:

PRMs ,i	=	(TOTCAPs, i – FIRMPKLDs, ii) / FIRMPKLDs, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PRM s, i | % | Planning Reserve Margin—The Planning Reserve Margin for the peak Load season s for year i. |
| TOTCAP s, i | MW | Total Capacity—Total Capacity available during the peak Load season s for the year i. |
| FIRMPKLD s, i | MW | Firm Peak Load—Firm Peak Load for the peak Load season s for the year i. |
| i | None | Year. |
| s | None | Peak Load Season as defined above. |

8.2	Minimum ERCOT Planning Reserve Margin Criterion

The minimum ERCOT Planning Reserve Margin (PRM) criterion is 13.75%.

8.3	ERCOT Planning Reserve Margin Calculation Methodology

ERCOT shall prepare and publish on the ERCOT website, at least annually, a report containing an estimate of the Planning Reserve Margin (PRM) for the current summer and winter Peak Load Seasons as well as a minimum of ten future summer and winter peak Load periods.  The format and content of this report shall be developed by ERCOT, subject to Technical Advisory Committee (TAC) approval.  The estimate of the PRM shall be based on the methodology in Section 8.3.1, Peak Load Estimate, and Section 8.3.2, Total Capacity Estimate.

8.3.1	Peak Load Estimate

ERCOT shall prepare, at least annually, an estimate of the total peak Load for both summer and winter peak Load periods for the current year and a minimum of ten future years using an econometric forecast, taking into account econometric inputs, weather conditions, demographic data and other variables as deemed appropriate by ERCOT.  The firm peak Load forecast shall be determined by the following equation:

FIRMPKLDs ,i	=	TOTPKLDs, i – LRRRSs, i – LRNSRSs, i – EILSs, i – CLR s, i – ENERGYEFF s, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FIRMPKLDs, i | MW | Firm Peak Load Estimate—The Firm Peak Load Estimate for the Peak Load Season s for the year i. |
| TOTPKLDs, i | MW | Total Peak Load Estimate—The Total Peak Load Estimate for the Peak Load Season s for the year i. |
| LRRRSs, i | MW | Load Resource providing Responsive Reserve (RRS)—The amount of RRS a Load Resource is providing for the Peak Load Season s for the year i. |
| LRNSRSs, i | MW | Load Resource providing Non-Spinning Reserve (Non-Spin)— The amount of Non-Spin a Load Resource is providing for the Peak Load Season s for the year i. |
| EILSs, i | MW | Emergency Interruptible Load Service (EILS)— The amount of EILS for the peak Load Season s for the year i based on: (a) For the winter Peak Load Season of the current year, the amount of EILS procured by ERCOT for the October to January EILS Contract Period using the simple average of two time period procurements (Business Hours and Non-Business Hours); (b) For the summer Peak Load Season of the current year, the amount of EILS procured by ERCOT for the May procurement (Business Hours); and (c) For all subsequent years and Peak Load Seasons, escalate the amount of EILS from the previous Season by 10%. |
| CLRs, i | MW | Amount of Controllable Load Resource—Amount of Controllable Load Resource that is available for Dispatch by ERCOT during the current year i for the Peak Load Season s not already included in LRRRS or LRNSRS. |
| ENERGYEFFs, i | MW | Amount of Energy Efficiency Programs Procured—Amount of energy efficiency programs procured by Transmission and/or Distribution Service Providers (TDSPs) pursuant to P.U.C. Subst. R. 25.181, Energy Efficiency Goal, for the peak Load Season s for the year i. |
| i | None | Year. |
| s | None | Peak Load Season (summer or winter as defined above). |

8.3.2	Total Capacity Estimate

The total capacity estimate shall be determined based on the following equation:

TOTCAPs ,i	=	INSTCAPs, i + PUNCAP s, i + WINDCAP s, i  + RMRCAPs, i + DCTIECAP s, i + SWITCHCAP s, i + MOTHCAP s, i + PLANNON s, i + PLANWIND s, i – UNSWITCH s, i – RETCAP s, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| INSTCAPs, i | MW | Seasonal Net Max Sustainable Rating—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource asset registration process for each operating Generation Resource for the year i excluding Wind-powered Generation Resources (WGRs), Resources operating under Reliability Must-Run (RMR) Agreements, and Generation Resources capable of “switching” from ERCOT Region to another power region. |
| PUNCAPs, i | MW | Private Use Network Capacity—The Private Use Network capacities as provided to ERCOT pursuant to Protocol Section 3.10.7.3, Modeling of Private Use Networks. |
| WINDCAPs, i | MW | Effective Load Carrying Capability of WGRs—The effective Load carrying capability of all existing WGRs as determined by ERCOT for the Peak Load Season s for the year i. |
| RMRCAPs, i | MW | Seasonal Net Max Sustainable Rating for Generation Resource providing RMR Service—The seasonal net max sustainable rating for the peak Load Season s as reported in the approved Resource asset registration process for each Generation Resource providing RMR Service for the year i until the approved exit strategy for the RMR Resource is expected to be completed. |
| DCTIECAPs, i | MW | Seasonal Net Max Sustainable Rating for Direct Current Tie (DC Tie) Resource—The seasonal net max sustainable rating for the peak Load Season s as reported in the approved Resource asset registration process for each DC Tie Resource for the year i multiplied by 50%. |
| SWITCHCAPs, i | MW | Seasonal Net Max Sustainable Rating for Switching Generation Resource—The seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource asset registration process for each Generation Resource for the year i that can electrically connect (i.e., “switch”) from the ERCOT Region to another power region. |
| MOTHCAPs, i | MW | Seasonal Net Max Sustainable Rating for Mothballed Generation Resource—The seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource asset registration process for each Mothballed Generation Resource for the year i based on the lead time and probability information furnished by the owners of Mothballed Generation Resources pursuant to Protocol Section 3.14.1.9, Generation Resource Return to Service Updates. |
| PLANNONs, i | MW | New, non-Wind Generating Capacity—The amount of new, non-wind generating capacity for the peak Load Season s and year i that: (a) has a Texas Commission on Environmental Quality (TCEQ)-approved air permit, and (b) has a signed Standard Generation Interconnect Agreement (SGIA), or a public, financially-binding agreement between the Resource owner and Transmission Service Provider (TSP) under which generation interconnection facilities would be constructed; or for a Municipally Owned Utility (MOU) or Electric Cooperative (EC), a public commitment letter to construct a new Resource. |
| PLANWINDs, i | MW | Effective Load Carrying Capability of New Intermittent Renewable Resource (IRR) Capacity—The effective Load carrying capability of new IRR capacity as determined by ERCOT for the Peak Load Season s and year i that has an SGIA or other public, financially-binding agreement between the Resource owner and TSP under which generation interconnection facilities would be constructed or, for a MOU or EC, a public commitment letter to construct a new IRR. |
| UNSWITCHs, i | MW | Capacity of Unavailable Switchable Generation Resource—The amount of capacity reported by the owners of a switchable Generation Resource that will be unavailable to ERCOT during the Peak Load Season s and year i pursuant to paragraph (2) of Protocol Section 16.5.4, Maintaining and Updating Resource Entity Information. |
| RETCAPs, i | MW | Capacity Pending Retirement—The amount of capacity in Season s of year i that is pending retirement based on information that has been submitted on a Notification of Suspension of Operations form pursuant to Protocol Section 3.14.1.11, Budgeting Eligible Costs, but is under review by ERCOT pursuant to Protocol Section 3.14.1.2, ERCOT Evaluation, that has not otherwise been considered in any of the above defined categories. |
| i | None | Year. |
| s | None | Peak Load Season (summer or winter as defined in Section 8.1, ERCOT Planning Reserve Margin). |