| NPRR Number | 357 | NPRR Title | Multi-Month CRR Auction and Revisions to Collateral Requirements |
| --- | --- | --- | --- |
| Date |  | June 13, 2011 |  |
| Submitter’s Information |  |  |  |
| Name |  | Randa Stephenson |  |
| E-mail Address |  | Randa.stephenson@luminant.com |  |
| Company |  | Luminant Energy Company LLC |  |
| Phone Number |  | 512-349-6491 |  |
| Cell Number |  | 214-498-6661 |  |
| Market Segment |  | Investor Owned Utility (IOU) |  |

Comments

Luminant Energy Company LLC (Luminant) appreciates the comments it received on this NPRR at the June 9, 2011 CWG/MCWG joint meeting.  The language changes shown below attempt to address each of the comments received.

1.	Backup Plan

Luminant believes that a move to a rolling monthly Congestion Revenue Right (CRR) Auction in late 2011 that begins with the month of January 2012 would be beneficial to the market and preferable to having an annual auction in late 2011 and delaying the rolling monthly auction until the end of the first quarter of 2012.  The changes proposed in these comments have been written to assume that a rolling CRR Auction that begins with the month of January is achievable and can be implemented.  However, ERCOT has indicated their concern with being able to commit to a rolling CRR Auction that begins with January due to high workload expected in the fall of this year.  If this NPRR (as revised by these comments) proves not to be implementable as it is written, then it is important that a “backup” plan be developed that would allow Market Participants to otherwise obtain CRRs at auction for a period of at least the months of 2012.   The changes shown in Section 7.5.2, CRR Auction Offers and Bids, and Section 9.8, CRR Auction Award Invoices, also relate to a CRR Auction fee that may be useful for ERCOT to obtain the resources it needs to successfully implement the vision of this NPRR and overcome its workload issues.

2.	Payment for Pre-Assigned Congestion Revenue Rights (PCRRs)

One commenter suggested that some attention needed to be given to the process by which Non-Opt-In Entities (NOIEs) pay for PCRRs.  Given that a rolling monthly CRR Auction will establish multiple prices for each instrument, a methodology for determining the price of PCRRs must be developed.  Luminant has not developed proposed language to address this matter but encourages NOIEs to propose a solution for how PCRRs may be priced.

There were some follow-up issues from a recent joint MCWG/CWG meeting

- CRR Auction Pre-screening: Are the adders path specific? 
This is addressed with the inclusion of subscripts j, k in the formula in Section 7.5.5.3, Auction Process.

- Clarify the timing of settlement for the future sale of CRRs.
This is addressed in the language in Section 7.5.6.1, Payment of an Awarded CRR Auction Offer.

- When does the a CRR account holder become the legal owner of that CRR?
This issue, and how it relates to the bilateral sell of CRR, is being considered by legal groups and will addressed in the future.

Revised Proposed Protocol Language

7.5	CRR Auctions

7.5.1	Nature and Timing

(1)	The Congestion Revenue Right (CRR) Auction auctions the available network capacity of the ERCOT transmission system not allocated as described in Section 7.4, Allocation of Preassigned Congestion Revenue Rights, or sold in a previous auction.  The CRR Auction also allows CRR Owners an opportunity to offer for sale CRRs that they hold.  Each annual and monthly CRR Auction allows for the purchase of CRR products as described in paragraph (6) of Section 7.3, Types of Congestion Revenue Rights to Be Auctioned, in one-month strips and allows for the reconfiguration of all CRR blocks that were previously awarded.

(2)	The CRR Network Model must be based on, but is not the same as, the Network Operations Model.  The CRR Network Model must, to the extent practicable, include the same topology, contingencies, and operating procedures as used in the Network Operations Model as reasonably expected to be in place for each month.  The expected network topology used in the CRR Network Model for any month must include all Outages from the Outage Scheduler and identified by ERCOT as expected to have a significant impact upon transfer capability during the month.  These Outages included in the CRR Network Model shall be posted on the Market Information System (MIS) Secure Area consistent with model posting requirements by ERCOT with accompanying cause and duration information, as indicated in the Outage Scheduler.  Transmission system upgrades and changes must be accounted for in the CRR Network Model for CRR Auctions held after the month in which the element is placed into service, which avoids the overselling of grid capacity that could potentially result from schedule delays associated with transmission system upgrades.  As referenced in Section 3.3, Economic-Driven Project Evaluation, of the Technical Advisory Committee (TAC)-approved ERCOT Regional Planning Group Charter and Procedures, clearing prices of CRRs may be considered a visible ERCOT market indicator for the purposes of determining the consumer benefit associated with transmission upgrades.  The combination of the progressive nature of selling the transmission capacity in the monthly 12 month CRR Auctions as described in paragraph (7) below and Section 3.3 of the ERCOT Regional Planning Group Charter and Procedures, creates the opportunity to identify and in some cases, find quickly implementable transmission upgrades (e.g. terminal equipment upgrade, Remedial Action Plans (RAPs), etc.) that can be implemented before the remaining transmission capacity has been auctioned off in subsequent auctions thus allowing the market to recognize lower congestion cost.  Additionally, paragraph (2) of Nodal Operating Guide Section 5.3, Transmission Reliability Studies, requires the provision of Transmission Facilities, RAPs, Special Protection Systems (SPSs), etc. to resolve unacceptable study results identified during the planning horizon.  It is important for the buyer of CRRs to realize that it is highly likely that subsequent transmission upgrades may occur after the CRR Auction date for all of the reasons outlined above, which may create more transmission capacity than was available at the time of the auction.

(a)	ERCOT shall use Dynamic Ratings in the CRR Network Model as required under Section 3.10.8, Dynamic Ratings.

(b)	The CRR Network Model must use the peak Load conditions of the month being modeled.

(c)	ERCOT’s criteria for determining if an Outage should be in the CRR Network Model shall be in accordance with these Protocols and described in the ERCOT Operating Guides.

(3)	ERCOT shall model bids and offers into the CRR Auction as flows based on the MW offer and defined source and sink.  When the Simultaneous Feasibility Test (SFT) is run, the model must weight the Electrical Buses and Hub Buses included in a Hub or Load Zone appropriately to determine the system impacts of the CRRs.

(a)	To distribute injections and withdrawals to buses within a Hub, ERCOT shall use distribution factors specified in Section 3.5.2, Hub Definitions.

(b)	To distribute injections and withdrawals to Electrical Buses in Load Zones, ERCOT shall use the Load-weighted distribution factors for On-Peak Hours in each Load Zone.  For monthly CRR Auctions, ERCOT shall derive CRR Auction Load distribution factors with the set of Load distribution factors constructed in accordance with the ERCOT Load distribution factor methodology specified in paragraph (5) of Section 4.5.1, DAM Clearing Process, for use in the Day-Ahead Market (DAM).  For Annual CRR Auctions, ERCOT shall derive CRR Auction Load distribution factors from the corresponding planning model or with the set of Load distribution factors constructed in accordance with the ERCOT Load distribution factor methodology specified in paragraph (5) of Section 4.5.1, for use in the DAM.  ERCOT shall notify the market as to which method was used for the annual CRR Auction model in the corresponding auction notice.  ERCOT shall post the CRR Auction Load distribution factors as part of the CRR Network Model pre-auction posting.

(4)	ERCOT shall conduct CRR Auctions with the frequency, on the dates, and for the terms specified as follows:

(a)	Point-to-Point (PTP) Options, PTP Obligations, and Flowgate Rights (FGRs) in monthly auctions for one-month terms beginning with the month prior to the Texas Nodal Market Implementation Date.

(b)	ERCOT shall conduct a monthly CRR Auction during the month preceding the month during which the CRRs being auctioned are effective.  ERCOT shall publish a calendar of relevant auction dates each year for the following year’s activities.

(c)	No later than December 2011, ERCOT shall conduct each month a multi-month auction for PTP Options, PTP Obligations, and FGRs for one-month strips for each of the succeeding 12 forward months.

(d)	Upon approval by the ERCOT Board, ERCOT may conduct once a year, beginning after October 1, 2012, a second-year CRR Auction offering one-month strips for each of the calendar months of the second year and an annual strip for that second year.

(5)	ERCOT shall auction the following products:

(a)	In each monthly 12 month CRR Auction:

(i)	PTP Options in one-month strips for one-month terms;

(ii)	PTP Obligations in one-month strips for one-month terms; and

(iii)	FGRs (on any defined flowgates) in one-month strips for one-month terms.

(6)	ERCOT shall offer network capacity in the one-time annual CRR Auction equal to

the expected network topology for that month in the CRR Network Model scaled down to 20%, including PCRRs scaled to the same percentage.

(7)	ERCOT shall offer network capacity for each monthly 12 month CRR Auction equal to the difference between:

(a)	The expected transmission network topology in the CRR Network Model of the month for which the CRRs are effective, scaled down to the following percentages for each of the succeeding 12 months; and

| Month | Scaled Capacity | Month | Scaled Capacity |
| --- | --- | --- | --- |
| Prompt | 90.00% | 7 | 52.50% |
| 2 | 83.75% | 8 | 46.25% |
| 3 | 77.50% | 9 | 40.00% |
| 4 | 71.25% | 10 | 33.75% |
| 5 | 65.00% | 11 | 27.50% |
| 6 | 58.75% | 12 | 21.25% |

(b)	All  PCRRs that were previously awarded or allocated for the month scaled to the same percentage.

7.5.2	CRR Auction Offers and Bids

(1)	To submit bids or offers into a CRR Auction, an Entity must become a CRR Account Holder and satisfy financial assurance criteria required to participate, under Section 16.8, Registration and Qualification of Congestion Revenue Rights Account Holders.

(2)	In order to enforce a volume limitation on the number of market transactions (bids and offers) submitted into the CRR Auction, ERCOT shall evaluate the maximum number transactions which are available prior to the auction, and evenly divide the limit across the CRR Account Holders.  This limit shall be designated as the allocated CRR transaction limit.  The allocated CRR transaction limitation for all CRR Account Holders will be communicated as part of the CRR Auction Notice prior to each auction.

(a)	No CRR Account Holder shall submit more than 10,000 transactions in any CRR Auction.

(b)	If the total number of transactions submitted by all Market Participants into the CRR Auction does not exceed the maximum number of transactions available prior to the auction, then the allocated CRR transaction limit will not apply and all transactions will be accepted.

(c)	If the total number of transactions submitted by CRR Account Holders exceeds the maximum number of transactions available prior to the auction, ERCOT shall notify all CRR Account Holders within one hour of the close of each CRR Auction that the maximum number of transactions has been exceeded.  Each CRR Account Holder shall then adjust their transactions to be less than or equal to the allocated CRR transaction limitation for the affected CRR Auction within one Business Day.  If the Market Participant fails to reduce its transactions to the allocated CRR transaction limitation within one Business Day, ERCOT shall reject all transactions submitted by that CRR Account Holder into the affected CRR Auction.  ERCOT will then execute the CRR Auction using the updated set of transactions as revised by Market Participants.

(d) 	Each Counter-Party is limited to a total of three CRR Account Holders.

(e)	ERCOT shall charge a fee on each PTP Option bid awarded in each CRR Auction as described in Section 7.7, Point-to-Point (PTP) Option Award Fee.

(3)        ERCOT shall charge a fee per CRR bid/offer in each CRR Auction, as described in the ERCOT Board approved document “ ERCOT User Fee for CRR Auctions”.

7.5.5.3	Auction Process

(1)     	Prior to the actual Auction Process, ERCOT will conduct a pre-Auction screening process to determine the maximum potential exposure of each CRR Account Holder.  The calculated exposure for each CRR Account Holder is:

CEOBLBIDo=ΣdΣtΣ(j,k) [[ Max[ 0 , Min[ Pbid, MCEP(j,k),c,t,d,z ]]  + A(j,k),t,d] * QOBLB(j,k),o,t,d ] + [ FAOBL(j,k),t,d * QFDOBLB(j,k),o,t,d ]

CEOPTBIDo = ΣdΣtΣ(j,k)[Max[ 0 , Min[ Pbid, MCEP(j,k),c,t,d,z ]] * QOPTB(j,k),o,t,d] + [ FAOPT(j,k),t,d * QFDOPTB(j,k),o,t,d ]

CEOBLOFFERo=ΣdΣtΣ(j,k)[-1 * Min[ 0 , Min[ Poffer, MCEP(j,k),c,t,d,z ]] * QOBLO(j,k),o,t] + [-1*FAOBL(j,k),t,d * QFDOBLO(j,k),o,t,d ]

CEOPTOFFERo=ΣdΣtΣ(j,k)[-1 * Min[ 0 , Min[ Poffer, MCEP(j,k),c,t,d,z ]] * QOPTO(j,k),o,t]+ [-1*FAOPT(j,k),t,d * QFDOPTO(j,k),o,t,d ]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| CEc,a | $ | Credit Exposure for a Counter-Party for an auction — The calculated credit exposure for all CRRs awarded to a Counter-Party. |
| CEOBLBIDo,a | $ | Credit Exposure for PTP Obligations Bid for a CRR Account Holder for an auction |
| CEOPTBIDo,a | $ | Credit Exposure for PTP Options Bid for a CRR Account Holder for an auction |
| CEOBLOFFERo,a | $ | Credit Exposure for PTP Obligations Offered for a CRR Account Holder for an auction |
| CEOPTOFFERo,a | $ | Credit Exposure for PTP Options Offered for a CRR Account Holder for an auction |
| MCEP(j,k),c,t,d,z | $/MW | Maximum Credit Exposure Price for Hedge Type for a Counter-Party for a time of use block for a delivery month per a source and sink pair – For each Counter-Party ERCOT shall calculate per the unique combination of source, sink, Time of Use (TOU), delivery month, PTP type, a single price that results in the maximum product of the quantity submitted multiplied by the submitted price |
| QOBLB (j,k),o,t,d | MWh | Quantity for PTP Obligations Bid for a CRR Account Holder for a Time Of Use block, for the prompt delivery month per a source and sink pair |
| QOPTB(j,k),o,t,d | MWh | Quantity for PTP Options Bid for a CRR Account Holder for a Time Of Use block, for the prompt delivery month per a source and sink pair |
| QFDOBLB(j,k),o,t,d | MWh | Quantity for forward delivery months for PTP Obligations Bid for a CRR Account Holder for a Time Of Use block, for delivery months exceeding the prompt month per a source and sink pair |
| QFDOPTB(j,k),o,t,d | MWh | Quantity for forward delivery months for PTP Options Bid for a CRR Account Holder for a Time Of Use block, for delivery months exceeding the prompt month per a source and sink pair |
| QOBLO(j,k),o,t,d | MWh | Quantity for PTP Obligations Offered for a CRR Account Holder for a Time Of Use block, for a delivery month per a source and sink pair |
| QOPTO(j,k),o,t,d | MWh | Quantity for PTP Options Offered for a CRR Account Holder for a Time Of Use block, for a delivery month per a source and sink pair |
| A(j,k),t,d | $/MW | Adder per source and sink pair for a time of use block for the prompt delivery month |
| FAOBL(j,k),t,d | $/MW | Adder for Obligation per source and sink pair for a time of use block per delivery month for all delivery months greater than the prompt month |
| FAOPT(j,k),t,d | $/MW | Adder for Option per source and sink pair for a time of use block per delivery month for all delivery months greater than the prompt month |
| a | None | A CRR Auction |
| d | None | Delivery Month |
| j | None | A source settlement point. |
| k | None | A sink settlement point. |
| t | none | TOU block |
| o | none | A CRR Account Holder |
| c | none | Counter-Party |
| z | none | CRR Type (i.e. Obligation or Option) |

(a)	The values of A(j,k),t,d and FA(j,k),t,d shall be posted on the MIS Public Area.  TAC shall review these values at least annually and may recommend to the ERCOT Board, changes to these values that become effective at least 30 days prior to a monthly CRR Auction and 60 days prior to an annual CRR Auction.  Any changes to these values shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval.

(b)	The values of A(j,k),t,d and FA(j,k),t,d will be defined and modified using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”).

(2)	The auction must be a single-round, simultaneous auction for selling the CRRs available for all auction products, with the following steps:

(a)	If the sum of the calculated Credit Exposure for a CRR Account Holder is greater than the Account Holder’s collateralized quantity prior to the Auction, ERCOT shall enter into the CRR Auction engine model a credit constraint for each CRR Account Holder.  A CRR Account Holder’s CRR Auction credit limit is equal to the lesser of the credit limit as determined in Section 16.11.4.6.1, Credit Requirements for CRR Auction Participation, or, if provided, the Counter-Party’s self-imposed CRR Auction credit limit.  The credit constraint for each Counter-Party ensures that the following calculated exposure for all of the Counter-Party’s CRR Account Holders is less than or equal to the Counter-Party’s CRR Auction credit limit:

(i)	All awarded CRR Auction Bids multiplied by the absolute value of the corresponding bid price; plus

(ii)	All awarded CRR Auction Offers with negative offer prices multiplied by the absolute value of their corresponding offer price; plus

Σo

ΣdΣtΣ(j,k)

ΣdΣtΣ(j,k)

ΣdΣtΣ(j,k)

ΣdΣtΣ(j,k)

(iii)	The additional credit requirement for all awarded PTP Obligations, which is $A per MW per hour, plus the absolute value of the PTP Obligation bid.  The values of A(j,k),t,d and FA(j,k),t,d shall be posted on the MIS Public Area.  TAC shall review these values at least annually and may recommend to the ERCOT Board, changes to these values that become effective at least 30 days prior to a monthly CRR Auction and 60 days prior to an annual CRR Auction.  Any changes to these values shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval.

(iv)	The values of A(j,k),t,d and FA(j,k),t,d will be defined and modified using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”).

(b)	ERCOT shall award CRRs in quantities truncated to the nearest tenth MW (0.1 MW).

(c)	The CRR clearing price is equal to the corresponding Shadow Price for that CRR product.

(d)	When a CRR Account Holder is awarded CRRs as a result of a CRR Auction, the CRRs do not become the property of the winning CRR Account Holder, and the CRRs may not be placed in their CRR accounts, until the CRR Invoices have been paid in full.

(e)	When a CRR Account Holder sells PTP Obligations as a result of an auction at a negative price, the CRR Account Holder is not relieved of the PTP Obligations until the CRR Invoices have been paid in full.

(3)	ERCOT shall use a linear programming auction engine model for each CRR Auction that evaluates all CRR Auction Bids and CRR Auction Offers submitted, and selects a combination of CRR Auction Bids and CRR Auction Offers that:

(a)	Makes the solution simultaneously feasible within the limits of the ERCOT network capability over the auction term; and

(b)	Maximizes the objective function, which is equal to the total economic value (as expressed in the CRR Auction Bids) of the awarded CRR Auction Bids, less the total economic cost (as expressed in CRR Auction Offers) of the awarded CRR Auction Offers, while observing all applicable constraints.

(4)	The CRR Network Model must, to the extent practicable, reflect the continuous and post-contingency system operating limits and operational procedures (i.e., Special Protection Systems and Remedial Action Plans) in the Network Operations Model used by ERCOT during Real-Time Operations, as discussed below in Section 7.5.5.4, Simultaneous Feasibility Test.

(5)	Once a CRR Auction is complete, ERCOT shall archive and keep the CRR Auction system and all models used to finalize the CRR Auction results under ERCOT’s data retention policy as that policy applies to data that may be needed to resolve requests for billing adjustments under applicable billing adjustment procedures.

7.5.6	CRR Auction Settlements

7.5.6.1	Payment of an Awarded CRR Auction Offer

(1)	Following each CRR Auction, ERCOT shall pay each CRR Account Holder its PTP Obligation offers previously awarded for the prompt month only.  Payments related to PTP Obligation offers previously awarded for months exceeding the following prompt month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion of the monthly aAction that is associated with the delivery month related to the PTP Obligation offers awarded. Within each monthly CRR Auction net Invoice, the payment for each source and sink pair for a given Operating Hour is calculated as follows:

OBLSAMT crrh, (j, k), a	=	(-1) * OBLPR (j, k), a * OBLS crrh, (j, k), a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| OBLSAMT crrh, (j, k), a | $ | PTP Obligation Sale Amount per CRR Account Holder per source and sink pair per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation offers with the source j and the sink k awarded in CRR Auction a, for the hour. |
| OBLPR (j, k), a | $/MW per hour | PTP Obligation Price per source and sink pair per CRR Auction—The clearing price of a PTP Obligation with the source j and the sink k in CRR Auction a, for the hour. |
| OBLS crrh, (j, k), a | MW | PTP Obligation Sale per CRR Account Holder per source and sink pair per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Obligation offers associated with the source j and the sink k awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |

(2)	Following each CRR Auction, ERCOT shall pay each CRR Account Holder  its PTP Option offers previously awarded for the following prompt month only.  Payments related to PTP Option offers previously awarded for months exceeding the following prompt month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion of the monthly auction that is associated with the delivery month related to the PTP Option offers awarded. Within each monthly CRR Auction net Invoice, the payment for each source and sink pair for a given Operating Hour is calculated as follows:

OPTSAMT crrh, (j, k), a	=	(-1) * OPTPR (j, k), a * OPTS crrh, (j, k), a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| OPTSAMT crrh, (j, k), a | $ | PTP Option Sale Amount per CRR Account Holder per source and sink pair per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids with the source j and the sink k awarded in CRR Auction a, for the hour. |
| OPTPR (j, k), a | $/MW per hour | PTP Option Price per source and sink pair per CRR Auction—The clearing price of a PTP Option with the source j and the sink k in CRR Auction a, for the hour. |
| OPTS crrh, (j, k), a | MW | PTP Option Sale per CRR Account Holder per source and sink pair per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Option offers with the source j and the sink k awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |

(3)	Following each CRR Auction, ERCOT shall pay each CRR Account Holder  its FGR offers previously awarded for the following prompt month only.  Payments related to FGR offers previously awarded for months exceeding the following prompt month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion of the monthly auction that is associated with the delivery month related to the FGR offers awarded. Within each monthly CRR Auction net Invoice, the payment for each flowgate for a given Operating Hour is calculated as follows:

FGRSAMT crrh, f, a	=	(-1) * FGRPR f, a * FGRS crrh, f, a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FGRSAMT crrh, f, a | $ | Flowgate Right Sale Amount per CRR Account Holder per flowgate per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR offers associated with FGR f awarded in CRR Auction a, for the hour. |
| FGRPR f, a | $/MW per hour | Flowgate Right Price per flowgate per CRR Auction—The clearing price of FGR f in CRR Auction a, for the hour. |
| FGRS crrh, f, a | MW | Flowgate Right Sale per CRR Account Holder per flowgate per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s FGR offers associated with FGR f awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| f | none | An FGR. |
| a | none | A CRR Auction. |

7.5.6.2	Charge of an Awarded CRR Auction Bid

(1)	Following each CRR Auction, ERCOT shall charge each CRR Account Holder  its PTP Obligation bids previously awarded for the following prompt month only.  Charges related to PTP Obligation bids previously awarded for months exceeding the following prompt month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion of the monthly auction that is associated with the delivery month related to the PTP Obligation bid awards. Within each monthly CRR Auction net Invoice, the charge for each source and sink pair for a given Operating Hour is calculated as follows:

OBLPAMT crrh, (j, k), a	=	OBLPR (j, k), a * OBLP crrh, (j, k), a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| OBLPAMT crrh, (j, k), a | $ | PTP Obligation Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation bids with the source j and the sink k awarded in CRR Auction a, for the hour. |
| OBLPR (j, k), a | $/MW per hour | PTP Obligation Price per source and sink pair per CRR Auction—The clearing price of a PTP Obligation with the source j and the sink k in CRR Auction a, for the hour. |
| OBLP crrh, (j, k), a | MW | PTP Obligation Purchase per CRR Account Holder per source and sink pair per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Obligation bids associated with the source j and the sink k awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |

(2)	Following each CRR Auction, ERCOT shall charge each CRR Account Holder its PTP Option bids previously awarded for the following prompt month only.  Charges related to PTP Option bids previously awarded for months exceeding the following prompt month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion of the monthly auction that is associated with the delivery month related to the PTP Option bid awards. Within each monthly CRR Auction net Invoice, the charge for each source and sink pair for a given Operating Hour is calculated as follows:

OPTPAMT crrh, (j, k), a	=	OPTPR (j, k), a * OPTP crrh, (j, k), a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| OPTPAMT crrh, (j, k), a | $ | PTP Option Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids with the source j and the sink k awarded in CRR Auction a, for the hour. |
| OPTPR (j, k), a | $/MW per hour | PTP Option Price per source and sink pair per CRR Auction—The clearing price of a PTP Option with the source j and the sink k in CRR Auction a, for the hour. |
| OPTP crrh, (j, k), a | MW | PTP Option Purchase per CRR Account Holder per source and sink pair per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Option bids associated with the source j and the sink k awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |

(3)	Following each CRR Auction, ERCOT shall charge each CRR Account Holder  its flowgate bids previously awarded for the following prompt month only.  Charges related to flowgate bids previously awarded for months exceeding the following prompt month will be subsequently invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion of the monthly auction that is associated with the delivery month related to the flowgate bid awards. Within each monthly CRR Auction net Invoice, the charge for each flowgate for a given Operating Hour is calculated as follows:

FGRPAMT crrh, f, a	=	FGRPR f, a * FGRP crrh, f, a

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FGRPAMT crrh, f, a | $ | Flowgate Right Purchase Amount per CRR Account Holder per flowgate per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR bids associated with FGR f awarded in CRR Auction a, for the hour. |
| FGRPR f, a | $/MW per hour | Flowgate Right Price per flowgate per CRR Auction—The clearing price of FGR f in CRR Auction a, for the hour. |
| FGRP crrh, f, a | MW | Flowgate Right Purchase per CRR Account Holder flowgate per CRR Auction—The MW quantity that represents the total of CRR Account Holder ccrh’s FGR bids associated with FGR f awarded in CRR Auction a, for the hour. |
| crrh | none | A CRR Account Holder. |
| f | none | An FGR. |
| a | none | A CRR Auction. |

7.5.6.3	Charge of PCRRs Pertaining to a CRR Auction

(1)	For pre-assigned PTP Obligations allocated before each CRR Auction (annual or monthly auction), following each CRR Auction ERCOT shall charge each CRR Account Holder for the following prompt month only.  Charges related to pre-assigned PTP Obligations allocated for months exceeding the following prompt month will be invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion of the monthly auction that is associated with the delivery month related to the pre-assigned PTP Obligations allocated. Within each monthly CRR Auction net Invoice,the charge for each source and sink pair for a given Operating Hour is calculated as follows:

If OBLPR (j, k), a >0

PCRROBLAMT crrh, (j, k), a, tech =	PCRROBLF tech * OBLPR (j, k), a

* PCRROBL crrh, (j, k), a, tech

Otherwise

PCRROBLAMT crrh, (j, k), a, tech =	OBLPR (j, k), a * PCRROBL crrh, (j, k), a, tech

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRROBLAMT crrh, (j, k), a, tech | $ | PCRR PTP Obligation Amount per CRR Account Holder per source and sink pair per CRR Auction by resource technology—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Obligations associated with the source j and the sink k allocated before CRR Auction a based on Resources of the technology tech, for the hour. |
| PCRROBLF tech |  | PCRR PTP Obligation pricing Factor per resource technology—The pricing factor of pre-allocated PTP Obligations based on Resources of the technology tech. See Section 7.4.2, PCRR Allocation Terms and Conditions, item (f)(ii). |
| OBLPR (j, k), a | $/MW per hour | PTP Obligation Price per source and sink pair per CRR Auction—The clearing price of a PTP Obligation with the source j and the sink k in CRR Auction a, for the hour. |
| PCRROBL crrh, (j, k), a, tech | MW | PCRR PTP Obligation per CRR Account Holder per source and sink pair per CRR Auction by resource technology—The MW quantity that represents the total of CRR Account Holder ccrh’s PTP Obligations associated with the source j and the sink k allocated before CRR Auction a based on Resources of the technology tech, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |
| tech | none | A Resource technology. See Section 7.4.2, PCRR Allocation Terms and Conditions, item (f). |

(2)	For pre-assigned PTP Options allocated before each CRR Auction (annual or monthly auction), following each CRR Auction ERCOT shall charge each CRR Account Holder for the following prompt month only.  Charges related to pre-assigned PTP Options allocated for months exceeding the following prompt month will be invoiced on a net basis within the CRR Auction Invoice that is issued on the first Business Day following the completion of the monthly auction that is associated with the delivery month related to the pre-assigned PTP Options allocated. Within each monthly CRR Auction net Invoice, the charge for each source and sink pair for a given Operating Hour is calculated as follows:

PCRROPTAMT crrh, (j, k), a, tech	=	PCRROPTF tech * OPTPR (j, k), a * 					PCRROPT crrh, (j, k), a, tech

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PCRROPTAMT crrh, (j, k), a, tech | $ | PCRR PTP Option Amount per CRR Account Holder per source and sink pair per CRR Auction by resource technology—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Options associated with the source j and the sink k allocated before CRR Auction a based on Resources of the technology tech, for the hour. |
| PCRROPTF tech |  | PCRR PTP Option pricing Factor per resource technology—The pricing factor of pre-allocated PTP Options based on Resources of the technology tech. See Section 7.4.2, PCRR Allocation Terms and Conditions, item (f) (i). |
| OPTPR (j, k), a | $/MW per hour | PTP Option Price per source and sink pair per CRR Auction—The clearing price of a PTP Option with the source j and the sink k in CRR Auction a, for the hour. |
| PCRROPT crrh, (j, k), a, tech | MW | PCRR PTP Option per CRR Account Holder per source and sink pair per CRR Auction by resource technology—The MW quantity that represents the total of CRR Account Holder crrh’s PTP Options with the source j and the sink k allocated before CRR Auction a based on Resources of the technology tech, for the hour. |
| crrh | none | A CRR Account Holder. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| a | none | A CRR Auction. |
| tech | none | A Resource technology. See Section 7.4.2, PCRR Allocation Terms and Conditions, item (f). |

9.8	CRR Auction Award Invoices

(1)	ERCOT shall prepare invoices for each Congestion Revenue Right (CRR) Auction (CRR Auction Invoice) on a net basis.  Invoices must be issued on the first Business Day following the completion of a CRR Auction on the date specified in the Settlement Calendar.  For each CRR Auction Invoice, the CRR Account Holder to whom the Invoice is addressed (“Invoice Recipient”) is either a net payee or net payor.  The Invoice Recipient is responsible for accessing the CRR Auction Invoice on the Market Information System (MIS) Certified Area once posted by ERCOT.

(2)	Each Invoice Recipient shall pay any net debit and be entitled to receive any net credit shown on the CRR Auction Invoice on the payment due date.  Payments for CRR Auction Invoices are due on the applicable payment due date, whether or not there is any Settlement and billing dispute regarding the amount of the payment.

(3)	ERCOT shall post on the MIS Certified Area for each Invoice Recipient a CRR Auction Invoice based on CRR Auction charges and payments as set forth in:

(a)	Section 7.5.6.1, Payment of an Awarded CRR Auction Offer;

(b)	Section 7.5.6.2, Charge of an Awarded CRR Auction Bid;

(c)	Section 7.5.6.3, Charge of PCRRs Pertaining to a CRR Auction; and

(d)       ERCOT Board approved document “ERCOT User Fee for CRR Auctions”, ERCOT CRR bid/offer Fee.

[NPRR320: Insert paragraph (3)(e) below upon system implementation:] (e) Section 7.7, Point-to-Point (PTP) Option Award Fee.

(4)	CRR Auction Invoices must contain the following information:

(a)	The Invoice Recipient’s name;

(b)	The ERCOT identifier (Settlement identification number issued by ERCOT);

(c)	Net Amount Due/Payable – the aggregate summary of all charges owed to or due from the Invoice Recipient summarized by CRR Auction;

(d)	Time Period – the CRR Auction for which the Invoice is generated;

(e)	Run Date – the date on which ERCOT created and published the Invoice;

(f)	Invoice Reference Number – a unique number generated by ERCOT for payment tracking purposes;

(g)	Product Description – a description of each product awarded in, sold in, or allocated before the CRR Auction, or of any applicable fee;

(h)	Payment Date – the date and time that Invoice amounts are to be paid or received; and

(i)	Remittance Information Details – details including the account number, bank name and electronic transfer instructions of the ERCOT account to which any amounts owed by the Invoice Recipient are to be paid or of the Invoice Recipient’s account from which ERCOT may draw payments due.

16.11.4.5	Determination of the Counter-Party Future Credit Exposure

(1)	ERCOT shall monitor and calculate the Counter-Party’s FCE for all CRRs held by the Counter-Party as owner of record at ERCOT, for all Operating Days for which a DAM has not executed and for CRRs that have not settled, using the formula below.

FCE o  =  FCEOBL o + FCEOPT o + FCRFGR o- Max[ 0, FDAR o ]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCE o | $ | Future Credit Exposure—Counter-Party FCE for all CRRs held by the Counter-Party as owner o of record at ERCOT, for all Operating Days that have not yet occurred and for CRRs that have not settled. |
| FCEOBL o | $ | Future Credit Exposure for PTP Obligations—Counter-Party FCE for all PTP Obligations held by the Counter-Party as owner o of record at ERCOT, for all Operating Days for which a DAM has not executed . |
| FCEOPT o | $ | Future Credit Exposure for PTP Options—Counter-Party FCE for all PTP Options held by the Counter-Party as owner o of record at ERCOT, for all Operating Days for which a DAM has not executed. |
| FCEFGR o | $ | Future Credit Exposure for FGRs—Counter-Party FCE for all Flowgate Rights (FGRs) held by the Counter-Party as owner o of record at ERCOT, for all Operating Days for which a DAM has not executed. |
| FDAR o | $ | Future Discounted Auction Revenue— ERCOT shall determine, for each forward operating month for which CRRs have been auctioned, the expected CRR monthly revenues the Counter-Party as owner o is forecasted to realize. |
| o | none | A CRR Owner |

(2)	The Counter-Party’s FCE for all PTP Obligations (FCEOBL) held by the Counter-Party as owner of record at ERCOT for

Where:

aelated to PTP Obligations that exceed the current month for which ERCOT and the Counter-Party have not settled payment for an awarded CRR, FCEOBL is calculated as follows:

FCEOBL o   = Max (ACPEOBL o, p - FMMOBL o, p) – Min (0, FMMOBLo) + INTMOBL o

Where:

ACPEOBL o,p	=	Σ(h) Σ(j,k)(ACPE h, p(j,k) * OBLMW o, h, p(j,k))

FMMOBL o,p = Σ(h)Σ(j,k)[(W1 * ACP h,p (j, k) + W2 * TOBLV h, (j, k) + W3*  FDOBLV h, (j,k) + W4 * PMOBLV h, (j, k)) * OBLMWo,p, h, (j, k)]

INTMOBL o =Σ(h) Σ(j,k)(INTM h, (j,k) * OBLMW o, h, (j,k))

FMMOBL o =Σ(h)Σ(j,k)[(ACP h, (j, k) - OBLPR (j, k), a)* OBLMWo, h, (j, k)]

Σ(h)Σ(j,k)

Σ(h)Σ(j,k)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCEOBL o | $ | Future Credit Exposure for PTP Obligations—Counter-Party FCE for all PTP Obligations held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a DAM has not executed and for CRRs that have not settled. |
| INTMOBL o | $ | Initial Margin Exposure for all PTP Obligations held by the Counter-party as owner o of record at ERCOT for all Operating Days for which a DAM has not executed and for CRRs that have not settled. |
| ACPE h, p, (j, k) | $ | Auction Clearing Price Exposure for PTP Obligations with the source j and the sink k for hour h, in the prompt month, p - Exposure level calculated as follows: - if the PTP Obligation Auction Clearing Price is greater than $Y per MW, then Y * X divided by the PTP Obligation Auction Clearing Price; - if the PTP Obligation Auction Clearing Price is between $0 and $Y per MW, then $X per MW; and - if the PTP Obligation Auction Clearing Price is negative, then $X per MW, plus the absolute value of the PTP Obligation Auction Price per MW. |
| ACPEOBL o,p | $ | Auction Clearing Price Exposure for all PTP Obligations held by the Counter-party as owner o of record at ERCOT for all Operating Days and for CRRs that have not settled, in the prompt month, p. |
| INTM h, (j, k) | $/MW per hour | Initial Margin Exposure for PTP Obligations with the source j and the sink k for hour h - Exposure level calculated using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”). The procedures will be used to define and modify INTM h, (j, k). - The procedures to determine INTM and the value of INTM h, (j, k) shall be posted on the MIS Public Area. The TAC shall review the procedures at least annually and may recommend to the ERCOT Board changes to the parameters. If changes to these parameters are approved by the ERCOT Board, such revised procedures shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval. - ERCOT shall generally notify Counter-Parties two Bank Business Days prior to implementing either a change in procedures to determine INTM or the value of INTM h, (j, k) - ERCOT may adjust any INTM h, (j, k) immediately if, in its sole discretion, ERCOT determines that either the parameters or the reported value of INTM h, (j, k) does not adequately match the financial risk created by the PTP Obligation. |
| FMMOBL o | $ | Forward Mark-to-Market for PTP Obligations—Estimate of the forward mark-to-market value of PTP Obligations held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a DAM has not executed and for CRRs that have not settled. |
| FMMOBL o, p | $ | Forward Mark-to-Market for PTP Obligations, Prompt Month—Estimate of the forward mark-to-market value of PTP Obligations held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a DAM has not executed and for CRRs that have not settled in the prompt month. |
| ACP h, (j, k) | $/MW per hour | Auction Clearing Price—The auction clearing price for a PTP Obligation within the most recent CRR Auction corresponding to the CRR with a delivery month m, with the source j and the sink k for hour h. ERCOT has the discretion to use a price from the bilateral and/or over-the-counter market for a delivery month m, with the source j and the sink k for hour h if the price originated from an executed transaction and has a transaction date in excess of the most recent CRR Auction associated with delivery month m. |
| ACP h, p (j, k) | $/MW per hour | Auction Clearing Price—The auction clearing price for a PTP Obligation within the most recent CRR Auction corresponding to the CRR with the source j and the sink k for hour h, in the prompt month, p. ERCOT has the discretion to use a price from the bilateral and/or over-the-counter market for the prompt month, p, with the source j and the sink k for hour h if the price originated from an executed transaction and has a transaction date in excess of the most recent CRR Auction associated with the prompt month, p. |
| W1 – W4 | none | Weighting—The weighting associated with the pricing components that sum to 1. The values of these factors must be determined by the Credit Work Group (Credit WG) and posted on the MIS Public Area. The weighting factors may be customizable for the month to which a CRR applies. |
| TOBLV h, (j, k) | $/MW per hour | Today’s PTP Obligation Value—The difference in current day’s most recent DAM Settlement Point Price between the sink k and the source j of the CRR for the hour h owned. If the DAM is executed but specific DAM Settlement Point Prices are not available, ERCOT may use the appropriate Auction Clearing Prices instead. If the DAM is not executed for an Operating Day, ERCOT shall use the RTM Settlement Point Prices for that Operating Day. |
| FDOBLV h, (j, k) | $/MW per hour | Five-day PTP Obligation Value—Average of the most recent rolling five-day difference in DAM Settlement Point Price between the sink k and the source j of the CRR for the hour h owned. If the DAM is executed but specific DAM Settlement Point Prices are not available, ERCOT may use the appropriate Auction Clearing Prices instead. If the DAM is not executed for an Operating Day, ERCOT shall use the RTM Settlement Point Prices for that Operating Day. |
| PMOBLV h, (j, k) | $/MW per hour | Previous Month’s PTP Obligation Value—Average of the previous month’s daily difference in DAM Settlement Point Price between the sink k and the source j of the CRR for the hour h owned. If the DAM is executed but specific DAM Settlement Point Prices are not available, ERCOT may use the appropriate Auction Clearing Prices instead. If the DAM is not executed for an Operating Day, ERCOT shall use the RTM Settlement Point Prices for that Operating Day. |
| OBLPR (j, k), a | $/MW per hour | PTP Obligation Price per source and sink pair per CRR Auction—The clearing price of the PTP Obligation with the source j and the sink k in CRR Auction a, for the hour. |
| OBLMW o, h, (j, k) | MW | PTP Obligation with the source j and the sink k for hour h owned by the Counter-Party as owner o for all Operating Days for which a DAM has not executed and for CRRs that have not settled. |
| a | none | A CRR Auction. |
| j | none | A source Settlement Point |
| k | none | A sink Settlement Point |
| h | none | An Operating Hour. |
| o | none | A CRR Owner |
| p | none | The prompt month |

(3)		The Counter-Party’s FCE for all PTP Options (FCEOPT) held by the Counter-Party as owner of record at ERCOT for

Σ(h)Σ(j,k)W

all hours related to PTP Options that exceed the current month for which ERCOT and the Counter-Party have not settled payment for an awarded CRR, FCEOPT is calculated as follows.

FCEOPT o   = - FMMOPT o, p – FMMOPT o

Where:

FMMOPT o,p = Σ(h)Σ(j,k)[(W1 * ACP h,p (j, k) + W2 * TOPTV h, (j, k) + W3*  FDOPTV h, (j,k) + W4 * PMOPTV h, (j, k)) * OPTMWo,p, h, (j, k)

FMMOPT o = Σ(h)Σ(j,k)[Max [- OPTPR (j, k), a , ( ACP h, (j, k) - OPTPR (j, k), a - ACPE h, (j,k))] * OPTMW o, h, (j, k)]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCEOPT o | $ | Future Credit Exposure for PTP Options—Counter-Party FCE for all PTP Options held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a DAM has not executed and for CRRs that have not settled. |
| FMMOPT o | $ | Forward Mark-to-Market for PTP Options—Estimate of the forward mark-to-market value of PTP Options held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a DAM has not executed and for CRRs that have not settled. |
| FMMOPT o, p | $ | Forward Mark-to-Market for PTP Options—Estimate of the forward mark-to-market value of PTP Options held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a DAM has not executed and for CRRs that have not settled, in the prompt month p. |
| INTM h, (j, k) | $/MW per hour | Initial Margin Exposure for PTP Options with the source j and the sink k for hour h - Exposure level calculated using A TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”). The procedures will be used to define and modify INTM h, (j, k). - The procedures to determine INTM and the value of INTM h, (j, k) shall be posted on the MIS Public Area. The TAC shall review the procedures at least annually and may recommend to the ERCOT Board changes to the parameters. If changes to these parameters are approved by the ERCOT Board, such revised procedures shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval. - ERCOT shall generally notify Counter-Parties two Bank Business Days prior to implementing either a change in procedures to determine INTM or the value of INTM h, (j, k) - ERCOT may adjust any ACPE h, (j, k) immediately if, in its sole discretion, ERCOT determines that either the parameters or the reported value of INTM h, (j, k) does not adequately match the financial risk created by the PTP Options. |
| ACP h, (j, k) | $/MW per hour | Auction Clearing Price— The auction clearing prices for a PTP Option within the most recent CRR Auction corresponding to the CRR with a delivery month m, with the source j and the sink k for hour h. ERCOT has the discretion to use a price from the bilateral and/or over-the-counter market for a delivery month m, with the source j and the sink k for hour h if the price originated from an executed transaction and has a transaction date in excess of the most recent CRR Auction associated with delivery month m.. |
| ACP h, p (j, k) | $/MW per hour | Auction Clearing Price—The auction clearing price for a PTP Obligation within the most recent CRR Auction corresponding to the CRR with the source j and the sink k for hour h, in the prompt month, p. ERCOT has the discretion to use a price from the bilateral and/or over-the-counter market for the prompt month, p, with the source j and the sink k for hour h if the price originated from an executed transaction and has a transaction date in excess of the most recent CRR Auction associated with the prompt month, p. |
| W1 – W4 | none | Weighting—The weighting associated with the pricing components that sum to 1. The values of these factors must be determined by the Credit WG and posted on the MIS Public Area. The weighting factors may be customizable for the month to which a CRR applies. |
| TOPTV h, (j, k) | $/MW per hour | Today’s PTP Option Value—The greater of zero or the difference in current day’s most recent DAM Settlement Point Price between the sink k and the source j of the CRR for the hour h owned. If the DAM is executed but specific DAM Settlement Point Prices are not available, ERCOT may use the appropriate Auction Clearing Prices instead. If the DAM is not executed for an Operating Day, ERCOT shall use the RTM Settlement Point Prices for that Operating Day. |
| FDOPTV h, (j, k) | $/MW per hour | Five-day PTP Option Value—Average of the most recent rolling five-day amount given by the greater of zero or the difference in DAM Settlement Point Price between the sink k and the source j of the CRR for the hour h owned. If the DAM is executed but specific DAM Settlement Point Prices are not available, ERCOT may use the appropriate Auction Clearing Prices instead. If the DAM is not executed for an Operating Day, ERCOT shall use the RTM Settlement Point Prices for that Operating Day. |
| PMOPTV h, (j, k) | $/MW per hour | Previous Month’s PTP Option Value—Average of the previous month’s daily amount given by the greater of zero or the difference in DAM Settlement Point Price between the sink k and the source j of the CRR for the hour h owned. If the DAM is executed but specific DAM Settlement Point Prices are not available, ERCOT may use the appropriate Auction Clearing Prices instead. If the DAM is not executed for an Operating Day, ERCOT shall use the RTM Settlement Point Prices for that Operating Day. |
| OPTMW o, h, (j, k) | MW | PTP Option with the source j and the sink k owned by the Counter-Party as owner o for hour h. |
| OPTMW o, p, h, (j, k) | MW | PTP Option with the source j and the sink k owned by the Counter-Party as owner o for hour h, in the prompt month, p. |
| OPTPR (j, k), a | $/MW per hour | PTP Option Price per source and sink pair per CRR Auction—The clearing price of a PTP Option with the source j and the sink k in CRR Auction a, for the hour. |
| a | none | A CRR Auction. |
| j | none | A source settlement point. |
| k | none | A sink settlement point. |
| h | none | An operating hour. |
| o | none | A CRR owner |
| p | none | The prompt month |

(4)		The Counter-Party’s FCE for all FGRs (FCEFGR) held by the Counter-Party as owner of record at ERCOT for

FMMFGR o = Σ(h)Σ(f)[(W1 * ACP h, f + W2 * TFGRV h, f + W3 * FDFGRV h, f + W4 * PMFGRV h, f ) * FGRMW o, h, f]

rs relaeo PPOgatot excedtecr monhch ERCOTadteCer-Prot settle amn an aa FCEFGR i acltds:

FCEFGR o  = - FMMFGR o, p - FMMFGR o + INTMFGR o

Where:

FMMFGR o,p = Σ(h)Σ(j,k)[(W1 * ACP h,p (j, k) + W2 * TFGRV h, f + W3*  FDFGRV h, f + W4 * PMFGRV h, f) * FGRMWo,p, h, (j, k)]

INTMFGR o =Σ(h) Σ(f)(INTM h, f * FGRMW o, h, f)

FMMFGR o =Σ(h)Σ(f)[(ACP h, f - FGRPR f, a )* FGRMWo, h, f]

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| FCEFGR o | $ | Future Credit Exposure for FGRs—Counter-Party FCE for all FGRs held by the Counter-Party as owner o of record at ERCOT for all Operating Days for which a DAM has not executed and for CRRs that have not settled. |
| INTM h, f | $/MW per hour | Initial Margin Exposure for Flow Gate Rights on the flowgate f - Exposure level calculated using A TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”). The procedures will be used to define and modify INTM h, f . - The procedures to determine INTM and the value of INTM h, f shall be posted on the MIS Public Area. The TAC shall review the procedures at least annually and may recommend to the ERCOT Board changes to the parameters. If changes to these parameters are approved by the ERCOT Board, such revised procedures shall be posted on the MIS Public Area within three Business Days of ERCOT Board approval. - ERCOT shall generally notify counterparties two Bank Business Days prior to implementing either a change in procedures to determine INTM or the value of ACPE h, f. - ERCOT may adjust any INTM h, f immediately if, in its sole discretion, ERCOT determines that either the parameters or the reported value of ACPE h, f does not adequately match the financial risk created by the FGRs. |
| FMMFGR o | $ | Forward Mark-to-Market for FGRs—Estimate of the forward mark-to-market value of FGRs held by the Counter-Party as owner o of record at ERCOT for all Operating Days that have not yet occurred and for CRRs that have not settled. |
| FMMFGR o, p | $ | Forward Mark-to-Market for FGRs—Estimate of the forward mark-to-market value of FGRs held by the Counter-Party as owner o of record at ERCOT for all Operating Days that have not yet occurred and for CRRs that have not settled, in the prompt month p. |
| ACP h, p (j, k) | $/MW per hour | Auction Clearing Price—The auction clearing price for a PTP Obligation within the most recent CRR Auction corresponding to the CRR with the source j and the sink k for hour h, in the prompt month, p. ERCOT has the discretion to use a price from the bilateral and/or over-the-counter market for the prompt month, p, with the source j and the sink k for hour h if the price originated from an executed transaction and has a transaction date in excess of the most recent CRR Auction associated with the prompt month, p. |
| ACP h, f | $/MW per hour | Auction Clearing Price— The auction clearing prices for a FGR within the most recent CRR Auction corresponding to the CRR with a delivery month m, with the flowgate f for hour h. ERCOT has the discretion to use a price from the bilateral and/or over-the-counter market for a delivery month m, with the flowgate f for hour h if the price originated from an executed transaction and has a transaction date in excess of the most recent CRR Auction associated with delivery month m.. |
| W1 – W4 | None | Weighting—The weighting associated with the pricing components that sum to 1. The values of these factors must be determined by the Credit WG and posted on the MIS Public Area. The weighting factors may be customizable for the month to which a CRR applies. |
| TFGRV h, f | $/MW per hour | Today’s FGR Value—The current day’s most recent DAM price of the FGR on the flowgate f for the hour h. If the DAM is not executed for an operating day, ERCOT shall use the RTM Settlement Point Prices for that Operating Day. |
| FDFGRV h, f | $/MW per hour | Five-day FGR Value—Average of the most recent rolling five-day price of the FGR on the flowgate f for the hour h. If the DAM is not executed for an Operating Day, ERCOT shall use the RTM Settlement Point Prices for that Operating Day. |
| PMFGRV h, f | $/MW per hour | Previous Month’s FGR Value—Average of the previous month’s daily price of the FGR on the flowgate f for the hour h. If the DAM is not executed for an Operating Day, ERCOT shall use the RTM Settlement Point Prices for that Operating Day. |
| FGRMW o, p, h, (j, k) | MW | FGR on the flowgate f owned by the Counter-Party as owner o for hour h of (a) the remaining hours in the current month, and (b) all hours in the following month. |
| FGRMW o, h, f | MW | FGR on the flowgate f owned by the Counter-Party as owner o for hour h . |
| FGRPR f, a | $/MW per hour | Flow Gate Right Price per source and sink pair per CRR Auction—The clearing price of the Flow Gate Rights with the flowgate f in CRR Auction a, for the hour. |
| a | none | A CRR Auction. |
| f | none | A Flowgate Right |
| h | none | An Operating Hour |
| o | none | A CRR Owner |
| p | none | The prompt month |

(5)	ERCOT shall determine the Counter-Party’s Future Discounted Auction Revenue (FDAR) for each operating month associated with either awarded CRRs or allocated Pre-Congestion Revenue Rights (PCRRs) that are unrelated to settled CRRs for which ERCOT has distributed CRR Auction revenues as calculated under Section 7.5.7, Method for Distributing CRR Auction Revenues, and Section 9.10, CRR Auction Revenue Distribution Invoices. The Counter-Party’s FDAR o is calculated as follows

FDAR o = ΣqΣz [(-1) * [Σa(FDCRRZREV z, a + FDPCRRZREV z, a) * FDMLRSZ z, q + Σa (FDCRRNZREV z, a + FDPCRRNZREV a) * FDMLRS q]]

Where

FDCRRZREV z, a =Σh(ΣcrrhΣjΣkOBLSAMTcrrh,(j,k),z,a,h + ΣcrrhΣjΣkOPTSAMTcrrh,(j,k),z,a,h + ΣcrrhΣfFGRSAMT crrh,f,z,a,h + ΣcrrhΣjΣkFDOBLPAMT crrh,(j,k),z,a,h + ΣcrrhΣjΣkFDOPTPAMTcrrh,(j,k),z,a,h + ΣcrrhΣfFDFGRPAMT crrh,f,z,a,h)

FDPCRRZREV z, a	=	Σh(ΣcrrhΣjΣkΣtechFDPCRROBLAMT crrh,(j,k),z,a,tech,h + ΣcrrhΣjΣkΣtechFDPCRROPTAMT crrh,(j,k),z,a,tech,h)

FDCRRNZREV a =Σh(ΣcrrhΣjΣkOBLSAMTcrrh,(j,k),a,h + ΣcrrhΣjΣkOPTSAMTcrrh,(j,k),a,h + ΣcrrhΣfFGRSAMT crrh,f,a,h + ΣcrrhΣjΣkFDOBLPAMT crrh,(j,k),a,h + ΣcrrhΣjΣkFDOPTPAMTcrrh,(j,k),a,h + ΣcrrhΣfFDFGRPAMT crrh,f,a,h)

FDPCRRNZREV a	=	Σh(ΣcrrhΣjΣkΣtechFDPCRROBLAMT crrh,(j,k),a,tech,h + ΣcrrhΣjΣkΣtechFDPCRROPTAMT crrh,(j,k),a,tech,h)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FDCRRZREV z, a | $ | Future Discounted CRR Zonal Revenue per zone per CRR Auction— The probabilistic revenue for a given month produced from CRRs that source and sink within the same 2003 ERCOT Congestion Management Zone (CMZ) z, cleared through CRR Auction offers and CRR Auction bids in CRR Auction a. Using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) probabilistic revenue shall be defined. |
| OBLSAMT crrh, (j, k), z, a, h | $ | PTP Obligation Sale Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per hour—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation offers awarded in CRR Auction a with the source j and the sink k, both in CMZ z, for the hour h. |
| OPTSAMT crrh, (j, k), z, a, h | $ | PTP Option Sale Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per hour—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids awarded in CRR Auction a with the source j and the sink k, both in CMZ z, for the hour h. |
| FGRSAMT crrh, f, z, a, h | $ | Flowgate Right Sale Amount per CRR Account Holder per flowgate per zone per CRR Auction per hour—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR offers awarded in CRR Auction a associated with FGR f in CMZ z, for the hour h. |
| FDOBLPAMT crrh, (j, k), z, a, h | $ | Future Discounted PTP Obligation Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation bids awarded in CRR Auction a with the source j and the sink k, both in CMZ z, for the hour h. |
| FDOPTPAMT crrh, (j, k), z, a, h | $ | Future Discounted PTP Option Purchase Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids awarded in CRR Auction a with the source j and the sink k, both in CMZ z, for the hour h. |
| FDFGRPAMT crrh, f, z, a, h | $ | Future Discounted Flowgate Right Purchase Amount per CRR Account Holder per flowgate per zone per CRR Auction per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR bids awarded in CRR Auction a associated with FGR f in CMZ z, for the hour h. |
| FDMLRS q | none | Future Discounted Monthly Load Ratio Share (LRS) per QSE—The LRS calculated for QSE q for the peak-Load 15-minute Settlement Interval in the month. See the TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) for defining probable LRS values. |
| FDMLRSZ q, z | none | Future Discounted Monthly Load Ratio Share Zonal per QSE per zone—The LRS of QSE q for its Load in CMZ z, for the peak-Load 15-minute Settlement Interval in the month. See the TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) for defining probable LRS values. |
| FDCRRNZREV a | $ | Future Discounted CRR Non-Zonal Revenue—The probabilistic revenue for a given month produced from CRRs that source and sink in different 2003 ERCOT CMZs, cleared through CRR Auction offers and CRR Auction bids in CRR Auction a. Using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) probabilistic revenue shall be defined. |
| OBLSAMT crrh, (j, k), a, h | $ | PTP Obligation Sale Amount per CRR Account Holder per source and sink pair per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation offers awarded in CRR Auction a with the source j and the sink k in different CMZs, for the hour h. |
| OPTSAMT crrh, (j, k), a, h | $ | PTP Option Sale Amount per CRR Account Holder per source and sink pair per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option offers awarded in CRR Auction a with the source j and the sink k in different CMZs, for the hour h. |
| FGRSAMT crrh, f, a, h | $ | Flowgate Right Sale Amount per CRR Account Holder per flowgate per CRR Auction—The payment calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR offers awarded in CRR Auction a associated with FGR f across CMZs z, for the hour h. |
| FDOBLPAMT crrh, (j, k), a, h | $ | Future Discounted PTP Obligation Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Obligation bids awarded in CRR Auction a with the source j and the sink k in different CMZs, for the hour h. |
| FDOPTPAMT crrh, (j, k), a, h | $ | Future Discounted PTP Option Purchase Amount per CRR Account Holder per source and sink pair per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total PTP Option bids awarded in CRR Auction a with the source j and the sink k in different CMZs, for the hour h. |
| FDFGRPAMT crrh, f, a, h | $ | Future Discounted Flowgate Right Purchase Amount per CRR Account Holder per flowgate per CRR Auction—The charge calculated for CRR Account Holder crrh of the MW quantity that represents the total FGR bids awarded in CRR Auction a associated with FGR f across CMZs z, for the hour h. |
| FDPCRRZREV z, a | $ | Future Discounted PCRR Zonal Revenue per zone per CRR Auction—The probabilistic revenue for a given month produced from PCRRs that source and sink within the same 2003 ERCOT CMZ z, pertaining to each CRR Auction. The revenue resulted from the PCRRs that source and sink in CMZ z, pertaining to CRR Auction a. Using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) probabilistic revenue shall be defined. |
| FDPCRROBLAMT crrh, (j, k), z, a, tech, h | $ | Future Discounted PCRR PTP Obligation Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per resource technology per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Obligations pertaining to CRR Auction a with the source j and the sink k in CMZ z, based on Resources of the technology tech, for the hour h. |
| FDPCRROPTAMT crrh, (j, k), z, a, tech, h | $ | Future Discounted PCRR PTP Option Amount per CRR Account Holder per source and sink pair per zone per CRR Auction per resource technology per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Options pertaining to CRR Auction a with the source j and the sink k in CMZ z, based on Resources of the technology tech, for the hour h. |
| FDPCRRNZREV a | $ | Future Discounted PCRR Non-Zonal Revenue per CRR Auction—The probabilistic revenue for a given month produced from PCRRs that source and sink in different 2003 ERCOT CMZs, pertaining to each CRR Auction. The revenue resulted from the PCRRs that source and sink in different CMZs, pertaining to CRR Auction a. Using a TAC-recommended and ERCOT Board-approved procedure (“Procedures for Setting Independent Amounts Related to Auction Clearing Price Exposure”) probabilistic revenue shall be defined. |
| FDPCRROBLAMT crrh, (j, k), a, tech, h | $ | Future Discounted PCRR PTP Obligation Amount per CRR Account Holder per source and sink pair per CRR Auction per resource technology per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Obligations pertaining to CRR Auction a with the source j and the sink k in different CMZs, based on Resources of the technology tech, for the hour h. |
| FDPCRROPTAMT crrh, (j, k), a, tech, h | $ | Future Discounted PCRR PTP Option Amount per CRR Account Holder per source and sink pair per CRR Auction per resource technology per hour—The charge calculated for CRR Account Holder crrh of the MW quantity that represents its total PTP Options pertaining to CRR Auction a with the source j and the sink k in different CMZs, based on Resources of the technology tech, for the hour h. |
| a | none | A CRR Auction. |
| q | none | A QSE. |
| z | none | A 2003 ERCOT CMZ. |
| crrh | none | A CRR Account Holder that paid the invoice in full. |
| tech | none | A Resource technology. |
| j | none | A source Settlement Point. |
| k | none | A sink Settlement Point. |
| f | none | An FGR. |
| h | none | An hour in the month. |