| NPRR Number | 336 | NPRR Title | Authorize ERCOT to Procure Additional RRS During Severe Cold Weather |
| --- | --- | --- | --- |
| Date |  | June 17, 2011 |  |
| Submitter’s Information |  |  |  |
| Name |  | Sherry Looney |  |
| E-mail Address |  | sherry.looney@luminant.com |  |
| Company |  | Luminant Energy |  |
| Phone Number |  | (214) 875-9278 |  |
| Cell Number |  | (469) 688-7607 |  |
| Market Segment |  | Investor-Owned Utility (IOU) |  |

Comments

| Overall Market Benefit | Improved reliability of the ERCOT System. |
| --- | --- |
| Overall Market Impact | Significant impacts are expected but are needed in order to maintain reliability of the ERCOT System. |
| Consumer Impact | Unknown. |

Luminant Energy Company LLC (Luminant) supports the concept of authorizing ERCOT to procure additional generating reserves during severe weather.  However, Luminant believes that rather than specify a particular amount of Responsive Reserve (RRS) for ERCOT to procure, it is better to provide ERCOT the discretion and flexibility to procure a discretional amount of Non-Spin Reserve Service (NSRS) necessary to maintain adequate system reliability in response to weather forecasting contingency issues.  Luminant also believes that it is necessary to define a new term, Conditional Reliability Unit Commitment (CRUC) to allow ERCOT to start Generation Resources with start times in excess of 10 hours which may be needed beyond the next Operating Day to ensure system reliability.

Revised Proposed Protocol Language

| NPRR Number | 336 | NPRR Title | Authorize ERCOT to Procure Additional NSRS During Severe Cold Weather |
| --- | --- | --- | --- |

2.1	DEFINITIONS

Watch

The third of four possible levels of communication issued by ERCOT in anticipation of a possible Emergency Condition, detailed in Section 6.5.9, Emergency Operations.

Cold Weather Watch

A Watch declared when temperatures are forecasted to be in between 20°F and 26°F and the maximum temperature is expected to remain 32°F or below in three or more Weather Zones in the ERCOT Region.

Severe Cold Weather Watch

A Watch declared when temperatures are forecasted to be below 20°F and the maximum temperatures are expected to remain between 20°F and 26°F or below in five or more Weather Zones in the ERCOT Region.

Conditional Reliability Unit Commitment

A RUC instruction which is issued by ERCOT prior to the Day Ahead Market for an    Operating Day due to the forecasted loads requiring a Resource with longer than 10 hour start time.  A Resource that receives a Conditional Reliability Unit Commitment prior to the DAM for an Operating Day will be eligible to participate in the DAM or self commit prior to the DRUC process for the Operating Day and if not otherwise committed, will receive a RUC instruction with no clawback provisions.

- 6.5.9.3.3	Watch

(1)	A Watch is the third of four possible levels of communication issued by ERCOT in anticipation of a possible Emergency Condition.

(2)	ERCOT shall issue a Watch when ERCOT determines that:

(a)	Conditions have developed such that additional Ancillary Services are needed in the current Operating Period;

(b)	There are insufficient Ancillary Services or Energy Offers. in the DAM or in a Supplemental Ancillary Services Market (SASM);

(c)	Market-based congestion management techniques embedded in SCED as specified in these Protocols will not be adequate to resolve transmission security violations;

(d)	Forced Outages or other abnormal operating conditions have occurred, or may occur that require operations with active transmission security violations; or

(e)	Forecasted weather conditions meet the Cold Weather Watch and Severe Cold Weather Watch criteria.

(3)	With the issuance of a Watch pursuant to paragraph (2)(a) above, ERCOT may exercise its authority to immediately procure the following services from existing offers:

(a)	Regulation Services;

(b)	RRS services; and

(c)	Non-Spinning Reserve Services.

(4)	If the actions in paragraph (3) above do not relieve the insufficiency described in paragraph (2)(a) above, then ERCOT may issue Dispatch Instructions to Resources certified to provide the insufficient service, even though there is not an existing Ancillary Service Offer for that Resource.

(5)	If ERCOT issues a Watch because insufficient Ancillary Service Offers were received in the DAM, and if the Watch does not result in sufficient offer and the DAM is executed with insufficient offers, then ERCOT shall acquire the insufficient amount of Ancillary Services in the Day-Ahead Reliability Unit Commitment (DRUC) and shall issue Dispatch Instructions to the QSEs for Resources that were RUC-committed to provide Ancillary Services, informing them of the requirement that the Resources be prepared to provide those Ancillary Services.

(6)	If ERCOT issues a Watch because insufficient Ancillary Service Offers were received in a SASM, and if the Watch does not result in sufficient offer and the SASM is executed with insufficient offers, then ERCOT shall acquire the insufficient amount of Ancillary Services in the next Hourly Reliability Unit Commitment (HRUC) and shall issue Dispatch Instructions to the QSEs for Resources that were RUC-committed to provide Ancillary Services, informing them of the requirement that the Resources be prepared to provide those Ancillary Services.

(7)	ERCOT shall post the Watch message electronically to the MIS Secure Area and shall provide verbal notice to all TSPs and QSEs via the Hotline.  Corrective actions identified by ERCOT must be communicated through Dispatch Instructions to all TSPs, DSPs and QSEs required to implement the corrective action.  Each QSE shall immediately notify the Market Participants that it represents of the Watch.  To minimize the effects on the ERCOT System, each TSP or DSP shall identify and prepare to implement actions, including restoration of transmission lines as appropriate and preparing for Load shedding.  ERCOT may instruct TSPs or DSPs to reconfigure ERCOT System elements as necessary to improve the reliability of the ERCOT System.  On notice of a Watch, each QSE, TSP, and DSP shall prepare for an emergency in case conditions worsen.  ERCOT may require information from QSEs representing Resources regarding the Resources’ fuel capabilities.  Requests for this type of information shall be for a time period of no more than seven days from the date of the request.  The specific information that may be requested shall be defined in the Operating Guides.  QSEs representing Resources shall provide the requested information in a timely manner, as defined by ERCOT at the time of the request.

(8)	If ERCOT issues a Cold Weather or Severe Cold Weather Watch, ERCOT shall procure a discretional amount of additional NSRS reserves.

(9)	If ERCOT issues a Severe Cold Weather Watch, ERCOT shall evaluate the need for additional Resources which have greater than 10 hour lead times and may be required for loads forecasted beyond the next day.  ERCOT should issue a Conditional RUC instruction for Resources which are needed beyond the next Operating Day to ensure system reliability.