| NPRR Number | 373 | NPRR Title | Revised State Estimator Reports on Individual Transmission Load on Electrical Buses |
| --- | --- | --- | --- |
| Date Posted |  | June 3, 2011 |  |
| Nodal Protocol Section Requiring Revision |  | 6.5.7.1.13, Data Inputs and Outputs for the Real-Time Sequence and SCED |  |
| Requested Resolution |  | Normal |  |
| Revision Description |  | The Nodal Protocol Revision Request (NPRR) reinstates the State Estimator Load Reports, which will be published every five minutes on the Market Information System (MIS) Secure Area. The NPRR requires ERCOT to redact station codes with less than ten Electric Service Identifiers (ESI IDs) to protect confidential information. |  |
| Reason for Revision |  | When NPRR329, Security Classification Changes for Extracts/Reports, became effective on May 18, it suspended the posting of State Estimator Load Reports that revealed individual Load data. WMS believes there is value of Load data being available for Market Participants without releasing Protected Information. The State Estimator Task Force (SETF) worked on a methodology to redact station codes based off ERCOT’s current systems. During market trials and after Nodal go-live, ERCOT was publishing this report every five minutes before the concerns arose of confidentiality. |  |
| Overall Market Benefit |  | Positive- improves market transparency. |  |
| Overall Market Impact |  | Positive- improves market transparency. |  |
| Consumer Impact |  | Positive- improves market transparency. |  |
| Credit Implications |  | To be determined. |  |

| Quantitative Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | None. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Improves market transparency. | Unknown. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Eric Goff on behalf of WMS |
| E-mail Address | egoff@reliant.com |
| Company | Reliant Energy Retail Services, LLC |
| Phone Number |  |
| Cell Number | 512-284-4908 |
| Market Segment | Independent Retail Electric Provider (IREP) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

Comments

Please note that the following NPRRs also propose revisions to Section 6.5.7.1.13:

- NPRR327, State Estimator Data Redaction Methodology

- NPRR360, Summary Report of HDL and LDL

Proposed Protocol Language Revision

6.5.7.1.13	Data Inputs and Outputs for the Real-Time Sequence and SCED

(1)	Inputs:  The following information must be provided as inputs to the Real-Time Sequence and SCED.  ERCOT may require additional information as required, including:

(a)	Real-Time data from TSPs including status indication for each point if that data element is stale for more than 20 seconds;

(i)	Transmission Electrical Bus voltages;

(ii)	MW and MVAr pairs for all transmission lines, transformers, and reactors;

(iii)	Actual breaker and switch status for all modeled devices; and

(iv)	Tap position for auto-transformers;

(b)	State Estimator results (MW and MVAr pairs and calculated MVA) for all modeled Transmission Elements;

(c)	Transmission Element ratings from TSPs;

(i)	Data from the Network Operations Model:

(A)	Transmission lines – Normal, Emergency, and 15-Minute Ratings (MVA); and

(B)	Transformers and Auto-transformers – Normal, Emergency, and 15-Minute Ratings (MVA) and tap position limits;

(ii)	Data from QSEs:

(A)	Generator step-up transformers tap position;

(B)	Resource HSL (from telemetry); and

(C)	Resource LSL (from telemetry); and

(d)	Real-Time weather, from WGRs, and where available from TSPs or other sources.  ERCOT may elect to obtain other sources of weather data and may utilize such information to calculate the dynamic limit of any Transmission Element.

(2)	ERCOT shall validate the inputs of the Resource Limit Calculator as follows:

(a)	The calculated SURAMP and SDRAMP are each greater than or equal to zero; and

(b)	Other provision specified under Section 3.18, Resource Limits in Providing Ancillary Service.

(3)	Outputs for ERCOT Operator information and possible action include:

(a)	Operator notification of any change in status of any breaker or switch;

(b)	Lists of all breakers and switches not in their normal position;

(c)	Operator notification of all Transmission Element overloads detected from telemetered or State-Estimated data;

(d)	Operator notification of all Transmission Element security violations; and

(e)	Operator summary displays:

(i)	Transmission system status changes;

(ii)	Overloads;

(iii) 	System security violations; and

(iv)	Base Points.

(4)	Every hour, ERCOT shall post on the MIS Secure Area the following information:

(a)	Status of all breakers and switches used in the NSA except breakers and switches connecting Resources to the ERCOT Transmission Grid;

(b)	All binding transmission constraints and the contingency or overloaded element pairs that caused such constraint; and

(c)	Shift Factors by Resource Node.

(5)	Every five minutes, ERCOT shall post on the MIS Secure Area the individual transmission Load on Electrical Buses for station codes that have greater than ten Electric Service Identifiers (ESI_IDs), sum of the Load in each Load Zone, and total Load on Electrical Buses in the ERCOT System from the State Estimator.

(6)	Sixty days after the applicable Operating Day, ERCOT shall post on the MIS Secure Area, the following information:

(a)	Hourly transmission line flows and voltages from the State Estimator, excluding transmission line flows and voltages for Private Use Networks; and

(b)	Hourly transformer flows, voltages and tap positions from the State Estimator, excluding transformer flows, voltages, and tap positions for Private Use Networks.

(7)	Notwithstanding paragraph (6) above, ERCOT, in its sole discretion, shall release relevant State Estimator data less than 60 days after the Operating Day if it determines the release is necessary to provide complete and timely explanation and analysis of unexpected market operations and results or system events including, but not limited to, pricing anomalies, recurring transmission congestion, and system disturbances.  ERCOT’s release of data under this paragraph shall be limited to intervals associated with the unexpected market or system event as determined by ERCOT.  The data release shall be made available simultaneously to all Market Participants.

(8)	Notwithstanding paragraph (6) above, ERCOT shall develop and post a redacted version of the State Estimator data, as soon as reasonably practicable after collection of the data, so long as a redacted version excludes information (including, but not limited to, voltages, transmission flows and transformer flows) from which resource-specific output levels or offer curves could continually and systematically be derived.

(9)	Every hour, ERCOT shall post on the MIS Public Area, the sum of ERCOT generation, and flow on the DC Ties, all from the State Estimator.