| NPRR Number | 382 | NPRR Title | Verbal RUC Committed or Decommitted Resources Report |
| --- | --- | --- | --- |
| Date Posted |  | June 8, 2011 |  |
| Nodal Protocol Sections Requiring Revision |  | 5.3, ERCOT Security Sequence Responsibilities |  |
| Requested Resolution |  | Normal. |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) creates two new reports on the Market Information System (MIS) Secure Area for verbal Reliability Unit Commitment (RUC) commitments/decommitments, one following the Day-Ahead Reliability Unit Commitment (DRUC) and one following the Hourly Reliability Unit Commitment (HRUC) to stay consistent with the current RUC data delivery process. |  |
| Reason for Revision |  | To provide market transparency regarding all RUC commitments including the Verbal Dispatch Instruction (VDI) commitments. The reports will provide useful information that will help Market Participants pursue their business interests. This information is consistent with the information currently provided for commitments and decommiments both for HRUC and DRUC. This provides additional verbal RUC information for the RUC study period. |  |
| Overall Market Benefit |  | Provides a more complete RUC data set. |  |
| Overall Market Impact |  | Beneficial as it provides Market Participants access to additional RUC data. |  |
| Consumer Impact |  | None. |  |
| Credit Implications |  | No. |  |

Quantitative Impacts and Benefits

| Assumptions | 1 | Will not contain deselected RUC data. |  |
| --- | --- | --- | --- |
|  | 2 | Will follow frequency and format of current HRUC and DRUC reports. |  |
|  | 3 | Two new MIS Secure Area links. |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | None. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Complete data set for RUC information. |  |
|  | 2 | Additional access to RUC data for Market Participants. |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Jackie Ashbaugh on behalf of the MIS User Group |
| E-mail Address | jashbaugh@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3937 |
| Cell Number |  |
| Market Segment | N/A |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sandra Tindall |
| E-Mail Address | stindall@ercot.com |
| Phone Number | 512-248-3867 |

Proposed Protocol Language Revision

- 5.3	ERCOT Security Sequence Responsibilities

(1)	ERCOT shall start the Day-Ahead Reliability Unit Commitment (DRUC) process at 1430 in the Day Ahead.

(2)	For each DRUC, ERCOT shall use a snapshot of Resource commitments taken at 1430 in the Day-Ahead to settle Reliability Unit Commitment (RUC) charges.  For each Hourly Reliability Unit Commitment (HRUC), ERCOT shall use a snapshot of Resource commitments from each Qualified Scheduling Entity’s (QSE’s) most recently submitted Current Operating Plan (COP) before HRUC execution to settle RUC charges.

(3)	For each RUC process, ERCOT shall:

(a)	Execute the Security Sequence described in Section 5.5, Security Sequence, Including RUC, including:

(i)	Validating Three-Part Supply Offers, defined in Section 4.4.9.1, Three-Part Supply Offers; and

(ii)	Reviewing the Resource commitment recommendations made by the RUC algorithm; and

(b)	Post to the Market Information System (MIS) Secure Area, all Resources that were committed or decommitted by the RUC process including verbal RUC commitments and decommitments;

(c)	Post to the MIS Public Area, all active and binding transmission constraints (contingency and overloaded element pair information where available) used as inputs to the RUC; and

(d)	Issue Dispatch Instructions to notify each QSE of its Resource commitments or decommittments.

(4)	ERCOT shall provide each QSE with the information necessary to pre-validate their data for DRUC and HRUC including:

(a)	Publishing validation rules for offers, bids, and trades; and

(b)	Posting any software documentation and code that is not Protected Information to the MIS Secure Area within five Business Days of receipt by ERCOT.