ERCOT Operations Report

Aug 2011 ROS & OWG

Frequency Control

| Time | Freq(Hz) | Duration (s) |  |
| --- | --- | --- | --- |
| 8/1/2011 10:25 | 59.864 | 272 | Multiple Unit trips with 1100 MW online |
| 8/24/2011 2:22 | 59.866 | 52 | Unit trip with 625 MW online |
| 8/31/2011 21:36 | 59.879 | 16 | Unit trip with 585 MW |

Note: Time of frequency excursions outside the range of +/- 0.1 Hz.

Deployed Responsive (Reported Above)

| Time deployed | Stop Time | Duration | Max | Details |
| --- | --- | --- | --- | --- |
| 8/3/2011 15:28 | 8/3/2011 17:05 | 1:37:26 | 450 | Out of up reg |
| 8/4/2011 14:14 | 8/4/2011 14:35 | 0:21:09 | 1000 | Out of up reg |
| 8/5/2011 15:41 | 8/5/2011 15:42 | 0:00:40 | 200 | Out of up reg |
| 8/24/2011 2:22 | 8/24/2011 2:25 | 0:03:09 | 357.14 | Out of up reg |
| 8/24/2011 14:50 | 8/24/2011 15:11 | 0:20:48 | 800 | Out of up reg |
| 8/24/2011 15:49 | 8/24/2011 15:54 | 0:04:44 | 200 | Out of up reg |
| 8/24/2011 15:58 | 8/24/2011 16:00 | 0:02:11 | 100 | Out of up reg |

RUC Capacity Purchases

| Weather Zone | Unit Days |
| --- | --- |
| COAST | 7 |
| EAST | 4 |
| NORTH CENTRAL | 21 |
| SOUTH | 2 |
| SOUTH CENTRAL | 13 |
| WEST | 4 |
| Grand Total | 51 |

SCED Congestion Management Activity Congestion affected by outages  Remedial action plan has been developed for the element

| Contingency | Overloaded Element | Days |
| --- | --- | --- |
| Amoco North Cowden Tap - Moss Switch 138 kV | Odessa North Transformer 138/69 kV | 22 |
| Olinger - Ben Davis & Elmgrove 138 kV | Firewheel - Olinger 138 kV | 14 |
| Moss Switch – Odessa EHV Switch 345 kV | Odessa - Odessa North 138 kV | 11 |
| Basecase | Matador - Paducah Rea Tap 69 kV | 8 |
| DCKT Greens Bayou - Gable & Hardy - Crocket 138 kV | Garrott - Midtown 138 kV | 8 |
| Cagnon 345/138 kV XFMR1 | Cagnon 345/138 kV XFM4H | 8 |
| Orange Grove Switching Station - Lon Hill 138 kV | Sinton - Skidmore 69 kV | 7 |
| Amoco North Cowden Tap - Moss Switch 138 kV | Odessa North - North Cowden 69 kV | 5 |
| Fort Worth Substation - West Denton 138 kV | West Denton - Jim Christal Substation 138kV | 5 |
| Royse Switch - Allen Switch 345 kV | Apollo - Richardson East 138 kV | 5 |
| Elkton - Forney Switch 345 kV & Tri Corner - Martin Lake SES 345 kV | Troup - Wlgrv 138 kV | 5 |
| Temple Switch - Salado Switch 345 kV | Bell County Switch - Temple Switch 345 kV | 4 |
| Basecase | Golden Switch - Brand (Oncor) 138 kV | 3 |
| Gail Sub - Key Sub 138 kV | Sparenburg - Lamesa 69 kV | 3 |
| San Angelo Red Creek - Comanche Switch 345 kV | Flat Creek - Leon Switch 138 kV | 3 |
| Calmont Switch - Benbrook Switch 138 kV | Saginaw Switch - American Mfg Tap 138 kV | 3 |
| Venus Switch 3 - Big Brown SES 345 kV | Trinidad SES - Richland Chambers 345 kV | 3 |
| Caney Creek - Dyann 138 kV | Texas Gulf Sulphur - New Gulf 138 kV | 3 |
| Ph Robinson Transformer AT2 345/138 kV | Ph Robinson Transformer AT1 345/138 kV | 3 |
| Paloduro Sub - Pearsall Switching Station 138 kV | Pearsall Switching Station 138/69 kV XFMER | 3 |
| Bell County East Switch - Temple Switch 345 kV (425) | Bell County East Switch - Temple Switch 345 kV (435) | 2 |
| Baytown Energy Center - Chambers 345 kV | Baytown Energy Center Transformer AT-2 138/345 kV | 2 |
| Bluff Creek 345/138 kV Transformer 2 | Oak Creek CSW - Nicole 138 kV | 2 |
| Lewisville Switch FMR1 345/138 kV | Lakepointe TNP - Carrollton Northwest 138 kV | 2 |
| Bellso - Bellno 138 kV | Chaphi - Waller 138 kV | 2 |
| Brownwood Switch - Zephyr BEPC 138 kV | Rising Star - Downing 69 kV | 2 |
| Elm Mott - Tradinghouse SES 345 kV | Rogers Hill - Elm Mott 138 kV | 2 |
| DCKT Roans Prairie -Kuykendahl-King& Singleton -Tomball 345 kV | Singleton - Zenith 345 kV | 2 |
| Amoco North Cowden Tap - Moss Switch 138 kV | Odessa North - Odessa Basin Switch 69 kV | 1 |
| Basecase | South Lane City - New Gulf 138 kV | 1 |
| Orange Grove Switching Station - Lon Hill 138 kV | Smith - Lon Hill 69 kV | 1 |
| Fort Worth Substation - West Denton 138 kV | West Denton - Jim Christal Substation 138kV | 1 |
| Bell County East Switch - Temple Switch 345 kV (425) | Bell County East Switch - Temple Switch 345 kV (435) | 1 |
| Liggett Switch - Venus Switch 345 kV | Davis Tap - Euless Trinity Blvd 138 kV | 1 |
| Liggett Switch - Venus Switch 345 kV | Mayfield Tap North - Cedar Hill Switch 138 kV | 1 |
| Watermill Switch - Tri Corner & Trinidad SES 345 kV | Kleberg Tap - Seagoville 138 kV | 1 |
| Watermill Switch - Tri Corner & Trinidad SES 345 kV | Seagoville Switch - Tri Corner 345 kV | 1 |
| Calallen Substation - Lon Hill 69 kV | Lon Hill - Robstown Tap 69 kV | 1 |
| DCKT DOW Chemical - Oasis 345 kV | Danevang - Danevang Sub 69 kV | 1 |
| Cabaniss - Airline CSW138 kV | Naval Base - North Padre 69kV | 1 |
| DCKT Austrop - Garfield & Zorn 345 kV | Austrop 345/138kV AT1 | 1 |
| Fort Stockton - Barilla 138 kV | Fort Stockton Switch - Barilla 69 kV | 1 |
| Morgan Creek SES - China Grove Switch 138 kV | Morgan Creek SES - China Grove Switch 138 kV | 1 |
| Gail Sub - Key Sub 138 kV | Sparenburg - Lamesa 69 kV | 1 |
| Norwood Switch - Irving Norwood 138kV | Liggett Switch 345/138 kV FMR2 | 1 |
| Moore Switching Station - Downie Switching Station 138 kV | Uvalde CSW - West Batesville 138 kV | 1 |
| Sun Switch - Golden Switch 138 kV | China Grove Switch - Bluff Creek Switch 138 kV | 1 |
| Collin SES - Collin Switch 138 kV | Allen Switch - Plano Custer Road 138 kV | 1 |
| Sun Switch - Golden Switch 138 kV | China Grove Switch - Bluff Creek Switch 138 kV | 1 |
| Brazoria - West Columbia Main 138 kV | Old Ocean TNP - Sweeny TNP 69 kV | 1 |
| DCKT WA Parish - Obrien 345 kV | Garrott - Midtown 138 kV | 1 |
| Gaf TNP _ Greenbelt TNP 138 kV | Heights TNP - PH Robinson 138 kV | 1 |
| DCKT Nueces Bay - Koch Up River & Lon Hill 138 kV | Hiway - Air Liquide 69 kV | 1 |
| Formosa 138/69 kV XFMR TR22 | Formosa 138/69 kV XFMR TR21 | 1 |
| Venus Switch - Webb Station 345 kV | Cedar Hill Switch - Fish Creek Switch 138 kV | 1 |
| DCKT Garfield - Hicross & Onion - Pilot 138 kV | Advance - Burleson 138 kV | 1 |
| DCKT Temple Pecan Creek - Tradinghouse & Temple Switch - Lake Creek 345 kV | Troy Sub - Eddy Substation 69 kV | 1 |
| Everman Switch 345/138 kV AT2 | Everman Switch 345/138 kV AT1 | 1 |
| DCKT Jewett - Singleton 345 kV | Twin Oak Switch - BTU Jack Creek Switch 345 kV | 1 |
| Jim Christal Substation - West Denton 138 kV | West Denton - Fort Worth 138 kV | 1 |
| DCKT Watermill Switch - Limestone 345 kV | Jewett - Big Brown SES 345 kV | 1 |
| Meadow - Oasis 345 kV | Garrott - Midtown 138 kV | 1 |

Generic Transmission Limits (GTL’s)

| GTL | Days |
| --- | --- |
| West – North | 2 |

New RAP, PCAP (Date indicates last update of RAP, PCAP not implementation date.)

None

SPS (RAS) (New, Modified, Removed)

08/24 No Trees SPS #34

New Procedures/Forms/Operations Bulletins

| Date | Subject | Bulletin No. |
| --- | --- | --- |
| 08/24/2011 | Reliability Unit Commitment Desk V1 Rev8 | 550 |
| 08/03/2011 | Transmission and Security Desk V1 Rev9 | 549 |
| 08/03/2011 | Real Time Desk V1 Rev9 | 548 |

## Load Shed Incidences

None

EEA

08/02 14:40	ERCOT issued an EEA Level 1 (PRC dropped below 2,300 MW) at 14:40. At 18:00 EEA Level 1 was cancelled.

08/03 14:50	ERCOT issued an EEA Level 1 (PRC dropped below 2,300 MW) at 14:50. At 18:10 EEA Level 1 was cancelled.

08/04 13:45	ERCOT issued an EEA Level 1 (PRC dropped below 2,300 MW) at 13:45. At 14:32 ERCOT issued an EEA Level 2A (PRC dropped below 1,750 MW).  At 15:44 ERCOT declared EEA Level 2B. EILS deployed. At 17:53 ERCOT move to EEA Level 2A from EEA Level 2B. At 18:09 ERCOT move to EEA Level 1 from EEA Level 2A. At 18:52 EEA Level 1 was cancelled.

08/05 15:10	ERCOT issued an EEA Level 1 (PRC dropped below 2,300 MW) at 15:10. At 18:05 EEA Level 1 was cancelled.

08/23 15:43	ERCOT issued an EEA Level 1 (PRC dropped below 2,300 MW) at 15:43. At 18:00 EEA Level 1 was cancelled.

08/24 14:18	ERCOT issued an EEA Level 1 (PRC dropped below 2,300 MW) at 14:18. At 15:08 ERCOT issued an EEA Level 2A (PRC dropped below 1,750 MW. At 17:26 ERCOT move to EEA Level 1 from EEA Level 2A. At 18:08 EEA Level 1 was cancelled.

System Voltage Problems

None

Application Issues

08/02 13:45 	ERCOT issued an Emergency notice due to SCED failure.

08/12 13:45 	ERCOT issued an Emergency notice due to SCED failure.

08/13 17:06 	ERCOT issued an Emergency notice due to SCED failure.

Communication Issues

None

## ERCOT peak (Unofficial)

08/03 17:00 68,294 MW

Yearly MTLF Error Comparison

| Yearly Mid-Term (Hourly) Load Forecast Error Comparison for the ERCOT Forecast |  |  |  |  |
| --- | --- | --- | --- | --- |
| Month | MAPE for MTLF at 16:00 Day-Ahead |  |  |  |
|  | 2008 | 2009 | 2010 | 2011 |
| January | 3.30 | 4.11 | 3.78 | 2.91 |
| February | 3.59 | 3.49 | 3.79 | 3.46 |
| March | 3.29 | 3.24 | 2.24 | 2.02 |
| April | 2.45 | 2.66 | 2.47 | 3.55 |
| May | 3.01 | 2.90 | 2.63 | 3.27 |
| June | 2.81 | 2.89 | 2.84 | 2.28 |
| July | 2.55 | 2.83 | 2.39 | 1.63 |
| August | 3.66 | 3.07 | 2.29 | 2.28 |
| September | 4.99 | 3.68 | 3.14 | NA |
| October | 2.79 | 2.92 | 2.37 | NA |
| November | 2.60 | 1.93 | 3.46 | NA |
| December | 4.54 | 3.70 | 2.60 | NA |
| Annual Average | 3.29 | 3.11 | 2.83 | 2.68 |

OCN, Advisory, Watch & Emergency Notices

Capacity

| Date | Start | Event |
| --- | --- | --- |
| 8/1/2011 | 03:00 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1400-2100 |
| 8/1/2011 | 12:48 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/1/2011 | 14:24 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/2/2011 | 00:40 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1400-2000 |
| 8/2/2011 | 13:21 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/2/2011 | 13:57 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/2/2011 | 14:00 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/3/2011 | 00:30 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1400-2000. |
| 8/3/2011 | 13:01 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/3/2011 | 14:04 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/4/2011 | 00:15 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1400-2100. |
| 8/4/2011 | 12:34 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/4/2011 | 13:16 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/5/2011 | 00:45 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1400-2000 |
| 8/5/2011 | 13:26 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/5/2011 | 14:06 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/6/2011 | 07:45 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1600-1900. |
| 8/6/2011 | 16:12 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/8/2011 | 02:10 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1600-2000. |
| 8/8/2011 | 13:57 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/8/2011 | 14:54 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/9/2011 | 02:04 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2200. |
| 8/9/2011 | 13:47 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/10/2011 | 01:20 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2000. |
| 8/10/2011 | 02:40 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500 2000 10 2011 |
| 8/10/2011 | 14:23 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/10/2011 | 16:03 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/15/2011 | 03:25 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1600-1900. |
| 8/15/2011 | 13:14 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/16/2011 | 00:53 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2100 |
| 8/16/2011 | 10:30 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1600-1800. |
| 8/16/2011 | 15:25 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/17/2011 | 00:52 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2100. |
| 8/17/2011 | 08:11 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 15-21 2500. |
| 8/17/2011 | 14:00 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/18/2011 | 00:09 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2000. |
| 8/18/2011 | 13:35 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/18/2011 | 15:25 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/19/2011 | 02:36 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-1900. |
| 8/19/2011 | 07:18 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-1900. |
| 8/19/2011 | 14:50 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/20/2011 | 03:05 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1400-2000. |
| 8/20/2011 | 13:20 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/21/2011 | 02:40 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2000. |
| 8/21/2011 | 12:35 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/22/2011 | 02:40 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2000. |
| 8/22/2011 | 13:20 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/22/2011 | 15:30 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/22/2011 | 17:57 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/23/2011 | 02:25 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-1900 |
| 8/23/2011 | 10:48 | ERCOT issued a Watch for projected reserve capacity shortage with no market solution available. RMR Resources will be committed. Watch ended at 19:03 |
| 8/23/2011 | 13:25 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/23/2011 | 15:15 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/24/2011 | 03:00 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500 -2000. |
| 8/24/2011 | 08:46 | ERCOT issued a Watch for projected reserve capacity shortage with no market solution available. RMR Resources will be committed. Watch ended at 20:01 |
| 8/24/2011 | 12:50 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/24/2011 | 14:00 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/25/2011 | 01:15 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2100. |
| 8/26/2011 | 02:34 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1700. |
| 8/26/2011 | 14:14 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/27/2011 | 02:02 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2000. |
| 8/27/2011 | 13:27 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/27/2011 | 15:00 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2000. |
| 8/27/2011 | 15:29 | ERCOT issued a Watch for PRC below 2500 MW. |
| 8/28/2011 | 00:06 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2000. |
| 8/28/2011 | 10:55 | ERCOT issued a Watch for projected reserve capacity shortage with no market solution available. RMR Resources will be committed. Watch ended at 04:45 (08/29). |
| 8/28/2011 | 13:43 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/29/2011 | 00:18 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-2000. |
| 8/29/2011 | 08:32 | ERCOT issued a Watch for projected reserve capacity shortage with no market solution available. RMR Resources will be committed. Watch ended at 20:09. |
| 8/29/2011 | 14:29 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |
| 8/31/2011 | 07:41 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 1500-1900. |
| 8/31/2011 | 09:19 | ERCOT issued a Watch for projected reserve capacity shortage with no market solution available. RMR Resources will be committed. Watch ended at 18:16. |
| 8/31/2011 | 15:41 | ERCOT issued an Advisory due to Physical Responsive Capability (PRC) below 3000 MW. |

Transmission Watch

08/09 14:54	ERCOT issued a Transmission Watch due to post contingency overloads.

Emergency Notice

None

Texas RE Event Analysis

08/14 15:47	ERCOT experienced the simultaneous loss of three 138 KV lines as a result of the 138 kV bus trip. There were thunderstorms in the area.

08/23 22:36	ERCOT experienced the simultaneous loss of 4 138 KV line and 1 69 kV line as a result of a 138 kV bus trip. There was a loss of 26 MW of load in the area.

08/27 12:45	ERCOT experienced the simultaneous loss of one 138 KV line and 3 generating units connected to the line.

| Forced | New_Forced | Maintenance | De-rated | Planned | Total Lost Capacity |
| --- | --- | --- | --- | --- | --- |
| 3438.293 | 684.5868191 | 558.3745876 | 599.753 | 658.376 | 5939.382952 |

All the values are in average MWs.