| NPRR Number | 382 | NPRR Title | Verbal RUC Committed or Decommitted Resources Report |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Approved |
| Date of Decision |  | September 20, 2011 |  |  |
| Effective Date |  | Upon system implementation. |  |  |
| Priority and Rank Assigned |  | Priority – High; Rank – 32.6 |  |  |
| Nodal Protocol Section Requiring Revision |  | 5.3, ERCOT Security Sequence Responsibilities |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) creates two new reports on the Market Information System (MIS) Secure Area for verbal Reliability Unit Commitment (RUC) commitments/decommitments, one following the Day-Ahead Reliability Unit Commitment (DRUC) and one following the Hourly Reliability Unit Commitment (HRUC) to stay consistent with the current RUC data delivery process. |  |  |
| Reason for Revision |  | To provide market transparency regarding all RUC commitments including the Verbal Dispatch Instruction (VDI) commitments. The reports will provide useful information that will help Market Participants pursue their business interests. This information is consistent with the information currently provided for commitments and decommiments both for HRUC and DRUC. This provides additional verbal RUC information for the RUC Study Period. |  |  |
| Overall Market Benefit |  | Provides a more complete RUC data set. |  |  |
| Overall Market Impact |  | Beneficial as it provides Market Participants access to additional RUC data. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR382 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 6/8/11, NPRR382 was posted. - On 6/23/11, PRS considered NPRR382. - On 7/14/11, an Impact Analysis and Cost Benefit Analysis were posted. - On 7/21/11, PRS considered the 6/23/11 PRS Report, Impact Analysis and Cost Benefit Analysis for NPRR382. - On 8/4/11, TAC considered NPRR382. - On 9/20/11, the ERCOT Board considered NPRR382. |  |  |
| PRS Decision |  | On 6/23/11, PRS unanimously voted to recommend approval of NPRR382 as submitted. All Market Segments were present for the vote. On 7/21/11, PRS unanimously voted to endorse and forward the 6/23/11 PRS Report, Impact Analysis and Cost Benefit Analysis for NPRR382 to TAC with a recommended priority of High and a rank of 32.6. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 6/23/11, the issue of confidentially of RUC disclosures was referred to WMS. On 7/21/11, ERCOT Staff reviewed the Cost Benefit Analysis for NPRR382 explaining that it was based on non-quantifiable benefits. It was stated that the benefit of NPRR382 was the transparency it provides. |  |  |
| TAC Decision |  | On 8/4/11, TAC unanimously voted to recommend approval of NPRR382 as recommended by PRS in the 7/21/11 PRS Report. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 8/4/11, there was no discussion. |  |  |
| Board Decision |  | On 9/20/11, the ERCOT Board approved NPRR382 as recommended by TAC in the 8/4/11 TAC Report. |  |  |

| Quantitative Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 | Will not contain deselected RUC data. |  |
|  | 2 | Will follow frequency and format of current HRUC and DRUC reports. |  |
|  | 3 | Two new MIS Secure Area links. |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | None. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Complete data set for RUC information. |  |
|  | 2 | Additional access to RUC data for Market Participants. |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Jackie Ashbaugh on behalf of the MIS User Group (MISUG) |
| E-mail Address | jashbaugh@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3937 |
| Cell Number |  |
| Market Segment | N/A |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sandra Tindall |
| E-Mail Address | stindall@ercot.com |
| Phone Number | 512-248-3867 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None. |  |

Comments

Please note that NPRR401, Clarification of Timing for a Generation Resource to be Considered Self-Committed, also proposes revisions to Section 5.3.

Proposed Protocol Language Revision

- 5.3	ERCOT Security Sequence Responsibilities

(1)	ERCOT shall start the Day-Ahead Reliability Unit Commitment (DRUC) process at 1430 in the Day Ahead.

(2)	For each DRUC, ERCOT shall use a snapshot of Resource commitments taken at 1430 in the Day-Ahead to settle Reliability Unit Commitment (RUC) charges.  For each Hourly Reliability Unit Commitment (HRUC), ERCOT shall use a snapshot of Resource commitments from each Qualified Scheduling Entity’s (QSE’s) most recently submitted Current Operating Plan (COP) before HRUC execution to settle RUC charges.

(3)	For each RUC process, ERCOT shall:

(a)	Execute the Security Sequence described in Section 5.5, Security Sequence, Including RUC, including:

(i)	Validating Three-Part Supply Offers, defined in Section 4.4.9.1, Three-Part Supply Offers; and

(ii)	Reviewing the Resource commitment recommendations made by the RUC algorithm; and

(b)	Post to the Market Information System (MIS) Secure Area, all Resources that were committed or decommitted by the RUC process including verbal RUC commitments and decommitments;

(c)	Post to the MIS Public Area, all active and binding transmission constraints (contingency and overloaded element pair information where available) used as inputs to the RUC; and

(d)	Issue Dispatch Instructions to notify each QSE of its Resource commitments or decommittments.

(4)	ERCOT shall provide each QSE with the information necessary to pre-validate their data for DRUC and HRUC including:

(a)	Publishing validation rules for offers, bids, and trades; and

(b)	Posting any software documentation and code that is not Protected Information to the MIS Secure Area within five Business Days of receipt by ERCOT.