| NPRR Number | 327 | NPRR Title | State Estimator Data Redaction Methodology |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Tabled |
| Date of Decision |  | September 22, 2011 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 3.20, Process for Redacting State Estimator Output for Real-Time Publication (new) 3.20.1, Methodology For Redaction of State Estimator Data (new) 3.20.2, Methodology of Identification of Redacted Load (new) 6.5.7.1.13, Data Inputs and Outputs for the Real-Time Sequence and SCED |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) inserts language to define a redaction methodology for the posting of State Estimator data. |  |  |
| Reason for Revision |  | In Public Utility Commission of Texas (PUCT) Project No. 38470, Confidentiality of Electric Generation Information in the Electric Reliability Council of Texas Region, the PUCT amended P.U.C. Subst. R. 25.505, Resource Adequacy in the Electric Reliability Council of Texas Power Region, to address the timing for release of transmission system information contained in State Estimator reports. As part of that amendment, subsection (f)(3)(E)(ii) of P.U.C. Subst. R. 25.505 states the following: “Concurrently, in conjunction with the Independent Market Monitor and the commission Staff, ERCOT, through its stakeholder process, shall develop protocols that detail, at a minimum, the methodology, duration, and posting requirement of a redacted version of the State Estimator data. The redacted report methodology developed through the stakeholder process shall be completed within 90 days of the start of the nodal market.” ERCOT, in cooperation with the Independent Market Monitor (IMM) and PUCT Staff, submits this NPRR as a cost effective solution for redaction of the State Estimator data. |  |  |
| Overall Market Benefit |  | Data increases grid operations transparency to the market. |  |  |
| Overall Market Impact |  | Redaction system. |  |  |
| Consumer Impact |  | None. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR327 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 2/22/11, NPRR327, a preliminary Impact Analysis and a preliminary Cost Benefit Analysis were posted. - On 3/24/11, PRS considered NPRR327. - On 7/18/11, WMS comments were posted. - On 7/21/11, Morgan Stanley comments were posted. - On 7/21/11, PRS again considered NPRR327. - On 8/12/11, ERCOT comments were posted. - On 8/18/11, PRS again considered NPRR327. - On 9/19/11, an Impact Analysis was posted. - On 9/21/11, a Cost Benefit Analysis was posted. - On 9/21/11, a second set of ERCOT comments were posted. - On 9/22/11, PRS considered the 7/21/11 PRS Report, Impact Analysis, and Cost Benefit Analysis for NPRR327. |  |  |
| PRS Decision |  | On 3/24/11, PRS unanimously voted to table NPRR327. All Market Segments were present for the vote. On 7/21/11, PRS unanimously voted to recommend approval of NPRR327 as amended by the 7/21/11 Morgan Stanley comments and as revised by PRS. All Market Segments were present for the vote. On 8/18/11, PRS unanimously voted to table NPRR327. All Market Segments were present for the vote. On 9/22/11, PRS unanimously voted to table NPRR327. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 3/24/11, concerns were raised regarding the two day delay in posting a redacted version of the State Estimator data. It was stated that NPRR327 is being discussed by the State Estimator Task Force. On 7/21/11, the 7/21/11 Morgan Stanley comments were discussed. ERCOT Staff requested that language be reinstated for ERCOT to develop a publish list of State Estimator data not included in the approved redacted list as new elements are redacted by default. Additional edits were made to clarify the language to assist in preparation of the Impact Analysis. On 8/18/11, ERCOT Staff requested additional time to complete the Impact Analysis citing that further information is needed related to the conceptual business process for redaction. ERCOT Staff stated that the quarterly review and the requests to remove and repost elements of the State Estimator data will have an impact on ERCOT resources. Market Participants offered that although they do not believe there will be frequent requests for the removal of elements, they are committed to re-evaluating the process as needed and if quarterly reviews become burdensome, then can look at extending the time between reviews. It was also recommended that when completing the Cost Benefit Analysis, the obligation to comply with P.U.C. Subst. R. 25.505 should be included. On 9/22/11, there was discussion regarding the cost effectiveness of the proposed redaction methodology that would result in ongoing operational requirements and two Full Time Employees (FTEs). There was agreement to refer the issue to WMS and the State Estimator Task Force (SETF) in order to review the redaction methodology and identify a more cost effective method while still protecting confidential information. |  |  |

Quantitative Impacts and Benefits

| Assumptions | 1 | In no event shall Resource output levels be disclosed in Real-Time. |  |
| --- | --- | --- | --- |
|  | 2 | In no event shall offer curves be disclosed in Real-Time. |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | Redaction system. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Provides valuable Information. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Alex Sills |
| E-mail Address | asills@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3128 |
| Cell Number |  |
| Market Segment | N/A |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sandra Tindall |
| E-Mail Address | stindall@ercot.com |
| Phone Number | (512) 248-3867 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| WMS 071811 | Endorsed NPRR327 as revised by WMS. |
| Morgan Stanley 072111 | Proposed stylistic edits. |
| ERCOT 081211 | Requested additional time to complete the Impact Analysis. |
| ERCOT 092111 | Provided the assumptions used by ERCOT in developing the Impact Analysis report for NPRR327; and clarification of the language. |

Comments

Please note that NPRR360, Summary Report of HDL and LDL, also proposes revisions to Section 6.5.7.1.13.

Please also note that the baseline Nodal Protocol language in Section 6.5.7.1.13 has been updated due to the unboxing of NPRR329, Security Classification Changes for Extracts/Reports, in the May 18, 2011 Nodal Protocols.

Proposed Protocol Language Revision

3.20	Process for Redacting State Estimator Output for Real-Time Publication

(1)	State Estimator data that is published on the ERCOT Market Information System (MIS) shall exclude information (including but not limited to, voltages, transmission flows and transformer flows) from which resource-specific output levels or offer curves could continually and systematically be derived.

(2)	The appropriate Technical Advisory Committee (TAC) subcommittee shall be assigned to develop a list of State Estimator elements including Loads to be redacted using the methodology and reviewed for approval at the scheduled February, May, August, and November TAC meetings.

(3)	ERCOT shall develop a publish list of State Estimator data not included in the approved redacted list as described in Section 6.5.7.1.13, Data Inputs and Outputs for the Real-Time Sequence and SCED.

(4)	Distribution Service Providers (DSPs) and Transmission Service Providers (TSPs) will have access to all State Estimator data Real-Time and the data will not be subject to the redaction methodology as described in Section 3.20.1, Methodology of Quarterly Redaction of State Estimator Data.

- 3.20.1	Methodology For Redaction of State Estimator Data

(1)	This method seeks to introduce at least two transmission flow unknowns at, or near, every generation plant and Load station, as identified in Section 3.20.2, Methodology of Identification of Redacted Load, electrical connection.

(2)	The power flow from the following transmission elements shall be redacted:

(a)	Generator step up transformers;

(b)	Radial lines serving generating plants;

(c)	Transformers serving redacted Load; and

(d)	Radial lines serving redacted Load.

(3)	The power flow from the transmission elements determined using any of the following methods shall be redacted:

(a)	A transmission branch path that provides connection from the plant generation bus or redacted Load to another plant generation bus or redacted Load where the same owner exists on both ends;

(b)	A transmission branch path that provides connection from at least three independently owned generation plants or redacted Load to a common bus;

(c)	A transmission branch path that provides connection from plant generation bus to substation Load whose summer peak value is equal to 25% of generation plant capability; or

(d)	A transmission branch path that provides connection from redacted Load to substation Load whose summer peak value is equal to 25% of redacted Load.

3.20.2 	Methodology of Identification of Redacted Load

(1)	The following Loads will be considered Redacted Load:

(a)	Individual Load on Electrical Buses for station codes that have ten or fewer Electric Service Identifiers (ESI IDs) in DSP or TSP competitive areas;

(b)	Any Non-Opt-In Entity (NOIE) Load unless the NOIE informs ERCOT of the substation or Electrical Buses that can be published and do not conflict with Protected Information as described in Section 1.3.1, Restrictions on Protected Information.

6.5.7.1.13	Data Inputs and Outputs for the Real-Time Sequence and SCED

(1)	Inputs:  The following information must be provided as inputs to the Real-Time Sequence and SCED.  ERCOT may require additional information as required, including:

(a)	Real-Time data from TSPs including status indication for each point if that data element is stale for more than 20 seconds;

(i)	Transmission Electrical Bus voltages;

(ii)	MW and MVAr pairs for all transmission lines, transformers, and reactors;

(iii)	Actual breaker and switch status for all modeled devices; and

(iv)	Tap position for auto-transformers;

(b)	State Estimator results (MW and MVAr pairs and calculated MVA) for all modeled Transmission Elements;

(c)	Transmission Element ratings from TSPs;

(i)	Data from the Network Operations Model:

(A)	Transmission lines – Normal, Emergency, and 15-Minute Ratings (MVA); and

(B)	Transformers and Auto-transformers – Normal, Emergency, and 15-Minute Ratings (MVA) and tap position limits;

(ii)	Data from QSEs:

(A)	Generator Step-Up (GSU) transformers tap position;

(B)	Resource HSL (from telemetry); and

(C)	Resource LSL (from telemetry); and

(d)	Real-Time weather, from WGRs, and where available from TSPs or other sources.  ERCOT may elect to obtain other sources of weather data and may utilize such information to calculate the dynamic limit of any Transmission Element.

(2)	ERCOT shall validate the inputs of the Resource Limit Calculator as follows:

(a)	The calculated SURAMP and SDRAMP are each greater than or equal to zero; and

(b)	Other provision specified under Section 3.18, Resource Limits in Providing Ancillary Service.

(3)	Outputs for ERCOT Operator information and possible action include:

(a)	Operator notification of any change in status of any breaker or switch;

(b)	Lists of all breakers and switches not in their normal position;

(c)	Operator notification of all Transmission Element overloads detected from telemetered or State-Estimated data;

(d)	Operator notification of all Transmission Element security violations; and

(e)	Operator summary displays:

(i)	Transmission system status changes;

(ii)	Overloads;

(iii) 	System security violations; and

(iv)	Base Points.

(4)	Every hour, ERCOT shall post on the MIS Secure Area the following information:

(a)	Status of all breakers and switches used in the NSA except breakers and switches connecting Resources to the ERCOT Transmission Grid;

(b)	All binding transmission constraints and the contingency or overloaded element pairs that caused such constraint; and

(c)	Shift Factors by Resource Node.

(5)	Sixty days after the applicable Operating Day, ERCOT shall post on the MIS Secure Area, the following information:

(a)	Hourly transmission line flows and voltages from the State Estimator, excluding transmission line flows and voltages for Private Use Networks; and

(b)	Hourly transformer flows, voltages and tap positions from the State Estimator, excluding transformer flows, voltages, and tap positions for Private Use Networks.

(6)	Notwithstanding paragraph (5) above, ERCOT, in its sole discretion, shall release relevant State Estimator data less than 60 days after the Operating Day if it determines the release is necessary to provide complete and timely explanation and analysis of unexpected market operations and results or system events including, but not limited to, pricing anomalies, recurring transmission congestion, and system disturbances.  ERCOT’s release of data under this paragraph shall be limited to intervals associated with the unexpected market or system event as determined by ERCOT.  The data release shall be made available simultaneously to all Market Participants.

(7)	Notwithstanding paragraph (5) above, ERCOT shall post a redacted version of the State Estimator data every five minutes to the MIS Secure Area with a validation check if State Estimator does not reach a valid solution.  ERCOT shall only disclose State Estimator data for elements in the publish list as described in Section 3.20, Process for Redacting State Estimator Output for Real-Time Publication.  If a Market Participant determines that publishing an element of the State Estimator data, as described in Section 3.20, allows systematic and continual derivation of Resource-specific output levels and Resource Status, the Market Participant may request ERCOT remove the element from the publish list and the request will be posted on the MIS Secure Area.  ERCOT will remove the report as soon as practicable and review the element according to Section 3.20.1, Methodology For Redaction of State Estimator Data.  ERCOT shall validate the request as soon as practicable, and repost the element as soon as it is determined that it no longer meets the redaction criteria.  As soon as practicable, ERCOT shall resume posting the State Estimator Data.  .

(8) 	Every five minutes, ERCOT shall post on the MIS Secure Area from the State Estimator, individual Load on Electrical Buses utilizing the methodology described in 3.20.2, Methodology of Identification of Redacted Load, sum of Load in each Load Zone and total Load on Electrical Buses in the ERCOT System.

(9)	Every hour, ERCOT shall post on the MIS Public Area, the sum of ERCOT generation, and flow on the DC Ties, all from the State Estimator.