This Transmission Element Naming Convention standard is in three parts but constitutes one standard:

Part 1, Connectivity Node Group-PSS/E Bus Name Standard;

Part 2, Naming Convention for Lines, Breakers and Switches; and

Part 3, Naming Convention for CIM Station ID Names.

Transmission Element Naming Convention Revision Process

Revisions to the Transmission Element Naming Convention shall be made according to the approval process as prescribed in Protocol Section 3.10.7.1, Modeling of Transmission Elements and Parameters.

Part 1

Connectivity Node Group-PSS/E Bus Name Standard

The 2 to 12 character PSS/E Bus Name in the Annual Planning Models and the attribute associated with a ConnectivityNodeGroup in ERCOT’s Common Information Model (CIM) must follow these requirements.

The following technical requirements must be followed:

- PSS/E Bus names shall only include uppercase alpha-numeric values (A to Z and 0 to 9),

- The only special character allowed is the underscore ( '_' ),

- No spaces are allowed, and

- Names must be unique within ERCOT.

This naming standard does not apply to the The Electrical Bus class in ERCOT’s CIM which represents an abstract market construct.  Appropriate sections in the ERCOT Network Modeling Guidelines provide additional clarification for naming instances in ERCOT’s CIM.

Part 2

Naming Convention for Lines, Breakers and Switches

Breaker and Switch names shall be unique within the same substation.  Appropriate sections in the ERCOT Network Modeling Guidelines provide additional clarification when naming various Transmission Elements in ERCOT’s CIM.

All other Transmission Elements representing the same Transmission Element in the Network Operation Model, Updated Network Operations Model, and CRR Network Model shall have the first 14 characters unique within the Transmission Element type within all of ERCOT.

Part 3

Naming Convention for CIM Station ID Names

The CIM Station ID shall be unique for all substations within ERCOT and shall be limited to 8 characters.

CIM Station ID’s are not used in the Annual Planning Model and CRR Network Model.