| NPRR Number | 394 | NPRR Title | Outage Reporting |
| --- | --- | --- | --- |
| Date |  | September 26, 2011 |  |
| Submitter’s Information |  |  |  |
| Name |  | Isabel Flores |  |
| E-mail Address |  | iflores@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-6531 |  |
| Cell Number |  | 512-565-9117 |  |
| Market Segment |  | N/A |  |

Comments

ERCOT has performed further analysis of the impact introduced by NPRR394 and recommends applying the same Outage reporting criteria for all Resources.  The language proposed in these comments should reduce the reporting error created by the current process and the process suggested by NPRR394

Revised Cover Page Language

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | Market Benefit: - Applies the same Outage reporting requirements for all Resources. |
|  | 2 |  |
|  | 3 |  |
|  | 4 |  |
|  | 5 |  |

Revised Proposed Protocol Language

- 2.1	Definitions

- Forced Derate

The portion of the Resource removed from service must exceed the greater of ten MW or two percent of its seasonal net maximum sustainable rating provided through the Resource registration process.  For Qualified Scheduling Entities (QSEs) representing Wind-powered Generation Resources (WGRs), the loss of a portion of the capacity shall be due to the unavailability of a portion of the equipment and shall not include capacity changes due to changes in wind speed at the WGR facility.

- 3.1.4.8	Deratings

The Resource Entity or its designee must enter material deratings that are expected to last more than 48 hours in the ERCOT Outage Scheduler.  .  For QSEs representing Wind-powered Generation Resources (WGRs), the loss of a portion of the capacity shall be due to the unavailability of a portion of the equipment and shall not include capacity changes due to changes in wind speed at the WGR facility.  For QSEs representing WGRs, the percentage calculation will be determined using the seasonal net maximum sustainable rating of the WGR.  ERCOT will consider the Resource’s capability as the lesser of the latest seasonal Net Dependable Capability test, or the resource registration process.