| NPRR Number | 394 | NPRR Title | Outage Reporting |
| --- | --- | --- | --- |
| Date |  | October 17, 2011 |  |
| Submitter’s Information |  |  |  |
| Name |  | Eric Goff on behalf of the Wholesale Market Subcommittee (WMS) |  |
| E-mail Address |  | egoff@reliant.com |  |
| Company |  | Reliant Energy Retail Services, LLC |  |
| Phone Number |  | 512-284-4908 |  |
| Market Segment |  | Not applicable |  |

Comments

At its October 12, 2011 meeting, WMS reviewed Nodal Protocol Revision Request (NPRR) 394.  After discussion, WMS voted to endorse NPRR394 as amended by the 10/12/11 Luminant Energy comments.  There was one opposing vote from the Cooperative Market Segment and one abstention from the Cooperative Market Segment.

Following the vote, WMS continued discussion of NPRR394 and considered further revisions.  After this continued discussion, WMS unanimously voted to table NPRR394.  Accordingly, WMS recommends that PRS table NPRR394 to permit further discussion by WMS.

Revised Proposed Protocol Language

- 2.1	Definitions

- Forced Derate

The portion of the Resource removed from service which exceeds the maximum of ten MW or 5% of its prior HSL for all Generation Resources.  For Qualified Scheduling Entities (QSEs) representing Wind-powered Generation Resources (WGRs), the loss of a portion of the capacity shall be due to the unavailability of a portion of the equipment and shall not include capacity changes due to changes in wind speed at the WGR facility.  For QSEs representing WGRs, the percentage calculation will be determined using the seasonal net maximum sustainable rating of the WGR.

- 3.1.4.8	Deratings

The Resource Entity or its designee must enter material deratings that are expected to last more than 48 hours in the ERCOT Outage Scheduler.  A derating is considered to be material when the Resource’s capability is reduced by the maximum of ten MW or 5% of its prior HSL for all Generation Resources.  For QSEs representing Wind-powered Generation Resources (WGRs), the loss of a portion of the capacity shall be due to the unavailability of a portion of the equipment and shall not include capacity changes due to changes in wind speed at the WGR facility.  For QSEs representing WGRs, the percentage calculation will be determined using the seasonal net maximum sustainable rating of the WGR.  ERCOT will consider the Resource’s capability as the lesser of the latest seasonal Net Dependable Capability test, or the resource registration process.