| PGRR Number | 011 | PGRR Title | Planning Criteria Clarifications and Enhancements To Narrow The Gap Between Operations and Planning |
| --- | --- | --- | --- |

| Date | October 21, 2011 |
| --- | --- |

| Submitter’s Information |  |
| --- | --- |
| Name | Rob Lane |
| E-mail Address | Robert.lane@luminant.com |
| Company | Luminant Energy |
| Phone Number | 214-875-8063 |
| Cell Number |  |
| Market Segment | Investor Owned Utility (IOU) |

Comments

Luminant Energy submits the following questions to ERCOT, pertaining to ongoing consideration of PGRR011, NPRR409, NOGRR078, and NPRR385, for the upcoming October 21, ROS hosted workshop.

- How big is the gap?

  - Define duration of an “insecure state” as described in Nodal Protocol Section 6.5.7.1.10 (i.e. predicted post contingency loadings in excess of 100% of the Emergency Rating of the Transmission Facility) that ERCOT is comfortable allowing in Real-Time operations.  To the degree that this answer is different for IROLs vs. SOLs, please answer for both.

Update:  Luminant Energy believes that ERCOT is still evaluating the answer to this question; however, in order to facilitate market participant discussion around answers to the remaining questions below, we have targeted two different time durations: 15 minutes (t+15) and 30 minutes (t+30).  The 15 minute duration would be consistent with ERCOT’s philosophy of requiring a 15 minute facility rating in order for Remedial Action Plans to be utilized, whereas, the 30 minute rating would be more consistent with NERC requirements for IROLs.

  - Which specific transmission constraints (by month) since Nodal Go-Live have experienced an “insecure state” of operations as defined in Nodal Protocol Section 6.5.7.1.10 for a continuous period of time that exceeds the answer(s) provided in 1a) above, and what was the highest post contingency loading levels experienced for each constraint.  Please be sure to include “inactive” constraints that were not enforced by ERCOT Operations in Real-Time as allowed by Nodal Operating Guide Section 2.2.2 (2) due to an Emergency Condition being declared, but were in actuality operating on a post contingency basis above 100% of the Emergency Rating of the Transmission Facility.

Update: Luminant Energy has processed the transmission constraint data posted to the ERCOT Nodal MIS since Nodal Go-Live (12/1/2010) through September 30, 2011 as “active contingencies” and in the attached files is only reporting, those where SCED was not able to resolve an “insecure state” for either a continuous 15 minutes or 30 minute duration.  Separately, Luminant Energy also examined the “in-active contingencies” constraints that were posted to the ERCOT Nodal MIS during EEA periods, to understand which constraints were actually operating on a post contingency basis above 100% of the Emergency Rating of the Transmission Facility (without an apparent RAP or MP in place that would resolve it); however, ERCOT was not enforcing these constraints so that it had maximum availability to all generation capacity as allowed by Nodal Operating Guide Section 2.2.2 (2).

When ERCOT was not operating in a state of EEA,

- “Insecure states” lasting 15 minutes or longer were experienced on:

  - 139 transmission facilities

  - Total duration of 327 hours

  - 466 episodes

  - Average duration of 42.1 minutes (ie. (327 / 466) * 60 minutes).

- “Insecure states” lasting 30 minutes or longer were experienced on:

  - 68 transmission facilities

  - Total duration of 256 hours

  - 209 episodes

  - Average duration of 73.5 minutes

Similarly, when ERCOT was operating in a state of EEA (~50 hours) post contingency loadings greater than 100% of the emergency rating of facilities were examined:

- 120 transmission facilities

- Average duration of 51.6 minutes

- Loadings as high as 185% were observed

Note: for the purposes of the statistics referenced during EEA events, both the total number of episodes reported, as well as, the total duration of episodes may be greater than would initially be intuitive if there were multiple contingencies that simultaneously resulted in post-contingency overloads for the same binding facility experiencing post-contingency overloads above 100% of its emergency rating.

  - To the degree that ERCOT expects that some portion of the “insecure states” of Operations listed in 1b) above, should not reasonably have been expected to be identified in the companion Planning studies (e.g. Transmission Planning or Outage Planning studies), please specify which constraints, for which time periods, for which reasons (e.g. loads in excess of 90th percentile, generator unavailability in excess of 90th percentile, etc…).

  - How much total estimated congestion cost (i.e. Shadow Price * Limit as currently reported in the Monthly System Planning Reports to ROS) have occurred since Nodal Go-Live and what percentage of costs occurred due to constraints operating in an “insecure state”, as defined in Nodal Protocol Section 6.5.7.1.10, reported in 1b) above?

Update: Luminant Energy has processed the transmission constraint data posted to the ERCOT Nodal MIS since Nodal Go-Live (12/1/2010) through September 30, 2011 as “active contingencies” and estimates that the total estimated congestion cost (i.e. Shadow Price * Limit as currently reported in the Monthly System Planning Reports to ROS) to be $588 million.

Of this total congestion cost of $588 million,

- $249 million (42%) occurred for constraints that were insecure for 15 minutes or longer;

- Whereas, $191 million (32%) occurred for constraints that were insecure for 30 minutes or longer.

Note: since “insecure states” are not constrained upon in SCED during an EEA as allowed for by Nodal Operating Guides Section 2.2.2 (2), the congestion figures reported above do not include any cost for these high system stress time periods, even though a constraint may have been insecure and incurring congestion cost prior to the EEA initiation.  The Holistic Solution being considered at TAC and the Board that encourages ERCOT to not relax constraints during emergency situations may impact these statistics on a going forward basis.

  - To the degree possible, please determine which, if any, of these specific “insecure states” of operation may have been associated with system conditions that involved transmission clearances.  A simplistic approach to this analysis will be acceptable, such as assuming that all “insecure states” that were occurring some time period other than during the time frame of 2:00 P.M. to 8:00 P.M. of July – August (e.g. when it is highly unlikely that a planned transmission outage would have been granted due to high system loads).

Update: Luminant Energy has utilized the simplistic approach described above of categorizing all “insecure states” with both a duration of 15+ minutes, as well as, 30+ minutes that occurred inside the time window of 2:00 P.M. to 8:00 P.M. during July and August (e.g. when it is highly unlikely that a planned transmission outage would have been granted due to high system loads) and categorized these not related to transmission clearances and thus potential Transmission Planning process issues.  Similarly, all other “insecure states” that occurred outside this time window were categorized as potentially related to transmission clearances since they were occurring at time periods when the system wasn’t at or near peak loading conditions and are thus potential Outage Planning / Execution process issues.  This categorization was performed both for the EEA periods and non EEA periods as described in our response to #1b) above.

For the time periods categorized as Transmission Planning the following findings were observed:

- Non EEA, “insecure states” lasting 15 minutes or more:

  - 27 transmission facilities

  - Total duration of 47 hours

  - 63 episodes

  - Average duration of 44.5 minutes

  - $44.2 million congestion cost (17.8% of “insecure state” total from #1d)

- Non EEA, “insecure states” lasting 30+ minutes:

  - 14 transmission facilities

  - Total duration of 37.0 hours

  - 26 episodes

  - Average duration of 85.4 minutes

  - $38.5 million congestion cost (20.2% of “insecure state” total from #1d)

- EEA, post-contingency loadings 100+%:

  - 73 transmission facilities

  - Average duration of 58 minutes

  - Loadings as high as 157% were observed

For the time periods categorized as Outage Planning / Execution the following findings were observed:

- Non EEA, “insecure states” lasting 15 minutes or longer:

  - 123 transmission facilities

  - Total duration of 280 hours

  - 404 episodes

  - Average duration of 41.6 minutes

  - $204.6 million congestion cost (82.1% of “insecure state” total from #1d)

- Non EEA, “insecure states” lasting 30+ minutes:

  - 59 transmission facilities

  - Total duration of 219.0 hours

  - 183 episodes

  - Average duration of 71.8 minutes

  - $152.1 million congestion cost (79.8% of “insecure state” total from #1d)

- EEA, post-contingency loadings 100+%:

  - 82 transmission facilities

  - Average duration of 41 minutes

  - Loadings as high as 185% were observed

- What are the main drivers of the gap?

  - For the constraints experiencing an “insecure state” as identified in 1b) above, that were not excluded by 1c) above, please determine if the following drivers may have been present:

    - Load – Real-Time load in excess of the 50th percentile temperature driven load for that specific time period.

    - Generation Availability - To the degree that generation unavailability (e.g. Planned Outages, Forced Outages, and operational deratings) was experienced in Real-Time operations that would have provided Transmission Facility loading relief, please identify each case where the shift factor aggregated unavailability of this generation exceeded the transmission loading relief requirement of Planning Guide Section 2, Credible Single Contingency for Transmission Planning (4). Does ERCOT perform the same G-1 / N-1 (with any other generation pre-emptively redispatched) studies in Outage Planning studies, which has historically been a long standing requirement of Nodal Operating Guides?  If not, why not?

    - Very Low Wind Conditions - Please identify each case where very low levels (e.g. near zero output) of wind generation in a wide area (e.g. West Texas) may have been a contributing factor.

    - Ancillary Services - Please identify which of these constraint loadings, if any, were aggravated in Real-Time Operations because some portion of the Generation Resources that could have theoretically provided Transmission Facility loading relief, and was assumed to do so in the respective Transmission Planning or Outage Planning study, but had been separately reserved by ERCOT (i.e. double counted) in the integrated Day Ahead Market to provide “up” Ancillary Service capacity (e.g. Responsive Reserve, Regulation Up, and Non Spinning Reserve).

    - Consistent Representation of Generic Constraints - For the generic constraints experiencing an “insecure state” (e.g. Valley import), please determine if different levels of transfer capability were being enforced in Real-Time Operations than were assumed in Transmission Planning or Outage Planning studies for the same operating conditions.

    - Dynamic Ratings - Please identify which of these constraint loadings occurred on Dynamically Rated transmission facilities that were experiencing ratings below the nominal rating.

    - Operator Actions Not Modeled in SCED – Please identify which of these constraints were approved by Outage Planning studies based on the development of an Operator Action that would be utilized during the duration of that outage (does ERCOT refer to these as Temporary Outage Action Plans or TOAPs?) to maintain reliability in a defined adverse operation condition (e.g. identical to a RAP), but were not being modeled in SCED.

  - For the constraints experiencing “insecure states” of operation as identified in 2a) above, please identify any other potential drivers of the gap between operation and planning than those examined in 2a) i) thru vii) above, that ERCOT believes should be addressed, such as, but not limited to:

    - Combined Cycle Train – does ERCOT believe that it would be helpful for determining if a “secure state” of operation is reasonably expected to exist in Real-Time for planning studies (both Transmission and Outage) to consider each feasible configuration of a Combined-Cycle Train while fulfilling the G-1 / N-1 study requirements of Planning Guide Section 2, Credible Single Contingency for Transmission Planning (4)?

    - Autotransformer – given that the size of many autotransformers in the ERCOT system have become comparable in size (e.g. 750 MVA to 1,000 MVA in some cases) to the larger Generation Resources in the system, combined with the fact that some long-term outages can last multiple months and potentially longer than one year, does ERCOT believe that it would be helpful for determining if a “secure state” of operation is reasonably expected to be available in Real-Time via planning studies similar to those of Planning Guide Section 2, Credible Single Contingency for Transmission Planning (4)?

Update:  Luminant Energy has not yet had a chance to perform analysis that answers each of the questions raised above; however, we have performed analysis to examine the potential for the planning enhancements embodied in PGRR011 for Transmission Planning to serve in closing the Planning vs. Operations gap as described in detail for our response to questions #3 below.

- What are the appropriate solutions to close the gap(s)?

  - Load -  To the degree that load conditions occurred as examined in 2a) i) above, what percentile temperature driven load forecast does ERCOT believe reflects “Good Utility” practice in:

    - Transmission Planning Studies – for identifying constraints associated with “reasonable variations of load level” that TSPs involved should plan to resolve through provision of Transmission Facilities, RAPs, SPSs, or other means as appropriate in order to fulfill the obligations in ERCOT Planning Guide Section 4.1.1.1?

    - Outage Planning Studies – for identifying constraints associated with “reasonable variations of load level” that has historically been a long standing requirement of Nodal Operating Guide Section 5.3 (2) prior to being deleted on 9/30/11 by NOGRR058?

  - Generation Availability - To the degree that generation unavailability (e.g. Planned Outages, Forced Outages, and operational deratings) as examined in 2a) ii) above, has been experienced in Real-Time operations where the shift factor aggregated unavailability of this generation exceeded the loading relief requirement of Planning Guide Section 2, Credible Single Contingency for Transmission Planning (4), (e.g. constraints in the middle of the system where 20,000 to 30,000 MW of generation have an unloading effect are in excess of the single largest unit currently being studied), how does ERCOT recommend closing this gap?  Is there a legitimate reason for utilizing a different (higher) generation availability assumption in transmission adequacy studies than for resource adequacy studies, where 90th percentile is currently used?

  - Wind – Given the uncontrollable nature of wind resources (certainly it can’t be RUC’d on when the wind isn’t blowing) as examined in 2a) iii) above, does ERCOT believe that it would represent “Good Utility” practice to study conditions of no wind generation across both a local and wide area for both Outage Planning and Transmission Planning studies to ensure that a “secure state” of operations is reasonably expected to occur during those conditions?  If not, what guidelines would represent “Good Utility” practice in this area?

  - Ancillary Services – Does ERCOT believe that it is beneficial to utilize a process similar to that outlined in PGRR011 that John Dumas helped to develop, to minimize the historical double counting in many cases, of generation capacity in planning studies (where ERCOT Operations doesn’t have a process in place to allow for location specific deployments of Ancillary Services) being utilized for congestion relief, that in operations is reasonably expected to be reserved for A/S?  If not, is there a better approach to avoid this continued double counting of generation capacity issue?

  - Consistent Representation of Generic Constraints - To the degree that generic constraints are being applied in Real-Time operations, but not in planning, does ERCOT see any value in not closing this gap from a consistency standpoint?

  - Dynamic Ratings - Does ERCOT believe that it would be appropriate to utilize the same percentile (e.g. 90th) temperature driven dynamic line rating in planning studies as is recommended by ERCOT in 2a) above, for load sensitivities?  If not, why not given that the same high degree of correlation between temperature, load, and dynamic line ratings?

  - Operator Actions Not Modeled in SCED – Does ERCOT believe that it would minimize the potential false reporting of “insecure states” of operation in some cases by SCED, where the Outage Planning process has resulted in the development of a temporary RAPs (TOAPs?), to model all these in SCED the same way that Nodal Protocol Section 6.5.7.1.10 (3) (a) and Nodal Operating Guides Section 4.3.1 and 4.3.2 require for SPSs and RAPs?  If not, please explain why.

Update:  Luminant Energy has selected a representative ½ dozen (some 345 kV, 138 kV, and 69 kV spread across the system) constraints that were segmented into the Transmission Planning category in our response to question #1e) above and attempted to apply all of the elements of PGRR011 to test out the effectiveness of these proposed criteria enhancements and clarifications to closing the Planning vs. Operations gap.  Although Luminant Energy selected representative constraints from the Transmission Planning category to examine (primarily due to the ease of selecting an SSWG base case for the appropriate study period without having to apply the myriad of transmission outages that may have been present during another time period) the effectiveness of PGRR011 proposed solutions, it is our belief that since these same enhancements / criteria clarifications are embodied in NPRR409, that a detailed study of a representative sample of those constraints would have similar findings as long as all of the appropriate system outages were accurately represented in the studies.

For the ½ dozen constraints examined, the maximum post contingency loading at t+30 minutes ranged between 101% and 108% of the Emergency Rating of the Transmission Facility as reported on the Nodal MIS as described previously.  Individual sensitivities were performed to assess the impact of proposed PGRR011 / NPRR409 changes:

- 90th Percentile Weather Zone Load

- 90th Percentile Generation Unavailability net of Largest Unit

- North / West wind reduced to zero

- Generator Capacity reserved for Ancillary Services

- Dynamic Line deratings (Ops) relative to Nominal (Planning) ratings

- Full Combined-Cycle Train in all cases vs. current methodology

- Long-term auto transformer outage (assuming spare equipment strategy doesn’t exist within a 12 week window)

The table attached examines the collective impact of the first five sensitivities above as a group (with individual contributions shown), since given their nature (e.g. extreme weather driven load, high generator unavailability, no / low wind conditions, generator capacity reservation for A/S, dynamic line deratings) these are each reasonably likely to have actually occurred at least individually, if not jointly, during the limited historical time period where the “insecure states” have been examined (i.e. July & August 2011 from 2:00 P.M. to 8:00 P.M.)  The impact of the full Combined-Cycle Train and autotransformer outage are also illustrated; however, it is important to keep in mind if either or/both of these two drivers weren’t actually being experienced in the small historical time period where these insecure constraints are being examined, then they will not be a key driver in explaining the Planning vs. Operations gap for this data set, but may be in the future events.

When considering the impact of the first five sensitivities above as a group for the ½ dozen constraints examined, the maximum post contingency loading at t+30 minutes which in actual history ranged from 101% to 108% dropped to a range of 88% to 95%, suggesting that had these five sensitivities been applied in the Transmission Planning process, then all 6 of these Transmission Planning related insecure states in Real Time would not have occurred during the historical period being examined.  Additionally, the sensitivities of #6 and #7 above, to the degree that each of them did not already occur in the small historical time period being examined, are quite useful for illustrating the degree to which post contingency loadings would be even further increased above their Emergency Rating than was already experienced, if either or both of these events were to occur in Operations in the future.  The impact of the Full Combined-Cycle Train sensitivity has the potential to contribute between 0% to 18% increased loading, while the autotransformer outage sensitivity has the potential to contribute between 3% and 25% increased loading.

Overall, it is Luminant Energy’s belief that this review of ½ dozen actual real life examples of “insecure states” of operation this past summer that were occurring outside of the EEA time horizon, illustrate the potential for the concepts encompassed within PGRR011 and NPRR409 to be materially beneficial in closing the gap between Planning and Operations.

- Why are “insecure states” of operation (e.g. SCED Irresolvability) apparently more common, or more of a concern, today than previously?

  - Does ERCOT foresee that the “insecure state” conditions identified in 1e) above, which is where the majority of these conditions are occurring, may have been falsely reported in many cases by SCED in Real-Time due to TOAPs that were developed in the Outage Planning process, but not modeled in Nodal SCED for operations purposes such as outlined in 3g) above being a new problem that was not present in Zonal due to how Local Congestion was managed?

Update:  Luminant Energy understands from discussion with ERCOT that TOAPs (i.e. Temporary Outage Action Plan) in many cases are being developed as a part of the Outage Planning process, but are not being loaded into the EMS and MMS because this would potentially require up to daily database loads to accomodate.  It is our understanding that supporting daily database loads would require non-trivial increases to support staff to accomplish this function (i.e. modeling all TOAPs in all market systems, including SCED, when they are being relied upon by Operations to manage a post-contingency overload during a planned transmission outage).  Given that the intent of the Nodal Protocols, as stated in Section 3.10.7.4 is for all SPSs and RAPs used by ERCOT to be defined in the Network Operations Model and reflected in the market pricing, and that TOAPs are an undefined Protocol term (unlike SPS and RAP, TOAP is not a Protocol defined term) for short-term RAPs that have been developed by ERCOT to support a transmission clearance, doesn’t it make sense as a market to find a way to correct this gap between Planning and Operations?  Especially, when considering that the Outage Planning / Execution category as Luminant Energy has defined it in #1e) above accounts for $204.6 million in congestion cost (82.1% of all the congestion associated with insecure states last 15+ minutes)?

There could be several options for bringing all active TOAPs into the Network Operations Model, at least two of which are:

- Daily database loads to reflect all TOAPs that are active for that specific day only

- Utilize the current schedule of database loads every two weeks, however, some TOAPs would be defined in the model before they are absolutely required and beyond when they are absolutely needed, which should not be a problem.

If either of these two options are pursued, it may not be possible for TOAPs that were developed to accommodate last minute outage request to be reflected in the database loads even if option #1 were ultimately selected.  However, this remaining gap could potentially be addressed by ERCOT only developing TOAPs to support transmission outage requests that were submitted far enough in advance to allow them to be developed and modeled in the Network Operations Model, consistent with Nodal Protocol intent.

Another improvement opportunity may be to include leaving the TOAP associated contingency as “inactive” by not affirmatively placing it into “active” mode as would be the normal case if a TOAP did not exist.

  - Does ERCOT believe that the process recently implemented to develop load levels for use in the SSWG base cases (see slide 3 of the September 15th Joint PLWG / CMWG update to ROS for more details), is contributing to, or has the potential to contribute to, under studying of load conditions in transmission planning studies going forward, especially without clear guidance of what “reasonable variations of load level” mean in the context of Planning Guide Section 4.1.1.1?  If not, why not?

Update:  Luminant Energy continues to believe as outlined on slide 3 of the September 15, 2011, Joint PLWG / CMWG Assignment update to ROS presentation, that changes instituted in the last few years to the SSWG case building process to bring alignment of the SSWG basecase loads to the ERCOT-wide peak load forecast, without simultaneously providing guidance to the Transmission Planner and ERCOT / PUCT reviewer regarding what “reasonable variations of load” means for planning studies, may be a key driver in why “insecure states” are becoming more common and must be systematically addressed.

  - To the degree that ERCOT’s answer to 1e) is longer than one SCED interval (e.g. ~5 minutes), does ERCOT believe that SCED pricing congestion as “irresolvable” (i.e. at the respective shadow price cap of the various post-contingency transmission constraints) before the time period defined in 1e) has elapsed, creating a false perception of what constitutes a reliability concern, as well as, charging the market for ‘reliability violating’ levels of congestion before the time period defined in 1e) above has elapsed?  If so, what solutions might potentially resolve this over statement of ‘reliability violations’ by SCED?

Update:  Would it be possible to create a subroutine for SCED to track how long a transmission facility has been operating in excess of 100% of its Emergency Rating and adjust the Shadow Price cap of the constraint to tier its way into the current shadow price cap levels over a 15 minute (or 30 minute) time period.  In the alternative, would it be possible to create a ramped shadow price cap that had tiered shadow price caps as a function of how overloaded the transmission facility is (e.g. lower shadow price cap for a facility at 101% of its Emergency Rating and higher for a 120% overload).

- Aren’t most of the transmission facilities that experienced “insecure states” identified in #1e) above as Transmission Planning related (i.e. occurred between 2PM to 8PM during July and August), already identified in the Five-Year Plan for upgrade, perhaps in a later year?

- Update:  Luminant Energy has observed that:

  - 27 Transmission Planning constraints (as defined in #1e) in summer 2011 experienced an insecure state for 15 or more minutes

    - 21 out of the 27 were not identified in the latest 5-Year Plan

      - Apparently there are key drivers of grid insecurity that are missing from the Reliability Planning process and causing this gap between Planning and Operations.

      - As outlined in our response to #3 above, Luminant Energy believes that PGRR011 will enable ERCOT and the TSPs to close that gap.

    - 6 of the 27 were identified in the 5 Year Plan

      - 4 as 2011 projects

        - West Denton – Jim Christal 138 kV Line

          - 5-Year Plan called for a MP by summer 2011

          - No project, RAP, or Mitigation Plan implemented prior to summer 2011

        - North Denton – Jim Christal 138 kV Line

          - 5-Year Plan called for a MP by summer 2011

          - No project, RAP, or Mitigation Plan implemented prior to summer 2011

        - Apollo – East Richardson 138 kV Line

          - 5-Year Plan called for a MP by summer 2011

          - No project, RAP, or Mitigation Plan implemented prior to summer 2011

        - Odessa – Odessa North 138 kV Line

          - 5-Year Plan called for a MP by summer 2011

          - No project, RAP, or Mitigation Plan implemented prior to summer 2011

      - 2 as 2012 projects

        - Lon Hill – Robstown 69 kV Line

          - 5-Year Plan called for a project by 2012

        - Troup – Walnut Grove 138 kV Line

          - 5-Year Plan called for a project by 2012

- Were the project upgrades and RAPs / MPs identified in the 2010 Five-Year Plan as needed for Summer 2011 implemented on time to avoid “insecure states” of operation as intended?  If not, are there unnecessary barriers in this process that can be eliminated?

  - The 2010 5-Year Plan, which was published on March 23, 2011, identified the reliability need for the following by summer 2011:

    - 26 Transmission Projects

      - 1 project was completed prior to summer 2011

        - Pegasus – South Pegasus 138 kV line conversion

    - 21 transmission facilities required the development of RAPs or MPs to protect them since there was insufficient time to build the project prior to summer 2011

      - 6 RAPs / MPs were completed prior to summer 2011

        - Victoria – N. Victoria 69 kV Line

        - Odessa Basin Sw. – Odessa Basin 69 kV Line

        - Odessa Basin – Goldsmith 69 kV Line

        - Goldsmith – Ector Shell 69 kV Line

        - Ector Shell – Holt 69 kV Line

        - Holt – Emma Tap 69 kV Line

  - The 2010 5-Year Plan identified 14 reliability projects and five operating configuration changes required to be in-service by summer 2012:

    - 7 projects are scheduled to be completed on-time, and presumably the one terminal equipment project will be completed on-time.

    - 2 projects are scheduled to be completed past 2012 and there is currently no RAP or MP developed to protect either facility.

    - 1 project is in the “Planning Stage” and there is currently no RAP or MP developed.

    - 3 projects required by summer 2012 and listed in the TPIT 5-Year Plan tab, have no TSP acknowledgement as all the TSP data fields within TPIT have been left blank

  - Potential barriers in the process that contributed to the TSPs being unable to complete 25 out of 26 transmission projects that were required for 2011:

    - The 2010 Five-Year Plan was published nearly 3 months after 2010

      - Suggested solution

        - Set an April 30th deadline for publishing the 5-Year Plan for that year (e.g. April 30, 2012 for the 2012 Five-Year Plan)

        - This schedule could be codified in the Protocols to ensure that it is strictly adhered to

    - 17 out of the 26 projects needed by summer 2011 had been newly identified in the 2010 Five-Year plan as needed in 2011 (i.e. not  previously identified by the 2009 Five-Year plan as needed in 2011)

      - RPG review required for many of these projects will take 6 to 9 months in many cases for end-to-end cycle time

      - Suggested solutions

        - Modify the RPG approval process to exempt all Five-Year Plan reliability endorsed projects costing less than $50 million

        - Grant ‘critical’ RPG review status to any remaining project endorsed by the 5-Year Plan with a need date of less than 36 months away

        - Create reporting metric via the monthly System Planning Report (on a quarterly basis tied to TPIT publishing schedule) to ROS of:

          - All specified projects in the Five-Year Plan that have missed or are tracking toward missing their target need date

  - Apparently there must be some barriers to the development of RAPs and MPs as specified by the Five-Year Plan that needs to be addressed:

    - 15 of the 21 RAPs / MPs specified by the Five-Year Plan weren’t completed for summer 2011

    - Suggested solutions

      - Create reporting metric via the monthly System Planning Report to ROS of all specified projects / RAPs / MPs that have not been completed as specified by the Five-Year Plan

      - Create a tighter tracking process for RAPs / PCAPs / MPs

        - Operating procedures developed to mitigate a reliability issue identified in the Five-Year Plan, should not be retired until the project has been completed to replace it or a subsequent Five-Year Plan finds it no longer necessary

        - Create a folder on the Nodal MIS for all retired RAPs / PCAPs / MPs

          - The reason and date of retirement should be captured