| NPRR Number | 414 | NPRR Title | UFE Calculation Clarifications |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | October 20, 2011 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 11.4.6, Unaccounted for Energy Calculation and Allocation 11.4.6.1, Calculation |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) clarifies language in the Unaccounted for Energy (UFE) calculation. |  |  |
| Reason for Revision |  | This NPRR will provide clarification in the Protocols in regards to the utilization of Direct Current Tie (DC Tie) Schedules in the UFE calculations and remove “metered” from the UFE definition since non-metered Loads are also included in the UFE calculation. |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - On 9/12/11, NPRR414 and the associated Impact Analysis were posted. - On 10/20/11, PRS considered NPRR414. |  |  |
| PRS Decision |  | On 10/20/11, PRS unanimously voted to recommend approval of NPRR414 as revised by PRS. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 10/20/11, ERCOT staff explained that the revisions to NPRR414 align the Protocols with current ERCOT business practices. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | Provide clarity in Protocols. |
|  | 2 | No changes to existing ERCOT practices or systems. |
|  | 3 |  |
|  | 4 |  |
|  | 5 |  |

| Sponsor |  |
| --- | --- |
| Name | Don Tucker |
| E-mail Address | dtucker@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3913 |
| Cell Number | 512-560-2378 |
| Market Segment | Not applicable. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sonja B. Mingo |
| E-Mail Address | smingo@ercot.com |
| Phone Number | 512-248-6463 |

Proposed Protocol Language Revision

## DEFINITIONS

Unaccounted for Energy (UFE)

The difference between total Load for each Settlement Interval, adjusted for applicable Distribution Losses and Transmission Losses, and total ERCOT generation.

11.4.6	Unaccounted for Energy Calculation and Allocation

The DAS shall adjust the net loss adjusted Load for each aggregated group for UFE.  The Data Aggregation process will calculate the difference between net loss adjusted Load for the entire ERCOT System, which has been adjusted for Distribution Losses and Transmission Losses, and the total system Load (generation) in order to determine the total UFE.  The calculated UFE for each Settlement Interval is then allocated to Loads.  For the purpose of the UFE calculation, scheduled flow out of ERCOT on a Direct Current Tie (DC Tie) will be deemed as Load, and scheduled flow into ERCOT on a DC Tie will be deemed as generation.

11.4.6.1	Calculation of ERCOT-Wide Unaccounted For Energy

The DAS will calculate ERCOT-wide UFE as the difference between the total ERCOT generation and the total Load, adjusted for losses in ERCOT during each Settlement Interval.  UFE may be positive or negative in any single Settlement Interval.

UFE i (MWh)	= ERCOT Generation i Total – ERCOT Net Loss Adjusted Load i Total