| NPRR Number | 351 | NPRR Title | SCED Look-Ahead Step 1: Pricing: Calculate Non-Binding Prices and Basepoints for Initial Research into SCED Look-Ahead and allow Consumers to have a Forward Price Projection (formerly “Calculate and Post Projected Non-Binding LMPs for the Next 15 Minutes”) |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Remanded |
| Date of Decision |  | November 3, 2011 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 6.3.2, Activities for Real-Time Operations 6.5.7.3, Security Constrained Economic Dispatch |  |  |
| Revision Description |  | Using the inputs specified in this Nodal Protocol Revision Request (NPRR), ERCOT will calculate prices and base points over the next hour, using an hour-long optimization instead of a five-minute optimization. These calculated prices and base points will initially be non-binding, although it is anticipated that these will become binding as Security-Constrained Economic Dispatch (SCED) Look-Ahead progresses in future phases. Not covered by this NPRR are the larger pieces of SCED Look-Ahead, like Real-Time commitment of generators and Load Resources (possibly replacing or eliminating the Non-Spinning Reserve (Non-Spin) product), Real-Time co-optimization of energy and Ancillary Services (including determining when the co-optimization occurs), whether to charge make-whole charges and to whom they should be charged to, and how these revisions will support the overall market design and Resource adequacy requirements. |  |  |
| Reason for Revision |  | ERCOT expects this NPRR will be the first step of a multi-phase implementation of SCED Look-Ahead, with the results of this NPRR primarily being used for research purposes to determine how to best calculate prices and base points for future periods. In addition, the ERCOT Steel Mills and at least one retailer have expressed a strong desire to use the results to make Real-Time decisions on how to consume power in Real-Time based on prices. Primarily, this will be used to determine the accuracy (and improve it, if possible) of the SCED Look-Ahead Real-Time Dispatch process. At issue will be whether the improved algorithm delivers better results than the current one. There are a number of areas to analyze, including: - The predictability of the wind generation over the next hour. - How near-term Load growth and generator Dispatch will create binding contingencies over the next hour. - What sort of uplift costs will be created by the process, if any, once it becomes binding. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR351 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 4/13/11, NPRR351 was posted. - On 5/19/11, ERCOT comments were posted. - On 5/19/11, PRS considered NPRR351. - On 6/9/11, Steel Companies comments were posted. - On 6/23/11, PRS again considered NPRR351. - On 7/7/11, the ERCOT Steel Mills appeal was posted. - On 8/3/11, Bob Wittmeyer comments were posted. - On 8/4/11, TAC considered the ERCOT Steel Mills appeal. - On 8/18/11, PRS again considered NPRR351. - On 9/20/11, Trefny-Crockett comments were posted. - On 9/20/11, a preliminary Impact Analysis was posted. - On 9/21/11, a second set of ERCOT comments were posted. - On 9/22/11, PRS considered the 8/18/11 PRS Report and preliminary Impact Analysis for NPRR351. - On 10/11/11, a third set of ERCOT comments were posted. - On 10/13/11, a fourth set of ERCOT comments were posted. - On 10/17/11, WMS comments were posted. - On 10/20/11, PRS again considered the 8/18/11 PRS Report and preliminary Impact Analysis for NPRR351 - On 10/20/11, a second ERCOT Steel Mills appeal was posted. - On 10/25/11, a second set of ERCOT Steel Mills comments were posted. - On 10/27/11, TIEC comments were posted. - On 11/1/11, Champion Energy Services comments were posted. - On 11/2/11, a third set of ERCOT Steel Mills comments were posted. - On 11/2/11, Reliant Energy Retail Services comments were posted. - On 11/3/11, TAC considered the ERCOT Steel Mills appeal. |  |  |
| PRS Decision |  | On 5/19/11, PRS unanimously voted to table NPRR351 for one month. All Market Segments were present for the vote. On 6/23/11, PRS voted to reject NPRR351 via roll call vote. There were five opposing votes from the Municipal (3), Investor-Owned Utility (IOU) (1), and Consumer Market Segments and eight abstentions from the Cooperative, IOU, Independent Generator (2), Independent Retail Electric Provider (IREP), and Independent Power Marketer (IPM) (3) Market Segments. All Market Segments were present for the vote. On 8/18/11, PRS voted to recommend approval of NPRR351 as revised by PRS for the sole purpose of ERCOT’s preparation of an Impact Analysis for PRS review. There was one opposing vote from the Independent Generator Market Segment. All Market Segments were present for the vote. On 9/22/11, PRS unanimously voted to table NPRR351 for one month; to request that ERCOT prepare comments outlining the impacts associated with Option 2 (as defined in the 9/21/11 ERCOT comments) and provide examples; and to request that WMS review NPRR351 and provide input by the October 20, 2011 PRS meeting. All Market Segments were present for the vote. On 10/20/11, PRS voted via roll call vote to table NPRR351 to allow further discussion in a workshop and further specify inputs, optimization, and outputs. There were nine opposing votes from the Consumer (5), Cooperative (2), Independent Generator and Municipal Market Segments and two abstentions from the IPM and Municipal Market Segments. A motion to grant NPRR351 Urgent status then failed via roll call vote. There were nine opposing votes from the Municipal, Independent Generator (2), IPM (3), IREP and IOU (2) Market Segments and two abstentions from the Consumer and IREP Market Segments. All Market Segments were present for the votes. |  |  |
| Summary of PRS Discussion |  | On 5/19/11, it was requested that PRS table NPRR351 to provide additional time to evaluate the 5/19/11 ERCOT comments. On 6/23/11, participants discussed whether NPRR351 or NPRR378, Posting of the ERCOT Short-Term Load Forecast and the Aggregated High Dispatch Limit (HDL) and Low Dispatch Limit (LDL) used in SCED, would benefit parties other than consumers; whether providing future price estimates is an appropriate task for ERCOT and its relation to effective demand response; whether ERCOT is capable of generating reasonably accurate estimates; and whether ERCOT’s proposed SCED Look-Ahead project addresses the issues raised in NPRR351 and NPRR378. ERCOT Staff provided an overview of the SCED Look-Ahead project. On 8/18/11, PRS reviewed the actions taken by TAC on 8/4/11 on NPRR351 and NPRR378. ERCOT Staff reviewed proposed revisions that were developed during discussions with the sponsor of the appeals of PRS’s rejection of NPRR351 and NPRR378, and explained that the revisions were intended to clarify the intent of NPRR351 to facilitate ERCOT’s completion of an Impact Analysis. Concern was raised that PRS was being asked to rule on the merits of NPRR351 without an Impact Analysis, which was outside the scope of TAC’s directive. ERCOT Staff explained that it did not have an opinion on the merits of the NPRR at this time and clarified that the proposed revisions were drafted for PRS consideration so that the scope of the NPRR is better established before ERCOT begins work on the Impact Analysis. On 9/22/11, ERCOT Staff reviewed the 9/21/11 ERCOT comments and explained that Option 2 would be the first phase of the longer-term SCED Look-Ahead project. Participants discussed the need to better understand the costs associated with Option 2; whether adopting Option 2 commits stakeholders to certain design parameters for the SCED Look-Ahead project; the importance of having a solution in place by the summer of 2012; and the potential effect of non-binding forward price projections on binding Real-Time prices. It was requested that ERCOT provide information on the costs associated with Option 2 and numerical examples for discussion at the October 12, 2011 WMS meeting. On 10/20/11, the 10/11/11 and 10/13/11 ERCOT comments were reviewed. Participants discussed whether additional detail is required in the Protocols to implement Option 2 beyond the revisions proposed in the 10/11/11 ERCOT comments. ERCOT Staff noted that Option 2 only represents the first phase of the SCED Look-Ahead project, and that stakeholders would have ample opportunity to provide input on Protocol language and design as the additional phases of the project are developed. It was emphasized that the look-ahead prices and base points being provided through NPRR351 would be non-binding. Some participants requested that a workshop be held on NPRR351 to further define the inputs, optimization, and outputs in regards to NPRR351, while others encouraged PRS to forward the NPRR to TAC. |  |  |
| TAC Decision |  | On 8/4/11, TAC voted via roll call vote to grant the ERCOT Steel Mills appeal and remand NPRR351 to PRS; to direct PRS to work with ERCOT to resolve any questions needed to complete the Impact Analysis; to request that ERCOT perform an Impact Analysis; and to direct PRS to recommend action on NPRR351 to TAC after the completion of the Impact Analysis. There were three abstentions from the IPM Market Segment. All Market Segments were present for the vote. On 11/3/11, TAC unanimously voted via roll call vote to grant the 10/20/11 ERCOT Steel Mills appeal and to remand NPRR351 to PRS as revised by TAC; to encourage WMS and PRS to continue discussions with ERCOT on the details of the SCED Look-Ahead project through workshops, special meetings, or other means; to instruct PRS to vote on either recommending approval or rejecting NPRR351 by the January 2012 PRS meeting and to report back to TAC; and to request that ERCOT provide more information to PRS at the 11/17/11 PRS meeting (or prior to the next TAC meeting if the PRS meeting is too soon) about its intentions on how to actually implement this NPRR in regards to wind forecasts, wind base points, ramp rates, State Estimator inputs, Dynamic Ratings, binding constraints, LDLs and HDLs, and the disclosure of forecasted base points to operators and the market. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 8/4/11, participants reviewed the purpose and procedural history of NPRR351 and related NPRR378 and the discussion and vote at the 6/23/11 PRS meeting. Participants discussed which Market Participants are likely to use the projections required by NPRR351. It was agreed that better understanding the implementation costs would be beneficial in considering the merits of both NPRRs. On 11/3/11, participants discussed when a final decision on NPRR351 is needed in order to have price projections delivered by Summer 2012 and how this timing might impact ERCOT’s implementation of phase 1 of the SCED Look-Ahead project. ERCOT Staff stated that it intends to move forward with the implementation of phase 1 of the SCED Look-Ahead project regardless of the final outcome of NPRR351, but that without the revisions proposed in NPRR351, the Protocols will not specify what price projections ERCOT will be required to post. Participants also discussed what information would be useful for ERCOT to provide to assist stakeholders’ deliberations and whether this information is needed before a decision on the merits of NPRR351 can be made. ERCOT Staff noted that the full SCED Look-Ahead project will require substantial stakeholder discussion and additional Protocol revisions, any of which will be vetted through the standard NPRR process. |  |  |

Business Case

| Business Case | 1 | Benefits - Posting of estimates of future Locational Marginal Prices (LMPs) facilitates consumers’ ability to reduce their consumption in response to high market prices. Furthermore, projecting non-binding prices will assist REPs in designing and implementing effective demand response programs for residential and small business consumers using Direct Load Control (DLC), etc. - Market response to Real-Time prices allows Generation Resource owners opportunities to better use their equipment. - This NPRR will allow Market Participants to begin to assess how a part of the changing Resource Dispatch will affect them, and ERCOT and Market Participants can identify ways to improve the forecast methodology. - Reduction of Load during periods of high prices frees up additional natural gas Resources to supply other generation and natural gas consumers. |
| --- | --- | --- |
|  | 2 | Costs: - MIS User Interface - new information to be obtained from the MIS. Cost unknown. |
|  | 3 | It is assumed that this NPRR will result in a new system in place in production that will be used for research purposes and price projections. |
|  | 4 | ERCOT has stated that this is a necessary first step to achieve SCED Look-Ahead, which is a policy goal of the Public Utility Commission of Texas (PUCT). |

| Sponsor |  |
| --- | --- |
| Name | Eric Goff on behalf of WMS |
| E-mail Address | EGoff@reliant.com |
| Company | Reliant Energy Retail Services, LLC |
| Cell Number | 512-284-4908 |
| Market Segment | IREP |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 051911 | Proposed as a long-term solution the forward Settlement Point Price forecast that will be part of the Real-Time Market for Co-Optimizing Energy & Ancillary Services Combined with Demand Response RTM upgrade project; proposed revisions requiring ERCOT to provide additional Real-Time data sets as an interim solution. |
| Steel Companies 060911 | Responded to 5/19/11 ERCOT comments and encouraged passage of NPRR351 as submitted. |
| ERCOT Steel Mills 070711 | Appealed PRS rejection of NPRR351. |
| Bob Wittmeyer 080311 | Requested that TAC consider the “Guiding Principles for ERCOT Project Prioritization” and the potential low prioritization of NPRR351. |
| Trefny-Crockett 092011 | Provided Business Case for NPRR351. |
| ERCOT 092111 | Provided a summary of and proposed Protocol revisions for alternative “Option 2” methodology for PRS consideration. |
| ERCOT 101111 | Proposed revisions to allow for flexibility in implementing Option 2. |
| ERCOT 101311 | Provided a summary of impacts associated with Option 2. |
| WMS 101711 | Recommended that Option 1 be rejected and that Option 2 be explored further in a workshop. |
| ERCOT Steel Mills 102011 | Appealed PRS tabling of NPRR351. |
| ERCOT Steel Mills 102511 | Provided substantive comments in support of appeal; requested that TAC recommend approval of NPRR351 as amended by the 10/11/11 ERCOT comments and forward NPRR351 to the ERCOT Board in time for the December 2011 meeting; and proposed that WMS and PRS continue discussions of SCED Look-Ahead concepts. |
| TIEC 102711 | Supported ERCOT Steel Mills appeal and recommended approval of Option 2 without delay. |
| Champion Energy Services 110111 | Supported ERCOT Steel Mills appeal and recommended approval of NPRR351 as amended by the 10/11/11ERCOT comments (Option 2) without further delay. |
| ERCOT Steel Mills 110211 | Modified relief requested to instead recommend that TAC instruct PRS to either approve or reject NPRR351 at the 11/17/11 PRS meeting and report back to TAC at the 12/1/11 TAC meeting. |
| Reliant Energy Retail Services 110211 | Supported ERCOT Steel Mills appeal and encouraged TAC to request ERCOT to provide additional information at the 11/17/11 PRS meeting (or 12/1/11 TAC meeting if the PRS meeting is too soon); and proposed revisions to cover page. |

Comments

Please note that NPRR405, Clarification of DC Tie Load into Operational Systems and Processes, also proposes revisions to Section 6.3.2.

Please also note that the baseline Protocol language in Section 6.3.2 has been updated due to the following:

- NPRR329, Security Classification Changes for Extracts/Reports (unboxed in the May 18, 2011 Protocols)

- NPRR350, Change to the Security Classification of the System Ancillary Service Capacity Monitor Dashboard (unboxed in the August 24, 2011 Protocols)

- NPRR384, Revisions to Support SCR766, Load Zone and Hub LMPs Distributed by ICCP (unboxed in the October 27, 2011 Protocols)

Proposed Protocol Language Revision

6.3.2	Activities for Real-Time Operations

(1)	Activities for Real-Time operations begin at the end of the Adjustment Period and conclude at the close of the Operating Hour.

(2)	The following table summarizes the timeline for the Operating Period and the activities of QSEs and ERCOT during Real-Time operations where “T” represents any instant within the Operating Hour.  The table is intended to be only a general guide and not controlling language, and any conflict between this table and another section of the Protocols is controlled by the other section:

| Operating Period | QSE Activities | ERCOT Activities |
| --- | --- | --- |
| During the first hour of the Operating Period |  | Execute the Hour-Ahead Sequence, including HRUC, beginning with the second hour of the Operating Period Review and communicate HRUC commitments Snapshot the Scheduled Power Consumption for Controllable Load Resources |
| Before the start of each SCED run | Update Output Schedules for DSRs | Validate Output Schedules for DSRs Execute Real-Time Sequence |
| SCED run |  | Execute SCED |
| During the Operating Hour | Telemeter the Ancillary Service Resource Responsibility for each Resource Acknowledge receipt of Dispatch Instructions Comply with Dispatch Instruction Review Resource Status to assure current state of the Resources is properly telemetered Update COP with actual Resource Status and limits and Ancillary Service Schedules Communicate Resource Forced Outages to ERCOT Communicate to ERCOT Resource changes to Ancillary Service Resource Responsibility via telemetry in the time window beginning 30 seconds prior to the five-minute clock interval and ending ten seconds prior to that five-minute clock interval | Communicate all Base Points, Dispatch Instructions and LMPs for energy and Ancillary Services using Inter-Control Center Communications Protocol (ICCP) or Verbal Dispatch Instructions (VDIs) Monitor Resource Status and identify discrepancies between COP and telemetered Resource Status Restart Real-Time Sequence on major change of Resource or Transmission Element Status Monitor ERCOT total system capacity providing Ancillary Services Validate COP information Monitor ERCOT control performance Distribute by ICCP, and post on the MIS Public Area, the LMPs created by each SCED process for each Resource Node, Load Zone and Hub. These prices shall be posted immediately subsequent to deployment of Base Points from SCED with the time stamp the prices are effective Post LMPs for each Electrical Bus on the MIS Public Area. These prices shall be posted immediately subsequent to deployment of Base Points from SCED with the time stamp the prices are effective Post on the MIS Public Area the 15-minute projected LMPs created by each SCED process for each Resource Node and the 15-minute projected Hub LMPs and Load Zone LMPs. These projected prices shall be posted immediately subsequent to deployment of Base Points with the time stamp of the study process that produced the projections. Post each hour on the MIS Public Area SCED Shadow Prices and active binding transmission constraints by Transmission Element name (contingency /overloaded element pairs) Post the Settlement Point Prices for each Settlement Point immediately following the end of each Settlement Interval Post parameters as required by Section 6.4.8, Ancillary Services Capacity During the Adjustment Period and in Real-Time, on the MIS Public Area |

(3)	At the beginning of each hour, ERCOT shall post on the MIS Public Area the following information:

(a)	Changes in ERCOT System conditions that could affect the security and dynamic transmission limits of the ERCOT System, including:

(i)	Changes or expected changes, in the status of Transmission Facilities as recorded in the Outage Scheduler for the remaining hours of the current Operating Day and all hours of the next Operating Day; and

(ii)	Any conditions such as adverse weather conditions as determined from the ERCOT-designated weather service;

(b)	Updated system-wide Load forecasts;

(c)	The quantities of Reliability Must-Run (RMR) Services deployed by ERCOT for each previous hour of the current Operating Day;

(d)	Total ERCOT System Demand, from Real-Time operations, integrated over each Settlement Interval; and

(e)	Updated Electrical Bus Load distribution factors and other information necessary to forecast Electrical Bus Loads for each hour of the current Operating Day and all hours of the next Operating Day.

6.5.7.3	Security Constrained Economic Dispatch

(1)	The SCED process is designed to simultaneously manage energy, the system power balance and network congestion through Resource Base Points and calculation of LMPs every five minutes.  The SCED process uses a two-step methodology that applies mitigation prospectively to resolve network Non-Competitive Constraints for the current Operating Hour.  The SCED process evaluates Energy Offer Curves and Output Schedules to produce a least cost dispatch of On-Line Generation Resources to the total current generation requirement determined by LFC, subject to power balance and network constraints.  The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP.

[NPRR257: Replace paragraph (1) above with the following upon system implementation:] (1) The SCED process is designed to simultaneously manage energy, the system power balance and network congestion through Resource Base Points and calculation of LMPs every five minutes. The SCED process uses a two-step methodology that applies mitigation prospectively to resolve Non-Competitive Constraints for the current Operating Hour. The SCED process evaluates Energy Offer Curves and Output Schedules to produce a least cost dispatch of On-Line Generation Resources to the total current generation requirement determined by LFC, subject to power balance and network constraints. The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP.

(2)	The SCED solution must monitor cumulative deployment of Regulation Services and ensure that Regulation Services deployment is minimized over time.

(3)	For use as SCED inputs, ERCOT shall use the available capacity of all committed Generation Resources by creating proxy Energy Offer Curves for certain Resources as follows:

(a)	Non-WGRs and Dynamically Scheduled Resources (DSRs) without Energy Offer Curves

ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below for:

(i)	Each non-WGR for which its QSE has submitted an Output Schedule instead of an Energy Offer Curve; and

(ii) 	Each DSR that has not submitted Incremental and Decremental Energy Offer Curves.

| MW | Price (per MWh) |
| --- | --- |
| HSL | System-Wide Offer Cap (SWCAP) |
| Output Schedule MW plus 1 MW | SWCAP minus $0.01 |
| Output Schedule MW | -$249.99 |
| LSL | -$250.00 |

(b)	DSRs with Energy Offer Curves

For each DSR that has submitted incremental and decremental Energy Offer Curves, ERCOT shall create a monotonically increasing proxy Energy Offer Curve.  That curve must consist of the incremental Energy Offer Curve that reflects the available capacity above the Resource’s Output Schedule to its HSL and the decremental Energy Offer Curve that reflects the available capacity below the Resource’s Output Schedule to the LSL.  The curve must be created as described below:

| MW | Price (per MWh) |
| --- | --- |
| Output Schedule MW plus 1 MW to HSL | Incremental Energy Offer Curve |
| LSL to Output Schedule MW | Decremental Energy Offer Curve |

(c)	Non-WGRs without full-range Energy Offer Curves

For each non-WGR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the Resource’s available capacity, ERCOT shall create a proxy Energy Offer Curve that extends the submitted Energy Offer Curve to use the entire available capacity of the Resource using the SWCAP above the highest point on the Energy Offer Curve to the Resource’s HSL and the offer floor from the lowest point on the Energy Offer Curve to its LSL, using these points:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in Energy Offer Curve) | SWCAP |
| 1 MW above highest MW in Energy Offer Curve (if less than HSL) | SWCAP minus $0.01 |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(d)	WGRs

(i)	For each WGR that has not submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL | SWCAP |
| HSL minus 1 MW | -$249.99 |
| LSL | -$250.00 |

(ii)	For each WGR for which its QSE has submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in Energy Offer Curve) | SWCAP |
| 1 MW above highest MW in Energy Offer Curve (if less than HSL) | SWCAP minus $0.01 |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(4)	The Entity with decision making authority, as more fully described in Section 3.19.1, Annual Competitiveness Test, over how a Resource or Split Generation Resource is offered or scheduled, shall be responsible for all offers associated with each Resource, including offers represented by a proxy Energy Offer Curve.

[NPRR240: Insert paragraph (5) and renumber accordingly upon system implementation:] (5) Energy Offer Curves that were constructed in whole or in part with proxy Energy Offer Curves shall be so marked in all ERCOT postings or references to the energy offer.

(5)	The two-step SCED methodology referenced in paragraph (1) above is:

(a)	The first step is to execute the SCED process to determine Reference LMPs.  In this step, ERCOT executes SCED using the full Network Operations Model while only observing limits of Competitive Constraints.  Energy Offer Curves for all On-Line Generation Resources, whether submitted by QSEs or created by ERCOT under this Section, are used in the SCED to determine “Reference LMPs.”

(b)	The second step is to execute the SCED process to produce Base Points, Shadow Prices, and LMPs, subject to security constraints (including Competitive and Non-Competitive Constraints) and other Resource constraints.  The second step must:

(i)	Use Energy Offer Curves for all On-Line Generation Resources, whether submitted by QSEs or created by ERCOT.  Each Energy Offer Curve must be capped at the greater of the Reference LMP (from Step 1) at the Resource Node or the appropriate Mitigated Offer Cap and bounded at the lesser of the Reference LMP (from Step 1) at the Resource Node or the appropriate Mitigated Offer Floor; and

(ii)	Observe all Competitive and Non-Competitive Constraints.

(c)	ERCOT shall archive information and provide monthly summaries of security violations and any binding transmission constraints identified in Step 2 of the SCED process.  The summary must describe the limiting element (or identified operator-entered constraint with operator’s comments describing the reason and the Resource-specific impacts for any manual overrides).  ERCOT shall provide the summary to Market Participants on the MIS Secure Area and to the Independent Market Monitor (IMM).

(6)	For each SCED process, ERCOT shall calculate a rolling 15 minute projection of the Resource Node LMPs, Hub LMPs, Load Zone LMPs, and LMPs for each Electrical Bus based on the same inputs to the SCED process as described in this Section, except that the Resource’s HDL and LDL and the total generation requirement will be as estimated at the end of the next 15 minutes.  The Resource’s HDL and LDL will be calculated based on ramping over 15 minutes.  ERCOT shall estimate the 15 minute projected total generation requirement by estimating the change in generation requirements in the next 15 minutes and adding that to the current actual generation requirement as was determined by LFC and used to produce the current SCED interval prices.  ERCOT shall post the 15-minute projected LMPs for Resource Nodes, Hub LMPs and Load Zone LMPs on the MIS Public Area pursuant to Section 6.3.2, Activities for Real-Time Operations.