| NPRR Number | 397 | NPRR Title | Balance of the Day Ancillary Service Market |
| --- | --- | --- | --- |
| Date Posted |  | August 1, 2011 |  |
| Nodal Protocol Sections Requiring Revision |  | 6.4.8.2, Supplemental Ancillary Services Market 6.7.2.1, Charges for Ancillary Service Capacity Replaced Due to SASM Reconfiguration (New) |  |
| Requested Resolution |  | Normal |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) allows the Supplemental Ancillary Services Market (SASM) to be operated as a balance of the day market for Ancillary Services. |  |
| Reason for Revision |  | The current market locks entities in once they have sold Ancillary Services to ERCOT in the Day-Ahead Market (DAM). Conditions often change after the close of the DAM and Real-Time. This NPRR would allow Qualified Scheduling Entities (QSEs) the ability to modify their responsibilities and optimize the use of their Resources. |  |
| Overall Market Benefit |  | Greater efficiency in the Ancillary Services market. |  |
| Overall Market Impact |  | Reduced risk on Ancillary Service providers. |  |
| Consumer Impact |  | Lower Ancillary Service prices. |  |
| Credit Implications |  | No. |  |

| Quantitative Impacts and Benefits |  |  |  |
| --- | --- | --- | --- |
| Assumptions | 1 | This NPRR would improve efficiency of the Ancillary Services market by allowing Entities to “buy back” their responsibilities from the market closer to Real-Time. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Cost |  | Impact Area | Monetary Impact |
|  | 1 | No cost for Market Participant implementation as participation is not mandatory. | Not applicable. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Market Benefit |  | Impact Area | Monetary Impact |
|  | 1 | Reduced risk in Ancillary Service provision. | Estimated 1-2% improved efficiency in Ancillary Service prices – to be analyzed further during stakeholder process. |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Additional Qualitative Information | 1 | Greater market efficiency as QSEs may transfer responsibilities from more efficient units to ones that are less efficient and provide energy from those units, or from less efficient units that may then be decommitted from the market. |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
| Other Comments | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |

| Sponsor |  |
| --- | --- |
| Name | Clayton Greer |
| E-mail Address | Clayton.greer@morganstanley.com |
| Company | Morgan Stanley |
| Phone Number | 512-497-2986 |
| Cell Number | 512-639-3993 |
| Market Segment | Independent Power Marketer (IPM) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@gmail.com |
| Phone Number | 512-248-6464 |

Comments

The purpose of this NPRR is to create a balance of the day Ancillary Service market QSEs can use to optimize their current positions against the market.  The intent is to utilize current SASM and Settlement systems to the greatest extent possible.  An interface would be required to allow QSEs the ability to notify ERCOT of the capacity they would like to put into the market.  It is also the intent of the NPRR that QSEs be allowed to bid against their own obligations.  As an example:

A QSE is carrying 100 MW of Responsive Reserve (RRS) Service, but market prices have risen to a level where it is more efficient for the unit to provide energy.  The QSE notifies ERCOT that it would like to place its 100 MW RRS award in the reconfiguration SASM.  For the examples, we’ll assume the QSE was awarded at a price of $10/MW for the service and is willing to continue providing the responsibility at a cap price of $50/MW.

Potential outcomes:

Outcome 1:  The SASM fails to clear due to no offers being present (assume the QSE did not offer at $50).  In this case the responsibility will stay with the original QSE.  No Settlement change.  The QSE will be paid the $10/MW price from the DAM award.

Outcome 2:  The SASM clears at $20/MW.  In this case the responsibility will be transferred to another QSE.  The original QSE will be paid $10/MW for the RRS service awarded from the DAM, but will pay $20/MW for the RRS replacement.  The original QSE’s Reliability Unit Commitment (RUC) position will be decreased by the release of the RRS obligation, but if its capacity position changes (it decommits a unit for instance).

Outcome 3:  The SASM clears at $50/MW awarding the full RRS MWs to the original QSE.  In this case, the QSE will maintain the responsibility and will be paid the $10/MW award from the DAM.  The $50/MW will be paid by the QSE and paid back to the QSE for provision of the RRS, so the QSE remains flat to the SASM outcome.

Outcome 4:  The SASM clears at $50/MW awarding some portion of the MWs (assume 50 MW) to the original QSE and some (50 MW) to the market.  In this case, the QSE will be relieved of 50 MW of their responsibility and will pay $50/MW for that amount of transfer.  They will continue to provide the remaining 50 MWs of RRS not covered by the market.

This NPRR includes revisions proposed in NPRR370, Supplementary Ancillary Services Market Modifications, as recommended for approval by PRS on 7/21/11.  See author “NPRR370.”

Proposed Protocol Language Revision

6.4.8.2	Supplemental Ancillary Services Market

(1)	During the Adjustment Period, ERCOT may procure additional Regulation Up (Reg-Up), Regulation Down (Reg-Down), Responsive Reserve (RRS), and Non-Spinning Reserve (Non-Spin) services for the reasons, and in the amounts, specified in Section 6.4.8.1, Evaluation and Maintenance of Ancillary Service Capacity Sufficiency, using a SASM.

(2)	ERCOT shall allow QSEs to modify their Ancillary Service positions through a reconfiguration SASM run at the top of each clock hour.  This SASM allows QSEs to change their commitments from the hour two hours following the prompt hour through the end of the current trade day.  For the two hours prior to the end of the current trade day, the SASM shall be for the hours two hours following the prompt hour through the end of the following trade day.

(3)	The SASM process for acquiring more Ancillary Service capacity or the hourly Ancillary Service reconfiguration must use the following timelines:

(a)	For Ancillary Service capacity related to ERCOT desired increases, ERCOT shall send a notice at time “X” to all QSEs of the SASM.  Time X may be any time not less than two hours before the start of the Operating Hour for which the additional Ancillary Services capacity is required.

| SASM Process | QSE Activities: | ERCOT Activities: |
| --- | --- | --- |
| Time = X |  | Notify all QSEs of intent to procure additional Ancillary Services. Notify QSEs of any additional Ancillary Service Obligation, allocated to each Load Serving Entity (LSE) and aggregated to the QSE level. |
| Time = X plus 30 minutes | May submit additional Self-Arranged Ancillary Service Quantities limited to the additional Ancillary Services Obligation of the QSE. | Determine the amount of Ancillary Services to be procured. |
| Time = X plus 35 minutes |  | Execute SASM. |
| Time = X plus 45 minutes |  | Notify QSEs with awards of results. Post the quantities and MCPCs of Ancillary Services bought in the SASM. |
| Time = X plus 60 minutes | Submit updated COP with updated Ancillary Service Resource Responsibility. | Validate COPs for Ancillary Service Resource Responsibility. |

(b)	For replacement of Ancillary Service capacity related to undeliverability or failure of a QSE to provide one or more Ancillary Services, ERCOT shall have the option of executing a SASM at time “E” without providing preliminary notice to the market.  Time “E” may be any time not less than two hours before the start of the Operating Hour for which the additional Ancillary Services capacity is required.

| SASM Process | QSE Activities: | ERCOT Activities: |
| --- | --- | --- |
| Time = E |  | Execute SASM. |
| Time = E plus 15 minutes |  | Notify QSEs with awards of results. Post the quantities and MCPCs of Ancillary Services bought in the SASM. |
| Time = E plus 30 minutes | Submit updated COP with updated Ancillary Service Resource Responsibility. | Validate COPs for Ancillary Service Resource Responsibility. |

(c)	For the hourly Ancillary Services reconfiguration, ERCOT shall execute a SASM at the top of each clock hour (time “E”), for the hours two hours following the prompt hour through the end of the trade day.  For the two hours prior to the end of the current trade day, the SASM shall be for the hours two hours following the prompt hour through the end of the following trade day.

| SASM Process | QSE Activities: | ERCOT Activities: |
| --- | --- | --- |
| Time = E – 15 minutes | QSEs Nominate quantities of Ancillary Services that shall be included in the reconfiguration SASM. | ERCOT sets the quantities of Ancillary Services to be procured in the SASM. |
| Time = E |  | Execute SASM for hours following the prompt hour (h+1) through end of trade day. |
| Time = E plus 15 minutes |  | Notify QSEs with awards of results. Post the quantities and MCPCs of Ancillary Services bought in the SASM. |
| Time = E plus 30 minutes | Submit updated COP with updated Ancillary Service Resource Responsibility. | Validate COPs for Ancillary Service Resource Responsibility. |

(4)	Each QSE that is awarded capacity in a SASM is paid the SASM MCPC for the quantity it is awarded.

(5)	ERCOT shall allocate additional Ancillary Service Obligations to QSEs using the same percentages as the original Day-Ahead allocation of Ancillary Service Obligations.

6.7.2.1	Charges for Ancillary Service Capacity Replaced Due to SASM Reconfiguration

A charge to each QSE that reconfigures its Ancillary Service Supply Responsibility to a reduced level through a SASM is calculated based on the amount of the Ancillary Service reduction and the MCPC of the SASM in which the responsibility was reduced.  By Ancillary Service, the charge to each QSE for a given Operating Hour is calculated as follows:

(a)	For Reg-Up, if applicable:

RURQAMT q	=	MCPCRU m * RURQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RURQAMT q | $ | Reg-Up Reduced Quantity Amount per QSE—The charge to QSE q for its total capacity associated with reductions in its Ancillary Service Supply Responsibility for Reg-Up, for the hour. |
| MCPCRU m | $/MW per hour | Market Clearing Price for Capacity for Reg-Up by market—The MCPC for Reg-Up in the market m, for the hour. |
| RURQ q | MW | Reg-Up Reduced Quantity per QSE—QSE q’s total capacity associated with reductions in its Ancillary Service Supply Responsibility for Reg-Up, for the hour. |
| m | none | The SASM for the given reduction in responsibility. |
| q | none | A QSE. |

(b)	For Reg-Down, if applicable:

RDRQAMT q	=	MCPCRD m * RDRQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RDRQAMT q | $ | Reg-Down Reduced Quantity Amount per QSE—The charge to QSE q for its total capacity associated with reductions in its Ancillary Service Supply Responsibility for Reg-Down, for the hour. |
| MCPCRD m | $/MW per hour | Market Clearing Price for Capacity for Reg-Down by market—The MCPC for Reg-Down in the market m, for the hour. |
| RDRQ q | MW | Reg-Down Reduced Quantity per QSE—QSE q’s total capacity associated with reductions in its Ancillary Service Supply Responsibility for Reg-Down, for the hour. |
| m | none | The SASM for the given reduction in responsibility. |
| q | none | A QSE. |

(c)	For RRS, if applicable:

RRRQAMT q	=	MCPCRR m * RRRQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RRRQAMT q | $ | Responsive Reserve Reduced Quantity Amount per QSE—The charge to QSE q for its total capacity associated with reductions in its Ancillary Service Supply Responsibility for RRS, for the hour. |
| MCPCRR m | $/MW per hour | Market Clearing Price for Capacity for Responsive Reserve per market—The MCPC for RRS in the market m, for the hour. |
| RRRQ q | MW | Responsive Reserve Reduced Quantity per QSE—QSE q’s total capacity associated with reductions in its Ancillary Service Supply Responsibility for RRS, for the hour. |
| m | none | The SASM for the given reduction in responsibility. |
| q | none | A QSE. |

(d)	For Non-Spin, if applicable:

NSRQAMT q	=	MCPCNS m * NSRQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| NSRQAMT q | $ | Non-Spin Reduced Quantity Amount per QSE—The charge to QSE q for its total capacity associated with reductions in its Ancillary Service Supply Responsibility for Non-Spin, for the hour. |
| MCPCNS m | $/MW per hour | Market Clearing Price for Capacity for Non-Spin by market—The MCPC for Non-Spin in the market m, for the hour. |
| NSRQ q | MW | Non-Spin Reduced Quantity per QSE—QSE q’s total capacity associated with reductions in its Ancillary Service Supply Responsibility for Non-Spin, for the hour. |
| m | none | The SASM for the given reduction in responsibility. |
| q | none | A QSE. |