| NPRR Number | 402 | NPRR Title | Clarification of Pre-DAM RUC Instruction Sequence |
| --- | --- | --- | --- |
| Date |  | 10/12/2011 |  |
| Submitter’s Information |  |  |  |
| Name |  | David Maggio |  |
| E-mail Address |  | dmaggio@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 215-248-6998 |  |
| Market Segment |  | NA |  |

Comments

ERCOT submits these comments to make clear its intent to post Weekly Reliability Unit Commitment (WRUC) instructions to the Market Information System (MIS) Secure Area.  The language in Section 5.3, ERCOT Security Sequence Responsibilities, is updated to specifically call out the posting of WRUC Verbal Dispatch Instructions (VDIs).

Minor administrative edits have also been proposed to correct defined term usage.

Revised Cover Page Language

| Nodal Protocol Section Requiring Revision | 2.1, Definitions 2.2, Acronyms and Abbreviations 5.1, Introduction 5.3, ERCOT Security Sequence Responsibilities |
| --- | --- |

Revised Proposed Protocol Language

2.1	DEFINITIONS

Weekly Reliability Unit Commitment (WRUC)

A RUC instruction issued by ERCOT prior to 1000 in the Day-Ahead for an Operating Day.

2.2		ACRONYMS AND ABBREVIATIONS

WRUC		Weekly Reliability Unit Commitment

5.1	Introduction

(1)	Transmission security analysis and Reliability Unit Commitment (RUC) are used to ensure ERCOT System reliability and to ensure that enough Resource capacity, in addition to Ancillary Service capacity, is committed in the right locations to reliably serve the forecasted Load on the ERCOT System.

(2)	ERCOT shall conduct at least one Day-Ahead RUC (DRUC) and at least one Hourly RUC (HRUC) before each hour of the Operating Day.  ERCOT, in its sole discretion, may conduct a RUC at any time to evaluate and resolve reliability issues.

(3)	The DRUC must be run after the close of the Day-Ahead Market (DAM).

(4)	The DRUC uses Three-Part Supply Offers submitted before the DAM by Qualified Scheduling Entities (QSEs) that were considered in the DAM but not awarded in the DAM.  A QSE may not submit a Three-Part Supply Offer to be considered in the DRUC unless the offer was also submitted for consideration in the DAM.

(5)	ERCOT must initiate the HRUC process at least one hour before the Operating Hour to fine-tune the Resource commitments using updated Load forecasts and updated Outage information.

(6)	The RUC Study Period for DRUC is the next Operating Day.  The RUC Study Period for HRUC is the balance of the current Operating Day plus the next Operating Day if the DRUC for the Operating Day has been solved.

(7)	HRUC may decommit Resources only to maintain the reliability of the ERCOT System.

(8)	For each RUC Study Period, the RUC considers capacity requirements for each hour of the RUC Study Period with the objective of minimizing costs based on Three-Part Supply Offers and while substituting a proxy Energy Offer Curve for the Energy Offer Curve.  The proxy Energy Offer Curve is calculated in a way that minimizes the effect of the proxy Energy Offer Curves on optimization.

(9)	The calculated Resource commitments arising from each RUC process must be reviewed by ERCOT before issuing Dispatch Instructions to QSEs to commit, extend, or decommit Resources.

(10)	The Security Sequence is a set of prerequisite processes for RUC that describes the key system components and inputs that are required to support the RUC process, the RUC process itself, and the ERCOT review of the Resource commitment recommendations made by the RUC process.

(11)	The RUC process may not be used to buy Ancillary Service unless the Ancillary Service Offers submitted in the DAM are insufficient to meet the requirements of the Ancillary Service Plan.

(12)	After the use of market processes to the fullest extent practicable without jeopardizing the reliability of the ERCOT System, any ERCOT Dispatch Instructions for additional capacity that order a QSE to commit a specific Generation Resource to be On-Line shall be considered a RUC Dispatch for the purpose of the Settlement of payments and charges related to the committed Generation Resource.  An Operating Condition Notice (OCN), Advisory, Watch, or Emergency Notice requesting the available capacity of any currently available Generation Resources but not naming specific Generation Resources is not considered a RUC Dispatch for purposes of Settlement.

[NPRR314: Insert paragraph (13) below and renumber accordingly upon system implementation:] (13) ERCOT shall post on the Market Information System (MIS) Certified Area, for each Off-Line Generation Resource that may be selected by a RUC process, the current time since the Generation Resource last went Off-Line (in hours) and the corresponding start-up times ERCOT is using for each such Off-Line Generation Resource. The time since the Generation Resource last went Off-Line and start-up times shall be updated at least hourly.

(13)	Prior to 1000 in the Day-Ahead, ERCOT may issue a Weekly Reliability Unit Commitment (WRUC) Verbal Dispatch Instruction (VDI) to inform a QSE that a Resource is required to be On-Line for all or part of a future Operating Day.  Following the receipt of a WRUC:

(a)	The QSE may self-commit the Resource for the WRUC-committed hours by updating the Resource’s Current Operating Plan (COP) to reflect the appropriate On-Line Resource Status for the WRUC-committed hours prior to 1000 in the Day-Ahead.  Resources that have been self-committed by a QSE in accordance with a WRUC:

(i)	May not be decommitted by the QSE for a WRUC-committed hour unless verbally approved by ERCOT;

(ii)	Will not be issued a RUC commitment for the WRUC-committed hours that were self-committed; and

(iii)	May have a Three-Part Supply Offer submitted into the DAM, and any of the WRUC-committed hours in which the Three-Part Supply Offer is awarded in the DAM become DAM-Committed Intervals for the Resource.

(b)	If the QSE does not self-commit the Resource for a WRUC-committed hour, ERCOT will commit the Resource as part of the DRUC process for the relevant Operating Day.  For all purposes, including RUC Settlement, the Resource will be considered as committed by the DRUC for these hours.

5.3	ERCOT Security Sequence Responsibilities

(1)	ERCOT shall start the Day-Ahead Reliability Unit Commitment (DRUC) process at 1430 in the Day-Ahead.

(2)	For each DRUC, ERCOT shall use a snapshot of Resource commitments taken at 1430 in the Day-Ahead to settle Reliability Unit Commitment (RUC) charges.  For each Hourly Reliability Unit Commitment (HRUC), ERCOT shall use a snapshot of Resource commitments from each Qualified Scheduling Entity’s (QSE’s) most recently submitted Current Operating Plan (COP) before HRUC execution to settle RUC charges.

(3)	For each RUC process, ERCOT shall:

(a)	Execute the Security Sequence described in Section 5.5, Security Sequence, Including RUC, including:

(i)	Validating Three-Part Supply Offers, defined in Section 4.4.9.1, Three-Part Supply Offers; and

(ii)	Reviewing the Resource commitment recommendations made by the RUC algorithm; and

(b)	Post to the Market Information System (MIS) Secure Area, all Resources that were committed or decommitted by the RUC process including Weekly Reliability Unit Commitment (WRUC) VDI commitments and decommitments;

[NPRR382: Replace item (3)(b) above with the following upon system implementation:] (b) Post to the Market Information System (MIS) Secure Area, all Resources that were committed or decommitted by the RUC process including verbal RUC and Weekly Reliability Unit Commitment (WRUC) Verbal Dispatch Instruction (VDI) commitments and decommitments;

(c)	Post to the MIS Public Area, all active and binding transmission constraints (contingency and overloaded element pair information where available) used as inputs to the RUC; and

(d)	Issue Dispatch Instructions to notify each QSE of its Resource commitments or decommittments.

(4)	ERCOT shall provide each QSE with the information necessary to pre-validate their data for DRUC and HRUC including:

(a)	Publishing validation rules for offers, bids, and trades; and

(b)	Posting any software documentation and code that is not Protected Information to the MIS Secure Area within five Business Days of receipt by ERCOT.