| NPRR Number | 428 | NPRR Title | Energy Offer Curve Requirements for Generation Resources Assigned Non-Spin Responsibility |
| --- | --- | --- | --- |
| Date Posted |  | November 9, 2011 |  |
| Requested Resolution |  | Urgent. Urgency is requested as a result of the Public Utility Commission of Texas’ (Commission) discussion at its October 27, 2011 Open Meeting regarding Project No. 37897, PUC Proceeding Relating to Resource and Reserve Adequacy and Shortage Pricing, where the Commission requested that the proposed solutions captured within this Nodal Protocol Revision Request (NPRR) be presented to the ERCOT Board of Directors (Board) for disposition as soon as possible. |  |
| Nodal Protocol Section Requiring Revision |  | 6.4.3.1, Energy Offer Curve for Non-Spinning Reserve Capacity (new) |  |
| Revision Description |  | This NPRR adds a requirement for the Energy Offer Curve to be at or above $120 for On-Line Non-Spinning Reserve (Non-Spin) capacity and at or above $180 for Off-Line Non-Spin capacity. Qualified Scheduling Entity (QSE) updates to an Energy Offer Curve entered at the end of the Adjustment Period become effective in ERCOT systems at the next Operating Period. Therefore, if Non-Spin is moved from one Generation Resource to another in the QSE's portfolio during an Operating Hour, any change to the Energy Offer Curve will be effective in the next Operating Period. There will not be any validation rules to verify the requirements added by this NPRR. The onus is on all QSEs to make the submission/telemetry changes needed to implement these new requirements. |  |
| Reason for Revision |  | To address the Commission’s discussion raised at its October 27, 2011 Open Meeting regarding Project No. 37897. |  |
| Credit Implications (Yes or No, and summary of impact) |  | To be determined. |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | - Addresses the Commission’s discussion raised at its October 27, 2011 Open Meeting regarding Project No. 37897. |
|  | 2 | - This NPRR will mitigate price suppression during deployment of reserve services to more accurately reflect the value of the service. |
|  | 3 |  |
|  | 4 |  |
|  | 5 |  |

| Sponsor |  |
| --- | --- |
| Name | John Dumas |
| E-mail Address | jdumas@ercot.com |
| Company | ERCOT |
| Phone Number | (512) 248-3195 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Yvette M. Landin |
| E-Mail Address | ylandin@ercot.com |
| Phone Number | (512) 248-4513 |

Comments

Deletion of paragraph (c) of Section 6.4.3.1 is contingent upon the approval of NPRR427, Energy Offer Curve Requirements for Generation Resources Assigned Reg-Up and RRS.

Proposed Protocol Language Revision

6.4.3.1	Energy Offer Curve for Non-Spinning Reserve Capacity

The following applies to Generation Resources that a QSE assigns Non-Spinning Reserve (Non-Spin) Ancillary Service Resource Responsibility in its Current Operating Plan (COP) to meet the QSE’s Ancillary Service Supply Responsibility for Non-Spin and includes both On-Line and Off-Line Non-Spin assignments arising as the result of Day-Ahead Market (DAM) or Supplemental Ancillary Services Market (SASM) Ancillary Service awards, or Self-Arranged Ancillary Service Quantity.

(a)	Prior to the end of the Adjustment Period for an Operating Hour during which a Generation Resource is assigned Non-Spin Ancillary Service Resource Responsibility, the QSE shall assure that a valid Output Schedule or Energy Offer Curve for the Operating Hour has been submitted and accepted by ERCOT.  The Energy Offer Curves submitted by the QSE for these Generation Resources must meet the following requirements:

(i)	If the Generation Resource is providing On-Line Non-Spin capacity, the energy from this capacity may not be offered at less than $120 per MWh; or

(ii)	If the Generation Resource is providing Off-Line Non-Spin capacity, the energy form this capacity may not be offered at less than $180 per MWh.

(b)	Paragraph (a) above shall not restrict the ability of the QSE to update the Energy Offer Curves for its Generation Resources in accordance with these Protocols provided that in those Operating Hours in which a Generation Resource is assigned Non-Spin Ancillary Service Resource Responsibility, the portion of the Energy Offer Curve reserved for this Ancillary Service meets those requirements.

(c) 	To assure that the Energy Offer Curves submitted in accordance with this section meets the requirement for a monotonically increasing Energy Offer Curve, the energy offer for capacity reserved for Responsive Reserve (RRS) and Regulation Up (Reg-Up) must also be offered by the QSE at no less than the minimums set forth in items paragraph (a) above.