| NPRR Number | 427 | NPRR Title | Energy Offer Curve Requirements for Generation Resources Assigned Reg-Up and RRS |  |
| --- | --- | --- | --- | --- |
| Timeline | Urgent | Action |  | Recommended Approval |
| Date of Decision |  | November 17, 2011 |  |  |
| Proposed Effective Date |  | January 5, 2012 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Section Requiring Revision |  | 6.4.3.1 Energy Offer Curve for Responsive Reserve Service and Regulation Up Service Capacity (new) |  |  |
| Revision Description |  | This NPRR adds a requirement for the Energy Offer Curve to be set at the System-Wide Offer Cap (SWCAP) for the capacity reserved for Responsive Reserve (RRS) Service Ancillary Service Resource Responsibility and Regulation Up (Reg-Up) Ancillary Service Resource Responsibility. Exemptions are provided for Resources that experience Forced Outages, Forced Derates, or Startup Loading Failures. Qualified Scheduling Entity (QSE) updates to an Energy Offer Curve entered at the end of the Adjustment Period become effective in ERCOT systems at the next Operating Period. Therefore, if Non-Spin is moved from one Generation Resource to another in the QSE's portfolio during an Operating Hour, any change to the Energy Offer Curve will be effective in the next Operating Period. There will not be any validation rule put in to verify the rules added by the new Protocol language. The onus is on QSE’s to make the submission/telemetry changes needed to implement the new Protocol language. |  |  |
| Reason for Revision |  | To address concerns raised at the October 27, 2011 PUCT Open Meeting regarding Project No. 37897. |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - On 11/9/11, NPRR427 and an Impact Analysis were posted. - On 11/16/11, Austin Energy comments were posted. - On 11/17/11, PRS considered NPRR427. |  |  |
| PRS Decision |  | On 11/17/11, PRS unanimously voted to grant NPRR427 Urgent status. PRS then voted to recommend approval of NPRR427 as revised by PRS; an effective date of January 5, 2012; and to forward NPRR427 to TAC. There was one opposing vote from the Consumer Market Segment. All Market Segments were present for the votes. |  |  |
| Summary of PRS Discussion |  | On 11/17/11, in response to compliance concerns, language changes were proposed so that QSEs will be exempt from the Energy Offer Curve requirements proposed in NPRR427 only if the Generation Resource experiences a Forced Outage, Force Derate or Startup Loading Failure within 15 minutes and notifies ERCOT Operators of the event. Additional issues were raised including price reversal due to shifting of Ancillary Service obligations for economic reasons; unintended consequences that could limit shifting of Ancillary Service obligations in Real-Time; and ability to update Energy Offer Curves in the Operating Period. There was a request for NPRR427 to become effective on January 5, 2012 to allow time for ERCOT to host a market call to discuss the processes to support NPRR427 and operator training. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | - Addresses concerns expressed at the October 27, 2011 Open Meeting regarding Project No. 37897. |
|  | 2 | - This NPRR will mitigate price suppression during deployment of reserve services to more accurately reflect the value of the service. |
|  | 3 |  |
|  | 4 |  |
|  | 5 |  |

| Sponsor |  |
| --- | --- |
| Name | John Dumas |
| E-mail Address | jdumas@ercot.com |
| Company | ERCOT |
| Phone Number | (512) 248-3195 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sonja B. Mingo |
| E-Mail Address | smingo@ercot.com |
| Phone Number | 512-248-6463 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Austin Energy 111611 | Proposed an implementation date of January 5, 2011 to allow time for training. |

Proposed Protocol Language Revision

6.4.3.2	Energy Offer Curve for Responsive Reserve and Regulation Up Capacity

The following applies to Generation Resources that a QSE assigns Responsive Reserve (RRS) and/or Regulation Up (Reg-Up) Ancillary Service Resource Responsibility in its Current Operating Plan (COP) to meet the QSE’s Ancillary Service Supply Responsibility for RRS and/or Reg-Up in each Operating Hour.  This includes RRS and Reg-Up Ancillary Service Resource Responsibility arising as the result of Day-Ahead Market (DAM) or Supplemental Ancillary Services Market (SASM) Ancillary Service awards, or Self-Arranged Ancillary Service Quantity.

(a)	Prior to the start of each Operating Period that includes an Operating Hour during which a Generation Resource is assigned RRS and/or Reg-Up Ancillary Service Resource Responsibility, the QSE shall ensure that a valid Output Schedule or Energy Offer Curve for the Operating Hour has been submitted and accepted by ERCOT.  The Energy Offer Curves submitted by the QSE for these Generation Resources must meet the following requirements:

(i)	All energy output from the Generation Resource’s capacity reserved to provide Reg-Up must be priced at the System-Wide Offer Cap (SWCAP) in $/MWh; and

(ii)	All energy output for the Generation Resource’s capacity reserved to provide RRS must be priced at the SWCAP in $/MWh.

(b)	Paragraph (a) above shall not restrict the ability of the QSE to update the Energy Offer Curves for its Generation Resources in accordance with these Protocols, provided that in those Operating Hours in which a Generation Resource is assigned RRS and/or Reg-Up Ancillary Service Resource Responsibility, the portion of the Energy Offer Curve reserved for these Ancillary Services meet the requirements set forth in paragraph (a) above.

(c)	The QSE is exempted from the criteria described in paragraph (a)(i) and (a)(ii) of Section 6.4.3.2, Energy Offer Curve for Responsive Reserve and Regulation Up Capacity, if the Resource experiences a Forced Outage, Forced Derate, or Startup Loading Failure within 15 minutes and the QSE notifies ERCOT Operators of the event.