## X.X	QSE Request to Decommit in Operating Hour due to Negative LMPs

Procedure Purpose:  Negative prices at Resources Nodes signal that system conditions require a reduction in the amount of energy injected by Generation Resources.

| Protocol Reference |  |  |  |  |
| --- | --- | --- | --- | --- |
| Guide Reference |  |  |  |  |
| NERC Standard |  |  |  |  |

| Version: 1 | Revision: 0 | Effective Date: XX XX, 2011 |
| --- | --- | --- |

| Step | Action |
| --- | --- |
| Negative LMPs | IF: - A QSE requests to de-commit a Resource due to negative LMPs; THEN: - Verify that the Resource does not have A/S responsibility, - Verify that the Resource is not needed for capacity, - Grant permission to QSE to take Resource off-line, - Issue an electronic VDI - Choose “Resource” tab - Choose “QSE REQUESTED DECOMMIT” as the Instruction Type tab. |
| Determination | IF: - This decision can not be made while the QSE is on the phone; THEN: - Override the Resources HDL to their LSL, or to ramp down to the LSL. |
| Resource Needed | IF: - After consideration, it has been determined that this Resource is needed for capacity and needs to remain on-line; THEN: - Keep the override in place for as long as negative LMPs exist for the Resource. |
| Resource Not Needed | IF: - After consideration, it has been determined that this Resource is not needed for capacity and can come off-line; THEN: - Notify the QSE, release the override on the Resource and Grant permission to QSE to take Resource off-line, - Issue an electronic VDI - Choose “Resource” tab - Choose “QSE REQUESTED DECOMMIT” as the Instruction Type tab. |