| NPRR Number | 428 | NPRR Title | Energy Offer Curve Requirements for Generation Resources Assigned Non-Spin Responsibility |  |
| --- | --- | --- | --- | --- |
| Timeline | Urgent | Action |  | Recommended Approval |
| Date of Decision |  | December 1, 2011 |  |  |
| Proposed Effective Date |  | January 5, 2012 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Section Requiring Revision |  | 6.4.3.1, Energy Offer Curve for Non-Spinning Reserve Capacity (new) |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) adds a requirement for the Energy Offer Curve to be at or above $120 for On-Line Non-Spinning Reserve (Non-Spin) capacity and at or above $180 for Off-Line Non-Spin capacity. If Non-Spin Ancillary Service Resource Responsibility is reassigned to a Generation Resource during the Adjustment Period in accordance with Section 6.4.6, QSE-Requested Decommitment of Resources and Changes to Ancillary Service Resource Responsibility of Resources, the Qualified Scheduling Entity (QSE) shall submit an Energy Offer Curve that meets the requirements of Section 6.4.3.1 for the Generation Resource newly assigned Ancillary Service Resource Responsibility for Non-Spin prior to the end of the Adjustment Period. If the Ancillary Service Resource Responsibility for Non-Spin is reassigned in accordance with Section 6.4.6 from a Generation Resource that experiences a Forced Outage, Derate, or Startup Loading Failure during the Operating Period, the QSE shall submit an Energy Offer Curve that meets the requirements of Section 6.4.3.1 as soon as reasonably practicable after the event. QSE updates to an Energy Offer Curve entered at the end of the Adjustment Period become effective in ERCOT systems at the next Operating Period. Therefore, if Non-Spin is moved from one Generation Resource to another in the QSE's portfolio during an Operating Hour, any change to the Energy Offer Curve will be effective in the next Operating Period. There will not be any validation rules to verify the requirements added by this NPRR. The onus is on all QSEs to make the submission/telemetry changes needed to implement these new requirements. |  |  |
| Reason for Revision |  | To address the Commission’s discussion raised at its October 27, 2011 Open Meeting regarding Project No. 37897. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR428 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 11/9/11, NPRR428 and an Impact Analysis were posted. - On 11/16/11, Austin Energy comments were posted. - On 11/17/11, PRS considered NPRR428. - On 11/29/11, ERCOT comments were posted. - On 11/30/11, STEC comments were posted. - On 12/1/11, TAC considered NPRR428. |  |  |
| PRS Decision |  | On 11/17/11, PRS unanimously voted to grant NPRR428 Urgent status. PRS then voted to recommend approval of NPRR428 as revised by PRS; an effective date of January 5, 2012; and to forward NPRR428 to TAC. There was one opposing vote from the Consumer Market Segment. All Market Segments were present for the votes. |  |  |
| Summary of PRS Discussion |  | On 11/17/11, in response to compliance concerns, language changes were proposed so that QSEs will be exempt from the Energy Offer Curve requirements proposed in NPRR428 only if the Generation Resource experiences a Forced Outage, Force Derate or Startup Loading Failure within 15 minutes and notifies ERCOT Operators of the event. It was clarified that the Energy Offer Curve criteria would still be applicable to the portion of the Non-Spin Ancillary Service Resource Responsibility provided through power augmentation participating as Off-Line. Additional issues were raised including price reversal due to shifting of Ancillary Service obligations for economic reasons; unintended consequences that could limit shifting of Ancillary Service obligations in Real-Time; and ability to update Energy Offer Curves in the Operating Period. There was a request for NPRR428 to become effective on January 5, 2012 to allow time for ERCOT to host a market call to discuss the processes to support NPRR428 and operator training. |  |  |
| TAC Decision |  | On 12/1/11, TAC voted to recommend approval of NPRR428 as recommended by PRS in the 11/17/11 PRS Report, as amended by the 11/29/11 ERCOT comments and as revised by TAC. There was one abstention from the Cooperative Market Segment. All Market Segments were present for the vote |  |  |
| Summary of TAC Decision |  | On 12/1/11, the 11/30/11 STEC comments were discussed. It was noted that QSEs should be allowed the flexibility to reassign the Ancillary Service Resource Responsibility from one Generation Resource to another when Ancillary Services become undeliverable. Participants were concerned that limiting the shifting of Ancillary Service obligations will increase costs and introduce compliance concerns; that the goal of NPRR428 is to mitigate price suppression during deployment of Ancillary Services, but that inefficiencies should be addressed in a new NPRR or System Change Request (SCR). |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | - Addresses the Commission’s discussion raised at its October 27, 2011 Open Meeting regarding Project No. 37897. |
|  | 2 | - This NPRR will mitigate price suppression during deployment of reserve services to more accurately reflect the value of the service. |
|  | 3 |  |
|  | 4 |  |
|  | 5 |  |

| Sponsor |  |
| --- | --- |
| Name | John Dumas |
| E-mail Address | jdumas@ercot.com |
| Company | ERCOT |
| Phone Number | (512) 248-3195 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Yvette M. Landin |
| E-Mail Address | ylandin@ercot.com |
| Phone Number | (512) 248-4513 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Austin Energy 111611 | Proposed an implementation date of January 5, 2011 to allow time for training. |
| ERCOT 112911 | Proposed language to clarify paragraph (c) of Section 6.4.3.1. |
| STEC 113011 | Proposed language to allow QSEs to move Ancillary Service obligations from one Generation Resource to another. |

Proposed Protocol Language Revision

6.4.3.1	Energy Offer Curve for Non-Spinning Reserve Capacity

The following applies to Generation Resources that a QSE assigns Non-Spinning Reserve (Non-Spin) Ancillary Service Resource Responsibility in its Current Operating Plan (COP) to meet the QSE’s Ancillary Service Supply Responsibility for Non-Spin and includes both On-Line and Off-Line Non-Spin assignments arising as the result of Day-Ahead Market (DAM) or Supplemental Ancillary Services Market (SASM) Ancillary Service awards, or Self-Arranged Ancillary Service Quantity.

(a)	Prior to the end of the Adjustment Period for an Operating Hour during which a Generation Resource is assigned Non-Spin Ancillary Service Resource Responsibility, the QSE shall ensure that a valid Output Schedule or Energy Offer Curve for the Operating Hour has been submitted and accepted by ERCOT.  The Energy Offer Curves submitted by the QSE for these Generation Resources must meet the following requirements:

(i)	If the Generation Resource is providing On-Line Non-Spin capacity, the energy from this capacity may not be offered at less than $120 per MWh; or

(ii)	If the Generation Resource is providing Off-Line Non-Spin capacity, the energy from this capacity may not be offered at less than $180 per MWh.

(b)	Paragraph (a) above shall not restrict the ability of the QSE to update the Energy Offer Curves for its Generation Resources in accordance with these Protocols provided that in those Operating Hours in which a Generation Resource is assigned Non-Spin Ancillary Service Resource Responsibility, the portion of the Energy Offer Curve reserved for this Ancillary Service meets those requirements.

(c)	If, in accordance with an ERCOT Dispatch Instruction or in accordance with Section 6.4.6, QSE-Requested Decommitment of Resources and Changes to Ancillary Service Resource Responsibility of Resources, the QSE moves Non-Spin Ancillary Service Resource Responsibility from a Generation Resource that experiences a Forced Outage, Derate or Startup Loading Failure during the Operating Period described in paragraph (a) above, the QSE shall ensure that a valid Output Schedule or Energy Offer Curve that meets the requirements of this Section has been submitted as soon as reasonably practicable after the event.

(d)		For the portion of the Non-Spin Ancillary Service Resource Responsibility provided from power augmentation participating as Off-Line, the criteria described in paragraph (a)(ii) above applies.

(e) 	To ensure that the Energy Offer Curves submitted in accordance with this section meets the requirement for a monotonically increasing Energy Offer Curve, the energy offer for capacity reserved for Responsive Reserve (RRS) and Regulation Up (Reg-Up) must also be offered by the QSE at no less than the minimums set forth in paragraph (a) above.