| NPRR Number | 397 | NPRR Title | Balance of the Day Ancillary Service Market |
| --- | --- | --- | --- |
| Date |  | December 9, 2011 |  |
| Submitter’s Information |  |  |  |
| Name |  | Eric Goff on behalf of the Wholesale Market Subcommittee (WMS) |  |
| E-mail Address |  | egoff@reliant.com |  |
| Company |  | Reliant Energy Retail Services, LLC |  |
| Phone Number |  | 512-284-4908 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable |  |

Comments

At its December 7, 2011 meeting, WMS reviewed Nodal Protocol Revision Request (NPRR) 397.  After discussion, WMS voted to endorse NPRR397 as amended by the 11/17/11 Morgan Stanley comments and as revised by WMS.  WMS was in consensus to recommend that there be one reconfiguration Supplemental Ancillary Services Market (SASM) per day but did not have a preference as to whether this SASM is executed as 0800 or 0900.

Proposed Protocol Language Revision

6.4.8.2	Supplemental Ancillary Services Market

(1)	During the Adjustment Period, ERCOT may procure additional Regulation Up (Reg-Up), Regulation Down (Reg-Down), Responsive Reserve (RRS), and Non-Spinning Reserve (Non-Spin) services for the reasons, and in the amounts, specified in Section 6.4.8.1, Evaluation and Maintenance of Ancillary Service Capacity Sufficiency, using a SASM.

(2)	ERCOT shall allow QSEs to modify their Ancillary Service positions through a reconfiguration SASM run at  [0800 or 0900] daily.  This SASM allows QSEs to change their commitments from the hour three hours following the prompt hour through the end of the current trade day.

(3)	The SASM process for acquiring more Ancillary Service capacity or an Ancillary Service reconfiguration must use the following timelines:

(a)	For Ancillary Service capacity related to ERCOT desired increases, ERCOT shall send a notice at time “X” to all QSEs of the SASM.  Time X may be any time not less than two hours before the start of the Operating Hour for which the additional Ancillary Services capacity is required.

| SASM Process | QSE Activities: | ERCOT Activities: |
| --- | --- | --- |
| Time = X |  | Notify all QSEs of intent to procure additional Ancillary Services. Notify QSEs of any additional Ancillary Service Obligation, allocated to each Load Serving Entity (LSE) and aggregated to the QSE level. |
| Time = X plus 30 minutes | May submit additional Self-Arranged Ancillary Service Quantities limited to the additional Ancillary Services Obligation of the QSE. | Determine the amount of Ancillary Services to be procured. |
| Time = X plus 35 minutes |  | Execute SASM. |
| Time = X plus 45 minutes |  | Notify QSEs with awards of results. Post the quantities and MCPCs of Ancillary Services bought in the SASM. |
| Time = X plus 60 minutes | Submit updated COP with updated Ancillary Service Resource Responsibility. | Validate COPs for Ancillary Service Resource Responsibility. |

(b)	For replacement of Ancillary Service capacity related to undeliverability or failure of a QSE to provide one or more Ancillary Services, ERCOT shall have the option of executing a SASM at time “E” without providing preliminary notice to the market.  Time “E” may be any time not less than two hours before the start of the Operating Hour for which the additional Ancillary Services capacity is required.

| SASM Process | QSE Activities: | ERCOT Activities: |
| --- | --- | --- |
| Time = E |  | Execute SASM. |
| Time = E plus 15 minutes |  | Notify QSEs with awards of results. Post the quantities and MCPCs of Ancillary Services bought in the SASM. |
| Time = E plus 30 minutes | Submit updated COP with updated Ancillary Service Resource Responsibility. | Validate COPs for Ancillary Service Resource Responsibility. |

(c)	For the Ancillary Services reconfiguration, ERCOT shall execute a SASM at  [0800 or 0900] (time “E”), for the hours three hours following the prompt hour through the end of the trade day.

| SASM Process | QSE Activities: | ERCOT Activities: |
| --- | --- | --- |
| Time = E – 15 minutes | QSEs Nominate quantities of Ancillary Services that shall be included in the reconfiguration SASM. | ERCOT sets the quantities of Ancillary Services to be procured in the SASM. |
| Time = E |  | Execute SASM for hours following the prompt hour (h+1) through end of trade day. |
| Time = E plus 15 minutes |  | Notify QSEs with awards of results. Post the quantities and MCPCs of Ancillary Services bought in the SASM. |
| Time = E plus 30 minutes | Submit updated COP with updated Ancillary Service Resource Responsibility. | Validate COPs for Ancillary Service Resource Responsibility. |

(4)	Each QSE that is awarded capacity in a SASM is paid the SASM MCPC for the quantity it is awarded.

(5)	ERCOT shall allocate additional Ancillary Service Obligations to QSEs using the same percentages as the original Day-Ahead allocation of Ancillary Service Obligations.

6.7.2.1	Charges for Ancillary Service Capacity Replaced Due to SASM Reconfiguration

A charge to each QSE that reconfigures its Ancillary Service Supply Responsibility to a reduced level through a SASM is calculated based on the amount of the Ancillary Service reduction and the MCPC of the SASM in which the responsibility was reduced.  By Ancillary Service, the charge to each QSE for a given Operating Hour is calculated as follows:

(a)	For Reg-Up, if applicable:

RURQAMT q	=	MCPCRU m * RURQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RURQAMT q | $ | Reg-Up Reduced Quantity Amount per QSE—The charge to QSE q for its total capacity associated with reductions in its Ancillary Service Supply Responsibility for Reg-Up, for the hour. |
| MCPCRU m | $/MW per hour | Market Clearing Price for Capacity for Reg-Up by market—The MCPC for Reg-Up in the market m, for the hour. |
| RURQ q | MW | Reg-Up Reduced Quantity per QSE—QSE q’s total capacity associated with reductions in its Ancillary Service Supply Responsibility for Reg-Up, for the hour. |
| m | none | The SASM for the given reduction in responsibility. |
| q | none | A QSE. |

(b)	For Reg-Down, if applicable:

RDRQAMT q	=	MCPCRD m * RDRQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RDRQAMT q | $ | Reg-Down Reduced Quantity Amount per QSE—The charge to QSE q for its total capacity associated with reductions in its Ancillary Service Supply Responsibility for Reg-Down, for the hour. |
| MCPCRD m | $/MW per hour | Market Clearing Price for Capacity for Reg-Down by market—The MCPC for Reg-Down in the market m, for the hour. |
| RDRQ q | MW | Reg-Down Reduced Quantity per QSE—QSE q’s total capacity associated with reductions in its Ancillary Service Supply Responsibility for Reg-Down, for the hour. |
| m | none | The SASM for the given reduction in responsibility. |
| q | none | A QSE. |

(c)	For RRS, if applicable:

RRRQAMT q	=	MCPCRR m * RRRQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RRRQAMT q | $ | Responsive Reserve Reduced Quantity Amount per QSE—The charge to QSE q for its total capacity associated with reductions in its Ancillary Service Supply Responsibility for RRS, for the hour. |
| MCPCRR m | $/MW per hour | Market Clearing Price for Capacity for Responsive Reserve per market—The MCPC for RRS in the market m, for the hour. |
| RRRQ q | MW | Responsive Reserve Reduced Quantity per QSE—QSE q’s total capacity associated with reductions in its Ancillary Service Supply Responsibility for RRS, for the hour. |
| m | none | The SASM for the given reduction in responsibility. |
| q | none | A QSE. |

(d)	For Non-Spin, if applicable:

NSRQAMT q	=	MCPCNS m * NSRQ q

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| NSRQAMT q | $ | Non-Spin Reduced Quantity Amount per QSE—The charge to QSE q for its total capacity associated with reductions in its Ancillary Service Supply Responsibility for Non-Spin, for the hour. |
| MCPCNS m | $/MW per hour | Market Clearing Price for Capacity for Non-Spin by market—The MCPC for Non-Spin in the market m, for the hour. |
| NSRQ q | MW | Non-Spin Reduced Quantity per QSE—QSE q’s total capacity associated with reductions in its Ancillary Service Supply Responsibility for Non-Spin, for the hour. |
| m | none | The SASM for the given reduction in responsibility. |
| q | none | A QSE. |