| NPRR Number | 414 | NPRR Title | UFE Calculation Clarifications |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Approved |
| Date of Decision |  | January 17, 2012 |  |  |
| Effective Date |  | February 1, 2012 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 11.4.6, Unaccounted for Energy Calculation and Allocation 11.4.6.1, Calculation |  |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) clarifies language in the Unaccounted for Energy (UFE) calculation. |  |  |
| Reason for Revision |  | This NPRR will provide clarification in the Protocols in regards to the utilization of Direct Current Tie (DC Tie) Schedules in the UFE calculations and remove “metered” from the UFE definition since non-metered Loads are also included in the UFE calculation. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR414 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 9/12/11, NPRR414 and the associated Impact Analysis were posted. - On 10/20/11, PRS considered NPRR414. - On 11/17/11, PRS considered the 10/20/11 PRS Report and Impact Analysis for NPRR414. - On 12/1/11, TAC considered NPRR414. - On 1/17/12, the ERCOT Board considered NPRR414. |  |  |
| PRS Decision |  | On 10/20/11, PRS unanimously voted to recommend approval of NPRR414 as revised by PRS. All Market Segments were present for the vote. On 11/17/11, PRS unanimously voted to endorse and forward the 10/20/11 PRS Report as revised by PRS and Impact Analysis for NPRR414 to TAC. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 10/20/11, ERCOT staff explained that the revisions to NPRR414 align the Protocols with current ERCOT business practices. On 11/17/11, ERCOT staff suggested adding a table in Section 11.4.6.1 to define the variables used in calculating ERCOT-wide UFE. |  |  |
| TAC Decision |  | On 12/1/11, TAC unanimously voted to recommend approval of NPRR414 as recommended by PRS in the 11/17/11 PRS Report. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | 12/1/11, there was no discussion. |  |  |
| Board Decision |  | On 1/17/12, the ERCOT Board approved NPRR414 as recommended by TAC in the 12/1/11 TAC Report. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | Provide clarity in Protocols. |
|  | 2 | No changes to existing ERCOT practices or systems. |
|  | 3 |  |
|  | 4 |  |
|  | 5 |  |

| Sponsor |  |
| --- | --- |
| Name | Don Tucker |
| E-mail Address | dtucker@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3913 |
| Cell Number | 512-560-2378 |
| Market Segment | Not applicable. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sonja B. Mingo |
| E-Mail Address | smingo@ercot.com |
| Phone Number | 512-248-6463 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None. |  |

Proposed Protocol Language Revision

## DEFINITIONS

Unaccounted for Energy (UFE)

The difference between total Load for each Settlement Interval, adjusted for applicable Distribution Losses and Transmission Losses, and total ERCOT generation.

11.4.6	Unaccounted for Energy Calculation and Allocation

The DAS shall adjust the net loss adjusted Load for each aggregated group for UFE.  The Data Aggregation process will calculate the difference between net loss adjusted Load for the entire ERCOT System, which has been adjusted for Distribution Losses and Transmission Losses, and the total system Load (generation) in order to determine the total UFE.  The calculated UFE for each Settlement Interval is then allocated to Loads.  For the purpose of the UFE calculation, scheduled flow out of ERCOT on a Direct Current Tie (DC Tie) will be deemed as Load, and scheduled flow into ERCOT on a DC Tie will be deemed as generation.

11.4.6.1	Calculation of ERCOT-Wide Unaccounted For Energy

The DAS will calculate ERCOT-wide UFE as the difference between the total ERCOT generation and the total Load, adjusted for losses in ERCOT during each Settlement Interval.  UFE may be positive or negative in any single Settlement Interval.

UFE i (MWh)	= ERCOT Generation i Total – ERCOT Net Loss Adjusted Load i Total

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| UFE i | (MWh) | Total ERCOT system UFE per interval. |
| ERCOT Generation i Total | (MWh) | Total ERCOT internal generation plus sum of approved ERCOT DC Tie imports |
| ERCOT Net Loss Adjusted Load i Total | (MWh) | Total ERCOT load plus BLT exports plus sum of approved DC Tie exports, adjusted for distribution and transmission losses. Exports associated with Oklaunion exempt QSEs do not receive distribution or transmission losses. |
| i |  | Interval |