| NPRR Number | 393 | NPRR Title | SCED Constraint Management Transparency |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | January 19, 2012 |  |  |
| Proposed Effective Date |  | Upon system implementation and fulfillment of ERCOT staffing requirements. |  |  |
| Priority and Rank Assigned |  | Priority – 2012; Rank – 660 |  |  |
| Nodal Protocol Sections Requiring Revision |  | 6.5.7.1.10, Network Security Analysis Processor and Security Violation Alarm 6.5.7.1.11, Transmission Network and Power Balance Constraint Management |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) creates a reporting requirement to provide transparency into Security-Constrained Economic Dispatch (SCED) constraint management decisions. |  |  |
| Reason for Revision |  | Market Participants must understand how transmission constraints are managed in Real-Time in order to achieve liquidity in the bilateral market. While ERCOT operations have good reason for the actions that are taken to manage transmission congestion, those decisions are not fully transparent to Market Participants. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR393 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 7/14/11, NPRR393 was posted. - On 7/20/11, the motion to grant NPRR393 Urgent status failed via PRS e-mail vote. - On 7/20/11, ERCOT comments were posted. - On 8/18/11, PRS considered NPRR393. - On 9/14/11, J Aron comments were posted. - On 9/21/11, a preliminary Impact Analysis was posted. - On 9/22/11, PRS considered the 8/18/11 PRS Report and preliminary Impact Analysis for NPRR393. - On 10/12/11, a second set of ERCOT comments were posted. - On 10/17/11, WMS comments were posted. - On 11/8/11, a second set of J Aron comments were posted. - On 11/9/11, a second set of WMS comments were posted. - On 11/17/11, Luminant Energy comments were posted. - On 11/17/11, PRS again considered the 8/18/11 PRS Report and preliminary Impact Analysis for NPRR393. - On 12/15/11, PRS again considered the 8/18/11 PRS Report and preliminary Impact Analysis for NPRR393. - On 1/16/12, an Impact Analysis was posted. - On 1/16/12, a third set of ERCOT comments were posted. - On 1/19/12, PRS considered the 12/15/11 PRS Report and Impact Analysis for NPRR393. |  |  |
| PRS Decision |  | On 8/18/11, PRS unanimously voted to recommend approval of NPRR393 as submitted. All Market Segments were present for the vote. On 9/22/11, PRS unanimously voted to table NPRR393. All Market Segments were present for the vote. On 11/17/11, PRS unanimously voted to table NPRR393. All Market Segments were present for the vote. On 12/15/11, PRS voted to endorse the 8/18/11 PRS Report as amended by the 9/14/11 J Aron comments and the 11/17/11 Luminant Energy comments and as revised by PRS; and to bring an Impact Analysis back to PRS for consideration. There were two abstentions from the Independent Generator and Consumer Market Segments. All Market Segments were present for the vote. On 1/19/12, PRS unanimously voted to endorse and forward the 12/15/11 PRS Report as amended by the 1/16/12 ERCOT comments and as revised by PRS and the Impact Analysis for NPRR393 to TAC and to recommend a priority of 2012 and rank of 660. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 8/18/11, it was explained that NPRR393 is the component previously provided for in NPRR364, Clarify Active and Inactive SCED Constraint Reporting, that allows for transparency in Real-Time for constraints that are overloaded, leaving them inactive in SCED. Market Participants stated that constraint transparency is critical for understanding congestion and related pricing. On 9/22/11, PRS requested that WMS review NPRR393. On 11/17/11, it was requested that NPRR393 be tabled to allow additional time to review the 11/17/11 Luminant Energy comments. On 12/15/11, administrative edits were proposed. On 1/19/12, ERCOT Staff explained that NPRR393 creates two new reports that require analysis and subsequent reporting, resulting in the ongoing staffing impact of one Full Time Employee (FTE). Market Participants discussed the 1/16/12 ERCOT comments and their concerns that the comments changed what was intended to be a monthly reporting requirement to an annual one. ERCOT Staff clarified that the 1/16/12 ERCOT comments do not change the monthly reporting requirement, but rather provide for ERCOT to review the transmission and Outage planning process annually and to make recommendations accordingly. Additional revisions were proposed to remove ERCOT’s annual review and reporting obligations, as these obligations already currently fall with ROS. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | Qualitative benefits: - Increased transparency of Real-Time transmission constraint management. - Improved review of Real-Time transmission constraint management processes and procedures. - Increased liquidity in the bilateral market. |
|  | 2 | Impacts to market segments: - Market Participants require insight into how ERCOT manages transmission congestion. - The changes proposed in this NPRR393 will allow Market Participants to better understand the decision making process used by ERCOT to manage binding constraints which benefits all Market Segments. |

| Sponsor |  |
| --- | --- |
| Name | Bill Barnes |
| E-mail Address | bill.barnes@gs.com |
| Company | J Aron & Company |
| Phone Number | 212-934-4821 |
| Cell Number | 315-885-5925 |
| Market Segment | Independent Power Marketer (IPM) |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sandra Tindall |
| E-Mail Address | stindall@ercot.com |
| Phone Number | 512-248-3867 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 072011 | Recommended rejection of NPRR393. |
| J Aron 091411 | Provided the Business Case for NPRR393. |
| ERCOT 101211 | Proposed language revisions to address the Security Violation Analysis and Reporting and Constraint Management Transparency relationship between NPRRs 393 and 385, Security Violation Analysis and Reporting and Negative Price Floor, and combined language where appropriate. |
| WMS 101711 | Recommended that PRS table NPRR393. |
| J Aron 110811 | Proposed a backstop of five days for ERCOT to publish the report. |
| WMS 110911 | Endorsed NPRR393 as amended by the 11/8/11 J. Aron comments. |
| Luminant Energy 111711 | Proposed changes to avoid ambiguity of undefined terms. |
| ERCOT 011612 | Provided language clarifying that ERCOT shall make recommendations to the appropriate TAC subcommittee on an annual basis regarding the transmission and Outage planning process. |

Comments

Please note that NPRR385, Security Violation Analysis and Reporting and Negative Price Floor, also proposes revisions to Section 6.5.7.1.10.

Please also note that the baseline Protocol language in Section 6.5.7.1.11 has been updated due to the incorporation of NPRR364, Clarify Active and Inactive SCED Constraint Reporting, into the November 1, 2011 Protocols.

Proposed Protocol Language Revision

- 6.5.7.1.10	Network Security Analysis Processor and Security Violation Alarm

(1)	Using the input provided by the State Estimator, ERCOT shall use the NSA processor to perform analysis of all contingencies remaining in the active list.  For each contingency, ERCOT shall use the NSA processor to monitor the elements for limit violations.  ERCOT shall use the NSA processor to verify Electrical Bus voltage limits to be within a percentage tolerance as outlined in the ERCOT Operating Guides.  Contingency security violations for transmission lines and transformers occur if:

(a)	The predicted post-contingency MVA exceeds 100% of the Emergency Rating after adjustments for Real-Time weather conditions applicable to the contingency are incorporated; and

(b)	An RAP or SPS is not defined allowing relief within the time allowed by the security criteria.

(2)	When the NSA processor notifies ERCOT of a security violation, ERCOT shall immediately:

(a)	Initiate the process described in Section 6.5.7.1.11, Transmission Network and Power Balance Constraint Management;

(b)	Seek to determine what unforeseen change in system condition has arisen that has resulted in the security violation, especially those that were 125% or greater of the Emergency Rating for a single SCED interval or greater than 100% of the Emergency Rating for a duration of 30 minutes or more; and

(c)	Where, possible, seek to reverse the action (e.g. initiating a transmission clearance that the system was not properly pre-dispatched for) that has led to a security violation until further preventative action(s) can be taken.

(3)	If the SCED does not resolve an insecure state, ERCOT shall attempt to relieve the insecure state by:

(a)	Confirming that pre-determined relevant RAPs are properly modeled in the system;

(b)	Re-Dispatching generation through the mechanism of over-riding HDLs and LDLs to provide more capacity to SCED;

(c)	After declaring a Watch, as appropriate, manual Dispatch of generation;

(d)	Removing non-cascading contingency overload/constraints from the SCED process; and

(e)	If all other mechanisms have failed, ERCOT may authorize the use of a Mitigation Action Plan (MAP) previously reviewed by the appropriate TSP or DSP.  An MAP is a set of pre-defined actions taken beyond normal RAPs under emergency circumstances to relieve transmission security violations.

(4)	NSA must be capable of analyzing contingencies, including the effects of automatically deployed SPSs and RAPs.  The NSA must fully integrate into the evaluation and deployment of these SPSs and RAPs and notify the ERCOT Operator of the application of these SPSs and RAPs to the solution.

(5)	The Real-Time NSA may employ the use of appropriate ranking and other screening techniques to further reduce computation time by executing one or two iterations of the contingency study to gauge its impact and discard further study if the estimated result is inconsequential.

(6)	ERCOT shall report monthly:

(a)	All security violations that were 125% or greater of the Emergency Rating for a single SCED interval or greater than 100% of the Emergency Rating for a duration of 30 minutes or more during the prior reporting month and the number of occurrences and congestion cost associated with each of the constraints causing the security violations on a rolling 12 month basis.

(b)	Operating conditions on the ERCOT System that contributed to each security violation reported in paragraph (6)(a) above.  Analysis should be made to understand the root cause and what steps could be taken to avoid a recurrence in the future.

6.5.7.1.11	Transmission Network and Power Balance Constraint Management

(1)	ERCOT may not allow any constraint (contingency and limiting Transmission Element pair) identified by NSA to be activated in SCED until it has verified that the contingency definition in NSA associated with the constraint is accurate and appropriate given the current operating state of the ERCOT Transmission Grid.  ERCOT shall continuously post to the MIS Secure Area all constraint contingencies in the NSA.  ERCOT shall provide relevant constraint information, including, but not limited to, the contingency name as provided in the standard contingency list, whether or not the constraint is active in SCED, the overloaded Transmission Element name, the rating of the overloaded Transmission Element including Generic Transmission Limits (GTLs), and pre-contingency or post-contingency flows  For each Operating Day, ERCOT shall post to the MIS Secure Area within five days, a report listing all constraints with pre-contingency or post-contingency flows which exceeded the rating of the overloaded Transmission Element for at least 15 minutes consecutively that were not activated in SCED and an explanation of why each constraint was not activated.

(2)	ERCOT shall establish a maximum Shadow Price for each network constraint as part of the definition of contingencies.  The cost calculated by SCED to resolve an additional MW of congestion on the network constraint is limited to the maximum Shadow Price for the network constraint.

(3)	ERCOT shall establish a maximum Shadow Price for the power balance constraint.  The cost calculated by SCED to resolve either the addition or reduction of one MW of dispatched generation on the power balance constraint is limited to the maximum Shadow Price for the power balance constraint.

(4)	ERCOT shall determine the methodology for setting maximum Shadow Prices for network constraints and for the power balance constraint.  Following review and recommendation by TAC, the ERCOT Board shall review the recommendation and approve a final methodology.

(5)	The process for setting the maximum Shadow Prices as described above shall require ERCOT to obtain ERCOT Board approval of the values assigned to these caps along with the effective date for application of the cap.  Within two Business Days following approval by the ERCOT Board, ERCOT shall post the Shadow Price caps and effective dates on the MIS Public Area.

(6)	When ERCOT identifies a binding network constraint on a repeated basis ERCOT shall have procedures established to contact the appropriate TSP and validate the accuracy of the Network Operations Model according to paragraph (5) of Section 3.10.4, ERCOT Responsibilities.

(7)	If ERCOT determines that rating(s) in the Network Operations Model or configuration of the Transmission Facilities are not correct, then the TSP will provide the appropriate data submittals to ERCOT to correct the problem upon notification by ERCOT.