ERCOT BUSINESS PRACTICE

Non-Spinning Reserve Service Deployment and Recall Procedure

TAC Approved: November 4, 2010

Version _0.4

Document Revisions

| Date | Version | Description | Author(s) |
| --- | --- | --- | --- |
| 03/30/2007 | 0.1 | TAC approved version |  |
| 08/11/2010 | 0.2 | Updated to reflect changes to protocol and Current system implementation | Colleen Frosch John Dumas Resmi Surendran |
| 10/4/2010 | 0.3 | Included Non-Spinning Reserve Service Deployment and Recall Procedure Revision Process | Market Rules |
| 01/04/2012 | 0.4 | Updated to synchronize with current Operating Procedures as well as the Protocol requirements introduced by NPRR426 | Market Operations Support |

Approval Authority

Title: Manager, System Operations

Name: Colleen Frosch

Sign: Colleen Frosch	Date	01-04-12

PROTOCOL DISCLAIMER

This Business Practice describes ERCOT Systems and the response of these systems to Market Participant submissions incidental to the conduct of operations in the ERCOT Texas Nodal Market implementation and is not intended to be a substitute for the ERCOT Nodal Protocols (available at http://www.ercot.com/mktrules/nprotocols/current), as amended from time to time. If any conflict exists between this document and the ERCOT Nodal Protocols, the ERCOT Nodal Protocols shall control in all respects.

Table of Contents

1.	Nodal Market Non-Spinning Reserve Service Deployment and Recall Procedure	5

2.	Non-Spin Deployment	5

3.	Recall of Non-Spin Deployment	8

4.	Non-Spinning Reserve Service Deployment and Recall Procedure Revision Process	9

## Nodal Market Non-Spinning Reserve Service Deployment and Recall Procedure

For any Non-Spinning Reserve (Non-Spin) Service that is not continually deployed to Security-Constrained Economic Dispatch (SCED) as part of a standing On-Line Non-Spin deployment, there are three situations that will cause Non-Spin to be deployed:

- Detection of insufficient capacity for energy dispatch during periodic checking of available capacity.

- Disturbance conditions such as a unit trip, sustained frequency decay or sustained low frequency operations.

- SCED not having enough energy available to execute successfully.

In each of these cases, the ERCOT operator will make the final decision and initiate the deployment.  The ERCOT operator shall deploy Non-Spin in amounts sufficient to respond to the operational circumstances.  This means that Non-Spin may be deployed partially over time or may be deployed in its entirety.  If Non-Spin is deployed partially, it shall be deployed in increments of 100% of each Resource’s capacity.  To support partial deployment, ERCOT shall, following the Day-Ahead Market (DAM), rank, for each hour of the Operating Day, the Resources supplying Non-Spin in an economic order based on DAM Settlement Point Prices. Partial Non-Spin deployment and recall decisions shall be based on each Resource’s economic cost order.

## Non-Spin Deployment

ERCOT will deploy Non-Spin under the following conditions:

- When Ancillary Service Capacity Monitor shows (HDL – Gen) ≤ 200 MW during normal load ramps and (HASL – Gen) – (30-minute load ramp) < 500 MW, then deploy all of the available Non-Spin capacity

- When Ancillary Service Capacity Monitor shows (HDL – Gen) ≤ 200 MW during normal load ramps and (HASL – Gen) – (30-minute load ramp) < 1000 MW, then deploy half of the available Non-Spin capacity

- When PRC < 2500 MW, then deploy enough Non-Spin to recover PRC to 2500 MW or higher

- When PRC < 2300 MW, then deploy any remaining Non-Spin capacity that is still available to be deployed

- If the North-to-Houston (N_H) Voltage Stability Limit ≤ 300 MW, deploy Non-Spin (all or partial) Non-Spin in the Houston area as needed to restore reliability margin

If a condition other than those listed above indicates that additional capacity may need to be brought On-Line to manage reliability, operators will evaluate the system condition and deploy Non-Spin as needed if no other better options are available to resolve the system condition. Under emergency, the emergency process will govern the deployment of Non-Spin.

  - Off-Line Generation Resource reserved for Non-Spin

- The QSE will be sent a Resource specific Dispatch Instruction that Non-Spin has been deployed.

- The Dispatch Instruction must include the expected amount of capacity that will be available for SCED and the anticipated duration of the deployment.

- The QSE will ensure that the Non-Spin Ancillary Service Schedule telemetry for that unit has been reduced to zero within 20 minutes of the Dispatch Instruction.

- The QSE must have the Resource On-Line and ready for SCED Dispatch of Base Points within 25 minutes of the Dispatch Instruction.

- SCED will respond to the changes in Resource Status that are received by telemetry from the QSE.  A typical sequence would be from OFFNS to ONTEST and to ON once the Resource is ready for SCED Dispatch.

- Once the Resource is On-Line it is Dispatched as any other Generation Resource including any provisions for processing generation less than the Resource’s LSL.

- The Resource must, at a minimum, be capable of providing all the Non-Spin energy to SCED within 30 minutes of the Dispatch Instruction.

  - Load Resource reserved for Non-Spin

- The QSE will be sent a Resource specific Dispatch Instruction that Non-Spin has been deployed.

- The Dispatch Instruction must include the MW level of Load energy to be interrupted by the Load Resource and the estimated duration of the deployment.

- Non-Spin procured from a Load Resource block offer must be deployed as a block.

- The Load Resource must, at a minimum, interrupt not less than 95%, nor more than 150% of the requested deployment energy within 30 minutes of the Dispatch Instruction.

  - On-Line Generation Resource with an Energy Offer Curve

- For a Resource that will not use power augmentation to provide any portion of its Non-Spin Ancillary Service Resource Responsibility:

  - The QSE shall set the value of the Non-Spin Ancillary Service Schedule to zero within the 30-second window prior to the start of the delivery hour.

  - ERCOT will automatically calculate new HASL constraints for SCED using the telemetry of the Resource’s Non-Spin Ancillary Service Schedule.

  - The total amount of capacity reserved on that Resource for Non-Spin shall be considered as a standing Non-Spin deployment Dispatch Instruction for the duration of the Operating Hour.

  - A Non-Spin deployment Dispatch Instruction from ERCOT is not required for standing Non-Spin deployments.

- For a Resource that will use power augmentation to provide a specific MW portion of its Non-Spin Ancillary Service Responsibility:

  - The QSE shall set the value of the Non-Spin Ancillary Service Schedule to the appropriate value within the 30-second window prior to the start of the delivery hour.

  - The QSE may set the value of the Non-Spin Ancillary Service Schedule equal to the MW amount of Non-Spin that will be provided via power augmentation; otherwise, the QSE may set the value of the schedule to zero.

  - If the Non-Spin Ancillary Service Schedule is set to zero, then the total amount of capacity reserved on that Resource for Non-Spin shall be considered as a standing Non-Spin deployment Dispatch Instruction for the duration of the Operating Hour.

  - If the Non-Spin Ancillary Service Schedule is set to a non-zero value, then the QSE will be sent a Resource specific Dispatch Instruction indicating that Non-Spin has been deployed for the total amount of the Non-Spin Schedule.

    - The Dispatch Instruction must include the expected amount of capacity that will be available for SCED and the anticipated duration of the deployment.

    - The QSE shall reduce the Resource’s Non-Spin Ancillary Service Schedule to zero within 20 minutes following a deployment instruction.

  - ERCOT will automatically calculate new HASL constraints for SCED using the telemetry of the Resource’s Non-Spin Ancillary Service Schedule.

- The QSE must, at a minimum, ensure that the Normal Ramp Rate represented by the Resource’s ramp rate curve is sufficient to allow SCED to fully Dispatch the Resource’s Non-Spin Resource Responsibility within 30 minutes, regardless of whether or not the Resource uses power augmentation to provide the service.

  - On-Line Generation Resource with Output Schedules

- The QSE shall set the value of the Non-Spin Ancillary Service Schedule to zero within the 30-second window prior to the start of the delivery hour.

- ERCOT will automatically calculate new HASL constraints for SCED using the telemetry of the Resource’s Non-Spin Ancillary Service Schedule.

- If the QSE is sent a Resource-specific Dispatch Instruction indicating that Non-Spin has been deployed:

  - The Dispatch Instruction must include the additional amount of energy (MW) that needs to be produced by the Resource and the estimated duration of the deployment.

  - For DSRs providing Non-Spin, as soon as the QSE receives the deployment, the QSE shall adjust the telemetry Output Schedule to reflect the Non-Spin deployment.  A DSR QSE with a Load Resource that has provided Non-Spin will ensure that the Output Schedule is not reduced to reflect the Load deployment if the Load Resource is part of the DSR Load that the Resource follows.

  - For non-DSRs (with Output Schedules) providing Non-Spin, ERCOT shall increase the Output Schedule used in SCED by the difference between telemetered Non-Spin Ancillary Service Resource Responsibility and Ancillary Service Schedule to reflect the amount of Non-Spin energy that is to be provided by the Resource in response to the Non-Spin deployment.

## Recall of Non-Spin Deployment

Non-Spin will be recalled when (HDL- Gen) ≥ 500 MW and PRC is ≥ 3000 MW.

- After recall, the QSE will be allowed to control the Resource down to its LSL at the Normal Ramp Rate and then trip it off-line if the QSE wants to shutdown the Resource.  In this case, the Non-Spin Ancillary Service Schedule for that Resource will be reset to equal the Non-Spin Ancillary Service Responsibility for that Resource for that hour.  A QSE with a Generation Resource that was previously Off-Line will be allowed to keep the Resource On-Line after minimum On-Line time, provided that the difference between its HSL and LSL is greater than or equal to its Ancillary Service Resource Responsibility.

- A QSE with a Generation Resource (with an Energy Offer Curve) that will stay On-Line may set the value of the Non-Spin Ancillary Service Schedule equal to the MW amount of Non-Spin that will be provided via power augmentation; otherwise, the QSE will set the value of the Non-Spin Ancillary Service Schedule for that Resource to 0 MW.  For all SCED intervals during which the Resource remains On-Line and released to SCED, its Non-Spin Ancillary Service Resource Responsibility will be Dispatched according to the description given above in Section 2.3, On-Line Generation Resource with an Energy Offer Curve.

- A QSE with a DSR Generation Resource (with an Output Schedule) that will stay On-Line will back out the Non-Spin addition that was made to the Output Schedule.  This can be incrementally deleted depending on the size of the deployment and Normal Ramp Rate.  For non-DSR Generation Resources,   SCED will use the QSE submitted non-DSR Output Schedule once the Non-Spin has been recalled.

- A QSE with a Load Resource that has provided Non-Spin will ensure that the Load energy and Non-Spin capability is restored within three hours from the expiration of the Non-Spin deployment.  If it is not, the Non-Spin capability must be replaced by the QSE on other Generation or Load Resources capable of providing the service.

If Non-Spin has been deployed in the Houston area to help manage the N_H Voltage Stability Limit, the deployments will be recalled once reliability margins have been restored to a manageable level.

## Non-Spinning Reserve Service Deployment and Recall Procedure Revision Process

Revisions to the Non-Spinning Reserve Service Deployment and Recall Procedure shall be made according to the approval process as prescribed in Protocol Section 6.5.7.6.2.3, Non-Spinning Reserve Service Deployment.