| NPRR Number | 446 | NPRR Title | Correction of Non-Spin Ancillary Service Schedule Telemetry for Standing Non-Spin Deployment |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Approved |
| Date of Decision |  | July 17, 2012 |  |  |
| Effective Date |  | August 1, 2012 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Section Requiring Revision |  | 6.5.7.6.2.3, Non-Spinning Reserve Service Deployment |  |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) corrects the language that describes how Qualified Scheduling Entities (QSEs) will update the Non-Spinning Reserve (Non-Spin) Ancillary Service Schedule value in telemetry as introduced in NPRR426, Standing Non-Spin Deployment in the Operating Hour for Generation Resources Providing On-Line Non-Spin, which was approved by the ERCOT Board on December 12, 2011. |  |  |
| Reason for Revision |  | This NPRR corrects the Protocol language so that a QSE with a Resource that is telemetering an On-Line Resource Status and is reserving an amount of capacity for Non-Spin Ancillary Service Resource Responsibility provided via power augmentation will know how to correctly update the telemetered Non-Spin Ancillary Service Schedule in the Inter-Control Center Communications Protocol (ICCP). |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR446 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 2/29/12, NPRR446 and an Impact Analysis were posted. - On 3/22/12, PRS considered NPRR446. - On 4/19/12, PRS considered the 3/22/12 PRS Report and Impact Analysis for NPRR446. - On 6/7/12, TAC considered NPRR446. - On 7/17/12, the ERCOT Board considered NPRR446. |  |  |
| PRS Decision |  | On 3/22/12, PRS unanimously voted to recommend approval of NPRR446 as submitted. All Market Segments were present for the vote. On 4/19/12, PRS unanimously voted to endorse and forward the 3/22/12 PRS Report and Impact Analysis for NPRR446 to TAC. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 3/22/12, there was no discussion. On 4/19/12, there was no discussion. |  |  |
| TAC Decision |  | On 6/7/12, TAC unanimously voted to recommend approval of NPRR446 as recommended by PRS in the 4/19/12 PRS Report. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 6/7/12, there was no discussion. |  |  |
| ERCOT Opinion |  | ERCOT supports approval of NPRR446. |  |  |
| Board Decision |  | On 7/17/12, the ERCOT Board approved NPRR446 as recommended by TAC in the 6/7/12 TAC Report. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | Qualitative benefits: - Aligns Protocol language with current practices. |

| Sponsor |  |
| --- | --- |
| Name | Resmi Surendran |
| E-mail Address | rsurendran@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-3033 |
| Cell Number |  |
| Market Segment | Not applicable. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sandra Tindall |
| E-Mail Address | stindall@ercot.com |
| Phone Number | 512-248-3867 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None. |  |

Comments

Please note that the baseline Protocol language has been updated to reflect the unboxing of NPRR354, Revisions to Non-Spin Performance Criteria Language and Provision for ICCP Telemetry of Non-Spin Deployment, in the June 28, 2012 Protocols.

Proposed Protocol Language Revision

6.5.7.6.2.3	Non-Spinning Reserve Service Deployment

(1)	ERCOT shall deploy Non-Spin Service by operator Dispatch Instruction for the portion of On-Line Generation Resources that is only available through power augmentation and participating as Off-Line Non-Spin, Off-Line Generation Resources and Load Resources.  ERCOT shall develop a procedure approved by TAC to deploy Resources providing Non-Spin Service.  ERCOT Operators shall implement the deployment procedure when a specified threshold(s) in MW of capability available to SCED to increase generation is reached.  ERCOT Operators may implement the deployment procedure to recover deployed RRS or when other Emergency Conditions exist.  The deployment of Non-Spin must always be 100% of that scheduled on an individual Resource.

(2)	Once Non-Spin capacity from Off-Line Generation Resources providing Non-Spin is deployed and the Generation Resources are On-Line, ERCOT shall use SCED to determine the amount of energy to be dispatched from those Resources.

(3)	Off-Line Generation Resources providing Non-Spin (OFFNS Resource Status) are required to provide an Energy Offer Curve for use by SCED.

(4)	On receipt of a deployment instruction, Load Resources providing Non-Spin must, at a minimum, provide the requested deployment energy within 30 minutes of the Dispatch Instruction.  Within 20 minutes after receipt of the deployment instruction, the QSE shall reduce the Non-Spin Ancillary Service Schedule by the amount of the deployment to make the capacity available for SCED to dispatch.

(5)	Subject to the exceptions described in paragraphs (a) and (b) below, On-Line Generation Resources that are assigned Non-Spin Ancillary Service Resource Responsibility during an Operating Hour shall always be deployed in that Operating Hour.  This deployment shall be considered as a standing Protocol-directed Non-Spin deployment Dispatch Instruction.  Within the 30-second window prior to the top-of-hour clock interval described in paragraph (2) of Section 6.3.2, Activities for Real-Time Operations, the QSE shall respond to the standing Non-Spin deployment Dispatch Instruction for those Generation Resources assigned Non-Spin Ancillary Service Resource Responsibility effective at the top-of-hour by adjusting the Non-Spin Ancillary Service Schedule telemetry.  The QSE shall set the Non-Spin Ancillary Service Schedule telemetry equal to the portion of Non-Spin being provided from power augmentation if the portion being provided from power augmentation is participating as Off-Line Non-Spin, otherwise it shall be set to 0.  As described in Section 6.5.7.2, Resource Limit Calculator, ERCOT shall adjust the HASL and LASL based on the QSE’s telemetered Non-Spin Ancillary Service Schedule to account for such deployment and to make the energy from the full amount of the Non-Spin Ancillary Service Resource Responsibility available to SCED.  A Non-Spin deployment Dispatch Instruction from ERCOT is not required and these Generation Resources must be able to Dispatch their Non-Spin Ancillary Service Resource Responsibility in response to a SCED Base Point deployment instruction.  The provisions of this paragraph (5) do not apply to:

(a)	Quick Start Generation Resources (QSGRs) assigned Off-Line Non-Spin Ancillary Service Resource Responsibility and provided to SCED for deployment, which must follow the provisions of Section 3.8.3, Quick Start Generation Resources; or

(b)	The portion of On-Line Generation Resources that is only available through power augmentation if participating as Off-Line Non Spin.

(6)	Off-Line Generation Resources providing Non-Spin, while Off-Line and before the receipt of any deployment instruction, shall be capable of being dispatched to their Non-Spin Resource Responsibility within 30 minutes of a deployment instruction.  Following a deployment instruction, the QSE shall reduce the Non-Spin Ancillary Service Schedule by the amount of the deployment.  An Off-Line Generation Resource providing Non-Spin must also be brought On-Line with an Energy Offer Curve at an output level greater than or equal to P1 multiplied by LSL where P1 is defined in the “ERCOT and QSE Operations Business Practices During the Operating Hour.”  These actions must be done within a time frame that would allow SCED to fully dispatch the Resource’s Non-Spin Resource Responsibility within the 30 minute period using the Resource’s Normal Ramp Rate curve.  The Resource Status indicating that a Generation Resource has come On-Line with an Energy Offer Curve is ON as described in paragraph (4)(b)(i) of Section 3.9.1, Current Operating Plan (COP) Criteria.

(7)	For DSRs providing Non-Spin, on deployment of Non-Spin, the DSR’s QSE shall adjust its Resource Output Schedule to reflect the amount of deployment.  For non-DSRs with Output Schedules providing Non-Spin, on deployment of Non-Spin, ERCOT shall adjust the Resource Output Schedule for the remainder of the Operating Period to reflect the amount of deployment.  ERCOT shall notify the QSEs representing the non-DSR of the adjustment through the MIS Certified Area.

(8)	For On-Line Generation Resources providing Non-Spin, Base Points include Non-Spin energy as well as any other energy dispatched as a result of SCED.  These Resources’ Non-Spin Ancillary Service Resource Responsibility and Normal Ramp Rate curve should allow SCED to fully Dispatch the Resource’s Non-Spin Resource Responsibility within the 30-minute time frame according to the Resources’ Normal Ramp Rate curve.  For the portion of the Non-Spin Ancillary Service Resource Responsibility provided from power augmentation participating as Off-Line, SCED should be able to be dispatch it within 30 minutes of the Non-Spin deployment instruction.

(9)	Each QSE providing Non-Spin from a Resource shall inform ERCOT of the Non-Spin Resource availability using the Resource Status and Non-Spin Ancillary Service Resource Responsibility indications for the Operating Hour using telemetry and shall use the COP to inform ERCOT of Non-Spin Resource Status and Non-Spin Ancillary Service Resource Responsibility for hours in the Adjustment Period through the end of the Operating Day.

(10)	ERCOT may deploy Non-Spin at any time in a Settlement Interval.

(11)	ERCOT’s Non-Spin deployment Dispatch Instructions must include:

(a)	The Resource name;

(b)	A MW level of capacity deployment for Generation Resources with Energy Offer Curve, a MW level of energy for Generation Resources with Output Schedules, and interrupted amount for Load Resources; and

(c)	The anticipated duration of deployment.

(12)	ERCOT shall provide a signal via ICCP to the QSE of a deployed Generation or Load Resource indicating that its Non-Spin capacity has been deployed.

(13)	ERCOT shall, as part of its TAC-approved Non-Spin deployment procedure, provide for the recall of Non-Spin energy including descriptions of changes to Output Schedules and release of energy obligations from On-Line Resources with Output Schedules and from On-Line Resources that were previously Off-Line Resources providing Non-Spin capacity.

(14)	Non-Spin procured from a Load Resource block offer must be deployed as a block.

(15)	ERCOT shall provide a notification to all QSEs via the MIS Public Area when any Non-Spin capacity is deployed on the ERCOT System showing the time, MW quantity and the anticipated duration of the deployment.