| NPRR Number |  | NPRR Title | Changes to Ancillary Services Capacity Monitor |
| --- | --- | --- | --- |
| Date Posted |  |  |  |
| Requested Resolution |  | Normal |  |
| Nodal Protocol Section Requiring Revision |  | 6.5.7.5, Ancillary Services Capacity Monitor |  |
| Market Guide Section(s) Requiring Revision |  | Not applicable. |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) modifies the Ancillary Service Capacity Monitor information provided by ERCOT both through Inter-Control Center Communications Protocol (ICCP) telemetry and the Market Information System (MIS) Public Area. |  |
| Reason for Revision |  | This NPRR is the result of feedback from Market Participants regarding potential improvements to the Ancillary Service Capacity Monitor information provided by ERCOT. |  |
| Credit Implications |  | No. |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | Improves transparency of available capacity. |
|  | 2 | No negative impacts to any Market Segment. |
|  | 3 | [Any info re estimated cost to ERCOT?] |

| Sponsor |  |
| --- | --- |
| Name | [To be determined] |
| E-mail Address |  |
| Company | [To be determined] |
| Phone Number |  |
| Cell Number |  |
| Market Segment |  |

| Market Rules Staff Contact |  |
| --- | --- |
| Name |  |
| E-Mail Address |  |
| Phone Number |  |

Proposed Protocol Language Revision

6.5.7.5	Ancillary Services Capacity Monitor

(1)	ERCOT shall calculate the following every ten seconds and provide Real-Time summaries to ERCOT Operators and all Market Participants using ICCP, giving updates of calculations every ten seconds, and posting on the MIS Public Area, giving updates of calculations every five minutes, which show the Real-Time total system amount of:

(a)	Ancillary Service Resource Responsibility for RRS  from Generation Resources;

(b)	Ancillary Service Resource Responsibility for RRS  from Load Resources excluding Controllable Load Resources;

(c)	Ancillary Service Resource Responsibility for RRS  from Controllable Load Resources;

(d)	RRS deployed to Generation and Controllable Load Resources;

(e)	Ancillary Service Resource Responsibility for Non-Spin  from On-Line Generation Resources with Energy Offer Curves;

(f)	Ancillary Service Resource Responsibility for Non-Spin from On-Line Generation Resources with Output Schedules;

(g)	Ancillary Service Resource Responsibility for Non-Spin  from Load Resources;

(h)	Ancillary Service Resource Responsibility for Non-Spin  from Off-Line Generation Resources excluding Quick Start Generation Resources (QSGRs);

(i)	Ancillary Service Resource Responsibility for Non-Spin  from QSGRs;

(j)	Ancillary Service Resource Responsibility for Reg-Up;

(k)	Ancillary Service Resource Responsibility for Reg-Down;

(l)	Deployed Reg-Up and deployed Reg-Down;

(m)	HASL minus net real power for Generation Resources with Energy Offer Curves in the ERCOT System that can be used to increase Base Points in SCED;

(n)	LASL minus net real power for Generation Resources with Energy Offer Curves in the ERCOT System that can be used to decrease Base Points in SCED;

(o)	HASL minus net real power for Generation Resources without Energy Offer Curves in the ERCOT System that can be used to increase Base Points in SCED;

(p)	LASL minus net real power for Generation Resources without Energy Offer Curves in the ERCOT System that can be used to decrease Base Points in SCED; and

(q)	The ERCOT-wide Physical Responsive Capability (PRC) calculated as follows:

generationresourcei=onlineonlinegenerationAllresources

PRC1 =			Min(Max((RDF*HSL – Actual Net Telemetered Output)i , 0.0) , 0.2*RDF*HSLi)

generationresourcei=onlineonlinegenerationAllresources

PRC2 =	((Hydro-synchronous condenser output)i as qualified by item (9) of Operating Guide Section 2.3.1.2, Additional Operational Details for Responsive Reserve Providers))





resources

load

online

All

resource

load

online

i

PRC3 =			(RRS MW supplied from Load Resources)i

PRC =	PRC1 + PRC2 + PRC3

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| PRC1 | MW | Generation On-Line greater than 0 MW |
| PRC2 | MW | Hydro-synchronous condenser output |
| PRC3 | MW | RRS supplied from Load Resources |
| PRC | MW | Physical Responsive Capability |
| RDF |  | The currently approved Reserve Discount Factor |

(2)	Each QSE shall operate Resources providing Ancillary Service capacity to meet its obligations.  If a QSE experiences temporary conditions where its total obligation for providing Ancillary Service cannot be met on the QSE’s Resources, then the QSE may add additional capability from other Resources that it represents.  It adds that capability by changing the Resource Status and updating the Ancillary Service Schedules and Ancillary Services Resource Responsibility of the affected Resources and notifying ERCOT under Section 6.4.8.1, Evaluation and Maintenance of Ancillary Service Capacity Sufficiency.  If the QSE is unable to meet its total obligations to provide committed Ancillary Services capacity, the QSE shall notify ERCOT immediately of the expected duration of the QSE’s inability to meet its obligations.  ERCOT shall determine whether replacement Ancillary Services will be procured on behalf of the affected QSE according to Section 6.4.8.1.

[NPRR458: Replace paragraph (2) above with the following on January 1, 2013:] (2) Each QSE shall operate Resources providing Ancillary Service capacity to meet its obligations. If a QSE experiences temporary conditions where its total obligation for providing Ancillary Service cannot be met on the QSE’s Resources, then the QSE may add additional capability from other Resources that it represents. It adds that capability by changing the Resource Status and updating the Ancillary Service Schedules and Ancillary Services Resource Responsibility of the affected Resources and notifying ERCOT under Section 6.4.8.1, Evaluation and Maintenance of Ancillary Service Capacity Sufficiency. If the QSE is unable to meet its total obligations to provide committed Ancillary Services capacity, the QSE shall notify ERCOT immediately of the expected duration of the QSE’s inability to meet its obligations. ERCOT shall determine whether replacement Ancillary Services will be procured to account for the QSE’s shortfall according to Section 6.4.8.1.