NPRR Comments


	NPRR Number
	484
	NPRR Title
	 Revisions to Congestion Revenue Rights Credit Calculations and Payments

	
	

	Date
	October 10, 2012

	
	

	Submitter’s Information

	Name
	Patricia Egan / Sherry Looney

	E-mail Address
	Patricia.egan@energyfutureholdings.com / sherry.looney@luminant.com 

	Company
	Luminant Energy Company LLC

	Phone Number
	214-875-9685 / 214-875-9278

	Cell Number
	

	Market Segment
	Investor Owned Utility (IOU)


	Comments


Luminant Energy appreciates the opportunity to correct the description of the Portfolio Weighted Adder as used in the calculation of the FCEOBL in paragraph (2) of Section 16.11.4.5, Determination of the Counter-Party Future Credit Exposure. The path specific Day-Ahead Market (DAM) based adder as described in paragraph (3) of Section 7.5.5.3, Auction Process, does not form part of the calculation of the Portfolio Weighted Adder and was incorrectly included in this description.
	Revised Proposed Protocol Language


7.5.5.3
Auction Process

(1)
The CRR Auction must be a single-round, simultaneous auction for selling the CRRs available for all auction products.  ERCOT shall enter into the CRR Auction system a credit limit for each Counter-Party that has at least one CRR Account Holder.  A Counter-Party’s CRR Auction credit limit is equal to the lesser of the credit limit as determined in Section 16.11.4.6.1, Credit Requirements for CRR Auction Participation, or, if provided, the Counter-Party’s self-imposed CRR Auction credit limit.  
	[NPRR463:  Replace paragraph (1) above with the following upon system implementation:]
(1)
The CRR Auction must be a single-round, simultaneous auction for selling the CRRs available for all auction products.  ERCOT shall enter into the CRR Auction system a credit limit for each Counter-Party that has at least one CRR Account Holder.  A Counter-Party’s CRR Auction credit limit is equal to the lesser of the credit limit as determined in Section 16.11.4.6.1, Credit Requirements for CRR Auction Participation, or, if provided, the Counter-Party’s self-imposed CRR Auction credit limit for the CRR Monthly Auction or for a time-of-use within a CRR Auction held as part of a CRR Long-Term Auction Sequence.  


(2)
Prior to the CRR Auction, ERCOT will conduct a two-part pre-auction screening process.  First, if the Counter-Party’s CRR Auction credit limit is greater than that Counter-Party’s credit exposure as defined below, then the Counter-Party’s CRR Auction credit limit will be ignored as the CRR Auction is solved.  Second, for each CRR Account Holder of a Counter-Party, if the CRR Account Holder’s self-imposed credit limit is greater than that CRR Account Holder’s credit exposure as defined below, then the CRR Account Holder’s self-imposed credit limit will be ignored as the CRR Auction is solved. 

The calculated exposure for the pre-auction screening for each CRR Account Holder is the sum of the credit exposure for PTP Obligation bids, PTP Obligation offers, and PTP Option bids for that CRR Account Holder.  The calculated exposure for the pre-auction screening for each Counter-Party is the sum of the credit exposure for PTP Obligation bids, PTP Obligation offers, and PTP Option bids for that Counter-Party.  PTP Option offers have zero credit exposure.  Separately, for PTP Obligation bids, PTP Obligation offers, and PTP Option bids each, for each source/sink Settlement Point combination, the credit exposure will use the bid price and MW quantity that produces the maximum credit exposure that could result from the CRR Auction for that source/sink Settlement Point combination. 

(3)
The credit constraint for each Counter-Party is based on the following calculation:


ACR b 

= 
AOBLCR b  + AOPTCR b + AFGRCR b
Where:


AOBLCR b  =  
∑m ∑h ∑j, k [(BOBLMW m, h,(j, k), b * (Max(0, BPOBL m, h,(j, k), b) – Min(A ci99, m, h,(j, k), b, ACP m, h,(j, k) ) + S m, h,(j, k))) – (OOBLMW m, h,(j, k), b  *  Min(0, OPOBL m, h,(j, k), b))] 
AOPTCR b  = 
∑m ∑h ∑j, k [(BOPTMW m, h,(j, k), b * Max(0, BPOPT m, h,(j, k), b) – (OOPTMW m, h,(j, k), b * Min(0, OPOPT m, h,(j, k), b)))]

AFGRCR b = 
∑m ∑h ∑j, k   [(BFGRMW m, h,(j, k), b * Max(0,BPFGR m, h,(j, k), b) – OFGRMW m, h,(j, k), b * Min(0, OPFGR m, h,(j, k), b))]

 
The above variables are defined as follows:
	Variable
	Unit
	Description

	ACR b
	$
	Auction Credit Requirement – The auction credit requirement for a Counter-Party b.

	AOBLCR b
	$
	Auction PTP Obligation Credit Requirement – The auction credit requirement for all PTP Obligation bids submitted by a Counter-Party b for all Operating Days. 

	BOBLMW m, h, (j, k), b
	MW
	Bid PTP Obligation – The bid PTP Obligation with the source j and sink k for the hour h, and month m submitted by a Counter-Party b.

	BPOBL m, h, (j, k), b
	$/MW per hour
	Bid Price for PTP Obligation – Bid Price for PTP Obligation with the source j and sink k for the hour h, and month m submitted by a Counter-Party b. 

	A  ci 99, m, h, (j, k), b
	$/MW per hour
	Path-Specific DAM-Based Adder – The path-specific DAM-based adder with the source j and sink k for the hour h, and month m submitted by a Counter-Party b; calculated as 99th percentile of the average rolling consecutive DAM settled price for the reference CRR source/sink over a period that represents a month for each product type (18 days for 5*16, 8 days for 2*16, 28 days for 7*8).  A is either a negative number or equal to $0/MWh.  Specific historic DAM settled prices for source/ sink pairings can be excluded from the calculation if deemed no longer relevant following TAC review and ERCOT Board approval.

	ACP m, h, (j, k)
	$/MW per hour
	Auction Clearing Price – The auction clearing price with the source j and sink k for the hour h, and month m; and represents the most recent auction clearing price.

	S m, h, (j, k)
	$/MW per hour
	State Change Adder – The state change adder with the source j and sink k for the hour h, and month m will be set at a default of $0/MW per hour.  A change to this value will be initiated by ERCOT to mitigate against unforeseen increases to potential credit exposure and will require TAC approval to be in place for more than 60 days.  S is either zero or positive.

	OOBLMW  m, h, (j, k ), b
	MW
	Offer PTP Obligation – The offer PTP Obligation with source j and sink k for the hour h, and month m submitted by a Counter-Party b.

	OPOBL m, h, (j, k ), b
	$/MW per hour
	Offer Price for PTP Obligation – The offer price for PTP Obligation with the source j and sink k for the hour  h, and month m submitted by a Counter-Party b.

	AOPTCR b
	$
	Auction PTP Option Bid Credit Requirement – The auction credit requirement for all PTP Option bids submitted by a Counter-Party b.

	BOPTMW m, h, (j, k ),b
	MW
	Bid PTP Option – The bid PTP Option with the source j and sink k for the hour h, and month m submitted by a Counter-Party b. 

	BPOPT m, h, (j, k ), b
	$/MW per hour
	Bid Price for PTP Option – The bid price for PTP Option with the source j and sink k for the hour h, and month m submitted by a Counter-Party b.

	OOPTMW m, h, (j, k), b
	MW
	Offer PTP Option – The offer PTP Option with the source j and sink k for the hour h, and month m submitted by a Counter-Party b.

	OPOPT m, h, (j, k), b
	$/MW per hour
	Offer Price for PTP Option – The offer price for PTP Option with the source j and sink k for the hour h, and month m submitted by a Counter-Party b.

	AFGRCR b
	$
	Auction FGR Bid Credit Requirement – The auction credit requirement for all FGR bids submitted by a Counter-Party b for all Operating Days. 

	BFGRMW m, h, (j, k), b
	MW
	Bid FGR – The bid FGR with the source j and sink k for the hour h, and month m submitted by a Counter-Party b. 

	BPFGR m, h, (j, k), b
	$/MW per hour
	Bid Price for FGR – The bid price for FGR with the source j and sink k for the hour h, and month m submitted a Counter-Party b. 

	OFGRMW m, h, (j, k), b
	MW
	Offer FGR – The offer FGR with the source j and sink k for the hour h, and month m submitted by a Counter-Party b.

	OPFGR m, h, (j, k), b
	$/MW per hour
	Offer Price for FGR – The offer price for FGR with the source j and sink k for the hour h, and month m submitted by a Counter-Party b.

	b
	none
	A Counter-Party.

	m
	none
	An operating month.

	h
	none
	 An Operating Hour.

	j
	none
	 A source Settlement Point.

	k
	none
	 A sink Settlement Point.

	ci99
	none
	99th percentile confidence interval.








	




(4)
ERCOT may review preliminary CRR Auction results to ensure that post auction collateral requirements are satisfied for all CRR Account Holders participating in the CRR Auction.  If it is practicable to rerun the applicable CRR Auction, and the post CRR Auction collateral requirements for a Counter-Party are not satisfied, ERCOT:

(a)
Shall promptly notify the Counter-Party of the amount by which its Financial Security must be increased and allow it until 1500 on the next Bank Business Day from the date on which ERCOT delivered Notification to increase the Financial Security.  

(b)
If sufficient Financial Security is not received by 1500 on the next Bank Business Day, ERCOT shall void all of the Counter-Party’s bids and offers in the CRR Auction and rerun the CRR Auction without that Counter-Party’s activity.
(c)
ERCOT shall award CRRs in quantities truncated to the nearest tenth MW (0.1 MW). 
(d)
The CRR clearing price is equal to the corresponding Shadow Price for that CRR product.
(e)
When a CRR Account Holder is awarded CRRs as a result of a CRR Auction, the CRRs do not become the property of the winning CRR Account Holder, and the CRRs may not be placed in their CRR accounts, until either the required CRR Invoice has been paid, or the appropriate level of collateral as defined by the Future Credit Exposure calculation in Section 16.11.4, Determination and Monitoring of Counter-Party Credit Exposure, has been posted.
(f)
When a CRR Account Holder sells PTP Obligations as a result of an auction at a negative price, the CRR Account Holder is not relieved of the PTP Obligations until the CRR Invoices have been paid in full.  
(5)
ERCOT shall use a linear programming auction engine model for each CRR Auction that evaluates all CRR Auction bids and CRR Auction offers submitted, and selects a combination of CRR Auction bids and CRR Auction offers that:

(a)
Makes the solution simultaneously feasible within the limits of the ERCOT network capability over the auction term; and
(b)
Maximizes the objective function, which is equal to the total economic value (as expressed in the CRR Auction bids) of the awarded CRR Auction bids, less the total economic cost (as expressed in CRR Auction offers) of the awarded CRR Auction offers, while observing all applicable constraints.
(6)
The CRR Network Model must, to the extent practicable, reflect the continuous and post-contingency system operating limits and operational procedures (i.e., Special Protection Systems (SPSs) and Remedial Action Plans (RAPs)) in the Network Operations Model used by ERCOT during Real-Time Operations, as discussed below in Section 7.5.5.4, Simultaneous Feasibility Test.

(7)
Once a CRR Auction is complete, ERCOT shall archive and keep the CRR Auction system and all models used to finalize the CRR Auction results under ERCOT’s data retention policy as that policy applies to data that may be needed to resolve requests for billing adjustments under applicable billing adjustment procedures.

	[NPRR407:  Insert paragraph (7) below upon system implementation:]

(7)
Once a CRR Auction is complete, ERCOT will make available on the MIS Certified Area each active CRR Account Holder’s credit exposure calculated within the CRR Auction process (as defined in paragraphs (3)(a)(i) through (3)(a)(iii) above). 


9.9
Payment Process for CRR Auction Invoices

Payments for the Congestion Revenue Right (CRR) Auction are due only for awarded CRRs that settle in the prompt month, where the prompt month is the calendar month immediately following the current operating month.  For example, if the current operating month is September, October is the prompt month.  Awarded CRRs for all other months will be collateralized as part of the Future Credit Exposure (FCE) calculation defined in Section 16.11.4.1, Determination of Total Potential Exposure for Counter-Party.  Payment is required for all prompt month CRRs on a Business Day and Bank Business Day basis in a two-day, two-step process as detailed below.

9.9.1
Invoice Recipient Payment to ERCOT for the Awarded CRRs Settling In Prompt Month
(1)
The payment due date and time for the Invoice related to awarded CRRs that settle in the prompt month is 1700 on the third Bank Business Day after the CRR Monthly Auction Invoice date, unless third Bank Business Day is not a Business Day.  If the third Bank Business Day is not a Business Day, the payment is due by 1700 on the next Bank Business Day after the third Bank Business Day that is also a Business Day.  
(2)
All CRR Auction Invoices due, with funds owed by an Invoice Recipient, must be paid to ERCOT in U.S. Dollars (USDs) by Electronic Funds Transfer (EFT) in immediately available or good funds (i.e., not subject to reversal) on or before the payment due date. 

(3)
All CRR Auction Invoices must be paid in full on the Invoice due date.  
9.9.2
ERCOT Payment to Invoice Recipients for Awarded CRRs Settling In Prompt Month
(1)
Awarded prompt month CRR Auction Invoices with funds owed to an Invoice Recipient must be paid by ERCOT to the Invoice Recipient by 1700 on the next day that is both a Business Day and a Bank Business Day after the day that payments are due for that CRR Auction Invoice under Section 9.9.1, Invoice Recipient Payment to ERCOT for the Awarded CRRs Settling in Prompt Month, subject to ERCOT’s right to withhold payments under Section 16, Registration and Qualification of Market Participants or pursuant to the common law. 

(2)
ERCOT shall give irrevocable instructions to the ERCOT financial institution to remit, to each Invoice Recipient for same day value the amounts owed to each Invoice Recipient. 

16.11.4.5
Determination of the Counter-Party Future Credit Exposure

(1)
ERCOT shall monitor and calculate the Counter-Party’s FCE for all CRRs held by the Counter-Party as owner of record at ERCOT, for all Operating Days in the delivery month, where the delivery month starts immediately after the prompt month Invoice has been paid through the end of the next operating month.  For example, the delivery month for October would begin when the October Invoice is paid during the last week of September and end on October 31st; and in addition to CRRs held for delivery in forward months, where forward months are defined as any month further out than the delivery month, using the formula below.  
FCE o  =  FCEOBL o + FCEOPT o + FCEFGR o  -  (DMOPTMWo,  h, (j,k) * A ci99, h, (j, k))
The above variables are defined as follows:

	Variable
	Unit
	Description

	FCE o
	$
	Future Credit Exposure – FCE for all CRRs held by Counter-Party as CRR Owner o of record at ERCOT, for all Operating Days in the delivery month and all forward months that have not yet occurred.

	FCEOBL o
	$
	Future Credit Exposure for PTP Obligations – FCE for all PTP Obligations held by Counter-Party as CRR Owner o of record at ERCOT, for all Operating Days in the delivery month and all forward months that have not yet occurred. 

	FCEOPT o
	$
	Future Credit Exposure for PTP Options – FCE for all PTP Options held by the Counter-Party as CRR Owner o of record at ERCOT, for all Operating Days in the delivery month and all forward months that have not yet occurred. 

	FCEFGR o
	$
	Future Credit Exposure for FGRs – FCE for all Flowgate Rights (FGRs) held by the Counter-Party as CRR Owner o of record at ERCOT, for all Operating Days in the delivery month and all forward months that have not yet occurred. 

	DMOPTMWo,  h, (j,k)
	MW
	Delivery Month awarded PTP Option – PTP Option awarded in a delivery month with source j and sink k for the hour h, owned by the Counter-Party as CRR Owner o of record at ERCOT for all Operating Days in the delivery month.  Volumes roll-off as the delivery month rolls-off.

	A ci99,  h, (j, k)
	$/MW per hour
	Path Specific DAM Based Adder – Path specific DAM based adder calculated as 99th percentile of the average rolling consecutive DAM settled price for the reference  hour h CRR source  j and sink k over a period that represents a month for each product type (18 days for 5*16, 8 days for 2*16, 28 days for 7*8).  A is either a positive number or equal to $0/MWh.

	ci99
	none
	99th percentile confidence interval.

	o
	none
	A CRR Owner.

	h
	none
	An Operating Hour.

	j
	none
	A source Settlement Point.

	k
	none
	A sink Settlement Point.




	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


(2)
The Counter-Party’s FCE for all PTP Obligations (FCEOBL) held by the Counter-Party as owner of record at ERCOT for all Operating Days that have not yet occurred and for CRRs that have not settled is calculated as follows:

FCEOBL o
=  
∑m ∑h ∑j, k [(FPOBLMW m, h, (j, k), o * POBLACP m, h, (j, k), o) – (FSOBLMW m, h, (j, k), o  *  SOBLACP m, h, (j, k), o) + (NAOBLMW m, h, (j, k), o * -∑m ∑h∑j,k (Min(PWAci100, m, h, (j, k), o  , PWACP m, h,(j,k), o) +S m, h,(j,k)))]
The above variables are defined as follows:
	Variable
	Unit
	Description

	FCEOBL o
	$
	Future Credit Exposure for PTP Obligations – FCE for all PTP Obligations held by the Counter-Party as CRR Owner o of record at ERCOT for all Operating Days in the delivery month and all forward months. 

	FPOBLMW m, h, (j, k), o
	MW
	Forward Purchased PTP Obligations – Forward purchased PTP Obligations with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	POBLACP m, h, (j, k), o
	$/MW per hour
	 Purchased Auction Clearing Price – The auction clearing price of purchased PTP Obligations with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	FSOBLMW m, h, (j, k), o
	MW
	Forward Sold PTP Obligations – Forward sold PTP Obligations with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	SOBLACP m, h, (j, k), o
	$/MW per hour
	Sale Auction Clearing Price – The auction clearing price of  sold PTP Obligations with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	NAOBLMW m, h, (j, k), o
	MW
	Net Awarded PTP Obligations – Net awarded PTP Obligations with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.  Volumes will not roll-off until the delivery month is complete.

	PWA ci100, m, h, (j, k), o
	$/MW per hour
	Portfolio Weighted Adder – The portfolio weighted adder calculated as the 100th percentile of a volume weighted average rolling consecutive DAM settled price for each Counter-Party as owner o based on volumes owned by source j and sink k for the hour h, and month m, over a period that represents a month for each product type (18days for 5*16, 8 days for 2*16, 28 days for 7*8).  The portfolio weighted adder is either a negative number or equal to $0/MWh.  Specific historic DAM settled prices for source – sink pairings can be excluded from the calculation if deemed no longer relevant following TAC review and ERCOT Board approval.

	PWACP m, h, (j, k), o
	$/MW per hour
	Portfolio Weighted Auction Clearing Price – The portfolio weighted auction clearing price calculated as the volume weighted auction clearing price for each Counter-Party as CRR Owner o of record at ERCOT based on the most recent auction clearing price by source j and sink k for the hour h, and month m  and volumes owned by source j and sink k for the hour  h, and month m .

	S m, h, (j, k)
	$/MW per hour
	State Change Adder – The state change adder with the source j and sink k for the hour h, and month m will be set at default of $0/MWper hour.  A change to this value will be initiated by ERCOT to mitigate against unforeseen increases to potential credit exposure and will require TAC approval if in place for more than 60 days.

	j
	none
	A source Settlement Point.

	k
	none
	A sink Settlement Point.

	m
	none
	An operating month.

	h
	none
	An Operating Hour for all hours in the delivery month and all forward months.

	o
	none
	A CRR Owner.

	ci100
	none
	100th percentile confidence interval.













	
	
	

	
	
	

	
	
	

	
	
	
· 
· 
· 

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


 CRR ownerg hour for all hours in the Delivery Month and all forward monthsward months that have not yet occurred. 











 (3)
The FCE for PTP Options held by the Counter-Party for all Operating Days in the delivery month and all forward months is calculated as follows:

FCEOPTo 
=    ∑m ∑h ∑j, k [(FPOPTMW m, h, (j, k), o  * POPTACP m, h, (j, k), o) - (FSOPTMWm, h, (j, k), o  * SOPTACP m, h, (j, k), o)]


The above variables are defined as follows:
	Variable
	Unit
	Desciption

	FCEOPT o
	$
	Future Credit Exposure for PTP Options –  FCE for all PTP Options held by the Counter-Party as CRR Owner o of record at ERCOT in the forward months. 

	FPOPTMW  m, h, (j, k), o
	MW
	Forward Purchased PTP Options – Forward purchased PTP Options with the source j and sink k for the hour h and month m, owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	POPTACP m, h, (j, k), o
	$/MW per hour
	 Purchase Auction Clearing Price – The auction clearing price of purchased PTP Options with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	FSOPTMW  m, h, (j, k), o
	MW
	 Forward Sold PTP Options –  Forward sold PTP Options with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	SOPTACP m,  h, (j, k), o
	$/MW per hour
	 Sale Auction Clearing Price – The auction clearing price of  sold PTP Options with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	j
	none
	A source Settlement Point.

	k
	none
	A sink Settlement Point.

	m
	none
	An operating month.

	h
	none
	An Operating Hour in the delivery month or forward months.

	o
	none
	A CRR Owner.


 






	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


(4)
The FCE for FGRs (FCEFGR) held by the Counter-Party for all Operating Days in the forward months is calculated as follows:

 FCEFGR o 
= ∑m ∑h ∑j,k [(FPFGRMW m, h, (j, k), o  * PFGRACP m, h, (j, k), o) - (FSFGRMW m, h, (j, k), o  * SFGRACP m, h, (j, k), o)]

The above variables are defined as follows:

	Variable 
	Unit
	Description

	FCEFGR o
	$
	Future Credit Exposure for FGRs – FCE for all FGRs held by the Counter-Party as CRR Owner o of record at ERCOT in the forward months. 

	FPFGRMW , m,h, (j, k), o
	MW
	Forward Purchased FGRs  –  Forward Purchased FGRs with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	PFGRACP m, h, (j, k), o
	$/MW per hour
	Purchase Auction Clearing Price – The auction clearing price of purchased PTP FGRs with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	FSFGRMW , m, h, (j, k), o
	MW
	Forward Sold FGRs  – Forward sold FGRs with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	SFGRACP m, h, (j, k), o
	$/MW per hour
	 Sale Auction Clearing Price  – The auction clearing price of awarded Offer PTP FGRs with the source j and the sink k for the hour h, and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.

	j
	none
	A source Settlement Point.

	k
	none
	A sink Settlement Point.

	m
	none
	An operating month.

	h
	none
	An Operating Hour in the delivery month or forward months.

	o
	none
	A CRR Owner.











	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	

	
	
	


	[NPRR347:  Replace Section 16.11.4.5 above with the following upon system implementation:] 

16.11.4.5
Determination of the Counter-Party Future Credit Exposure

(1)
ERCOT shall monitor and calculate the Counter-Party’s FCE for all CRRs held by the Counter-Party as owner of record at ERCOT, for all Operating Days in the delivery month, where the delivery month starts immediately after the prompt month Invoice has been paid through the end of the next operating month.  For example, the delivery month for October would begin when the October Invoice is paid during the last week of September and end on Oct 31st; and in addition to CRRs held for delivery in forward months, where forward months are defined as any month further out than the delivery month, using the formula below.  

FCE o = FCEOBL o + FCEOPT o + FCEFGR o -  (DMOPTMWo,  h, (j,k) * A ci99, h, (j, k))
The above variables are defined as follows:

Variable

Unit

Description

FCE o
$

Future Credit Exposure – FCE for all CRRs held by the Counter-Party as CRR Owner o of record at ERCOT, for all Operating Days in the delivery month and all forward months that have not yet occurred.

FCEOBL o
$

Future Credit Exposure for PTP Obligations – FCE for all PTP Obligations held by the Counter-Party as CRR Owner o of record at ERCOT, for all Operating Days in the delivery month and all forward months that have not yet occurred.
FCEOPT o
$

Future Credit Exposure for PTP Options – FCE for all PTP Options held by the Counter-Party as CRR Owner o of record at ERCOT, for all Operating Days in the delivery month and all forward months that have not yet occurred.

FCEFGR o
$

Future Credit Exposure for FGRs – FCE for all Flowgate Rights (FGRs) held by the Counter-Party as CRR Owner o of record at ERCOT, for all Operating Days in the delivery month and all forward months that have not yet occurred.

DMOPTMWo,  h, (j,k)
MW
Delivery Month awarded PTP Option – PTP Option awarded in a delivery month with source j and sink k for the hour h, owned by the Counter-Party as CRR Owner o of record at ERCOT for all Operating Days in the delivery month.  Volumes roll-off as the delivery month rolls-off.
A ci99,  h, (j, k)
$/MW per hour
Path Specific DAM Based Adder – Path specific DAM based adder calculated as 99th percentile of the average rolling consecutive DAM settled price for the reference CRR source-sink over a period that represents a month for each product type (18 days for 5*16, 8 days for 2*16, 28 days for 7*8).  A is either a positive number or equal to $0/MWh.
ci99
none
99th percentile confidence interval.
o

none

A CRR Owner.
h
none
An Operating Hour.
j
none
A source Settlement Point.
k
none
A sink Settlement Point.
(2)
The Counter-Party’s FCE for all PTP Obligations (FCEOBL) held by the Counter-Party as owner of record at ERCOT for all Operating Days that have not yet occurred and for CRRs that have not settled is calculated as follows:

FCEOBL o 
=
∑m ∑h ∑j, k [(FPOBLMW m, h, (j, k), o * POBLACP m, h, (j, k), o) – (FSOBLMW m, h, (j, k), o  *  SOBLACP m, h, (j, k), o) + (NAOBLMW m, h, (j, k), o * -∑m ∑h∑j,k (Min(PWAci100, h, (j, k), o , PWACP m, h,(j,k), o) +S m, h,(j,k)))]










The above variables are defined as follows:

Variable

Unit

Description

FCEOBL o
$

Future Credit Exposure for PTP Obligations – FCE for all PTP Obligations held by the Counter-Party as CRR Owner o of record at ERCOT for all Operating Days in the delivery month and all forward months .







· 
· 
· 





















FPOBLMW m, h, (j, k), o
MW
Forward Purchased PTP Obligations – Forward purchased PTP Obligations with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
POBLACP m, h, (j, k), o
$/MW per hour
 Purchased Auction Clearing Price – The auction clearing price of purchased PTP Obligations with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
FSOBLMW m, h, (j, k), o
MW
Forward Sold PTP Obligations – Forward sold PTP Obligations with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
SOBLACP m, h, (j, k), o
$/MW per hour
Sale Auction Clearing Price – The auction clearing price of  sold PTP Obligations with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
NAOBLMW m, h, (j, k), o
MW
Net Awarded PTP Obligations – Net awarded PTP Obligations with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.  Volumes will not roll-off until the delivery month is complete.
PWA ci100, m, h, (j, k), o
$/MW per hour
Portfolio Weighted Adder – The portfolio weighted adder calculated as the 100th percentile of a volume weighted average rolling consecutive DAM settled price for each Counter-Party as CRR Owner o of record at ERCOT based on volumes owned by source j and sink k for the hour h, and month m, over a period that represents a month for each product type (18days for 5*16, 8 days for 2*16, 28 days for 7*8).  The portfolio weighted adder is either a negative number or equal to $0/MWh.  Specific historic DAM settled prices for source – sink pairings can be excluded from the calculation if deemed no longer relevant following TAC review and ERCOT Board approval.
PWACP m, h, (j, k), o
$/MW per hour
Portfolio Weighted Auction Clearing Price – The portfolio weighted auction clearing price calculated as the volume weighted auction clearing price for each Counter-Party as CRR Owner o of record at ERCOT based on the most recent Auction Clearing price by source j and sink k for the hour  h, and month m  and volumes owned by source j and sink k for the hour  h, and month m. 
S m, h, (j, k)
$/MW per hour
State Change Adder – The state change adder with the source j and sink k for the hour h, and month m will be set at default of $0/MWper hour.  A change to this value will be initiated by ERCOT to mitigate against unforeseen increases to potential credit exposure and will require TAC approval if in place for more than 60 days. 
j

none

A source Settlement Point.
k

none

A sink Settlement Point.
m
none
An operating month.
h

none

An Operating Hour for all hours in the delivery month and all forward months.
o

none

A CRR Owner.
ci100
none
100th percentile confidence interval.
(3)
The FCE for all PTP Options (FCEOPT) held by the Counter-Party for all Operating Days in the delivery month and all forward months is calculated as follows:
FCEOPT o 
=
∑m ∑h ∑j, k [(FPOPTMW m, h, (j, k), o  * POPTACP m, h, (j, k), o) - (FSOPTMWm, h, (j, k), o  * SOPTACP m, h, (j, k), o)]







The above variables are defined as follows:

Variable

Unit

Description

FCEOPT o
$

Future Credit Exposure for PTP Options – FCE for all PTP Options held by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.  





















FPOPTMW  m, h, (j, k), o
MW
Forward Purchased PTP Options – Forward purchased PTP Options with the source j and sink k for the hour h and month m, owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
POPTACP m, h, (j, k), o
$/MW per hour
 Purchase Auction Clearing Price – The auction clearing price of purchased PTP Options with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
FSOPTMW  m, h, (j, k), o
MW
Forward Sold PTP Options – Forward sold PTP Options with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
SOPTACP m,  h, (j, k), o
$/MW per hour
 Sale Auction Clearing Price – The auction clearing price of  sold PTP Options with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
j

none

A source Settlement Point.

k

none

A sink Settlement Point.

m
none
An operating month.
h

none

An Operating Hour in the delivery month or forward months.
o

none

A CRR Owner.
(4)
The FCE for all FGRs (FCEFGR) held by the Counter-Party as owner of record at ERCOT for all Operating Days in the forward months is calculated as follows:

FCEFGR o 
=
∑m ∑h ∑j,k [(FPFGRMW m, h, (j, k), o  * PFGRACP m, h, (j, k), o) - (FSFGRMW m, h, (j, k), o  * SFGRACP m, h, (j, k), o)]






The above variables are defined as follows:

Variable

Unit

Description

FCEFGR o
$

Future Credit Exposure for FGRs – FCE for all FGRs held by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.  





















FPFGRMW , m,h, (j, k), o
MW
Forward Purchased FGRs – Forward Purchased FGRs with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
PFGRACP m, h, (j, k), o
$/MW per hour
Purchase Auction Clearing Price – The auction clearing price of purchased PTP FGRs with the source j and the sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
FSFGRMW , m, h, (j, k), o
MW
Forward Sold FGRs – Forward sold FGRs with the source j and sink k for the hour h and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
SFGRACP m, h, (j, k), o
$/MW per hour
 Sale Auction Clearing Price – The auction clearing price of awarded Offer PTP FGRs with the source j and the sink k for the hour h, and month m owned by the Counter-Party as CRR Owner o of record at ERCOT in the forward months.
j
none
A source Settlement Point
k
none
A sink Settlement Point
m
none
An operating month.
h

none

An Operating Hour in the delivery month or forward months.
o

none

A CRR Owner.


	


PRR_Template.doc
Page 2 of 14
PRR_Template.doc
Page 2 of 14
484NPRR-02 Luminant Energy Comments 101012.doc
Page 1 of 22
PUBLIC

_1411358877.unknown

_1411358881.unknown

_1411358885.unknown

_1411358887.unknown

_1411358888.unknown

_1411358889.unknown

_1411358886.unknown

_1411358883.unknown

_1411358884.unknown

_1411358882.unknown

_1411358879.unknown

_1411358880.unknown

_1411358878.unknown

_1411358875.unknown

_1411358876.unknown

_1411358874.unknown

