ERCOT Operations Report

October 2012 ROS & OWG

Frequency Control

| Time | Freq(Hz) | Duration (s) |  |
| --- | --- | --- | --- |
| 10/3/2012 15:29 | 59.887 | 12 | Multiple Unit Trips with 501 MW net online. |
| 10/5/2012 16:41 | 59.869 | 109 | Unit Trip with 580 MW net online. |
| 10/12/2012 19:22 | 59.740 | 108 | Unit Trip with 850 MW net online. |
| 10/13/2012 2:28 | 59.845 | 20 | Unit Trip with 600 MW net online. |
| 10/14/2012 3:57 | 59.883 | 8 | East DC tie Trip with 467 MW net online. |
| 10/19/2012 12:55 | 59.892 | 8 | Unit Trip with 723 MW net online. |
| 10/25/2012 15:16 | 59.806 | 52 | Unit Trip with 615 MW net online. |

Note: Time of frequency excursions outside the range of +/- 0.1 Hz.

PMU Events

None

Deployed Responsive (Reported Above)

| Time deployed | Stop Time | Duration | Max | Details |
| --- | --- | --- | --- | --- |
| 10/3/12 15:29:42 | 10/3/12 15:31:30 | 0:01:48 | 164.67 |  |
| 10/5/12 16:41:54 | 10/5/12 16:45:34 | 0:03:40 | 228.35 |  |
| 10/12/12 19:22:22 | 10/12/12 19:26:10 | 0:03:48 | 1070.3 |  |
| 10/13/12 2:28:22 | 10/13/12 2:30:58 | 0:02:36 | 571.82 |  |
| 10/14/12 3:56:50 | 10/14/12 3:58:54 | 0:02:04 | 398.52 |  |
| 10/19/12 12:55:18 | 10/19/12 12:56:46 | 0:01:28 | 412.13 |  |
| 10/25/12 15:16:42 | 10/25/12 15:19:02 | 0:02:20 | 642.52 |  |

RUC Commitments

SCED Congestion Management Activity

Congestion affected by outages  Remedial action plan has been developed for the element

| Contingency | Overloaded Element | Days | MP |
| --- | --- | --- | --- |
| Pawnee Switching Station - Calaveras 345kV | Pawnee Switching Station Xf1 345/138 kV | 15 | N |
| Sun Switch - Morgan Creek SES 138 kV | China Grove Switch - Bluff Creek Switch 138 kV | 14 | N |
| Cisco - Putnam 138kV & Leon Switch 138kV | Putnam 138 kV - Leon Switch 138kV | 10 | N |
| Salt Creek Bepc - Spur And Scurry Switch 138kV | Aspermont AEP - Aspermont Continental 69 kV | 7 | N |
| Falcon Seaboard - Morgan Creek SES 345kV | Midland West - Windwood 138kV | 7 | N |
| Moss Switch - Holt Switch & Amoco North Cowden 138kV | Odessa North - Odessa Basin Switch 69 kV | 7 | Y |
| Fort Stockton Plant - Barrilla 138 kV | Fort Stockton Switch - Barrilla 69 kV | 7 | N |
| DCKT South Channel - Pair & Shell 138 kV | Conial - Waburn 138kV | 7 | N |
| Salt Creek Bepc - Spur And Scurry Switch 138 kV | Aspermont AEP 138_69T1 138/69 kV | 6 | N |
| Morgan Creek SES Axfmr1t (3) 345/138 kV | Morgan Creek SES Axfmr4t (3) 345/138 kV | 6 | N |
| Morgan Creek SES Axfmr4t (3) 345/138 kV | Morgan Creek SES Axfmr1t (3) 345/138 kV | 6 | N |
| Ennis Creek Switch - Knapp 138 kV | Sun Switch - Morgan Creek SES 138 kV | 6 | N |
| Midland East - Moss Switch 345 kV | General Tire Switch - Southwestern Portland Tap 138kV | 6 | N |
| DCKT Graham SES - Long Creek & Cook Field Road Switch 345 kV | Hamilton Road - Maverick 138 kV | 5 | N |
| DCKT Salado Switch - Killeen Switch 345 kV | Belton - Belton Southwest 138 kV | 5 | N |
| Ajo - Rio Hondo 345 kV | Las Pulgas - Rio Hondo 345 kV | 5 | N |
| DCKT Longshore Switch - Quail Switch & Morgan Creek SES 345 kV | Midland West - Windwood 138 kV | 4 | N |
| Scurry Switch - Sun Switch 138 kV | Aspermont AEP 138_69T1 138/69 kV | 4 | N |
| DCKT Big Brown SES - Jewett 345 kV | Jewett - Butler Tap 138 kV | 4 | N |
| Meadow Sub - PH Robinson 345 kV | Hastings TNP - Friendswood TNP 138 kV | 4 | N |
| Del Rio City - Buena Vista & Hamilton Road 138 kV | Hamilton Road - Maverick 138 kV | 4 | N |
| Scurry Switch - Sun Switch 138 kV | Aspermont AEP - Aspermont Continental 69 kV | 3 | N |
| Bluff Creek Switch - China Grove Switch 138 kV | Ennis Creek Switch Axfmr1 138/69 kV | 3 | N |
| Pawnee Switching Station Xf1 345/138 kV | Coleto Creek - Kenedy Swtich 138 kV | 3 | N |
| Snyder - China Grove Switch 138 kV | Ennis Creek Switch Axfmr1 138/69 kV | 3 | N |
| Haskell - Josselett - Munday AEP 69 kV | Munday AEP - Bkem Munday 69 kV | 3 | N |
| DCKT Skyline - Calaveras 345 kV | Cagnon Axfmr4L (3) 345/138 kV | 3 | N |
| DCKT Willow Creek Switch - Parker Switch 345 kV | Barton Switch - Oran Sub 138 kV | 3 | N |
| DCKT Longshore Switch - Quail Switch & Morgan Creek SES 345 kV | Glenhaven - Crmwd 8 Tap 138 kV | 2 |  |
| DCKT Longshore Switch - Quail Switch & Morgan Creek SES 345 kV | Odessa Ehv Switch - Big Three Odessa Tap 138 kV | 2 |  |
| Falcon Seaboard - Morgan Creek SES 345 kV | Spraberry Switch - Midland Downtown 138kV | 2 |  |
| Moss Switch - Holt Switch & Amoco North Cowden 138 kV | Odessa North Axfmr1 138/69 kV | 2 |  |
| Bluff Creek Switch - China Grove Switch 138 kV | Plainview Tap - Cogdell Tap 69 kV | 2 |  |
| DCKT Permian Basin SES - Wink Sub & Wickett Tap 138 kV | Northward - Wink Sub 69 kV | 2 |  |
| DCKT Permian Basin SES - Wink Sub & Wickett Tap 138 kV | Wickett TNP - Winkler County 6 TNP 69 kV | 2 |  |
| DCKT Navarro - Big Brown SES And Venus Switch 345 kV | Big Brown SES - Richland Chambers 345kV (20) | 2 |  |
| DCKT Morgan Creek SES - China Grove Switch 138kV | Colorado City - Morgan Creek SES 69kV | 2 |  |
| DCKT Watermill - Tricorner And Trinidad SES 345 kV | Seagoville - Kleberg Tap 138 kV | 2 |  |
| Lobo - San Miguel 345_138 KV Switchyards 345 kV | Laquinta - Lobo 138 kV | 2 |  |
| DCKT Wa Parish - Jeanetta 345kV | Smithers - Bellaire 345 kV | 2 |  |
| Kendall - Cagnon 345 kV | Comfort - Cico 138 kV | 2 |  |
| DCKT Greens Bayou - King 345 kV | Greens Bayou At2L (3) 345/138 kV | 2 |  |
| DCKT Dupont Swtich - Ingleside - Whitepoint 138 kV | Rincon - Whitepoint 138 kV (Rincon_White_2) | 2 |  |
| Aspermont AEP - Spur 138 kV | Spur 138_69_1 138/69 kV | 2 |  |
| Basecase | Greens Bayou At2L (3) 345/138 kV | 1 |  |
| Scurry Switch - Sun Switch 138 kV | Munday AEP - Weinert 69kV | 1 |  |
| Scurry Switch - Sun Switch 138 kV | Radium - Hamlin Rea 69kV | 1 |  |
| Scurry Switch - Sun Switch 138 kV | Longworth - Roby 69kV | 1 |  |
| Morgan Creek SES Axfmr1t (3) 345/138 kV | Sterling City - Sterling County 69 kV | 1 |  |
| Bluff Creek Switch - China Grove Switch 138 kV | Plainview Tap - Snyder 69kV | 1 |  |
| Meadow Sub - PH Robinson 345 kV | North Alvin TNP - Alvin TNP 138 kV | 1 |  |
| Pawnee Switching Station Xf1 345/138 kV | Sinton - Skidmore 69 kV | 1 |  |
| DCKT Navarro - Big Brown SES And Venus Switch 345 kV | Big Brown SES - Richland Chambers 345 kV (30) | 1 |  |
| DCKT Navarro - Big Brown SES And Venus Switch 345 kV | Richland Chambers - Trinidad SES 345 kV (21) | 1 |  |
| DCKT Long Creek - Sweetwater East Switch & Abilene Mulberry Creek 345kV | Putnam 138kV - Pecan Bayou 138kV | 1 |  |
| DCKT Long Creek - Sweetwater East Switch & Abilene Mulberry Creek 345kV | Hamilton Road - Maverick 138kV | 1 |  |
| DCKT Morgan Creek SES - China Grove Switch 138kV | Morgan Creek SES - Sun Switch 138kV | 1 |  |
| DCKT Watermill - Tricorner And Trinidad SES 345 kV | Prairie Creek - Parkdale Switch 138 kV | 1 |  |
| Lobo - San Miguel 345_138 KV Switchyards 345 kV | Hamilton Road - Maverick 138 kV | 1 |  |
| DCKT Watermill Switch - Desoto Switch & Cedar Hill Switch 345 kV | West Levee Switch Axfmr2h (3) 345/138 kV | 1 |  |
| DCKT Watermill Switch - Desoto Switch & Cedar Hill Switch 345 kV | West Levee Switch - Dallas West 138 kV | 1 |  |
| Coleto Creek - Pawnee Switching Station 345 kV | Victoria - Loop 463 Sub 138 kV | 1 |  |
| Coleto Creek - Pawnee Switching Station 345 kV | Coleto Creek - Kenedy Swtich 138 kV | 1 |  |
| Formosa Xf_Tr_21_L 138/69 kV | Formosa Xf_Tr_22_L 138/69 kV | 1 |  |
| Pawnee Switching Station - San Miguel 345_138 Kv Switchyards 345kV | San Miguel 345_138 Kv Switchyards Autb_L (3) 345/138 kV | 1 |  |
| Ena Snyder Wind Unit Ena1 | Ennis Creek Switch Axfmr1 138/69 kV | 1 |  |
| Lobo 2_345_138_H 345/138 KV | Laquinta - Lobo 138 kV | 1 |  |
| Longshore Switch - Quail Switch & Morgan Creek SES 345 kV (6050) | Morgan Creek SES Axfmr1h (3) 345/138 kV | 1 |  |
| DCKT Sam Switch - Tradinghouse SES & Venus Switch 345 kV | Elm Mott - Rogers Hill Bepc 138 kV | 1 |  |
| Falcon Seaboard - Midland East 345 kV | Morgan Creek SES Axfmr1h (3) 345/138 kV | 1 |  |
| Mcdonald Road - Big Spring Switch 138 kV | Forsan Tap - Crmwd 7 Tap 138 kV | 1 |  |
| Odessa Ehv Switch Axfmr2t (3) 345/138 kV | Odessa Ehv Switch Axfmr3t (3) 345/138 kV | 1 |  |
| Big Brown SES - Jewett 345 kV (40) | Big Brown SES - Jewett 345 kV (50) | 1 |  |
| DCKT Trinidad SES - Watermill Switch & Tri Corner 345 kV | Forest Grove Switch - Mabank Tap 138 kV | 1 |  |
| Bracketville - Odlaw Switchyard & Hamilton Road 138 kV | Hamilton Road - Maverick 138 kV | 1 |  |
| DCKT Northbelt - King & Cedar Bayou Plant 345 kV | Greens Bayou At2L (3) 345/138 kV | 1 |  |
| Lewisville Switch - Carrollton Northwest 345 kV (105_A) | Jones Street TNP - Lakepointe TNP 138 kV | 1 |  |
| Comanche Switch (Oncor) - Brown Switch 345 kV | Hamilton Road - Maverick 138 kV | 1 |  |
| San Miguel 345_138 kV Switchyards - Elmcreek 345 kV (SMG_ELMCRK2) | San Miguel 345_138 kV Switchyards Auto_L (3) 345/138 kV | 1 |  |
| Koch Up River - Gila 138 kV | Weil Tract - Citgo West 138 kV | 1 |  |
| DCKT Mt Enterprise - Tenaska Gateway & Nacogdoches Se & Trinidad SES and Martin Lake SES - Stryker Creek SES 345 kV | Tyler Grande - Whitehouse 138 kV | 1 |  |
| Orange Grove Switching Station - Lon Hill 138 kV | Sinton - Skidmore 69 kV | 1 |  |
| DCKT Lytton Springs - Holman Aen & Fayette Plant 2 345 kV | Austrop Au_At1L 345/138 kV | 1 |  |
| Holt Switch Axfmr1 138/69 kV | Andrews North Axfmr1 138/69 kV | 1 |  |
| Andrews North Axfmr1 and Axfmr2 138/69 kV | Holt Switch - Emma Tap 69 kV | 1 |  |
| Odessa Texas Instruments - Odessa Ehv Switch & Spraberry Switch 138 kV | Glenhaven - Crmwd 8 Tap 138 kV | 1 |  |
| Paducah Clare Street 138_69T1 138/69 kV | Paducah Clare Street - Paduca City 69 kV | 1 |  |
| Laquinta - Lobo 138 kV | Falfurrias 138_69A1 138/69 kV | 1 |  |
| DCKT Eagle Mountain SES - Alliance - Roanoke Switch - Parker Switch 345 kV | Parker Switch - Benbrook Switch 345 kV (6042) | 1 |  |
| South Channel - Jefferson 138 kV | Galena Park At1_h (3) 138/69 kV | 1 |  |
| Golden Switch - Sun Switch 138 kV | China Grove Switch - Bluff Creek Switch 138 kV | 1 |  |
| DCKT Roans Prairie - Kuykendahl - King and Singleton - Tomball 345 kV | Singleton - Zenith 345 kV (SNGZEN99) | 1 |  |
| DCKT Lewisville Switch - Carrollton Northwest 345 kV | Jones Street TNP - Lakepointe TNP 138 kV | 1 |  |
| Caney Creek - Wharton & East Bernard & Dyann 138 kV | Pledger - West Columbia 138 kV | 1 |  |
| Rule - Aspermont AEP 69 kV | Aspermont AEP - Aspermont Continental 69 kV | 1 |  |

Note: MP column refers to Mitigation plans involving load shed and were only listed which made our recurring congestion criteria.

Generic Transmission Limits (GTL’s)

| GTL | Days |
| --- | --- |
| West – North | 14 |

New RAP, PCAP (Date indicates last update of RAP, PCAP not implementation date.)

None

SPS (RAS) (New, Modified, Removed)

None

New Procedures/Forms/Operations Bulletins

None

## Load Shed Incidences

None

EEA

None

System Voltage Problems

None

Application Issues

None

Communication Issues

None

## ERCOT peak (Unofficial)

10/04 17:00 48,256 MW

Yearly MTLF Error Comparison

| Yearly Mid-Term (Hourly) Load Forecast Error Comparison for the ERCOT Forecast |  |  |  |  |
| --- | --- | --- | --- | --- |
| Month | MAPE for MTLF at 16:00 Day-Ahead |  |  |  |
|  | 2009 | 2010 | 2011 | 2012 |
| January | 4.11 | 3.78 | 2.91 | 2.94 |
| February | 3.49 | 3.79 | 3.46 | 2.51 |
| March | 3.24 | 2.24 | 2.02 | 3.49 |
| April | 2.66 | 2.47 | 3.55 | 2.78 |
| May | 2.90 | 2.63 | 3.27 | 3.09 |
| June | 2.89 | 2.84 | 2.28 | 2.85 |
| July | 2.83 | 2.39 | 1.63 | 2.85 |
| August | 3.07 | 2.29 | 2.28 | 2.96 |
| September | 3.68 | 3.14 | 2.70 | 3.43 |
| October | 2.92 | 2.37 | 3.51 | 2.99 |
| November | 1.93 | 3.46 | 3.38 | NA |
| December | 3.70 | 2.60 | 2.99 | NA |
| Annual Average | 3.11 | 2.83 | 2.83 | 2.99 |

OCN, Advisory, Watch & Emergency Notices

Capacity

| Date | Start | Event |
| --- | --- | --- |
| 10/11/2012 | 02:45 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 15:00 20:00 |
| 10/21/2012 | 13:04 | ERCOT issued an OCN for a Projected Reserve Capacity Shortage for HE 16:00 18:00 |

Transmission Watch

None

Watch

None

Emergency Notice

None

Texas RE Event Analysis

None