| NPRR Number | 505 | NPRR Title | ERS Weather-Sensitive Loads |
| --- | --- | --- | --- |
| Date |  | January 15, 2013 |  |
| Submitter’s Information |  |  |  |
| Name |  | Harry Holloway on behalf of the Reliability and Operations Subcommittee (ROS) |  |
| E-mail Address |  | harry.holloway@gdfsuezna.com |  |
| Company |  | GDF Suez Energy Marketing NA |  |
| Phone Number |  | (713) 636-1410 |  |
| Cell Number |  |  |  |
| Market Segment |  | Independent Generator |  |

Comments

At its January 10, 2013 meeting, ROS voted to endorse NPRR505 as amended by the 12/12/12 ERCOT comments and to request that ERCOT publish projected available MWs according to various weather conditions.  There were four opposing votes from the Independent Power Marketer (IPM) and Independent Generator (3) Market Segments; and four abstentions from the Independent Retail Electric Provider (IREP) and Investor Owned Utility (IOU) Market Segments.  All Market Segments were present for the vote.

Revised Proposed Protocol Language

## 2.1	DEFINITIONS

Emergency Response Service (ERS) Weather-Sensitive Load

A type of ERS Load determined by ERCOT to have Demand response capability that is influenced significantly by weather conditions.

3.14.3.5	Emergency Response Service Weather-Sensitive Loads

(1)	ERCOT may procure ERS from ERS Weather-Sensitive Loads under the provisions in this Section 3.14.3.5.  Unless specifically addressed in this Section, ERS Weather-Sensitive Loads are subject to the same market rules applicable to other ERS Resources.

(2)	In order to qualify as an ERS Weather-Sensitive Load, an ERS Load must meet one of the following criteria:

(a)	The ERS Load must consist exclusively of residential sites; or

(b)	The ERS Load must consist exclusively of non-residential sites and must qualify as weather-sensitive based on the accuracy of the regression baseline evaluation methodology as an indicator of actual interval Load.  This methodology is described in the document entitled “Emergency Response Service Default Baseline Methodologies” posted to the ERCOT website.  ERCOT shall establish minimum accuracy standards for qualification as a Weather-Sensitive Load under the regression baseline evaluation methodology.  An ERS Load must have at least nine months of interval meter data to qualify as weather-sensitive under the regression baseline evaluation methodology.  ERCOT’s determination that an ERS Load qualifies as an ERS Weather-Sensitive Load is independent of ERCOT’s determination of which baseline methodologies may be appropriate for purposes of evaluating the ERS Load’s performance.

(3)	An ERS Weather-Sensitive Load is eligible to participate during ERS Time Periods and ERS Contract Periods designated by ERCOT in the ERS Request for Proposal for ERS Weather-Sensitive Load participation.

(4)	An ERS Weather-Sensitive Load may elect to participate as an ordinary ERS Resource in any ERS Time Period, whether or not that ERS Time Period is designated for ERS Weather-Sensitive Load participation, provided that a QSE representing an aggregated ERS Weather-Sensitive Load that elects to participate in ERS Time Periods not designated for ERS Weather-Sensitive Load participation shall not be entitled to modify the number of sites in the aggregation during the Standard Contract Term as permitted by paragraph (7)(a) below.

(5)	An ERS Weather-Sensitive Load must be capable of meeting its event performance obligations in response to a Dispatch Instruction to its QSE.  ERCOT may Dispatch ERS Weather-Sensitive Loads separately or in conjunction with the Dispatch of other committed ERS Resources.

(6)	The minimum capacity offer for an ERS Weather-Sensitive Load is 0.5 MW.

(7)	A QSE may modify the population of an aggregated ERS Weather-Sensitive Load once per month during an ERS Standard Contract Term via a process defined by ERCOT.   Such adjustments shall be effective on the first day of each month following the first month.

(a)	During an ERS Standard Contract Term, a QSE may increase the number of sites in an aggregated ERS Weather-Sensitive Load that is participating only in designated weather-sensitive ERS Time Period(s) by no more than the greater of the following:

- (i)	100% of its initial size; or

- (ii)	Two MW times the QSE’s projection of the maximum number of sites in the aggregation during the ERS Standard Contract Term, divided by the MW capacity offered for the aggregation.

- (b)	Any sites added to an ERS Weather-Sensitive Load are subject to the same requirements for historical meter data as the other sites in the aggregation, as described in paragraph (1) of Section 8.1.3.1.5.1, Baseline Assignments for Emergency Response Service Weather-Sensitive Loads.

(8)	Each offer submitted by a QSE on behalf of an aggregated ERS Weather-Sensitive Load shall include the QSE’s projection of the maximum number of sites in the aggregation during the ERS Standard Contract Term.  ERCOT shall review this projection and the information provided about the initial size of each aggregated ERS Weather-Sensitive Load and shall reject any offer on behalf of such a Load if the maximum size of the Load projected by the QSE would violate the limits on site participation growth described paragraph (7) above.

(9)	An ERS Weather-Sensitive Load shall be subject to a maximum number of deployment events equal to two times the number of months of obligation in an ERS Contract Period.  The duration of the Sustained Response Period for each deployment event shall be a maximum of three hours.

(10)	ERS Weather-Sensitive Loads are eligible for the same renewal opt-in provisions that apply to other ERS Resources.

6.6.11.1	Emergency Response Service Capacity Payments

ERCOT shall pay, for each Emergency Response Service (ERS) Contract Period, the QSEs representing ERS Resources as follows:

ERSPAMT qcw(tp) = COMPAMT qcw(tp) + SPAMT qcw(tp)

ERSPAMTQSETOT q = cw(tp)ERSPAMT qcw(tp)

ERSPAMTTOT cw(tp) = qERSPAMT qcw(tp)

Where:

COMPAMT qcw(tp) = -1 * we=1coOFFERPRICE qcwe(tp) * COMPDELMW qcwe(tp)* TPH (tp)

SPAMTqcw(tp) = -1 * w(Min(SPCULqcw(tp), COMPAVGPRICE cw(tp) * SPDELMWqcew(tp) * TPH we(tp) )

COMPAVGPRICE cw(tp) = q=1ne=1co(OFFERPRICE qcwe(tp) * COMPDELMW qcwe(tp)) / COMPDELMWTOT cw(tp)

COMPDELMWTOT cw(tp) = q=1ne=1coCOMPDELMW qcwe(tp)

COMPDELMW qcwe(tp) = COMPOFFERMW qcwe(tp) * (ERSAFWT qcw * Min (ERSAFCOMB qrw,1) + (1 - ERSAFWT qcw) * Min (ERSEPF qrw,1))

SPDELMW qcwe(tp) = SPOFFERMW qcwe(tp) * (ERSAFWT qcw * Min(ERSAFCOMB qrw,1) + (1 – ERSAFWT qcw) * Min(ERSEPF qrw,1))

The ERS Self-Provision Capacity Upper Limit for each self-providing QSE shall be calculated by ERCOT using a two-pass process.  The first pass will consist of simultaneously solving for all QSEs’ ERS Self-Provision Capacity Upper Limits with the constraint that each QSE’s ERS Self-Provision Capacity Upper Limit will equal its LRS multiplied by the total ERS capacity awarded for competitive offers, plus the sum of all QSEs’ ERS Self-Provision Capacity Upper Limits.  The second pass will repeat the solution of the equations with a QSE’s delivered self-provided MW capacity (adjusted for availability and/or event performance) substituted for the ERS Self-Provision Capacity Upper Limit if the delivered MW capacity is less than the first pass calculation of the ERS Self-Provision Capacity Upper Limit.

Pass 1:

For QSE 1:

SPCUL 1cw(tp) = ERSLRS 1cw(tp) * (COMPDELMWTOT cw(tp) + SPCUL 1cw(tp) + SPCUL 2cw(tp) + … + SPCUL ncw(tp))

For QSE 2:

SPCUL 2cw(tp) = ERSLRS 2cw(tp) * (COMPDELMWTOT cw(tp) + SPCUL 1cw(tp) + SPCUL 2cw(tp) + … + SPCUL ncw(tp))

…

For QSE n:

SPCUL ncw(tp) = ERSLRS ncw(tp) * (COMPDELMWTOT cw(tp) + SPCUL 1cw(tp) + SPCUL 2cw(tp) + … + SPCUL ncw(tp))

Pass 2:

For QSE 1:

SPCUL 1cw(tp) = ERSLRS 1cw(tp) * (COMPDELMWTOT cw(tp) +

Min(SPDELMW 1cw(tp),SPCUL 1cw(tp)) +

Min(SPDELMW 2cw(tp),SPCUL 2cw(tp))

+ … + Min(SPDELMW ncw(tp),SPCUL ncw(tp)))

For QSE 2:

SPCUL 2cw(tp) = ERSLRS 2cw(tp) * (COMPDELMWTOT cw(tp) +

Min(SPDELMW 1cw(tp),SPCUL 1cw(tp)) +

Min(SPDELMW 2cw(tp),SPCUL 2cw(tp))

+ … + Min(SPDELMW ncw(tp),SPCUL ncw(tp)))

…

For QSE n:

SPCUL ncw(tp) = ERSLRS ncw(tp) * (COMPDELMWTOT cw(tp) +

Min(SPDELMW 1cw(tp),SPCUL 1cw(tp)) +

Min(SPDELMW 2cw(tp),SPCUL 2cw(tp))

+ … + Min(SPDELMW ncw(tp),SPCUL ncw(tp)))

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| q | None | A QSE. |
| c | None | ERS Contract Period. |
| r | None | ERS Standard Contract Term. |
| tp | None | Hours in an ERS Time Period. |
| e | None | An ERS Resource procured from a QSE for an ERS Contract Period. |
| co | None | The number of competitive ERS Resources procured from a QSE for an ERS Contract Period. |
| s | None | The number of self-provided ERS Resources procured from a QSE for an ERS Contract Period. |
| n | None | The number of QSEs for an ERS Contract Period. |
| w | None | ERS type (weather-sensitive / non-weather-sensitive) |
| ERSPAMT qcw(tp) | $ | ERS Payment Amount per QSE per ERS Contract Period per ERS Time Period—ERS total payment to QSE q for ERS Contract Period c, ERS type w and ERS Time Period tp. |
| COMPAMT qcw(tp) | $ | Competitive Amount per QSE per ERS Contract Period per ERS Time Period—ERS total payment to QSE q for all competitively procured ERS Resources delivered for ERS Contract Period c, ERS type w and ERS Time Period tp. |
| SPAMT qcw(tp) | $ | Self-Procured Amount per QSE per ERS Contract Period per ERS Time Period—ERS total payment to the QSE for its self-provided ERS Resources for ERS Contract Period c, ERS type w and ERS Time Period tp. |
| ERSPAMTQSETOT q | $ | ERS Payment QSE Total per QSE per ERS Contract Period—The total ERS total payments to QSE q. |
| ERSPAMTTOT cw(tp) | $ | ERS Payment Amount Total per ERS Contract Period per ERS Time Period—Total of all ERS payments for ERS Contract Period c, ERS type w and ERS Time Period tp. |
| OFFERPRICE qcwe(tp) | $/MW per hour | Offer Price per QSE per ERS Contract Period per ERS Resource per ERS Time Period—Contracted offer price for ERS Contract Period c, ERS type w, competitively procured ERS Resource e, and ERS Time Period tp. |
| COMPDELMW qcwe(tp) | MW | Competitive Delivered MW per QSE per ERS Contract Period per ERS Resource per ERS Time Period—ERS capacity delivered by the QSE q for ERS Contract Period c, ERS type w, competitive ERS Resource e and ERS Time Period tp. |
| TPH cwe(tp) | Hours | Hours in ERS Time Period tp for ERS type w and ERS Resource e. For ERS Resources e whose obligation is not exhausted in an ERS Contract Period c, ERS type w, the number of hours in that ERS Time Period tp in that ERS Contract Period c. For ERS type w and ERS Resources e whose obligation is exhausted in an ERS Contract Period c, the number of hours in that ERS Time Period tp from the beginning of the ERS Contract Period c to the end of the ERS Standard Contract Term. |
| COMPAVGPRICE cw(tp) | $ per MW per hour | Competitive Average Price per ERS Contract Period per ERS Time Period—Weighted average price of competitive capacity delivered for ERS Contract Period c, ERS type w and ERS Time Period tp. |
| SPDELMW qcw(tp) | MW | Self-Provided Delivered MW per QSE per ERS Contract Period per ERS Time Period—Total ERS capacity self-provided by QSE q for ERS Contract Period c, ERS type w and ERS Time Period tp. |
| COMPDELMWTOT cw(tp) | MW | Competitive Delivered MW Total per ERS Contract Period per ERS Time Period—Total ERS competitive capacity delivered by all QSEs for ERS Contract Period c, ERS type w, and ERS Time Period tp. |
| COMPOFFERMW qcwe(tp) | MW | Competitive Offered MW Total per QSE per ERS Contract Period per ERS Resource per ERS Time Period—ERS capacity offered by QSE q for ERS Contract Period c, ERS type w, competitive ERS Resource e for ERS Contract Period c and ERS Time Period tp. |
| ERSAFWT qcw | None | Availability Settlement weighting factor per QSE per ERS Contract Period—The weighting factor for QSE q for ERS Contract Period c, ERS type w to apply for Settlement as calculated pursuant to Section 8.1.3.1.3.3. |
| ERSAFCOMB qrw | None | Time- and Capacity-Weighted ERS Availability Factor per QSE per ERS Standard Contract Term—The availability factor for QSE q for ERS Standard Contract Term r, and ERS type w as calculated pursuant to Section 8.1.3.3.1, Performance Criteria for Qualified Scheduling Entities Representing Emergency Response Service Resources. |
| ERSEPF qrw | None | ERS Event Performance Factor per QSE per ERS Standard Contract Term—Event performance factor for QSE q in ERS Standard Contract Term r and ERS type w as calculated pursuant to Section 8.1.3.3.1. |
| SPCUL qcw(tp) | MW | Self-Provision Capacity Upper Limit per QSE per ERS Contract Period per ERS Time Period—The ERS Self-Provision Capacity Upper Limit calculated by ERCOT for a self-providing QSE for ERS Contract Period c, ERS type w and ERS Time Period tp by simultaneously solving for all QSEs’ obligations with the constraint that each QSE’s ERS Self-Provision Capacity Upper Limit does not exceed its obligation. |
| SPOFFERMW qcwe(tp) | MW | Self-Provision Offer MW per QSE per ERS Contract Period per ERS Resource per ERS Time Period—ERS capacity offered as self-provision by QSE q for ERS Contract Period c, ERS type w, ERS Resource e and ERS Time Period tp. |
| ERSLRS qcw(tp) | None | ERS Load Ratio Share per QSE per ERS Contract Period per ERS Time Period—ERS LRS for QSE q for ERS Contract Period c, ERS type w and ERS Time Period tp, calculated starting with the first hour of the ERS Contract Period and ending with the earlier of the last hour of the ERS Contract Period or the hour containing the recall instruction in an ERS deployment event that results in the exhaustion of a QSE portfolio’s ERS obligation. |

6.6.11.2	Emergency Response Service Capacity Charge

(1)	ERCOT shall allocate costs for an ERS Contract Period based on the LRS of each QSE during each ERS Time Period in an ERS Contract Period.  A QSE’s LRS for an ERS Time Period shall be the QSE’s total Load for the ERS Time Period divided by the total ERCOT Load in the ERS Time Period.  For the first Settlement of the ERS Contract Period as described in paragraph (1) of Section 9.14.5, Settlement of Emergency Response Service, LRS will be calculated using the latest Settlement Load for each Operating Day in the ERS Contract Period.  For the resettlement of the ERS Contract Period as described in paragraph (2) of Section 9.14.5, the LRS will be calculated using the true-up Load for each Operating Day in the ERS Contract Period.

(2)	ERCOT shall calculate each QSE’s ERS capacity charge as follows:

LAERSAMT  qcw(tp) = ERSLRS qcw(tp) * ERSPAMTTOT cw(tp)

LAERSAMTQSETOTq= cw(tp)w(tp)LAERSAMT qcw(tp)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| q | None | A QSE. |
| c | None | ERS Contract Period. |
| tp | None | An ERS Time Period. |
| w | None | ERS type (weather-sensitive / non-weather-sensitive) |
| ERSPAMTTOT cw(tp) | $ | ERS Payment Amount Total per ERS Contract Period per ERS Time Period—Total of all ERS payments for ERS Contract Period c, ERS type w and ERS Time Period tp. |
| ERSLRS qcw(tp) | None | ERS Load Ratio Share per QSE per ERS Contract Period per ERS Time Period—ERS LRS for QSE q for ERS Contract Period c, ERS type w and ERS Time Period tp, calculated starting with the first hour of the ERS Contract Period and ending with the earlier of the last hour of the ERS Contract Period or the hour containing the recall instruction in an ERS deployment event that results in the exhaustion of a QSE portfolio’s ERS obligation. |
| LAERSAMT qcw(tp) | $ | Load-Allocated ERS Amount per QSE per ERS Contract Period per ERS Time Period—ERS charge for QSE q for ERS Contract Period c, ERS type w and ERS Time Period tp. |
| LAERSAMTQSETOT q | $ | Load-Allocated ERS Amount QSE Total per QSE per ERS Contract Period—The total ERS charge for QSE q. |

8.1.3.1.5	Performance Criteria for Emergency Response Service Weather-Sensitive Loads

- ERS Weather-Sensitive Loads are subject to event performance, test performance, and availability requirements as described in Sections 8.1.3.1.5.1, Baseline Assignments for Emergency Response Service Weather-Sensitive Loads, through 8.1.3.1.5.5, Emergency Response Service Weather-Sensitive Load Payments.  Unless specifically addressed in these Sections, ERS Weather-Sensitive Loads are subject to the same performance criteria that apply to other ERS Resources.

8.1.3.1.5.1	Baseline Assignments for Emergency Response Service Weather-Sensitive Loads

- (1)	ERCOT shall assign a residential ERS Weather-Sensitive Load to either the regression baseline evaluation methodology or the control group baseline performance evaluation methodology.  Both methodologies are described in the document entitled “Emergency Response Service Default Baseline Methodologies” posted to the ERCOT website.  The control group baseline performance evaluation methodology shall only be available to ERS Weather-Sensitive Loads consisting entirely of residential sites.

- (a)	At least nine months of interval data for all sites within an ERS Weather-Sensitive Load are required for the Load to be eligible for the regression baseline evaluation methodology.  If one or more sites lack sufficient interval data, the ERS Weather-Sensitive Load will either be evaluated using the control group baseline performance evaluation methodology or will be disqualified from participation as an ERS Weather-Sensitive Load.

(b)	Sites in an ERS Weather-Sensitive Load assigned to the control group baseline are required to have fully functional interval metering in place at the start of an ERS Standard Contract Term, but are not required to have historical meter data prior to that time.

(c)	If ERCOT determines that the residential ERS Weather-Sensitive Load may be assigned to either baseline methodology, the QSE may select its preferred option.

- (2)	If the ERS Weather-Sensitive Load consists of non-residential sites, the ERS Load must qualify for and the QSE must select at least one ERS default baseline methodology, as described in paragraph (2) of Section 8.1.3.1.1, Baseline Assignments for Emergency Response Service Loads.

(3)	For an ERS Weather-Sensitive Load assigned to an ERS default baseline, ERCOT will assign each site in the ERS Weather-Sensitive Load to one of two numbered groups for purposes of testing and deployment event Dispatch.  Upon the request of a QSE, or upon a determination by ERCOT that assignment to two groups would result in test results that would inaccurately represent performance for the entire Load, ERCOT shall assign all sites within an ERS Weather-Sensitive Load on an ERS default baseline to a single group.  Group designations are subject to change if the QSE adjusts the population of the ERS Weather-Sensitive Load during the ERS Standard Contract Term, as described in paragraph (9) of Section 3.14.3.5, Emergency Response Service Weather-Sensitive Loads.

(4)	For an ERS Weather-Sensitive Load assigned to the control group baseline, ERCOT will divide the aggregation into multiple randomly assigned numbered groups for purposes of testing and deployment event Dispatch, and one or more of these groups will be designated as the control group(s) at time of Dispatch.  ERCOT will strive to minimize control group size while preserving the ability to achieve accurate Demand response measurement and verification.  The number of groups, group size and group designations are subject to change if the QSE adjusts the population of the ERS Weather-Sensitive Load during the ERS Standard Contract Term, as described in paragraph (9) of Section 3.14.3.5.

8.1.3.1.5.2	Event Performance Criteria for Emergency Response Service Weather-Sensitive Loads

(1)	For an ERS deployment event, ERCOT shall calculate EIPFs and an ERSEPF for an ERS Weather-Sensitive Load consistent with the provisions of paragraph (2)(b)(i) of Section 8.1.3.1.4, Event Performance Criteria for Emergency Response Service Resources.  No other provisions in paragraph (2) of Section 8.1.3.1.4 shall apply to ERS Weather-Sensitive Loads.

- (2)	Regardless of the number of enrolled sites in the ERS Weather-Sensitive Load at the time of an event or test, the contracted capacity value (OFFERMW) used will be the value submitted by the QSE in its offer.

- (3)	For a deployment event or test with two or more full intervals in the Sustained Response Period, if an ERS Weather-Sensitive Load achieves the EIPF for the first full interval of the Sustained Response Period that is less than 75% of the average EIPF for the remaining full intervals of the Sustained Response Period, the baseline used to evaluate the ERS Weather-Sensitive Load shall be reduced to the level at which the ERSEPF for that event or test is equal to the 0.75 times the ERSEPF determined by using the initial baseline.

8.1.3.1.5.3	Testing of Emergency Response Service Weather-Sensitive Loads

- (1)	ERCOT shall conduct unannounced testing of each ERS Weather-Sensitive Load at least once but no more than twice per month during an ERS Standard Contract Term, unless testing has been superseded by deployment events as described in paragraph (4) below.

- (a)	For no more than two of the tests of an ERS Weather-Sensitive Load in an ERS Standard Contract Term, the Sustained Response Period of the test will have a duration of at least one hour.

- (b)	The remaining tests will be conducted according to normal ERS testing procedures.

- (c)	At the time of Dispatch during a test, ERCOT will not advise the QSE of the test duration.

- (d)	ERCOT may conduct a test during any of an ERS Weather-Sensitive Load’s obligated hours.  However, tests will generally be targeted toward periods of peak weather conditions.

- (e)	For an ERS Weather-Sensitive Load assigned to an ERS default baseline, tests will be conducted by group.  The Dispatch Instruction shall identify the group being tested.

- (e)	For an ERS Weather-Sensitive Load assigned to the control group baseline, tests will target one or more of the designated groups.  The Dispatch Instruction shall identify the group(s) being tested.

- (i)	Non-tested groups will serve as the control group.

- (ii)	Selection of groups to be tested will be random and will cycle through the groups within the ERS Weather-Sensitive Load.

- (2)	ERCOT shall calculate a test performance factor for each test of an ERS Weather-Sensitive Load, using the event performance methodology described in Section 8.1.3.1.5.2, Event Performance Criteria for Emergency Response Service Weather-Sensitive Loads, except that the MW obligations shall be prorated based on the number of sites dispatched during the test.

- (3)	The QSE is responsible for managing group assignments and for deploying only the group(s) dispatched by ERCOT during a test.

- (4)	ERCOT may reduce the number of tests administered by the number of deployment events during the ERS Standard Contract Term.  However, a test shall not reduce an ERS Weather-Sensitive Load’s deployment event obligation as described in paragraph (8) of Section 3.14.3.5, Emergency Response Service Weather-Sensitive Loads.

8.1.3.1.5.4	Availability Criteria for Emergency Response Service Weather-Sensitive Loads

- The availability factor methodologies described in Section 8.1.3.1.3, Availability Criteria for Emergency Response Service Resources, shall not apply to ERS Weather-Sensitive Loads.  An ERS Weather-Sensitive Load’s availability factor for an ERS Contract Period shall always be set to 1.0.

8.1.3.1.5.5	Emergency Response Service WeatherSensitive Load Payments

- (1)	An ERS Weather-Sensitive Load’s ERS Contract Period ERSEPF shall be the basis for the QSE’s payment for that ERS Weather-Sensitive Load.  ERCOT shall compute an ERS Weather-Sensitive Load’s ERS Contract Period ERSEPF by calculating the time-weighted average of all test performance factors and all event performance factors, each capped at 1.0.

- (2)	Notwithstanding the aforementioned, ERCOT shall reduce a QSE’s payment for an ERS Weather-Sensitive Load as follows:

- (a)	If the maximum number of sites in the ERS Weather-Sensitive Load during the ERS Standard Contract Term is less than 90% of the maximum number of sites projected by the QSE at the time of offer submission, as described in paragraph (8) Section 3.14.3.5, Emergency Response Service Weather-Sensitive Loads, the baseline used to evaluate the ERS Weather-Sensitive Load shall be reduced to the level at which the ERS Contract Period ERSEPF is equal to the square of the ERSEPF determined by using the initial baseline.

- (b)	If the average Demand reduction value per site within the ERS Weather-Sensitive Load for all tests and deployment events during an ERS Contract Period is less than 90% of the average Demand reduction value per site as calculated based on the QSE’s offer, the baseline used to evaluate ERS Weather-Sensitive Load shall be reduced to the level at which the ERS Contract Period ERSEPF is equal to the square of the ERSEPF determined by using the initial baseline.  ERCOT shall adjust the results of each test and deployment event to normalized peak weather conditions before making this calculation.

- (c)	If ERCOT determines that the total Demand response value provided by the ERS Weather-Sensitive Load, as adjusted for normalized peak weather conditions, is equal to or greater than 90% of its contracted capacity value (OFFERMW), the squaring provision described in paragraphs (2)(a) or (2)(b) above shall be waived.

- (d)	If the provisions of both paragraph (2)(a) and (2)(b) above require the ERSEPF to be squared and paragraph (2)(c) does not apply, the baseline used to evaluate the ERS Weather-Sensitive Load shall be reduced to the level at which the ERS Contract Period ERSEPF for the ERS Weather-Sensitive Load ERSEPF is equal to the cube of the ERSEPF determined by using the initial baseline.

- (e)	If an ERS Weather-Sensitive Load’s obligation is exhausted during an ERS Contract Period, the provisions of paragraphs (2)(a), (2)(b), and (2)(d) above shall not be applied.

- (3)	For purposes of calculating the QSE’s payment for an ERS Weather-Sensitive Load for an ERS Contract Period, ERCOT shall calculate the following:

- (a)	ERCOT shall compute the QSE’s portfolio-level ERS Weather-Sensitive Load Demand reduction value by summing the product of each of the QSE’s Weather Sensitive-Load’s ERS Contract Period event performance factors and its contracted capacity value (OFFERMW).

- (b)	ERCOT shall then compute the QSE’s portfolio-level ERS Weather-Sensitive Load event performance factor (ERSEPFqrw) as the average of the ERS Contract Period ERSEPF, weighted according to Demand reduction value as calculated in paragraph (3)(a) above.

- 8.1.3.3.1	Performance Criteria for Qualified Scheduling Entities Representing Emergency Response Service Resources

(1)	A QSE’s ERS performance will be evaluated based on its portfolio’s performance during ERS deployment events and on the overall availability of its portfolio in an ERS Standard Contract Term.  ERCOT shall evaluate the performance and availability of a QSE’s portfolio of ERS Weather-Sensitive Loads separate from the QSE’s portfolio of all other ERS Resources, as follows:

(a)	The following availability calculations and criteria apply only to each QSE’s portfolio of ERS Resources that are not ERS Weather-Sensitive Loads:

(i)	ERCOT shall calculate a portfolio-level availability factor for each portfolio for each ERS Time Period in an ERS Contract Period using the methodologies defined in Section 8.1.3.1.3, Availability Criteria for Emergency Response Service Resources, except that the availability factor for each ERS Time Period will be allowed to exceed 1.0.  ERCOT shall then calculate a single time- and capacity-weighted availability factor for the portfolio for the ERS Contract Period using the methodologies defined in Section 8.1.3.1.3.  ERCOT shall then calculate a single time- and capacity-weighted availability factor for the portfolio for the ERS Standard Contract Term, capped at 1.0.

(ii)	For an ERS Standard Contract Term with a single ERS Contract Period, the QSE’s portfolio-level availability factor for the ERS Standard Contract Term shall be the portfolio-level availability factor for the ERS Contract Period.  For an ERS Standard Contract Term with multiple ERS Contract Periods, ERCOT shall compute a portfolio-level availability factor for the ERS Standard Contract Term by averaging the QSE’s availability factors across ERS Contract Periods and ERS Time Periods, weighted according to time and capacity obligations.

(iii)	The QSE’s portfolio-level availability factor for the ERS Standard Contract Term will determine both the availability component of the ERS payment to the QSE and whether the QSE has met its ERS availability requirements.  If the QSE’s portfolio-level availability factor for the ERS Standard Contract Term equals or exceeds 0.95, the QSE shall be deemed to have met its availability requirements for the ERS Standard Contract Term; otherwise, the QSE shall be deemed to have failed to meet this requirement.  If the QSE’s portfolio-level availability factor for the ERS Standard Contract Term is less than 1.0, the QSE’s ERS capacity payment shall be reduced according to the formulas in Section 6.6.11.1, Emergency Response Service Capacity Payments.

(b)	The availability factor for each QSE’s portfolio of ERS Weather-Sensitive Loads for an ERS Standard Contract Term shall be set to 1.0.

(c)	The following event performance calculations and criteria apply only to each QSE’s portfolio of ERS Resources that are not ERS Weather-Sensitive Loads:

(i)	QSEs representing ERS Resources must meet performance standards specified in Section 8.1.3.1.4, Event Performance Criteria for Emergency Response Service Resources, as applied on a portfolio-level basis.  ERCOT’s calculation of portfolio performance shall weight each ERS Resource according to its committed share of the QSE’s portfolio capacity measured in MW.  ERCOT shall determine a QSE’s portfolio-level event performance by calculating an

ERSEPFqrw.  For purposes of evaluating ERS Loads in the QSE’s portfolio, ERCOT shall establish a baseline representing the portfolio’s estimated Load in the absence of the ERS deployment event.  For purposes of evaluating ERS Generators, ERCOT shall compute portfolio-level injection of energy to the ERCOT System.  Using this data, ERCOT shall calculate an ERSEPFqrw for each ERS deployment event based on the methodologies defined in Section 8.1.3.1.4.  ERCOT shall then calculate an ERSEPFqrw for the ERS Standard Contract Term.  For an ERS Standard Contract Term with no ERS deployment events, this ERSEPFqrw for the ERS Standard Contract Term shall be set to 1.0.

(ii)	For an ERS Standard Contract Term with a single ERS deployment event in which ERS Resources other than ERS Weather-Sensitive Loads were deployed, the ERSEPFqrw for the ERS Standard Contract Term shall be the ERSEPFqrw for the event.  For an ERS Standard Contract Term with multiple ERS deployment events in which ERS Resources other than ERS Weather-Sensitive Loads were deployed, ERCOT shall compute the ERSEPFqrw for the ERS Standard Contract Term by averaging the ERSEPFqr for all of the deployment events, weighted according to the duration of the events and capacity obligations by interval.

(iii)	The ERSEPFqrw for an ERS Standard Contract Term will determine both the event performance component of the ERS payment to the QSE and whether the QSE has met its ERS event performance requirements.  If an ERSEPFqrw for an ERS Standard Contract Term is greater than or equal to 0.95, the QSE will be deemed to have met its event performance requirements for the ERS Standard Contract Term; otherwise, the QSE shall be deemed to have failed to meet this requirement.  If an ERSEPFqrw less than 1.0 for the Standard Contract Term, the QSE’s ERS capacity payment shall be reduced according to the formulas in Section 6.6.11.1, Emergency Response Service Capacity Payments.  For purposes of calculating an ERSEPFqrw, any ERS Resources that were not subject to Dispatch during the event shall be treated as having met their obligation.

(d)	The ERSEPFqrw of ERS Weather-Sensitive Loads shall be determined as required by paragraph (3)(b) of Section 8.1.3.1.5.5, Emergency Response Service Weather-Sensitive Load Payments.  A QSE is subject to a reduction in its payment as provided in paragraph (2) of Section 8.1.3.1.5.5.

(e)	Timely Deployment for a QSE’s portfolio of ERS Resources that are not ERS Weather-Sensitive Loads:  A QSE shall ensure that each of its ERS Resources deploys within the prescribed ramp period when required by an applicable VDI.  For each ERS deployment event, ERCOT shall assess each QSE’s compliance with this requirement by calculating a capacity-weighted EIPF for the first full interval of the Sustained Response Period, using the methodologies defined in Section 8.1.3.1.4.  If that EIPF is equal to or greater than 0.95 the QSE shall be deemed to have met the ten-minute deployment requirement for its portfolio of Resources that are not weather-sensitive; otherwise, the QSE shall be deemed to have failed to meet this requirement.

(f)	Deployment requirements for a QSE’s portfolio of ERS Weather-Sensitive Loads:  A QSE must meet the deployment requirements described in paragraph (3) of Section 8.1.3.1.5.2, Event Performance Criteria for Emergency Response Service Weather-Sensitive Loads.

(2)	A QSE violates these Protocols if its portfolio of ERS Resources other than ERS Weather-Sensitive Loads fails any of the performance criteria in items (1)(a), (1)(c) or (1)(e) above.  For any portfolio of ERS Weather-Sensitive Loads, a QSE violates these Protocols if more than 25% of its average awarded MW capacity, weighted by ERS Time Period, is subject to the payment reduction provided by paragraph (2)(d) of Section 8.1.3.1.5.5 for any two ERS Standard Contract Terms in any two-year period.  Such violations may be subject to administrative penalties by the PUCT and would potentially be in addition to suspension by ERCOT of the QSE pursuant to Section 8.1.3.3, Suspension of Qualification of Emergency Response Service Resources and/or their Qualified Scheduling Entities.

- (3)	Failure by a QSE portfolio to meet its ERS event performance or availability requirements shall not be cause for revocation of the QSE’s Ancillary Services qualification.