| NPRR Number | 520 | NPRR Title | Real-Time Mitigation Rules and Creation of a Real-Time Constraint Competitiveness Test |  |
| --- | --- | --- | --- | --- |
| Timeline | Urgent | Action |  | Recommended Approval |
| Date of Decision |  | February 21, 2013 |  |  |
| Proposed Effective Date |  | Upon system implementation. |  |  |
| Priority and Rank Assigned |  | Priority – 2013; Rank – 205 |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 3.19, Constraint Competitiveness Tests 3.19.1, Constraint Competitiveness Test Definitions 3.19.2, Element Competitiveness Index Calculation 3.19.3, Long-Term Constraint Competitiveness Test 3.19.4, Monthly Constraint Competitiveness Test 3.19.5, Daily Constraint Competitiveness Test 6.5.7.3, Security Constrained Economic Dispatch 6.5.7.4, Base Points |  |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | The primary changes in this Nodal Protocol Revision Request (NPPR) are the following: - The Monthly and Daily Constraint Competitiveness Tests (CCTs) will be replaced with a CCT that is performed as part of the Security-Constrained Economic Dispatch (SCED) process considering only those constraints that are active in SCED and using the current system conditions - The Element Competitiveness Index (ECI) calculation for the Long-Term and SCED CCTs will only be performed on the import side of a constraint - The designation of a constraint as competitive or non-competitive in the SCED CCT will not be dependent on the designation during the Long-Term CCT - The Independent Market Monitor (IMM) is given the ability to designate constraints as either competitive or non-competitive, regardless of CCT results - Only those Entities with decision making authority that have a significant impact in making a constraint non-competitive will be considered for mitigation in SCED - Only those Resources that have a significant contribution in getting the Entity with decision making authority considered for mitigation for a Non-Competitive Constraint will have mitigation applied in SCED - The formula for mitigating energy offers in SCED is changed to include a small adder to the reference price from SCED Step 1 in order to minimize issues caused by the tie breaking logic - The use of Mitigated Offer Floors in the SCED process is removed. |  |  |
| Reason for Revision |  | To address the following issues: - Mitigated prices or offers being applied in situations that are competitive or in which an Entity with decision making authority does not have a significant competitive advantage. - Base Point oscillations that occur with changes in the reference Locational Marginal Price (LMP) and Resources being unnecessarily mitigated. - Multiple Quick Start Generation Resources (QSGRs) being brought On-Line when not all needed due to having the same mitigated offer applied. These were some of the issues raised in the January 15, 2013 IMM report to the ERCOT Board to “implement the changes necessary to address the SCED ‘over-mitigation’ issues.” |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - On 2/6/13, NPRR520 and an Impact Analysis were posted. - On 2/12/13, NRG Texas comments were posted. - On 2/21/13, PRS considered NPRR520. |  |  |
| PRS Decision |  | On 2/21/13, PRS unanimously voted to grant NPRR520 Urgent status. PRS then unanimously voted to recommend approval of NPRR520 as submitted with a recommended priority of 2013 and rank of 205. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 2/21/13, ERCOT Staff stated that important components of NPRR520 could be implemented by Summer 2013; that additional implementation details and operational examples would be presented at the 2/26/13 Congestion Management Working Group (CMWG) meeting; and that the appropriate thresholds for determining the competitive designation of constraints will be developed and approved by TAC. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | This NPRR addresses the over mitigation issues by minimizing the set of Resources that are mitigated to only those Resources that have significant impacts to a Non-Competitive Constraint. This ensures that Resources that do not impact a Non-Competitive Constraint, when Dispatched up in SCED Step 2 to meet the power balance constraint, will set the price based on their competitive offers. |
|  | 2 | This NPRR minimizes Base Point oscillations for Resources that become marginal due to offer price floors for Non-Spinning Reserve (Non-Spin) Service, Reliability Unit Commitment (RUC) and Reliability Must-Run (RMR) Service, high QSGR offers priced above the Reference LMP, or high offers submitted based on a voluntary Mitigation Plan above the Reference LMP. |

| Sponsor |  |
| --- | --- |
| Name | Resmi Surendran |
| E-mail Address | rsurendran@ercot.com |
| Company | ERCOT |
| Phone Number | (512) 248-3033 |
| Cell Number | (512) 289-7131 |
| Market Segment | Not applicable. |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Yvette M. Landin |
| E-Mail Address | ylandin@ercot.com |
| Phone Number | (512) 248-4513 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| NRG Texas 021213 | Recommended that PRS grant NPRR520 Urgent status and requested a prioritization level that will ensure implementation prior to Summer 2013. |

Comments

Please note that the following NPRRs also propose revisions to Section 6.5.7.3:

- NPRR444, Supplemental Reliability Deployments

- NPRR486, Calculation of Generation to be Dispatched

- NPRR508, Setting of Real-Time LMPs During EEA ERS/Load Resource Deployment

Please also note that the baseline Protocol language in the following sections has been updated as follows:

- NPRR469, Modifications to CCTs (unboxing in the February 14, 2013 Protocols)

  - 3.19

  - 3.19.1

  - 3.19.2

  - 3.19.3

  - 3.19.4

  - 3.19.5

  - 6.5.7.3

- NPRR472, Implementation Clarifications for Consideration of DC Tie Lines and Outages in CCTs (unboxing in the February 14, 2013 Protocols)

  - 3.19.1

  - 3.19.3

Proposed Protocol Language Revision

- 2.1	Definitions

- Competitive Constraint

A contingency/limiting Transmission Element pair that is determined to be competitive by the Techical Advisory Committee (TAC).

[NPRR520: Replace the above definition “Competitive Constraint” with the following upon system implementation:] A contingency/limiting Transmission Element pair that is determined to be competitive using the process defined in Section 3.19, Constraint Competitiveness Tests.

- Non-Competitive Constraint

A Transmission Element that is not a Competitive Constraint.

[NPRR520: Replace the above definition “Non-Competitive Constraint” with the following upon system implementation:] A contingency/limiting Transmission Element pair that is not determined to be a Competitive Constraint.

- 3.19	Constraint Competitiveness Tests

(1)	The Technical Advisory Committee (TAC) shall consider the results of the Constraint Competitiveness Tests (CCTs) and other relevant factors in reaching its determination as to whether or not a Transmission Element pair should be determined to be a Competitive Constraint.  Any contingency/limiting Transmission Element pair not designated as a Competitive Constraint shall be deemed to be a Non-Competitive Constraint.

(2)	The appropriate TAC subcommittee may develop an alternative list through the analysis described below for determining Competitive Constraints.

(3)	The TAC shall perform the following analysis with the goal of developing an objective standard for determining Competitive Constraints:

(a)	Contingency analysis – based on reasonable generation dispatch that would lead into a set of elements to be studied; and

(b)	The Long-Term, Monthly, and Daily CCTs.

(4)	At a minimum, the CCT should be performed at least once per month and the results compared to the existing TAC-approved Competitive Constraints list.  Based on the comparison, the TAC may evaluate alternative methodologies or alternative Competitive Constraints and report the results of these evaluations to the TAC.

(5)	The Independent Market Monitor (IMM) may suspend a Competitive Constraint from being designated as competitive for a specified period of time necessary to allow for analysis, but not to exceed 60 days and shall notify the market of the estimated time needed to conduct the analysis.  The IMM shall also provide notification to the market prior to the suspension of any Competitive Constraint.

(6)	TAC shall approve the Competitive Constraints one month prior to the long-term Congestion Revenue Right (CRR) Auction sequence.  Prior to each monthly CRR Auction, TAC shall approve updates to the Competitive Constraints that are applicable for the following monthly CRR Auction.

(7)	ERCOT shall post the Competitive Constraints to the Market Information System (MIS) Secure Area at least five Business Days before any change takes effect. ERCOT shall post any Competitive Constraints that have been suspended and the duration of the suspension as soon as practicable to the MIS Secure Area.

[NPRR520: Delete paragraphs (1)-(7) above upon system implementation].

3.19.1	Constraint Competitiveness Test Definitions

(1)	The CCT checks the competitiveness of the constraint by evaluating Market Participant’s ability to exercise market power by physical withholding, economic withholding, predatory pricing, etc.  The CCT for a constrained Transmission Element evaluates whether the constraint can be overloaded given the system condition, if there is sufficient competition to resolve the constraint on the import and export sides by calculating the Element Competitiveness Index (ECI) on the import and export sides of the constraint, and determines the existence of a single Entity needed to resolve the constraint.

(2)	The competitiveness of a constraint is tested on a long-term, monthly, and daily basis.  To conduct the test, various definitions are needed, including:

(a)	“Available Capacity for a Resource” is defined as:

(i)	The Seasonal Net Max Sustainable Rating of a Generation Resource, as registered with ERCOT, including a Switchable Generation Resource (except a Wind-powered Generation Resource (WGR)) for the Long-Term and Monthly CCTs or Current Operating Plan (COP) High Sustained Limit (HSL) for the Daily CCT.

(A)	For WGRs:

(1)	Long-Term CCT - on the export side, the Seasonal Net Max Sustainable Rating, as registered as specified in its ERCOT- approved Resource Asset Registration Form, and on the import side, zero MW.

(2)	Monthly and Daily CCT - the expected on-peak wind generation output on the export side and zero MW on the import side.

(ii)	The full import capability of the Direct Current Tie (DC Tie) lines on the export side and zero MW on the import side.

(3)	“Managed Capacity for an Entity” is a Resource or Split Generation Resource for which the Entity or its Affiliates has the decision-making authority over how the Resource or Split Generation Resource is offered or scheduled (e.g., Output Schedules), in accordance with subsection (e) of P.U.C. Subst. R. 25.502, Pricing Safeguards in Markets Operated by the Electric Reliability Council of Texas.  Each Resource Entity that owns a Resource shall submit a declaration to ERCOT, using a form designated by ERCOT, as to which Entity has the decision-making authority for each of its Resources.  The declaration shall be signed by the Authorized Representative of the Resource Entity.  In addition, each Resource Entity that owns a Resource shall Notify ERCOT of any known changes in that declaration no later than 14 days prior to the date that the change takes effect or as soon as possible in a situation where the Resource Entity is unable to meet the 14-day Notice requirement.  Upon ERCOT’s request, each Resource Entity that owns a Resource shall provide ERCOT with sufficient information or documentation to verify control of the Resource.  ERCOT shall apply decision-making authority to Managed Capacity for an Entity effective the first Operating Hour of the Operating Day ERCOT satisfactorily confirms the Resource Entity’s most recent declaration, but not sooner than the effective date specified on the Resource Entity’s most recent declaration.

(4)	Shift Factors of all Electrical Buses are computed relative to the distributed load reference Bus.

(a)	For voltage, stability, and thermal-limited constraints, as well as interfaces represented by thermal limits, the Shift Factors should be computed with no other contingencies removed from the electrical network.

(b)	For contingency-limited constraints, the Shift Factors used should be computed with the contingencies removed from the electrical network.

3.19.2	Element Competitiveness Index Calculation

(1)	The ECI is one of several criteria used in the Long-Term, Monthly, and Daily CCTs to determine the competitiveness of a constraint.

(2)	To compute the ECI on the import and export side, first determine the “ECI Effective Capacity” available to resolve the constraint on the import and export sides, as follows:

(a)	Determine the ECI Effective Capacity that each Entity contributes to resolve the constraint on the import side by taking, for each Managed Capacity for an Entity having negative Shift Factors with absolute values greater than the minimum of one-third of the highest absolute value of any Resource Shift Factor with a negative value and 2%, the sum of the products of (A) the Available Capacity for a Resource and (B) the square of the Shift Factor of that Resource to the constraint.

(b)	Determine the ECI Effective Capacity that each Entity contributes to resolve the constraint on the export side by taking, for each Managed Capacity for an Entity having positive Shift Factors greater than the minimum of one-third of the highest positive Resource Shift Factor and 2%, the sum of the products of (A) the Available Capacity for a Resource and (B) the square of the Shift Factor of that Resource to the constraint.

(3)	Determine the ECI on the import and export side of the constraint, as follows:

(a)	Determine the total ECI Effective Capacity by each Entity and its Affiliates on the import and export side.

(b)	Determine the percentage of ECI Effective Capacity by each Entity and its Affiliates on the import and export side by taking each Entity and its Affiliates’ ECI Effective Capacity and dividing by the total ECI Effective Capacity on the import and export side.

(c)	The ECI on the import side is equal to the sum of the square of the percentages of ECI Effective Capacity by each Entity and its Affiliates on the import side.

(d)	The ECI on the export side is equal to the sum of the square of the percentages of ECI Effective Capacity by each Entity and its Affiliates on the export side.

3.19.3	Long-Term Constraint Competitiveness Test

(1)	The Long-Term CCT uses 12 monthly peak Load cases for all calculations.

(2)	A constraint is classified as competitive for the year if it is competitive in any of the ERCOT-selected cases for the year.  A constraint is competitive for a monthly case if the constraint can be overloaded in the monthly case and it doesn’t meet any of the following conditions:

(a)	The ECI is greater than 2,000 on the import side or the ECI is greater than 2,500 on the export side of the constraint; or

(b)	The constraint cannot be resolved by eliminating all Available Capacity for a Resource on the import side, except nuclear capacity and minimum-energy amounts of coal and lignite capacity, that is Managed Capacity for an Entity or its Affiliates during peak Load conditions; or

(c)	There are no positive Shift Factors corresponding to Electrical Buses with Available Capacity for a Resource that have a value greater than or equal to 2%, and there are no negative Shift Factors corresponding to Electrical Buses with Available Capacity for a Resource that have an absolute value greater than or equal to 2%.

[NPRR520: Replace Sections 3.19.1, 3.19.2, and 3.19.3 with the following upon system implementation:] 3.19.1 Constraint Competitiveness Test Definitions (1) The Constraint Competitiveness Test (CCT) checks the competitiveness of a constraint by evaluating each Market Participant’s ability to exercise market power by physical or economic withholding. The CCT for a constrained Transmission Element evaluates whether there is sufficient competition to resolve the constraint on the import side by calculating the Element Competitiveness Index (ECI) on the import side of the constraint and by determining whether a single Entity is needed to resolve the constraint. (2) The competitiveness of a constraint is tested both on a long-term basis and before each Security-Constrained Economic Dispatch (SCED) execution (3) The “Available Capacity for a Resource” is defined as follows: (a) For Generation Resources, including Switchable Generation Resources (excludes Wind-powered Generation Resources (WGRs)). (i) Long-Term CCT - the Seasonal Net Max Sustainable Rating, as registered with ERCOT. (ii) SCED CCT - the telemetered High Sustained Limit (HSL). (b) For WGRs: (i) Long-Term CCT - on the export side, the Seasonal Net Max Sustainable Rating, as registered as specified in its ERCOT- approved Resource Asset Registration Form, and on the import side, zero MW. (ii) SCED CCT - the telemetered HSL. (c) The full import capability of the Direct Current Tie (DC Tie) lines on the export side and zero MW on the import side for all CCTs. (3) “Managed Capacity for an Entity” is a Resource or Split Generation Resource for which the Entity or its Affiliates has the decision-making authority over how the Resource or Split Generation Resource is offered or scheduled (e.g., Output Schedules), in accordance with subsection (e) of P.U.C. Subst. R. 25.502, Pricing Safeguards in Markets Operated by the Electric Reliability Council of Texas. Each Resource Entity that owns a Resource shall submit a declaration to ERCOT, using a form designated by ERCOT, as to which Entity has the decision-making authority for each of its Resources. The declaration shall be signed by the Authorized Representative of the Resource Entity. In addition, each Resource Entity that owns a Resource shall Notify ERCOT of any known changes in that declaration no later than 14 days prior to the date that the change takes effect or as soon as possible in a situation where the Resource Entity is unable to meet the 14-day Notice requirement. Upon ERCOT’s request, each Resource Entity that owns a Resource shall provide ERCOT with sufficient information or documentation to verify control of the Resource. ERCOT shall apply decision-making authority to Managed Capacity for an Entity effective the first Operating Hour of the Operating Day ERCOT satisfactorily confirms the Resource Entity’s most recent declaration, but not sooner than the effective date specified on the Resource Entity’s most recent declaration. (4) Shift Factors of all Electrical Buses are computed relative to the distributed load reference Bus. (a) For voltage, stability, and thermal-limited constraints, as well as interfaces represented by thermal limits, the Shift Factors should be computed with no other contingencies removed from the electrical network. (b) For contingency-limited constraints, the Shift Factors used should be computed with the contingencies removed from the electrical network. (5) ERCOT shall post values of ECIT1, ECIT2, SFP1, SFP2, DMEECP, and SFP3 in a TAC-approved document on the Market Information System (MIS) Public Area. 3.19.2 Element Competitiveness Index Calculation (1) The ECI is one criteria used in the Long-Term and SCED CCTs to determine the competitiveness of a constraint. (2) To compute the ECI on the import side, first determine the “ECI Effective Capacity” available to resolve the constraint. The ECI Effective Capacity that each Entity contributes to resolve the constraint on the import side is determined by taking, for each Managed Capacity for an Entity having negative Shift Factors with absolute values greater than the minimum of one-third of the highest absolute value of any Resource Shift Factor with a negative value and SFP1%, the sum of the products of (A) the Available Capacity for a Resource and (B) the square of the Shift Factor of that Resource to the constraint. (3) Determine the ECI on the import of the constraint, as follows: (a) Determine the total ECI Effective Capacity by each Entity and its Affiliates on the import side. (b) Determine the percentage of ECI Effective Capacity by each Entity and its Affiliates on the import side by taking each Entity and its Affiliates’ ECI Effective Capacity and dividing by the total ECI Effective Capacity on the import side. (c) The ECI on the import side is equal to the sum of the square of the percentages of ECI Effective Capacity by each Entity and its Affiliates on the import side. 3.19.3 Long-Term Constraint Competitiveness Test (1) The Long-Term CCT provides a projection of Competitive Constraints for the future 12 months. (2) The Long-Term CCT uses monthly and peak Load cases for calculations and performs analysis on a selected set of constraints. (3) A constraint is classified as a Competitive Constraint for a monthly case part of the Long-Term CCT if it meets all of the following conditions: (a) The ECI is less than ECIT1 on the import side of the constraint; (b) The constraint can be resolved by eliminating all Available Capacity for a Resource on the import side, except nuclear capacity and minimum-energy amounts of coal and lignite capacity, that is Managed Capacity for an Entity or its Affiliates during peak Load conditions; and (c) There are negative Shift Factors corresponding to Electrical Buses with Available Capacity for a Resource that have an absolute value greater than or equal to SFP2%. (4) Any constraint that is analyzed and does not meet the conditions in paragraph (3) above will be designated as a Non-Competitive Constraint for the monthly case. (5) ERCOT shall update and post the list of Competitive Constraints identified by the Long-Term CCT for each of the monthly cases on the MIS Secure Area. The list of Competitive Constraints for each monthly case shall be posted prior to the bid window opening for the CRR Long-Term Auction Sequence that includes that month.

3.19.4	Monthly Constraint Competitiveness Test

(1)	The Monthly CCT uses the peak case for the particular month for all calculations.  The peak case must include planned transmission and generation Outages for the month.

(2)	Unless otherwise approved by TAC as a Competitive Constraint, the Monthly CCT shall change the treatment of a Competitive Constraint to a Non-Competitive Constraint for the particular month if the constraint meets any of the following conditions:

(a)	The ECI is greater than 2,500 on the import side or the ECI is greater than 3,000 on the export side; or

(b)	The constraint cannot be resolved by eliminating all Available Capacity for a Resource on the import side, except nuclear capacity and minimum-energy amounts of coal and lignite capacity that is Managed Capacity for an Entity or its Affiliates during peak Load conditions; or

(c)	There are no positive Shift Factors corresponding to Electrical Buses with Available Capacity for a Resource that have a value greater than or equal to 2%, and there are no negative Shift Factors corresponding to Electrical Buses with Available Capacity for a Resource that have an absolute value greater than or equal to 2%.

[NPRR520: Delete 3.19.4 above upon system implementation.]

3.19.5	Daily Constraint Competitiveness Test

(1)	The Daily CCT uses the peak hour of the particular day under test.  The peak hour case must include planned transmission and Resource Outages for the day.

(2)	Based on the set of the Competitive Constraints as determined in the Monthly CCT, the Daily CCT shall change the treatment of a Competitive Constraint to a Non-Competitive Constraint for the particular day if the constraints meet any of the following conditions:

(a)	The ECI is greater than 2,500 on the import side or the ECI is greater than 3,000 on the export side; or

(b)	The constraint cannot be resolved by eliminating all Available Capacity for a Resource on the import side, except nuclear capacity and minimum-energy amounts of coal and lignite capacity, that is Managed Capacity for an Entity or its Affiliates during the peak hour of the day; or

(c)	There are no positive Shift Factors corresponding to Electrical Buses with Available Capacity for a Resource that have a value greater than or equal to 2%, and there are no negative Shift Factors corresponding to Electrical Buses with Available Capacity for a Resource that have an absolute value greater than or equal to 2%.

(3)	ERCOT shall post the Competitive Constraints to the MIS Secure Area by 0600 in the Day-Ahead.

[NPRR520: Replace Section 3.19.5 above with the following upon system implementation:] 3.19.4 Security-Constrained Economic Dispatch Constraint Competitiveness Test (1) The SCED CCT uses current system conditions to evaluate the competitiveness of a constraint. (2) Before each SCED execution, CCT is performed for all active constraints in SCED. The SCED CCT shall classify a constraint as competitive for the current SCED execution if the constraint meets all of the following conditions: (a) The ECI is less than ECIT2 on the import side; (b) The constraint can be resolved by eliminating all Available Capacity for a Resource on the import side, except nuclear capacity and minimum-energy amounts of coal and lignite capacity, that is Managed Capacity for an Entity or its Affiliates; (c) There are no negative Shift Factors corresponding to Electrical Buses with Available Capacity for a Resource that have an absolute value greater than or equal to SFP2%; and (d) The constraint was not designated as non-competitive by a previous SCED CCT execution within the current Operating Hour. (3) Any constraint that is analyzed and is not designated as a Competitive Constraint under the conditions outlined in paragraph (2) above shall be designated as a Non-Competitive Constraint by the SCED CCT. (4) A constraint that is determined to be a Non-Competitive Constraint by the SCED CCT within an Operating Hour will not be re-evaluated for the remainder of that Operating Hour. SCED will re-evaluate the competitiveness of the Non-Competitive Constraint starting with the first SCED interval of the next Operating Hour if the constraint remains active in SCED. (5) The Independent Market Monitor (IMM) may designate any constraint as a Competitive Constraint or a Non-Competitive Constraint. Any such designation from the IMM shall override the result of the SCED CCT. ERCOT shall provide notice describing any constraint designation by the IMM three days prior to it taking effect. The notice shall also include justification for the constraint designation by the IMM and the duration for which the IMM designation will be applied. (6) Each hour, ERCOT shall post on the MIS Public Area whether each binding constraint was designated as a Competitive Constraint or as a Non-Competitive Constraint for each of the SCED executions during the previous Operating Hour (7) Mitigation will be applied to a Resource in the SCED Step 2, as described in Section 6.5.7.3, Security Constrained Economic Dispatch, when all of the following conditions are met: (a) A constraint has been determined to be a Non-Competitive Constraint by either the SCED CCT or the IMM; (b) The percentage of ECI Effective Capacity on the import side for the constraint is greater than DMEECP% for the Entity which has decision-making authority for the Resource or the constraint cannot be resolved by eliminating all Available Capacity for Resources on the import side, except nuclear capacity and minimum-energy amounts of coal and lignite capacity, that is Managed Capacity for the Entity which has decision-making authority for the Resource; and (c) The Resource has a shift factor on the import side of the constraint with an absolute value greater than SFP3%; (8) Once mitigation has been applied to a Resource for a SCED interval, it shall remain applied for the remainder of the Operating Hour regardless of the conditions listed in (7) above.

- 6.5.7.3	Security Constrained Economic Dispatch

(1)	The SCED process is designed to simultaneously manage energy, the system power balance and network congestion through Resource Base Points and calculation of LMPs every five minutes.  The SCED process uses a two-step methodology that applies mitigation prospectively to resolve network Non-Competitive Constraints for the current Operating Hour.  The SCED process evaluates Energy Offer Curves and Output Schedules to produce a least cost dispatch of On-Line Generation Resources to the total current generation requirement determined by LFC, subject to power balance and network constraints.  The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP.

[NPRR257: Replace paragraph (1) above with the following upon system implementation:] (1) The SCED process is designed to simultaneously manage energy, the system power balance and network congestion through Resource Base Points and calculation of LMPs every five minutes. The SCED process uses a two-step methodology that applies mitigation prospectively to resolve Non-Competitive Constraints for the current Operating Hour. The SCED process evaluates Energy Offer Curves and Output Schedules to produce a least cost dispatch of On-Line Generation Resources to the total current generation requirement determined by LFC, subject to power balance and network constraints. The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP.

(2)	The SCED solution must monitor cumulative deployment of Regulation Services and ensure that Regulation Services deployment is minimized over time.

(3)	For use as SCED inputs, ERCOT shall use the available capacity of all committed Generation Resources by creating proxy Energy Offer Curves for certain Resources as follows:

(a)	Non-WGRs and Dynamically Scheduled Resources (DSRs) without Energy Offer Curves

ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below for:

(i)	Each non-WGR for which its QSE has submitted an Output Schedule instead of an Energy Offer Curve; and

(ii)	Each DSR that has not submitted Incremental and Decremental Energy Offer Curves.

| MW | Price (per MWh) |
| --- | --- |
| HSL | System-Wide Offer Cap (SWCAP) |
| Output Schedule MW plus 1 MW | SWCAP minus $0.01 |
| Output Schedule MW | -$249.99 |
| LSL | -$250.00 |

(b)	DSRs with Energy Offer Curves

For each DSR that has submitted incremental and decremental Energy Offer Curves, ERCOT shall create a monotonically increasing proxy Energy Offer Curve.  That curve must consist of the incremental Energy Offer Curve that reflects the available capacity above the Resource’s Output Schedule to its HSL and the decremental Energy Offer Curve that reflects the available capacity below the Resource’s Output Schedule to the LSL.  The curve must be created as described below:

| MW | Price (per MWh) |
| --- | --- |
| Output Schedule MW plus 1 MW to HSL | Incremental Energy Offer Curve |
| LSL to Output Schedule MW | Decremental Energy Offer Curve |

(c)	Non-WGRs without full-range Energy Offer Curves

For each non-WGR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the Resource’s available capacity, ERCOT shall create a proxy Energy Offer Curve that extends the submitted Energy Offer Curve to use the entire available capacity of the Resource using the SWCAP above the highest point on the Energy Offer Curve to the Resource’s HSL and the offer floor from the lowest point on the Energy Offer Curve to its LSL, using these points:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in Energy Offer Curve) | SWCAP |
| 1 MW above highest MW in Energy Offer Curve (if less than HSL) | SWCAP minus $0.01 |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(d)	WGRs

(i)	For each WGR that has not submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL | SWCAP |
| HSL minus 1 MW | -$249.99 |
| LSL | -$250.00 |

(ii)	For each WGR for which its QSE has submitted an Energy Offer Curve, ERCOT shall create a monotonically increasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in Energy Offer Curve) | SWCAP |
| 1 MW above highest MW in Energy Offer Curve (if less than HSL) | SWCAP minus $0.01 |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(4)	The Entity with decision making authority, as more fully described in Section 3.19.1, Constraint Competitiveness Test Definitions, over how a Resource or Split Generation Resource is offered or scheduled, shall be responsible for all offers associated with each Resource, including offers represented by a proxy Energy Offer Curve.

[NPRR240: Insert paragraph (5) and renumber accordingly upon system implementation:] (5) Energy Offer Curves that were constructed in whole or in part with proxy Energy Offer Curves shall be so marked in all ERCOT postings or references to the energy offer.

(5)	The two-step SCED methodology referenced in paragraph (1) above is:

(a)	The first step is to execute the SCED process to determine Reference LMPs.  In this step, ERCOT executes SCED using the full Network Operations Model while only observing limits of Competitive Constraints.  Energy Offer Curves for all On-Line Generation Resources, whether submitted by QSEs or created by ERCOT under this Section, are used in the SCED to determine “Reference LMPs.”

(b)	The second step is to execute the SCED process to produce Base Points, Shadow Prices, and LMPs, subject to security constraints (including Competitive and Non-Competitive Constraints) and other Resource constraints.  The second step must:

(i)	Use Energy Offer Curves for all On-Line Generation Resources, whether submitted by QSEs or created by ERCOT.  Each Energy Offer Curve must be capped at the greater of the Reference LMP (from Step 1) at the Resource Node or the appropriate Mitigated Offer Cap and bounded at the lesser of the Reference LMP (from Step 1) at the Resource Node or the appropriate Mitigated Offer Floor; and

(ii)	Observe all Competitive and Non-Competitive Constraints.

[NPRR520: Replace paragraph (5)(b) above with the following upon system implementation:] (b) The second step is to execute the SCED process to produce Base Points, Shadow Prices, and LMPs, subject to security constraints (including Competitive and Non-Competitive Constraints) and other Resource constraints. The second step must: (i) Use Energy Offer Curves for all On-Line Generation Resources, whether submitted by QSEs or created by ERCOT. Each Energy Offer Curve must be bounded at the lesser of the Reference LMP (from Step 1) at the Resource Node or the appropriate Mitigated Offer Floor. In addition, each Energy Offer Curve subject to mitigation under the criteria described in Section 3.19.4, Security-Constrained Economic Dispatch Constraint Competitiveness Test, must be capped at the greater of the Reference LMP (from Step 1) at the Resource Node plus a variable not to exceed 0.01 multiplied by the lowest value of the Resource’s Mitigated Offer Cap curve or the appropriate Mitigated Offer Cap; and (ii) Observe all Competitive and Non-Competitive Constraints.

(c)	ERCOT shall archive information and provide monthly summaries of security violations and any binding transmission constraints identified in Step 2 of the SCED process.  The summary must describe the limiting element (or identified operator-entered constraint with operator’s comments describing the reason and the Resource-specific impacts for any manual overrides).  ERCOT shall provide the summary to Market Participants on the MIS Secure Area and to the Independent Market Monitor (IMM).

(6)	For each SCED process, in addition to the binding Base Points and LMPs, ERCOT shall calculate a non-binding projection of the Base Points and Resource Node LMPs, Hub LMPs and Load Zone LMPs at a frequency of every five minutes for at least 15 minutes into the future based on the same inputs to the SCED process as described in this Section, except that the Resource’s HDL and LDL and the total generation requirement will be as estimated at future intervals.  The Resource’s HDL and LDL will be calculated for each interval of the projection based on the ramp rate capability over the study period.  ERCOT shall estimate the projected total generation requirement by calculating a Load forecast for the study period.  ERCOT shall post the projected non-binding Base Points for each Resource for each interval study period on the MIS Certified Area and the projected non-binding LMPs for Resource Nodes, Hub LMPs and Load Zone LMPs on the MIS Public Area pursuant to Section 6.3.2, Activities for Real-Time Operations.

6.5.7.4	Base Points

(1)	ERCOT shall issue a Base Point for each On-Line Generation Resource on completion of each SCED execution.  The Base Point set by SCED must observe a Generation Resource’s HDL and LDL.  Base Points are automatically superseded on receipt of a new Base Point from ERCOT regardless of the status of any current ramping activity of a Resource.  ERCOT shall provide each Base Point using Dispatch Instructions issued over Inter-Control Center Communications Protocol (ICCP) data link to the QSE representing each Resource that include the following information:

(a)	Resource identifier that is the subject of the Dispatch Instruction;

(b)	MW output;

(c)	Time of the Dispatch Instruction;

(d)	Flag indicating SCED has dispatched a Generation Resource below HDL used by SCED;

(e)	Flag indicating SCED has dispatched a Generation Resource away from the Output Schedule submitted for that Generation Resource; and

[NPRR520: Insert paragraph (1)(f) below and renumber accordingly upon system implementation:] (f) Flag indicating SCED has mitigated the Generation Resource’s Energy Offer Curve; and

(f)	Other information relevant to that Dispatch Instruction.