| PGRR Number | 024 | PGRR Title | Clarify Planning Assumptions and Performance Criteria |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Approved |
| Date of Decision |  | March 19, 2013 |  |  |
| Effective Date |  | April 1, 2013 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Planning Guide Sections Requiring Revision |  | 2.1, Definitions 4.1, Introduction 4.1.1.1, Planning Assumptions 4.1.1.2, Performance Requirements for Credible Single Contingencies for Transmission Planning |  |  |
| Protocol Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | This Planning Guide Revision Request (PGRR) clarifies assumptions allowed for planning studies and planning performance criteria and better reflects current planning practices. |  |  |
| Reason for Revision |  | Clarifies the planning criteria and simplifies the format to more efficiently incorporate future ERCOT-specific criteria additions. |  |  |
| Procedural History |  | - On 10/31/12, PGRR024 was posted. - On 11/15/12, the Planning Working Group (PLWG) considered PGRR024. - On 11/21/12, an Impact Analysis was posted. - On 11/29/12, the PLWG considered the 11/15/12 PLWG Report and Impact Analysis for PGRR024. - On 12/6/12, ROS considered PGRR024. - On 1/3/13, TAC considered PGRR024. - On 3/19/13, the ERCOT Board considered PGRR024. |  |  |
| PLWG Decision |  | On 11/15/12, the PLWG was in consensus to recommend approval of PGRR024 as revised by the PLWG. On 11/29/12, the PLWG was in consensus to endorse and forward the 11/15/12 PLWG Report as revised by PLWG and the Impact Analysis for PGRR024 to ROS. |  |  |
| Summary of PLWG Discussion |  | On 11/15/12, whether ERCOT should make the final list of identified solutions to meet the reliability performance criteria available to all Market Participants was discussed. ERCOT Staff agreed to review whether providing the list of final solutions to all Market Participants is a confidentiality issue. On 11/29/12, ERCOT Staff expressed concern that as written, the requirement to provide the final list of identified solutions is vague and should be deleted. Additional non-substantive administrative changes were also proposed. |  |  |
| ROS Decision |  | On 12/6/12, ROS unanimously voted to recommend approval of PGRR024 as recommended by PLWG in the 11/29/12 PLWG Report and as revised by ROS. All Market Segments were present for the vote. |  |  |
| Summary of ROS Discussion |  | On 12/6/12, it was stated that paragraph (3) of Section 4.1 should be deleted as Entities are required to comply with applicable North American Electric Reliability Corporation (NERC) Reliability Standards. |  |  |
| TAC Decision |  | On 1/3/13, TAC voted to recommend approval of PGRR024 as recommended by ROS in the 12/6/12 ROS Report. There were two abstentions from the Independent Generator and Independent Power Marketer (IPM) Market Segments. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 1/3/12, it was explained how PGRR024 revises the Planning Guides to align with current planning practices. |  |  |
| ERCOT Opinion |  | ERCOT supports approval of PGRR024 as it aligns the Planning Guide with current planning practices. |  |  |
| Board Decision |  | On 3/19/13, the ERCOT Board approved PGRR024 as recommended by TAC in the 1/3/12 TAC Report. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | Added clarity will benefit ERCOT and Transmission Service Provider (TSP) planners. |

| Sponsor |  |
| --- | --- |
| Name | Sergio Garza on behalf of the PLWG |
| E-mail Address | Sergio.garza@lcra.org |
| Company | LCRA |
| Phone Number | (512) 578-4149 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Yvette M. Landin |
| E-Mail Address | ylandin@ercot.com |
| Phone Number | (512) 248-4513 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None. |  |

Comments

Please note that PGRR025, Addition of Criteria for Autotransformer Unavailability, also proposes revisions to Section 4.1.1.2.

Proposed Guide Language Revision

## 2.1		Definitions

- 4.1	Introduction

(1)	ERCOT employs both reliability criteria and economic criteria in evaluating the need for transmission system improvements.  The economic criteria are included in Protocol Section 3.11.2, Planning Criteria.  This Planning Guide provides the reliability criteria.

(2)	The ERCOT System consists of those generation and Transmission Facilities (60 kV and higher voltages) that are controlled by individual Market Participants and that function as part of an integrated and coordinated system.

(3)	To maintain reliable operation of the ERCOT System, it is necessary that all stakeholders observe and subscribe to certain minimum planning criteria.  The criteria set forth in this Section 4.1 constitute the aforementioned minimum planning criteria.  Tests outlined herein shall be performed to determine conformance to these minimum criteria; however, ERCOT recognizes that events more severe than those outlined in these criteria could cause grid separation and other tests may also be performed.

(4)	The complexity and uncertainty inherent in the planning and operation of the ERCOT System make exhaustive studies impracticable; therefore, to gain maximum benefit from the limited number of tests performed, the selection of the specific tests and the frequency of their performance will be made solely upon the basis of the expected value of the reliability information obtainable from the test.

(5)	ERCOT shall perform steady-state and dynamic analyses appropriate to ensure the reliability of the ERCOT System and identify appropriate solutions.

(6)	Each Transmission Service Provider (TSP) will perform steady-state, short circuit, and dynamic analyses appropriate to ensure the reliability of its portion of the ERCOT System and implement appropriate solutions to meet the reliability performance criteria in this Section 4.1.

(7)	The base cases created by the Steady-State Working Group (SSWG) and System Protection Working Group (SPWG) are available for use by Market Participants.

(8)	If a TSP has its own planning criteria in addition to those defined in this Planning Guide, the TSP shall provide documentation of those criteria to ERCOT.  ERCOT shall post the documentation on the Planning and Operations Information website.  The TSP shall notify ERCOT of any changes to their planning criteria and provide revised documentation within 30 days of such change.

- 4.1.1.1	Planning Assumptions

(1)	A contingency loss of an element includes the loss of an element with or without a single line-to-ground or three-phase fault.

(2)	A common tower outage is the contingency loss of a double-circuit transmission line consisting of two circuits sharing a tower for 0.5 miles or greater.

(3)	Unavailability of a single generating unit includes an entire Combined Cycle Train, if no part of the train can operate with one of the units Off-Line as provided in the Resource registration form.

(4)	The contingency loss of a single generating unit shall include the loss of an entire Combined Cycle Train, if that is the expected consequence.

(5)	The following assumptions may be applied to the SSWG base cases for use in planning studies:

(a)	Reasonable variations of Load forecast;

(b)	Reasonable variations of generation commitment and dispatch applicable to transmission planning analyses on a case-by-case basis may include, but are not limited to, the following methods:

(i)	Production cost model simulation, security constrained optimal power flow, or similar modeling tools that analyze the ERCOT System using hourly generation dispatch assumptions;

(ii)	Modeling of high levels of intermittent generation conditions; or

(iii)	Modeling of low levels of or no intermittent generation conditions.

- 4.1.1.2	Reliability Performance Criteria

(1)	The following performance criteria (summarized in Table 1, ERCOT-specific Reliability Performance Criteria, below) shall be applicable to planning analyses in the ERCOT Region:

- (a)	With all Facilities in their normal state, following a common tower outage, all Facilities shall be within their applicable Ratings, the ERCOT System shall remain stable with no cascading or uncontrolled Islanding, and there shall be no non-consequential Load loss.

- (b)	With any single generating unit unavailable, followed by Manual System Adjustments, followed by a common tower outage or the contingency loss of a transmission circuit or transformer, all Facilities shall be within their applicable Ratings, the ERCOT System shall remain stable with no cascading or uncontrolled Islanding, and there shall be no non-consequential Load loss.

| Initial Condition | Event | Facilities within Applicable Ratings and System Stable with No Cascading or Uncontrolled Outages | Non-consequential Load Loss Allowed |
| --- | --- | --- | --- |
| Normal System | Common tower outage | Yes | No |
| Unavailability of a generating unit, followed by Manual System Adjustments | Common tower outage; or Contingency loss of one of the following: 1. Transmission circuit; or 2. Transformer | Yes | No |

Table 1: ERCOT-specific Reliability Performance Criteria

(2)	ERCOT and the TSPs shall endeavor to resolve any performance deficiencies as appropriate.  If a Transmission Facility improvement is required to meet the criteria in this Section 4.1.1.2, but the improvement cannot be implemented in time to resolve the performance deficiency, an interim solution may be used to resolve the deficiency until the improvement has been implemented.