| NPRR Number | 489 | NPRR Title | Planning Reserve Margin |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Approved |
| Date of Decision |  | March 19, 2013 |  |  |
| Effective Date |  | April 1, 2013 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms and Abbreviations 3.2.6, Planning Reserve Margin (new) 3.2.6.1, Minimum ERCOT Planning Reserve Margin Criterion (new) 3.2.6.2, ERCOT Planning Reserve Margin Calculation Methodology (new) 3.2.6.2.1, Peak Load Estimate (new) 3.2.6.2.2, Total Capacity Estimate (new) 21.1, Introduction |  |  |
| Planning Guide Sections Requiring Revision |  | 1.2.1, Introduction 2.1, Definitions 2.2, Acronyms and Abbreviations 8, Planning Reserve Margin (delete) 8.1, ERCOT Planning Reserve Margin (delete) 8.2, Minimum ERCOT Planning Reserve Margin Criterion (delete) 8.3, ERCOT Planning Reserve Margin Calculation Methodology (delete) 8.3.1, Peak Load Estimate (delete) 8.3.2, Total Capacity Estimate (delete) |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) adds the Planning Reserve Margin (PRM) methodology, currently in the Planning Guide, to the Protocols. |  |  |
| Reason for Revision |  | The calculation methodology used by ERCOT to determine the PRM for the semi-annual Report on Capacity, Demand and Reserves in the ERCOT Region (CDR) has historically been developed by the Generation Adequacy Task Force (GATF), subsequently approved by the WMS, and then ultimately approved by the TAC. In 2010, Market Participants decided that adding the PRM methodology into the Planning Guide would increase the transparency of the PRM methodology, and also provide a formal change and approval procedure via the Planning Guide Revision Request (PGRR) process. Including the PRM methodology in the Planning Guide also provided for ERCOT Board approval. The PGRR approval requires a recommendation from the Planning Working Group (PLWG), the ROS, TAC, and finally ERCOT Board approval. While this approval path achieved its objective (i.e., to improve and formalize the PRM approval process) it did not include language acknowledging that the PRM methodology had historically been under GATF and WMS purview. Discussions at WMS and ROS resulted in both subcommittees agreeing the PRM calculation methodology: - Is important enough that it needs to be transparent; - Needs input from WMS, ROS, and any other interested stakeholder body or Market Participant; and - Should receive ERCOT Board approval. It was also determined the best way to meet these three goals is to move the PRM methodology from the Planning Guide to the Protocols. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR489 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 10/3/12, NPRR489 was posted. - On 10/17/12, ERCOT comments were posted. - On 10/17/12, Sierra Club comments were posted. - On 10/18/12, PRS considered NPRR489. - On 10/31/12, an Impact Analysis was posted. - On 11/15/12, PRS considered the 10/18/12 PRS Report and Impact Analysis for NPRR489. - On 11/29/12, TAC considered NPRR489. - On 12/7/12, NRG Energy, Inc. comments were posted. - On 1/3/13, TAC again considered NPRR489. - On 2/18/13, a second set of ERCOT comments were posted. - On 3/7/13, TAC comments were posted. - On 3/19/13, the ERCOT Board considered NPRR489. |  |  |
| PRS Decision |  | On 10/18/12, PRS voted to recommend approval of NPRR489 as amended by the 10/17/12 ERCOT comments and as revised by PRS. There were two abstentions from the Municipal and Independent Power Marketer (IPM) Market Segments. All Market Segments were present for the vote. On 11/15/12, PRS voted to endorse and forward the 10/18/12 PRS Report and Impact Analysis for NPRR489 to TAC. There was one abstention from the IPM Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 10/18/12, whether the parameters used for the Loss of Load Expectation (LOLE) study should be included in NPRR489 was discussed. It was stated that NPRR489 should move forward as submitted and that resource adequacy issues currently being discussed at the Public Utility Commission of Texas (PUCT) and the GATF should be addressed in future NPRRs. On 11/15/12, it was noted that adjustments to the CDR are necessary to better incorporate Demand response efficiency. |  |  |
| TAC Decision |  | On 11/29/12, TAC unanimously voted to table NPRR489. All Market Segments were present for the vote. On 1/3/13, TAC unanimously voted to recommend approval of NPRR489 as amended by the 12/7/12 NRG Energy, Inc. comments. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 11/29/12, whether the parameters used for the LOLE study should be included in NPRR489 was discussed. It was noted that NPRR489 will be reviewed by GATF to determine the feasibility of separating the CDR portion and that resource adequacy issues should be addressed in future NPRRs. On 1/3/13, it was reiterated that NPRR489 was intended to move the PRM language from the Planning Guides to the Protocols and that resource adequacy issues currently being discussed at the PUCT and the GATF should be addressed in future NPRRs. |  |  |
| ERCOT Opinion |  | ERCOT supports approval of NPRR489 as it provides additional transparency into the methodology used by ERCOT to determine the PRM for the semi-annual CDR. |  |  |
| Board Decision |  | On 3/19/13, the ERCOT Board approved NPRR489 as revised by the 2/18/13 ERCOT Comments. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | Improves the transparency and approval process for the PRM calculations performed by ERCOT for the semi-annual CDR. |

| Sponsor |  |
| --- | --- |
| Name | Adrian Pieniazek |
| E-mail Address | adrian.pieniazek@nrgenergy.com |
| Company | NRG Energy, Inc. |
| Phone Number | 512-473-8895 |
| Cell Number | 512-844-9888 |
| Market Segment | Independent Generator |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Yvette M. Landin |
| E-Mail Address | ylandin@ercot.com |
| Phone Number | 512-248-4513 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 101712 | Proposed changes to define “TOTCAPs ,i ” in Section 3.2.6.2.2 and to correct uses of Season and Peak Load Season, and proposed additional non-substantive changes. |
| Sierra Club 101712 | Recommended that PRS table NPRR489 to allow time for discussions at GATF, Demand Side Working Group (DSWG), and the PUCT. Also proposed new categories for the CDR. |
| NRG Energy, Inc. 120712 | Proposed changes to remove the linkage between the planning model assumptions and the CDR/PRM assumptions. |
| ERCOT 021813 | Proposed changes to Section 21.1 to allow modifications to the PRM to be processed via Administrative NPRRs. |
| TAC 030713 | Endorsed the 2/18/13 ERCOT comments. |

Proposed Protocol Language Revision

- 2.1	DEFINITIONS

- Interconnecting Entity (IE)

Any Entity that proposes to interconnect an All-Inclusive Generation Resource with the ERCOT System, upgrade the rated capacity of an existing All-Inclusive Generation Resource by ten MW or greater, re-power an All-Inclusive Generation Resource, or change the Point of Interconnection (POI) of an All-Inclusive Generation Resource.

- Peak Load Season

Summer months are June, July, August, and September; Winter months are December, January, and February.

- Planning Reserve Margin (PRM)

The net of total capacity for the Peak Load Season, less firm peak Load for the Peak Load Season, divided by the firm peak Load for the Peak Load Season (expressed as a percentage).

- 2.2	ACRONYMS AND ABBREVIATIONS

- IE		Interconnecting Entity

- PRM		Planning Reserve Margin

3.2.6	ERCOT Planning Reserve Margin

ERCOT shall calculate the Planning Reserve Margin (PRM) for each Peak Load Season as follows:

PRMs ,i	=	(TOTCAPs, i – FIRMPKLDs, i) / FIRMPKLDs, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| PRM s, i | % | Planning Reserve Margin—The Planning Reserve Margin for the Peak Load Season s for year i. |
| TOTCAP s, i | MW | Total Capacity—Total Capacity available during the Peak Load Season s for the year i. |
| FIRMPKLD s, i | MW | Firm Peak Load—Firm Peak Load for the Peak Load Season s for the year i. |
| i | None | Year. |
| s | None | Peak Load Season. |

3.2.6.1	Minimum ERCOT Planning Reserve Margin Criterion

The minimum ERCOT PRM criterion is 13.75%.  ERCOT shall periodically review and recommend to the ERCOT Board any changes to the minimum ERCOT PRM to ensure adequate reliability of the ERCOT System.

3.2.6.2	ERCOT Planning Reserve Margin Calculation Methodology

ERCOT shall prepare and publish on the ERCOT website, at least annually, the Report on Capacity, Demand and Reserves in the ERCOT Region  containing an estimate of the PRM for the current Peak Load Seasons as well as a minimum of ten future summer and winter peak Load periods.  The format and content of this report shall be developed by ERCOT, and subject to Technical Advisory Committee (TAC) approval.  The estimate of the PRM shall be based on the methodology in Section 3.2.6.2.1, Peak Load Estimate, and Section 3.2.6.2.2, Total Capacity Estimate.

3.2.6.2.1	Peak Load Estimate

ERCOT shall prepare, at least annually, a forecast of the total peak Load for both summer and winter Peak Load Seasons for the current year and a minimum of ten future years using an econometric forecast, taking into account econometric inputs, weather conditions, demographic data and other variables as deemed appropriate by ERCOT.  The firm Peak Load Season estimate shall be determined by the following equation:

FIRMPKLD s ,i	=	TOTPKLD s, i – LRRRS s, i – LRNSRS s, i – ERS s, i – CLR s, i – ENERGYEFF s, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FIRMPKLD s, i | MW | Firm Peak Load Estimate—The Firm Peak Load Estimate for the Peak Load Season s for the year i. |
| TOTPKLD s, i | MW | Total Peak Load Estimate—The Total Peak Load Estimate for the Peak Load Season s for the year i. |
| LRRRS s, i | MW | Load Resource providing Responsive Reserve (RRS)—The amount of RRS a Load Resource is providing for the Peak Load Season s for the year i. |
| LRNSRS s, i | MW | Load Resource providing Non-Spinning Reserve (Non-Spin)— The amount of Non-Spin a Load Resource is providing for the Peak Load Season s for the year i. |
| ERS s, i | MW | Emergency Response Service (ERS)— The amount of ERS for the Peak Load Season s for the year i based on: (a) For the winter Peak Load Season of the current year, the amount of ERS procured by ERCOT for the October to January ERS Contract Period using the simple average of two time period procurements (Business Hours and non-Business Hours); (b) For the summer Peak Load Season of the current year, the amount of ERS procured by ERCOT for the May procurement (Business Hours); and (c) For all subsequent years and Peak Load Seasons, escalate the amount of ERS from the previous season by 10%. |
| CLR s, i | MW | Amount of Controllable Load Resource—Amount of Controllable Load Resource that is available for Dispatch by ERCOT during the current year i for the Peak Load Season s not already included in LRRRS or LRNSRS. |
| ENERGYEFF s, i | MW | Amount of Energy Efficiency Programs Procured—Amount of energy efficiency programs procured by Transmission and/or Distribution Service Providers (TDSPs) pursuant to P.U.C. Subst. R. 25.181, Energy Efficiency Goal, for the Peak Load Season s for the year i. |
| i | None | Year. |
| s | None | Peak Load Season. |

3.2.6.2.2	Total Capacity Estimate

The total capacity estimate shall be determined based on the following equation:

TOTCAPs ,i	=	INSTCAPs, i + PUNCAP s, i + WINDCAP s, i  + RMRCAPs, i + DCTIECAP s, i + SWITCHCAP s, i + MOTHCAP s, i + PLANNON s, i + PLANWIND s, i – UNSWITCH s, i – RETCAP s, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| TOTCAP s, i | MW | Total Capacity—Total Capacity available during the Peak Load Season s for the year i. |
| INSTCAPs, i | MW | Seasonal Net Max Sustainable Rating—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource registration process for each operating Generation Resource for the year i excluding Wind-powered Generation Resources (WGRs), Resources operating under Reliability Must-Run (RMR) Agreements, and Generation Resources capable of “switching” from the ERCOT Region to a non-ERCOT Region. |
| PUNCAPs, i | MW | Private Use Network Capacity—The Private Use Network capacities as provided to ERCOT pursuant to Section 3.10.7.3, Modeling of Private Use Networks. |
| WINDCAPs, i | MW | Effective Load Carrying Capability of WGRs—The effective Load carrying capability of all existing WGRs as determined by ERCOT for the Peak Load Season s for the year i. |
| RMRCAPs, i | MW | Seasonal Net Max Sustainable Rating for Generation Resource providing RMR Service—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource registration process for each Generation Resource providing RMR Service for the year i until the approved exit strategy for the RMR Resource is expected to be completed. |
| DCTIECAPs, i | MW | Seasonal Net Max Sustainable Rating for Direct Current Tie (DC Tie) Resource—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource registration process for each DC Tie Resource for the year i multiplied by 50%. |
| SWITCHCAPs, i | MW | Seasonal Net Max Sustainable Rating for Switching Generation Resource—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource asset registration process for each Generation Resource for the year i that can electrically connect (i.e., “switch”) from the ERCOT Region to another power region. |
| MOTHCAPs, i | MW | Seasonal Net Max Sustainable Rating for Mothballed Generation Resource—The Seasonal net max sustainable rating for the Peak Load Season s as reported in the approved Resource registration process for each Mothballed Generation Resource for the year i based on the lead time and probability information furnished by the owners of Mothballed Generation Resources pursuant to Section 3.14.1.9, Generation Resource Return to Service Updates. |
| PLANNONs, i | MW | - New, non-Wind Generating Capacity—The amount of new, non-wind generating capacity for the Peak Load Season s and year i that: (a) has a Texas Commission on Environmental Quality (TCEQ)-approved air permit, and (b) has a signed Standard Generation Interconnect Agreement (SGIA), or a public, financially-binding agreement between the Resource owner and Transmission Service Provider (TSP) under which generation interconnection facilities would be constructed; or for a Municipally Owned Utility (MOU) or Electric Cooperative (EC), a public commitment letter to construct a new Resource. |
| PLANWINDs, i | MW | Effective Load Carrying Capability of New Intermittent Renewable Resource (IRR) Capacity—The effective Load carrying capability of new IRR capacity as determined by ERCOT for the Peak Load Season s and year i that has an SGIA or other public, financially-binding agreement between the Resource owner and TSP under which generation interconnection facilities would be constructed or, for a MOU or EC, a public commitment letter to construct a new IRR. |
| UNSWITCHs, i | MW | Capacity of Unavailable Switchable Generation Resource—The amount of capacity reported by the owners of a switchable Generation Resource that will be unavailable to ERCOT during the Peak Load Season s and year i pursuant to paragraph (2) of Section 16.5.4, Maintaining and Updating Resource Entity Information. |
| RETCAPs, i | MW | Capacity Pending Retirement—The amount of capacity in Peak Load Season s of year i that is pending retirement based on information a submitted on a Notification of Suspension of Operations form (Section 22, Attachment E, Notification of Suspension of Operations) pursuant to Section 3.14.1.11, Budgeting Eligible Costs, but is under review by ERCOT pursuant to Section 3.14.1.2, ERCOT Evaluation, that has not otherwise been considered in any of the above defined categories. |
| i | None | Year. |
| s | None | Peak Load Season. |

21.1	Introduction

(1)	A request to make additions, edits, deletions, revisions, or clarifications to these Protocols, including any attachments and exhibits to these Protocols, is called a Nodal Protocol Revision Request (NPRR).  Except as specifically provided otherwise in the following sentence or in other sections of these Protocols, Sections 21.2, Submission of a Nodal Protocol Revision Request or System Change Request, through 21.8, Review of Guide Changes, apply to all NPRRs.  ERCOT Members, Market Participants, Public Utility Commission of Texas (PUCT) Staff, Texas Reliability Entity (Texas RE) Staff, ERCOT, and any other Entities are required to utilize the process described herein prior to requesting, through the PUCT or other Governmental Authority, that ERCOT make a change to these Protocols, except for good cause shown to the PUCT or other Governmental Authority.

(2)	A request that ERCOT change its computer systems that does not require a revision to the Protocols is called a System Change Request (SCR).  Except as specifically provided in other sections of these Protocols, Sections 21.2 through 21.7, Review of Project Prioritization and Annual Budget Process, apply to all SCRs.

(3)	The “next regularly scheduled meeting” of the Protocol Revision Subcommittee (PRS), the Technical Advisory Committee (TAC), an Assigned TAC Subcommittee (as defined below), or the ERCOT Board shall mean the next regularly scheduled meeting for which required notice can be timely given regarding the item(s) to be addressed, as specified in the appropriate ERCOT Board or committee procedures.

(4)	ERCOT may make non-substantive corrections at any time during the processing of a particular NPRR.  Under certain circumstances, however, the Nodal Protocols can also be revised by ERCOT rather than using the NPRR process outlined in Section 21.4, Nodal Protocol Revision and System Change Procedure.

(a)	This type of revision is referred to as an “Administrative NPRR” or “Administrative Changes” and shall consist of non-substantive corrections, such as typos (excluding grammatical changes), internal references (including table of contents), improper use of acronyms, and references to ERCOT Protocols, PUCT Substantive Rules, the Public Utility Regulatory Act (PURA), North American Electric Reliability Corporation (NERC) regulations, Federal Energy Regulatory Commission (FERC) rules, etc.  Changes to the minimum ERCOT Planning Reserve Margin (PRM) in Section 3.2.6.1, Minimum ERCOT Planning Reserve Margin Criterion, approved by the ERCOT Board, shall also be processed as Administrative NPRRs.

(b)	ERCOT shall post such Administrative NPRRs to the ERCOT website and distribute the NPRR to PRS at least ten Business Days before implementation.  If no Entity submits comments to the Administrative NPRR in accordance with paragraph (1) of Section 21.4.4, Protocol Revision Subcommittee Review and Action, ERCOT shall implement it according to paragraph (4) of Section 21.6, Nodal Protocol Revision Implementation.  If any ERCOT Member, Market Participant, PUCT Staff, Texas RE Staff or ERCOT submits comments to the Administrative NPRR, then it shall be processed in accordance with the NPRR process outlined in Section 21.4.