| NPRR Number | 524 | NPRR Title | Resource Limits in Providing Ancillary Service |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Tabled |
| Date of Decision |  | March 21, 2013 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Nodal Protocol Section Requiring Revision |  | 3.18, Resource Limits in Providing Ancillary Service |  |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) will clarify and will allow Generating Resources with supplemental capacity that is not always frequency responsive to provide the supplemental capacity to the market and also continue to participate in the Responsive Reserve (RRS) Service market. |  |  |
| Reason for Revision |  | Many Generation Resources in the ERCOT Region have identified opportunities to produce additional energy above their traditional High Reasonability Limit (HRL) by using various types of supplemental MW enhancements. While the Generating Resource continues to provide frequency response when operating between the Low Reasonability Limit (LRL) and the traditional HRL, the new supplemental capacity above the traditional HRL may or may not be frequency responsive at all times. |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - On 3/5/13, NPRR524 was posted. - On 3/8/13, Calpine comments were posted. - On 3/11/13, GDF Suez Energy Marketing NA comments were posted. - On 3/20/13, ERCOT comments were posted. - On 3/21/13, PRS considered NPRR524. |  |  |
| PRS Decision |  | On 3/21/13, PRS unanimously voted to table NPRR524 and to refer the issue to ROS and WMS. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 3/21/13, some participants requested that ROS and WMS review NPRR524 in conjunction with NPRR527, Required Combined Cycle Telemetry for Operational Awareness and PDCWG Analysis, to provide an opportunity to better understand the implications of both NPRRs. It was noted that discussions are expected to occur at the Performance, Disturbance, Compliance Working Group (PDCWG) and the Qualified Scheduling Entity (QSE) Managers Working Group (QMWG). ERCOT Staff noted that both NPRRs are related to compliance with Texas Reliability Entity (Texas RE) Regional Reliability Standard BAL-001-TRE-01, Primary Frequency Response in the ERCOT Region. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | - This NPRR will allow otherwise stranded capacity to be brought to the energy and Ancillary Service markets. |
|  | 2 | - ERCOT will be able to monitor supplemental capacity that may not be responsive to frequency at all times, by mapping Inter-Control Center Communications Protocol (ICCP) data to existing ERCOT Real-Time operations displays. |

| Sponsor |  |
| --- | --- |
| Name | Robert Helton |
| E-mail Address | Robert.helton@gdfsuezna.com |
| Company | GDF Suez Energy Marketing NA, Inc. |
| Phone Number | 713-636-1814 |
| Cell Number | 832-435-7815 |
| Market Segment | Independent Generator |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| Calpine 030813 | Questioned the use of HRL as a boundary value. Recommended additional discussion regarding the provision of Responsive Reserve (RRS) and Fast-Responding Regulation Service (FRRS) without the potential stranding of power augmentation capability and the disparate treatment of Primary Frequency Response providers. |
| GDF Suez Energy Marketing NA 031113 | Revised NPRR to replace HRL with High Sustained Limit (HSL) as the threshold for requiring frequency responsiveness. |
| ERCOT 032013 | Expressed concern with NPRR as originally submitted. Proposed revisions to define “Frequency Responsive,” clarify that all RRS capacity must be Frequency Responsive, and require that the portion of Resource capacity that is available but is not Frequency Responsive be sent to ERCOT via ICCP telemetry. |

Proposed Protocol Language Revision

3.18	Resource Limits in Providing Ancillary Service

(1)	For both Generation Resources and Load Resources the High Sustained Limit (HSL) must be greater than or equal to the Low Sustained Limit (LSL) and the sum of the Resource-specific designation of capacity to provide Responsive Reserve (RRS), Regulation Up (Reg-Up), Regulation Down (Reg-Down), and Non-Spinning Reserve (Non-Spin).

(2)	For Non-Spin, the amount of Non-Spin provided must be less than or equal to the HSL for Off-Line Generation Resources.

(3)	For RRS Service:

(a)	The full amount of RRS provided from a Generation Resource must be less than or equal to 24% of thermal unit HSL for an Ancillary Service Offer, must be less than or equal to ten times the Emergency Ramp Rate;

(b)	RRS provided from a Generation Resource must be frequency responsive unless (i) the Generation Resource is operating at its High Reasonability Limit (HRL), or (ii) the Generation Resource is operating near its HRL and is producing supplemental capacity by various means of power augmentation.  If a Generation Resource is capable of producing supplemental capacity that is not always frequency responsive, a MW value reflecting the portion of the Generation Resource capacity that is not frequency responsive must be provided to ERCOT by Inter-Control Center Communications Protocol (ICCP) telemetry.

(c)	Hydro Generation Resources operating in the synchronous condenser fast-response mode may provide RRS up to the hydro Generation Resource’s proven 20-second response capability (which may be 100% of the HSL).  The initiation setting of the automatic under-frequency relay setting shall not be lower than 59.80 Hz;

(d)	For any hydro Generation Resource with a 5% droop setting operating as a generator, the amount of RRS provided may never be more than 24% of the HSL; and

(e)	The amount of RRS provided from a Load Resource must be less than or equal to the HSL minus the sum of the LSL, Reg-Up Resource Responsibility, Reg-Down Resource Responsibility, and Non-Spin Resource Responsibility.