| NPRR Number | 515 | NPRR Title | Day-Ahead Market Self-Commitment of Generation Resources |
| --- | --- | --- | --- |
| Date |  | April 4, 2013 |  |
| Submitter’s Information |  |  |  |
| Name |  | Carrie Bivens |  |
| E-mail Address |  | cbivens@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-6678 |  |
| Cell Number |  |  |  |
| Market Segment |  | Not applicable |  |

Comments

ERCOT submits these comments to provide clarification around the specific mechanism proposed in this Nodal Protocol Revision Request (NPRR), to address questions that have arisen through the different stakeholder meetings.

- For self-committed offers, the primary difference as compared to full Three-Part Supply Offers is that Resource constraints are ignored by the Day-Ahead Market (DAM) engine. All other attributes, such as Ancillary Services co-optimization (as designated with the inclusive/exclusive flag) remain.

- This NPRR does not add additional flexibility around Combined Cycle Generation Resources.  ERCOT explored the option of allowing one configuration to self-commit and another to offer up-transition costs in the same hour, but it was determined to be infeasible after discussions with our vendor.

- ERCOT systems presently allow self-commitment of Generation Resources in the (DAM) and we note that approximately 30 percent of the total MWs offered from Generation Resources is self-committed.  However, that self-commitment decision must currently be made for the whole Operating Day.  This NPRR proposes to change it to an hourly decision.