| NPRR Number | 514 | NPRR Title | Seasonal Generation Resource |  |
| --- | --- | --- | --- | --- |
| Timeline | Urgent | Action |  | Recommended Approval |
| Date of Decision |  | April 18, 2013 |  |  |
| Proposed Effective Date |  | To be determined. |  |  |
| Priority and Rank Assigned |  | To be determined. |  |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 3.9.1, Current Operating Plan (COP) Criteria 3.14.1.1, Notification of Suspension of Operations 3.14.1.2.1, ERCOT Evaluation of Seasonal Mothball Status (new) 3.14.1.9, Generation Resource Return to Service Updates 22, Attachment E, Notification of Suspension of Operations 22, Attachment H, Notification of Change of Generation Resource Designation |  |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) creates a new type of Generation Resource, a Seasonal Generation Resource, to allow Resource Entities to operate their Facilities during summer peak Load periods only without having to go through the existing mothballing process. This NPRR modifies the Protocols to allow a Resource Entity to designate that its Generation Resource will operate only on a seasonal basis (i.e., during the period June 1 to September 30). Additional language changes are provided to allow ERCOT to request the Seasonal Generation Resource to become available earlier than June 1 or remain available after September 30 if ERCOT determines that the Seasonal Generation Resource is needed for reliability reasons. |  |  |
| Reason for Revision |  | The Protocols define a Mothballed Generation Resource as “A Generation Resource for which a Generation Entity has submitted a Notification of Suspension of Operations, for which ERCOT has declined to execute an RMR Agreement, and for which the Generation Entity has not announced decommissioning and retirement of the Generation Resource.” Thus, the intent of “mothballed” status is to announce to the market that the particular Generation Resource is no longer economic to operate in the market and will likely retire unless economic conditions change. However, it is clear that the designation of “mothball” does not accurately reflect the status of a unit that intends to operate every summer but not be available at other times of the year. Because such seasonal operation may mean that the Resource would not be in operation for more than 180 days, the existing ERCOT process to “mothball” a Generation Resource comes into effect. That process requires substantial efforts on the part of both ERCOT and the Resource Entity to “mothball” then “un-mothball” the Resource when it returns to service. This NPRR creates a more efficient process by allowing the Seasonal Generation Resource to remain modeled in the ERCOT systems while it is not available for operation and allows the Seasonal Generation Resource to be shown as on Planned Outage in the Outage Scheduler during the period that it is not available for operation. To facilitate these changes, this NPRR adds a new checkbox on the Notice of Suspension of Operations form notifying ERCOT (and through the existing posting process, ERCOT Market Participants as well) that a Generation Resource owner plans to seasonally operate the Resource for a specific period of time. It also creates a new checkbox on the Notification of Change of Generation Resource Designation form to allow a Generation Resource to change from seasonal operation back to year-round operation if desired. This form is also revised to provide a Market Notice if the Generation Resource needs to modify its summer Operating Period (i.e., to start earlier than June 1 or to end later than September 30). The changes described in this NPRR will provide transparency to the market and simplify the process to allow Generation Resources to operate seasonally without all the complications of the existing mothball process. |  |  |
| Credit Impacts |  | To be determined. |  |  |
| Procedural History |  | - On 2/14/13, NPRR514 was posted. - On 2/21/13, PRS considered NPRR514. - On 3/18/13, ERCOT comments were posted. - On 3/20/12, Luminant Energy Company LLC comments were posted. - On 3/21/13, PRS again considered NPRR514. - On 4/15/13, WMS comments were posted. - On 4/18/13, PRS again considered NPRR514. |  |  |
| PRS Decision |  | On 2/21/13, PRS unanimously voted to table NPRR514 for one month. All Market Segments were present for the vote. On 3/21/13, PRS voted to table NPRR514 and refer the issue to WMS. There were two opposing votes from the Investor Owned Utility (IOU) and Independent Power Marketer (IPM) Market Segments and one abstention from the Consumer Market Segment. All Market Segments were present for the vote. On 4/18/13, PRS voted to grant Urgent status to NPRR514, to recommend approval of NPRR514 as amended by the 4/15/13 WMS comments, and to forward NPRR514 to TAC. There were two abstentions from the Consumer Market Segment. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 2/21/13, NPRR514 was tabled to allow additional time for ERCOT to review and analyze the NPRR. On 3/21/13, the 3/20/13 Luminant Energy Company LLC comments were reviewed. Participants stated that further discussion is needed to understand the logistical nature of ERCOT bringing a Seasonal Generation Resource back into operation and how this will be governed contractually; and the timelines regarding the notification period of the return to operation of a Seasonal Generation Resource. On 4/18/13, it was explained that the 4/15/13 WMS comments provided clarification that if ERCOT determines that a Resource operating under a Seasonal Operation Period needs to come On-Line outside of its Seasonal Operation Period, but has not reached mutual agreement with the Resource Entity or Qualified Scheduling Entity (QSE), ERCOT has the ability to bring that Resource into the market under a Reliability Must-Run (RMR) Agreement. It was also noted that no modifications were made to the timelines regarding the notification period for the return to operation of a Seasonal Generation Resource. |  |  |

| Business Case |  |  |
| --- | --- | --- |
| Business Case | 1 | Reduces time and resources necessary to allow a Generation Resource to operate seasonally in the ERCOT market. |
|  | 2 | Eliminates the complex process to “mothball” and “un-mothball” a Generation Resource in the ERCOT systems for a Resource that wants to operate seasonally. |
|  | 3 | Provides market transparency of all Generation Resources that plan to operate on a seasonal basis. |

| Sponsor |  |
| --- | --- |
| Name | Henry Durrwachter |
| E-mail Address | hdurrwachter@luminant.com |
| Company | Luminant Energy Company LLC |
| Phone Number | 214-875-9285 |
| Cell Number | 214-437-4622 |
| Market Segment | IOU |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Sandra Tindall |
| E-Mail Address | stindall@ercot.com |
| Phone Number | 512-248-3867 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 031813 | Proposed language to describe the evaluation process for Seasonal mothball status. |
| Luminant Energy Company LLC 032013 | Reinstated original language regarding returning Generation Resources to year-round operation. |
| WMS 041513 | Endorsed as amended by the 3/20/13 Luminant Energy Company LLC comments and as revised by WMS. |

Comments

Please note that the following NPRRs also propose revisions to the following Sections:

- NPRR429, HASL Offset Provision

  - Section 3.9.1

- NPRR522, Adjustment of Demand Response Performance for T&D Losses

  - Section 3.9.1

- NPRR532, Performance Measurement and Verification and Telemetry Requirements for Load Resources Providing Non-Spin

  - Section 3.9.1

Proposed Protocol Language Revision

## 2.1	Definitions

- Resource

- …

- Generation Resource

- …

Mothballed Generation Resource

A Generation Resource for which a Generation Entity has submitted a Notification of Suspension of Operations, for which ERCOT has declined to execute an RMR Agreement, and for which the Generation Entity has not announced decommissioning and retirement of the Generation Resource.

Split Generation Resource

Where a Generation Resource has been split to function as two or more independent Generation Resources in accordance with Section 10.3.2.1, Generation Resource Meter Splitting, and Section 3.10.7.2, Modeling of Resources and Transmission Loads, each such functionality independent Generation Resource is a Split Generation Resource.

- Seasonal Operation Period

The period in which a Generation Resource has identified it is available for operation.

3.9.1	Current Operating Plan (COP) Criteria

(1)	Each QSE that represents a Resource must submit a COP to ERCOT that reflects expected operating conditions for each Resource for each hour in the next seven Operating Days.

(2)	Each QSE that represents a Resource shall update its COP reflecting changes in availability of any Resource as soon as reasonably practicable, but in no event later than 60 minutes after the event that caused the change.

(3)	The Resource capacity in a QSE’s COP must be sufficient to supply the Ancillary Service Supply Responsibility of that QSE.

(4)	A COP must include the following for each Resource represented by the QSE:

(a)	The name of the Resource;

(b)	The expected Resource Status:

(i)	Select one of the following for Generation Resources synchronized to the ERCOT System that best describes the Resource’s status.  These Resource Statuses are to be used for COP and/or Real-Time telemetry purposes.

(A)	ONRUC – On-Line and the hour is a RUC-Committed Hour;

(B)	ONREG – On-Line Resource with Energy Offer Curve providing Regulation Service;

(C)	ON – On-Line Resource with Energy Offer Curve or Off-Line Quick Start Generation Resource (QSGR) available for Security-Constrained Economic Dispatch (SCED) Dispatch;

[NPRR272: Replace paragraph (4)(b)(i)(C) above with the following upon system implementation:] (C) ON – On-Line Resource with Energy Offer Curve;

(D)	ONDSR – On-Line Dynamically Scheduled Resource (DSR);

(E)	ONOS – On-Line Resource with Output Schedule;

(F)	ONOSREG – On-Line Resource with Output Schedule providing Regulation Service;

(G)	ONDSRREG – On-Line DSR providing Regulation Service;

(H)	ONTEST – On-Line blocked from SCED for operations testing  (while ONTEST, a Generation Resource may be shown on Outage in the Outage Scheduler);

(I)	ONEMR – On-Line EMR (available for commitment or dispatch only for ERCOT-declared Emergency Conditions; the QSE may appropriately set LSL and High Sustained Limit (HSL) to reflect operating limits); and

(J)	ONRR – On-Line as a synchronous condenser (hydro) providing Responsive Reserve (RRS) but unavailable for Dispatch by SCED and available for commitment by RUC;

[NPRR416: Insert paragraph (4)(b)(i)(K)below upon system implementation and renumber accordingly:] (K) ONOPTOUT – On-Line and the hour is a RUC Buy-Back Hour;

(K)	SHUTDOWN – The Resource is On-Line and in a shut-down sequence, and has no Ancillary Service Obligations.  This Resource Status is only to be used for Real-Time telemetry purposes;

(L)	STARTUP – The Resource is On-Line and in a start-up sequence and has no Ancillary Service Obligations.  This Resource Status is only to be used for Real-Time telemetry purposes;

[NPRR272: Insert paragraph (4)(b)(i)(M) upon system implementation:] (M) OFFQS – Off-Line but available for SCED deployment. Only qualified QSGRs may utilize this status.

(ii)	Select one of the following for Off-Line Generation Resources not synchronized to the ERCOT System that best describes the Resource’s status.  These Resource Statuses are to be used for COP and/or Real-Time telemetry purposes.

(A)	OUT – Off-Line and unavailable;

(B)	OFFNS – Off-Line but reserved for Non-Spinning Reserve (Non-Spin);

(C)	OFF – Off-Line but available for commitment in the Day-Ahead Market (DAM) and RUC; and

(D)	EMR – Available for commitment only for ERCOT-declared Emergency Condition events; the QSE may appropriately set LSL and HSL to reflect operating limits; and

(iii)	Select one of the following for Load Resources.  These Resource Statuses are to be used for COP and/or Real-Time telemetry purposes.

(A)	ONRGL – Available for Dispatch of Regulation Service;

(B)	ONRRCLR – Available for Dispatch of RRS Service as a Controllable Load Resource;

(C)	ONRL – Available for Dispatch of RRS Service or Non-Spin, excluding Controllable Load Resources; and

(D)	OUTL – Not available;

(c)	The HSL;

(d)	The LSL;

(e)	The High Emergency Limit (HEL);

(f)	The Low Emergency Limit (LEL); and

(g)	Ancillary Service Resource Responsibility capacity in MW for:

(i)	Regulation Up (Reg-Up);

(ii)	Regulation Down (Reg-Down);

(iii)	RRS Service; and

(iv)	Non-Spin.

(5)	For Combined Cycle Generation Resources, the above items are required for each operating configuration.  In each hour only one Combined Cycle Generation Resource in a Combined Cycle Train may be assigned one of the On-Line Resource Status codes described above.

(a)	During a RUC study period, if a QSE’s COP reports multiple Combined Cycle Generation Resources in a Combined Cycle Train to be On-Line for any hour, then until the QSE corrects its COP, the On-Line Combined Cycle Generation Resource with the largest HSL is considered to be On-Line and all other Combined Cycle Generation Resources in the Combined Cycle Train are considered to be Off-Line.  Furthermore, until the QSE corrects its COP, the Off-Line Combined Cycle Generation Resources as designated through the application of this process are ineligible for RUC commitment or de-commitment Dispatch Instructions.

(b)	For any hour in which QSE-submitted COP entries are used to determine the initial state of a Combined Cycle Generation Resource for a DAM or Day-Ahead Reliability Unit Commitment (DRUC) study and the COP shows multiple Combined-Cycle Generation Resources in a Combined Cycle Train to be in an On-line Resource Status, then until the QSE corrects its COP, the On-Line Combined Cycle Generation Resource that has been On-Line for the longest time from the last recorded start by ERCOT systems, regardless of the reason for the start, combined with the COP Resource Status for the remaining hours of the current Operating Day, is considered to be On-Line at the start of the DRUC study period and all other COP-designated Combined Cycle Generation Resources in the Combined Cycle Train are considered to be Off-Line.

(c)	ERCOT systems shall allow only one Combined Cycle Generation Resource in a Combined Cycle Train to offer Off-Line Non-Spin in the DAM or Supplemental Ancillary Services Market (SASM).

(i)	If there are multiple Non-Spin offers from different Combined Cycle Generation Resources in a Combined Cycle Train, then prior to execution of the DAM, ERCOT shall select the Non-Spin offer from the Combined Cycle Generation Resource with the highest HSL for consideration in the DAM and ignore the other offers.

(ii)	Combined Cycle Generation Resources offering Off-Line Non-Spin must be able to transition from the shutdown state to the offered Combined Cycle Generation Resource On-Line state and be capable of ramping to the full amount of the Non-Spin offered.

(d)	The DAM and RUC shall honor the registered hot, intermediate or cold Startup Costs for each Combined Cycle Generation Resource registered in a Combined Cycle Train when determining the transition costs for a Combined Cycle Generation Resource.  In the DAM and RUC, the Startup Cost for a Combined Cycle Generation Resource shall be determined by the positive transition cost from the On-Line Combined Cycle Generation Resource within the Combine Cycle Train or from a shutdown condition, whichever ERCOT determines to be appropriate.

(6)	ERCOT may accept COPs only from QSEs.

(7)	For the first 48 hours of the COP, a QSE representing a Wind-powered Generation Resource (WGR) must enter an HSL value that is less than or equal to the amount for that Resource from the most recent Short-Term Wind Power Forecast (STWPF) provided by ERCOT.

(8)	For hours 49 to 168 of the COP, a QSE representing a WGR shall enter an HSL value equal to their best estimate, which may be based on the wind power profile as published on the ERCOT website.

(9)	A QSE representing a Generation Resource that is not actively providing Ancillary Services may only use a Resource Status of SHUTDOWN to indicate to ERCOT through telemetry that the Resource is operating in a shut-down sequence or a Resource Status of ONTEST to indicate in the COP and through telemetry that the Generation Resource is performing a test of its operations either manually dispatched by the QSE or by ERCOT as part of the test.  A QSE representing a Generation Resource that is not actively providing Ancillary Services may only use a Resource Status of STARTUP to indicate to ERCOT through telemetry that the Resource is operating in a start-up sequence requiring manual control and is not available for Dispatch.

(10)	If a QSE has not  submitted a valid COP for any Generation Resource for any hour in the DAM or RUC Study Period, then the Generation Resource is considered to have a Resource Status as OUT thus not available for DAM awards or RUC commitments for those hours.

(11)	If a COP is not available for any Resource for any hour from the current hour to the start  of the DAM period or RUC study, then the Resource Status for those hours are considered equal to the last known Resource Status from a previous hour’s COP or from telemetry as appropriate for that Resource.

- 3.14.1.1	Notification of Suspension of Operations

(1)	Except for the occurrence of a Forced Outage, a Resource Entity must notify ERCOT in writing no less than 90 days prior to the date on which the Resource Entity intends to:

(a)	Cease or suspend operation of a Generation Resource for a period of greater than 180 days (i.e., becomes a Mothballed or Decommissioned Generation Resource); or

(b)	Mothball the Generation Resource on a seasonal basis and identify its Seasonal Operation Period.

(2)	In either paragraphs (1)(a) or (1)(b) above, the Resource Entity shall submit a completed Part I and Part II of the Notification of Suspension of Operations (found in Section 22, Attachment E, Notification of Suspension of Operations).  The Resource Entity may also complete Part III of the Notification and submit it along with Parts I and II, or may wait to submit Part III until ERCOT makes an initial determination of the need for the Generation Resource as an RMR Unit.  The Part I Notification must include the attestation of an officer of the Resource Entity that the Generation Resource is uneconomic to remain in service as currently designated and will be unavailable for Dispatch by ERCOT for a period specified in the Notification.

(3)	A Resource Entity ceasing or suspending operations as a result of a Forced Outage lasting greater than 180 days shall notify ERCOT as soon as practicable.  A Notification of Suspension of Operations submitted due to a Forced Outage will not be evaluated for RMR status and will not be posted on the MIS.

(4)	At least 60 days before the expiration of an existing RMR Agreement, the Resource Entity may apply to renew the RMR Agreement by submitting a new Notification (including both Part I and Part II).  Upon receipt of such a renewal request, ERCOT shall update and post to the MIS Secure Area, studies as set forth in Section 3.14.1, Reliability Must Run, within 15 Business Days.

3.14.1.2.1	ERCOT Evaluation of Seasonal Mothball Status

(1)	ERCOT shall evaluate requests to place Generation Resources on a seasonal mothball status pursuant to the guidelines provided in Section 3.14.1.2, ERCOT Evaluation, except as stated below.

(2)	Within 30 days after receiving the Notification of Suspension of Operations described in Section 3.14.1.1, Notification of Suspension of Operations, ERCOT shall make an initial assessment of whether the Generation Resource is required to support ERCOT System reliability during the portion of the year when the Generation Resource would be unavailable.

(a)	For the purpose of studying a Generating Resource operating under a Seasonal Operation Period, ERCOT shall use the Load value provided by the appropriate TSP as part of the annual SSWG study case development process for Load in the seasonal Generation Resource local area.  For the rest of the system, ERCOT shall use the weekly peak Load forecast between October 1 and May 31, as applicable, seasonally.  The forecast data is posted pursuant to P.U.C. Subst. R. 25.505, Resource Adequacy in the Electric Reliability Council of Texas Power Region.

(3)	Within 60 days after receiving the Notification of Suspension of Operations ERCOT shall make a final assessment of whether the Generation Resource is required to support ERCOT System reliability during the portion of the year when the Generation Resource would be unavailable.

- 3.14.1.9	Generation Resource Return to Service Updates

(1)	By April 1st and October 1st of each year and when material changes occur, every Resource Entity that owns or controls a Mothballed Generation Resource or an RMR Unit shall report to ERCOT, on a unit-specific basis, the estimated lead time required for each Resource to be capable of returning to service and, in percentage terms, report probable generation capacity from each Resource that the Resource Entity expects to return to service in each Season of each of the next ten years.

(2)	For modeling purposes, ERCOT and TSPs shall rely on the most recent submittal of the following two Notifications with respect to an RMR Unit, Mothballed Generation Resource or Decommissioned Generation Resource: Section 22, Attachment E, Notification of Suspension of Operations, or Section 22, Attachment H, Notification of Change of Generation Resource Designation.

(3)	A Mothballed Generation Resource that has identified a return to service date in Section 22, Attachment E, shall remain modeled in all ERCOT systems at all times and, when it is not available, shall be shown on Outage in the Outage Scheduler.

(4)	Unless the Resource Entity has identified a return to service date in its Notification of Suspension of Operations, a Resource Entity with a Mothballed Generation Resource shall notify ERCOT in writing no less than 30 days prior to the date on which the Resource Entity intends to return a Mothballed Generation Resource to service by completing a Notification of Change of Generation Resource Designation.  ERCOT shall post the Notification of Change of Generation Resource Designation on the MIS Secure Area as soon as practicable, but no later than five Business Days after receipt.

(5)	If a Resource Entity wishes to change the operational designation of a Generation Resource upon conclusion of an RMR Agreement, it must submit a Notification of Change of Generation Resource Designation no later than 60 days prior to the conclusion of the RMR Agreement.  ERCOT shall post on the MIS Secure Area information relating to a change of the operational designation of a Generation Resource pending conclusion of an RMR Agreement as soon as practicable but no later than five Business Days after receipt of such information.

(6)	A Resource Entity with a Generation Resource that operates under a Seasonal Operation Period shall notify ERCOT in writing no less than 15 days prior to the date on which the Resource Entity intends to begin its Seasonal Operation Period if the first date of operation is prior to June 1.  A Resource Entity with a Generation Resource that operates under a Seasonal Operation Period shall notify ERCOT in writing by September 15 if the Resource Entity intends to suspend operation later than September 30.  Notifications under this section shall be provided by the Resource Entity by completing a Notification of Change of Generation Resource Designation form (Section 22, Attachment H) which.  ERCOT shall post on the MIS Secure Area as soon as practicable, but no later than five Business Days after receipt.

(7)	Once the Resource Entity notifies ERCOT that a Generation Resource is operating under a Seasonal Operation Period, the Resource Entity does not need to annually notify ERCOT of such status.

(8)	A Resource Entity with a Generation Resource operating under a Seasonal Operation Period shall notify ERCOT in writing no less than 15 days prior to the date on which the Resource Entity intends to return the Generation Resource to year-round operation by completing a Notification of Change of Generation Resource Designation form (Section 22, Attachment H), which ERCOT shall post on the MIS Secure Area as soon as practicable, but no later than five Business Days after receipt.

(9)	A Resource Entity with a Generation Resource that operates under a Seasonal Operation Period must notify ERCOT in writing, by submitting Section 22, Attachment E, no less than 90 days before the date on which the Generation Resource is to be suspended indefinitely (i.e., becomes a Mothballed Generation Resource or Decommissioned Generation Resource);

(10)	ERCOT may request that a Generation Resource operating under a Seasonal Operation Period be available for operation earlier than June 1 or later than September 30 of any given calendar year.  If ERCOT identifies a specific Resource Entity or QSE with which it will discuss such a request in an attempt to reach a mutually agreeable resolution, ERCOT shall issue a Notice as soon as practicable.  The Notice shall include the Resource name and, as applicable, the Resource mnemonic, the Resource MW Rating by Season, and the potential duration of the extended operation period, including anticipated start and end dates.  If agreement is reached for the Generation Resource to be available for operation earlier than June 1 or later than September 30, the Resource Entity shall complete, within two Business Days, a Notification of Change of Generation Resource Designation form (Section 22, Attachment H), which ERCOT shall post on the MIS Secure Area as soon as practicable, but no later than five Business Days after receipt.

(11)	If ERCOT and the Resource Entity or QSE cannot reach a mutual agreement to make the Resource operating under a Seasonal Operation Period available earlier than June 1 or later than September 30 of any given calendar year, then ERCOT may exercise its ability to bring the Resource operating under a Seasonal Operating Period into the market under an RMR Agreement pursuant to sub paragraph (2) of Section 6.5.1.1, ERCOT Control Area Authority.

(12)	ERCOT may evaluate, on an annual basis, Generation Resources operating under a Seasonal Operation Period for RMR Service to address ERCOT System reliability during the portion of the year when the Generation Resource would be unavailable.

(13)	A Resource Entity that submitted a Notification of Suspension of Operations as a result of a Forced Outage must notify ERCOT of its intent to return to service as soon as practicable by updating its status in the Outage Scheduler and Current Operating Plan (COP) and is not required to submit a Notification of Change of Generation Resource Designation.

(14)	If a Generation Resource is designated as decommissioned and retired under either Notification, ERCOT will permanently remove the Generation Resource from the ERCOT registration systems in accordance with Section 3.10.1, Time Line for Network Operations Model Changes.  If a Resource Entity decides to bring a Decommissioned Generation Resource back to service at a later date, it will be considered a new Resource and must follow the Generation Resource Interconnection or Change Request process detailed in the Planning Guide.  If the Generation Resource is designated as mothballed, ERCOT and TSPs will consider the Generation Resource mothballed until the Resource Entity indicates a definitive return to service date pursuant to this Section.

ERCOT Nodal Protocols

Section 22

Attachment E:  Notification of Suspension of Operations

- January 1, 2012

Notification of Suspension of Operations of a Generation Resource

This Notification is required for providing notification of any Generation Resource suspension lasting greater than 180 days.  Information may be inserted electronically to expand the reply spaces as necessary.

The Notification must be signed, notarized and delivered to ERCOT.  Delivery may be accomplished via email to mpappl@ercot.com (if a scanned copy) or via facsimile (Attention: Market Participant Registration) at (512) 225-7079.

ERCOT may request additional information as reasonably necessary to support operations under the ERCOT Protocols.

Part I:

Resource Entity:

DUNS Number:

Resource Site Name:

Resource Site Location (County):

Unit Name(s):

Resource Name(s) (Unit Code/Mnemonic):

ESI ID:

Seasonal Net Max Sustainable Rating – Summer (MW):

Seasonal Net Minimum Sustainable Rating – Summer (MW):

Part II:

As of ___________ [date], the Generation Resource(s) will be limited or unavailable for Dispatch by ERCOT because Resource Entity will [check one]:

decommission and retire the Generation Resource(s) permanently,

suspend operation on a year-round basis and begin operation on a seasonal basis on___________ and ending on _________________ [date], ,

suspend operation (i.e., mothball) of the Generation Resource(s) for a period of not less than _________ months and not greater than ______ months, or

suspend operation (i.e., mothball) of the Generation Resource(s) indefinitely, or

suspend operation of the Generation Resource(s) due to a Forced Outage.  Resource Entity intends to bring the Generation Resource(s) back to service on ___________ [date].

Unless the Generation Resource(s) will be decommissioned and retired the estimated time to return the suspended Generation Resource(s) to service is _________ months.

Check if applicable:   Resource Entity believes that this Generation Resource(s) is inoperable due to emissions limitations or not being repairable.

Operational and Environmental Limitations (check and describe all that apply):

(a)	Operational:

Maximum annual hours of operation: _________________

Maximum annual MWhs: __________________________

Maximum annual starts: ___________________________

Other: __________________________________________

(b)	Environmental:

Maximum annual NOx emissions: ____________________

Maximum annual SO2 emissions: ____________________

Other: __________________________________________

Part III:

Proposed RMR Energy Price ($/MMBtu): ____________

Proposed Standby Cost ($/hr): ____________

I understand and agree that this Notification is not confidential and does not constitute Protected Information under the ERCOT Protocols.  This Notification is not intended to constitute an offer to enter into a binding Agreement, but is intended only as an offer to negotiate the terms of such an Agreement, in accordance with the ERCOT Protocols.

Unless the above Generation Resource(s) is inoperable due to emissions limitations or not being repairable, I certify that Resource Entity is willing to consider entering into an RMR Agreement for the Generation Resource(s).

The undersigned certifies that I am an officer of Resource Entity, that I am authorized to execute and submit this Notification on behalf of Resource Entity, and that the statements contained herein are true and correct.

Name:

Title:

Date:

STATE OF _______________

COUNTY OF _____________

Before me, the undersigned authority, this day appeared ___________________, known by me to be the person whose name is subscribed to the foregoing instrument, who, after first being sworn by me deposed and said:

“I am an officer of ______________, I am authorized to execute and submit the foregoing Notification on behalf of ______________, and the statements contained in such Notification are true and correct.”

SWORN TO AND SUBSCRIBED TO BEFORE ME, the undersigned authority on this the _____ day of ____________, 20__.

______________________________

Notary Public, State of ___________

My Commission expires __________

ERCOT Nodal Protocols

Section 22

Attachment H:  Notification of Change of Generation Resource Designation

- October 1, 2011

Notification of Change of Generation Resource Designation

This Notification is for changing a Generation Resource designation in accordance with the ERCOT Protocols.  Information may be inserted electronically to expand the reply spaces as necessary.

The Notification must be signed, notarized and delivered to ERCOT.  Delivery may be accomplished via email to mpappl@ercot.com (if a scanned copy) or via facsimile (Attention: Market Participant Registration) at (512) 225-7079. ERCOT may request additional information as reasonably necessary to support operations under the ERCOT Protocols.

Resource Entity: 			_______

DUNS No.: ____________________________________________________________________

Generation Resource(s) [plant and unit number(s)] 			______

Generation Resource(s) is currently [check one]

under a Reliability Must-Run (RMR) Agreement

seasonally mothballed

mothballed

As of ___________ [date], Resource Entity will change the Generation Resource(s) designation to [check one]

operational (for a Generation Resource operating seasonally, selecting this option means that the Generation Resource is returning to year round service)

Mothballed (a Generation Resource operating seasonally may not select this option, and must instead use the Section 22, Attachment E, Notification of Suspension of Operation form to be designated as mothballed)

Decommissioned and retired permanently (a Generation Resource operating seasonally may not select this option and must instead use the form in Section 22, Attachment E to be designated as decommissioned)

Generation Resource operating on a seasonal basis updating start date or end date of Seasonal Operation Period

As of ___________ [date], a Generation Resource will change its Seasonal Operation Period as follows:

change start date of Seasonal Operation Period from June 1, ____ to ________

change end date of Seasonal Operation Period from September 30, ____  to _____

The undersigned certifies that I am an officer of Resource Entity, that I am authorized to execute and submit this Notification on behalf of Resource Entity, and that the statements contained herein are true and correct.

Name:

Title:

Date:

STATE OF _______________

COUNTY OF _____________

Before me, the undersigned authority, this day appeared ___________________, known by me to be the person whose name is subscribed to the foregoing instrument, who, after first being sworn by me deposed and said:

“I am an officer of ______________, I am authorized to execute and submit the foregoing Notification on behalf of ______________, and the statements contained in such Notification are true and correct.”

SWORN TO AND SUBSCRIBED TO BEFORE ME, the undersigned authority on this the _____ day of ____________, 20__.

______________________________

Notary Public, State of ___________

My Commission expires __

Pursuant to Protocol Section 3.14.1.1, Notification of Suspension of Operations, this date must be at least 90 days from the date ERCOT receives this Notification, unless the suspension is the result of a Forced Outage, in which case the Generation Resource shall submit this Notification as soon as practicable.

ERCOT will remove the Generation Resource(s) from its registration systems if this option is selected.

A Generation Resource requesting seasonal Mothball status must be available June 1 through September 30.

ERCOT will remove the Generation Resource(s) from its registration systems if this option is selected, , in accordance with Section 3.14.1.9, Generation Resource Return to Service Updates, upon Resource Entity updating Resource registration accordingly.