| NPRR Number | 529 | NPRR Title | Congestion Management Plan |
| --- | --- | --- | --- |
| Date |  | June 27, 2013 |  |
| Submitter’s Information |  |  |  |
| Name |  | Amanda J. Frazier |  |
| E-mail Address |  | amanda.frazier@luminant.com |  |
| Company |  | Luminant Energy Company LLC |  |
| Phone Number |  | 512.349.6442 |  |
| Cell Number |  | 512.364.3275 |  |
| Market Segment |  | Investor Owned Utility (IOU) |  |

Comments

Luminant Energy Company LLC (“Luminant”) provides the comments below in support of ERCOT’s efforts to consolidate current Protocol and Operating Guide language regarding its use of congestion management plans, and the comments below are revisions to ERCOT’s comments of May 15, 2013.

The current Protocols and Operating Guides make several references to the use of congestion management plans for “market” purposes, and Luminant agrees that congestion management plans should continue to be used, when appropriate, to provide full deliverability of generation to customers.  These comments address those instances where Luminant believes the proposed language limits the flexibility that ERCOT enjoys under the current rules to manage the transmission grid efficiently.

The term “Congestion Management Plan” is newly defined in this NPRR, and attempts to summarize the types of action plans that ERCOT currently may use to address system conditions that cause or could lead to transmission security violations.  In Luminant’s view, the definition proposed in ERCOT’s May 15 comments could be interpreted to imply that these action plans can only be used where the Security Constrained Economic Dispatch cannot (either based on a study or in practice) resolve a transmission security violation.  Such an interpretation would significantly restrict the circumstances in which the current rules, and ERCOT’s practice, allow the use of Congestion Management Plans today.  Therefore, Luminant proposes to reinstate current Protocol and Operating Guide language (see current Protocols § 2 Definition of Remedial Action Plan, and § 3.1.6.5(1)(b),and Operating Guides § 4.3.1(1) and § 4.3.2) to more accurately describe Congestion Management Plans, and to ensure that ERCOT retains its current flexibility to use Congestion Management Plans where appropriate.

Additionally, Luminant has proposed to further distinguish Mitigation Plans from other types of Congestion Management Plans, by defining them as a “complex” set of more than three actions or actions that include load shedding.  This suggested edit is based on Luminant’s observation of ERCOT’s historical practice, in order clearly to distinguish a Mitigation Plan from a Remedial Action Plan.

Revised Proposed Protocol Language

2.1	Definitions

Congestion Management Plan (CMP)

A set of pre-defined actions executed in response to system conditions to prevent or resolve one or more thermal or non-thermal transmission security violations.    ERCOT will employ CMPs to facilitate the market use of the ERCOT Transmission Grid while maintaining system security and reliability in accordance with ERCOT and NERC Reliability Standards.  CMPs include Mitigation Plans, Remedial Action Plans (RAPs), Pre-Contingency Action Plans (PCAPs) and Temporary Outage Action Plans (TOAPs).

Mitigation Plan

A complex set of pre-defined actions (i.e., more than three actions, or includes Load shedding) to execute post-contingency in order to reduce overloading on one or more given, monitored Transmission Facilities to below their Emergency Rating.  A Mitigation Plan must be implementable and may include transmission switching and Load shedding.

Pre-Contingency Action Plan (PCAP)

A set of pre-defined actions to execute pre-contingency in order to reduce overloading on one or more given, monitored Transmission Facilities to below their Emergency Rating.  A PCAP may include transmission switching and does not include Load shedding. These plans may be relied upon in allowing additional market use of the transmission system.

Remedial Action Plan (RAP)

A set of pre-defined actions to execute post-contingency in order to reduce loading on one or more given, monitored Transmission Facilities to below their Emergency Rating defined in the Network Operations Model.  RAPs are sufficiently dependable to assume they can be executed within 15 minutes on a post-contingency basis without loss of reliability to the interconnected network, and do not include Load shedding.  These plans may be relied upon in allowing additional market use of the transmission system.

Temporary Outage Action Plan (TOAP)

A temporary set of pre-defined actions to execute post-contingency, during a specified Transmission Facility or Resource Outage, in order to reduce overloading on one or more given, monitored Transmission Facilities to below their Emergency Rating.  A TOAP must be implementable and may include transmission switching and/or Load shedding. These plans may be relied upon in allowing additional market use of the transmission system.

## ACRONYMS AND ABBREVIATIONS

CMP		Congestion Management Plan

PCAP		Pre-Contingency Action Plan

TOAP		Temporary Outage Action Plan

- 3.1.6.5	Evaluation of Proposed Resource Outage

(1)	If a proposed Resource Outage, in conjunction with previously accepted Outages, would cause a violation of applicable reliability standards, ERCOT shall:

(a)	Communicate with the requesting Qualified Scheduling Entity(ies) as required under Section 3.1.6.8, Resource Outage Rejection Notice;

(b)	Investigate possible Congestion Management Plans (CMPs) to resolve security violations, based upon security and reliability analysis results and strive to maximize transmission usage consistent with reliable operation; and

(c)	Consider modifying the previous acceptance or approval of one or more Transmission Facilities or reliability Resource Outages, considering order of receipt and impact to the ERCOT System.

(2)	If transmission security can be maintained using an alternative considered in item (1)(b) and (1)(c), then ERCOT, may, in its judgment, direct the selected alternatives and approve the proposed Resource Outage.

(3)	If ERCOT does not resolve transmission security issues by using the alternatives considered in item (1)(b) and (1)(c), then ERCOT shall reject the proposed Resource Outage.

- 3.10.7.4	Modeling of Special Protection Systems and Remedial Action Plans

(1)	All Special Protection Systems (SPSs) and Remedial Action Plans (RAPs) used by ERCOT and the TSPs to maintain a secure system must be defined in the Network Operations Model.

(2)	Proposed new SPSs and RAPs and proposed changes to SPSs and RAPs must be submitted to ERCOT for review and approval by ERCOT and all directly affected TSPs and Resource Entities under the applicable procedures in the Operating Guides.  Once a new or changed SPS or RAP is approved by ERCOT and all directly affected TSPs and Resource Entities, the TSP shall submit the approved SPS or RAP to ERCOT using an NOMCR.  The NOMCR must include a detailed description of the system conditions required to implement the SPS or RAP.  Execution of an SPS or RAP must be included or assumed in the calculation of LMPs as well as the Network Operations Model.   ERCOT shall post all SPSs and RAPs under consideration on the MIS Secure Area within five Business Days of receipt by ERCOT.

(3)	ERCOT shall model, and include in the security analysis, approved SPSs and RAPs. ERCOT shall post on the MIS Secure Area all approved SPSs and RAPs at least two Business Days before implementation, identifying the date of implementation.

- 6.5.7.1.10	Network Security Analysis Processor and Security Violation Alarm

(1)	Using the input provided by the State Estimator, ERCOT shall use the NSA processor to perform analysis of all contingencies in the active list.  For each contingency, ERCOT shall use the NSA processor to monitor the elements for limit violations.  ERCOT shall use the NSA processor to verify Electrical Bus voltage limits to be within a percentage tolerance as outlined in the ERCOT Operating Guides.  Contingency security violations for transmission lines and transformers occur if:

(a)	The predicted post-contingency MVA exceeds 100% of the Emergency Rating after adjustments for Real-Time weather conditions applicable to the contingency are incorporated; and

(b)	No defined  RAP or SPS exists that could provide  relief within the time allowed by the security criteria.

(2)	When the NSA processor notifies ERCOT of a security violation, ERCOT shall immediately initiate the process described in Section 6.5.7.1.11, Transmission Network and Power Balance Constraint Management.

[NPRR393: Replace paragraph (2) above with the following upon system implementation:] (2) When the NSA processor notifies ERCOT of a security violation, ERCOT shall immediately: (a) Initiate the process described in Section 6.5.7.1.11, Transmission Network and Power Balance Constraint Management; (b) Seek to determine what unforeseen change in system condition has arisen that has resulted in the security violation, especially those that were 125% or greater of the Emergency Rating for a single SCED interval or greater than 100% of the Emergency Rating for a duration of 30 minutes or more; and (c) Where possible, seek to reverse the action (e.g. initiating a transmission clearance that the system was not properly pre-dispatched for) that has led to a security violation until further preventative action(s) can be taken.

(3)	If the SCED does not resolve a transmission security violation, ERCOT shall attempt to relieve the violation by:

(a)	Confirming that pre-determined RAPs are properly modeled in the system;

(b)	Requesting that Resources follow Base Points from SCED if those Resources are not already doing so;

(c)	Requesting that Resources update the Resources Status in the Current Operating Plan (COP) from ONTEST to ON in order provide more capacity to SCED;

(d)	Requesting that Resources Redistribute Ancillary Services to provide capacity to SCED;

(e)	Deploying Resource-Specific Non-Spinning Reserve Service;

(f)	Committing additional Generation Resources through the Reliability Unit Commitment process;

(g)	Removing conflicting non-cascading constraints from the SCED process;

(h)	Re-Dispatching generation by over-riding High Dispatch Limits (HDLs) and Low Dispatch Limits (LDLs);

(i)	Instructing TSPs to utilize Reactive Power devices to manage voltage; and

(j)

If all other mechanisms have failed, ERCOT may authorize the expedited use of a Mitigation Plan .

(4)	NSA must be capable of analyzing contingencies, including the effects of  SPSs and RAPs.  The NSA must fully integrate the evaluation and deployment of SPSs and RAPs and notify the ERCOT Operator of the application of these SPSs and RAPs to the solution.

(5)	The Real-Time NSA may employ the use of appropriate ranking and other screening techniques to further reduce computation time by executing one or two iterations of the contingency study to gauge its impact and discard further study if the estimated result is inconsequential.

[NPRR393: Insert paragraph (6) below upon system implementation:] (6) ERCOT shall report monthly: (a) All security violations that were 125% or greater of the Emergency Rating for a single SCED interval or greater than 100% of the Emergency Rating for a duration of 30 minutes or more during the prior reporting month and the number of occurrences and congestion cost associated with each of the constraints causing the security violations on a rolling 12 month basis. (b) Operating conditions on the ERCOT System that contributed to each security violation reported in paragraph (6)(a) above. Analysis should be made to understand the root cause and what steps could be taken to avoid a recurrence in the future.