| NPRR Number | 538 | NPRR Title | Clarification of the Non-Spin Energy Offer Curve Requirements for QSGRs |  |
| --- | --- | --- | --- | --- |
| Timeline | Normal | Action |  | Recommended Approval |
| Date of Decision |  | July 2, 2013 |  |  |
| Proposed Effective Date |  | October 1, 2013 |  |  |
| Priority and Rank Assigned |  | Not applicable. |  |  |
| Nodal Protocol Section Requiring Revision |  | 6.4.3.2, Energy Offer Curve for Non-Spinning Reserve Capacity |  |  |
| Market Guide Section(s) Requiring Revision |  | None. |  |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) clarifies that the $120 per MWh Energy Offer Curve floor for Non-Spinning Reserve (Non-Spin) capacity is the floor that is applicable to Quick Start Generation Resources (QSGRs) that are providing Non-Spin and are available for deployment by Security-Constrained Economic Dispatch (SCED) pursuant to Section 3.8.3, Quick Start Generation Resources. |  |  |
| Reason for Revision |  | While it was the intent of NPRR428, Energy Offer Curve Requirements for Generation Resources Assigned Non-Spin Responsibility, that the $120 per MWh Energy Offer Curve floor for Non-Spin capacity be applicable to QSGRs that are made available for deployment by SCED, concerns were raised by Market Participants that this was not clearly stated within the Protocols. This NPRR remedies that concern. |  |  |
| Credit Impacts |  | ERCOT Credit Staff and the Credit Work Group (Credit WG) have reviewed NPRR538 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |  |  |
| Procedural History |  | - On 4/25/13, NPRR538 and an Impact Analysis were posted. - On 5/16/13, PRS considered NPRR538. - On 6/20/13, PRS considered the 5/16/13 PRS Report and Impact Analysis for NPRR538. - On 7/2/13, TAC considered NPRR538. |  |  |
| PRS Decision |  | On 5/16/13, PRS voted to recommend approval of NPRR538 as submitted. There was one abstention from the Consumer Market Segment. All Market Segments were present for the vote. On 6/20/13, PRS unanimously voted to endorse and forward the 5/16/13 PRS Report as revised by PRS and Impact Analysis for NPRR538 to TAC. All Market Segments were present for the vote. |  |  |
| Summary of PRS Discussion |  | On 5/16/13, there was no discussion. On 6/20/13, a minor edit was proposed to clarify that only the pricing criteria set forth in paragraph (a) of Section 6.4.3.2 applies to Non-Spin provided by power augmentation and QSGRs that are available for deployment by SCED. |  |  |
| TAC Decision |  | On 7/2/13, TAC unanimously voted to recommend approval of NPRR538 as recommended by PRS in the 6/20/13 PRS Report. All Market Segments were present for the vote. |  |  |
| Summary of TAC Discussion |  | On 7/2/13, there was no discussion. |  |  |
| ERCOT Opinion |  | ERCOT supports approval of NPRR538. |  |  |

| Business Case |  |
| --- | --- |
| 1 | - This NPRR clarifies the existing rules and expectations for QSGRs providing Non-Spin Service. |
| 2 | - As this NPRR solely clarifies existing rules, there should be no impacts to any Market Segments or ERCOT. |

| Sponsor |  |
| --- | --- |
| Name | David Maggio |
| E-mail Address | dmaggio@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-6998 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Jonathan Levine |
| E-Mail Address | jlevine@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| None. |  |

Comments

Please note that NPRR544, Energy Offer Curve Requirements for Generation Resources Assigned Non-Spin Responsibility, also proposes revisions to Section 6.4.3.2.

Proposed Protocol Language Revision

- 6.4.3.2	Energy Offer Curve for Non-Spinning Reserve Capacity

The following applies to Generation Resources that a QSE assigns Non-Spinning Reserve (Non-Spin) Ancillary Service Resource Responsibility in its Current Operating Plan (COP) to meet the QSE’s Ancillary Service Supply Responsibility for Non-Spin and includes both On-Line and Off-Line Non-Spin assignments arising as the result of DAM or Supplemental Ancillary Services Market (SASM) Ancillary Service awards, or Self-Arranged Ancillary Service Quantity.

(a)	Prior to the end of the Adjustment Period for an Operating Hour during which a Generation Resource is assigned Non-Spin Ancillary Service Resource Responsibility, the QSE shall ensure that a valid Output Schedule or Energy Offer Curve for the Operating Hour has been submitted and accepted by ERCOT.  The Energy Offer Curves submitted by the QSE for these Generation Resources must meet the following requirements:

(i)	If the Generation Resource is providing On-Line Non-Spin capacity, the energy from this capacity may not be offered at less than $120 per MWh; or

(ii)	If the Generation Resource is providing Off-Line Non-Spin capacity, the energy from this capacity may not be offered at less than $180 per MWh.

(b)	Paragraph (a) above shall not restrict the ability of the QSE to update the Energy Offer Curves for its Generation Resources in accordance with these Protocols provided that in those Operating Hours in which a Generation Resource is assigned Non-Spin Ancillary Service Resource Responsibility, the portion of the Energy Offer Curve reserved for this Ancillary Service meets those requirements.

(c)	If, in accordance with an ERCOT Dispatch Instruction or in accordance with Section 6.4.6, QSE-Requested Decommitment of Resources and Changes to Ancillary Service Resource Responsibility of Resources, the QSE moves Non-Spin Ancillary Service Resource Responsibility from a Generation Resource that experiences a Forced Outage, Derate or Startup Loading Failure during the Operating Period described in paragraph (a) above, the QSE shall ensure that a valid Output Schedule or Energy Offer Curve that meets the requirements of this Section has been submitted as soon as reasonably practicable after the event.

(d)	For the portion of the Non-Spin Ancillary Service Resource Responsibility provided from power augmentation participating as Off-Line, the pricing criteria described in paragraph (a)(ii) above applies.

(e)	For the Non-Spin Ancillary Service Resource Responsibility provided from Quick Start Generation Resources (QSGRs) that are available for deployment by SCED pursuant to Section 3.8.3, Quick Start Generation Resources, the pricing criteria described in paragraph (a)(i) above applies.

(f)	To ensure that the Energy Offer Curves submitted in accordance with this section meets the requirement for a monotonically increasing Energy Offer Curve, the energy offer for capacity reserved for Responsive Reserve (RRS) and Regulation Up (Reg-Up) must also be offered by the QSE at no less than the minimums set forth in paragraph (a) above.