| NPRR Number | 541 | NPRR Title | SPS and RAP Modeling Clarification |
| --- | --- | --- | --- |
| Date |  | June 14, 2013 |  |
| Submitter’s Information |  |  |  |
| Name |  | Ebby John |  |
| E-mail Address |  | ebby.john@centerpointenergy.com |  |
| Company |  | CenterPoint Energy |  |
| Phone Number |  | 713-207-2136 |  |
| Cell Number |  |  |  |
| Market Segment |  | Investor Owned Utility |  |

Comments

CenterPoint Energy appreciates the opportunity to provide comments on this topic.  The Company requests that Market Notices be issued whenever new or modified Congestion Management Plans are issued.  It should also be clarified that ERCOT is required to comply with the SPS Review and Distribution Operations Support Procedure dated December 1, 2010.

Revised Proposed Protocol Language

3.10.7.4	Definition of Special Protection Systems and Remedial Action Plans

(1)	All Special Protection Systems (SPSs) and Remedial Action Plans (RAPs) used by ERCOT and facility owners to maintain a secure system must be defined in the Network Operations Model.

(2)	Proposed new SPSs and RAPs and proposed changes to SPSs and RAPs must be submitted to ERCOT for review and approval. Affected Entities shall approve proposed new SPSs and RAPs and proposed changes to SPSs and RAPs in accordance with the process outlined in the Operating Guides.  ERCOT shall issue a Market Notice and post all SPSs and RAPs under consideration on the MIS Secure Area within five Business Days of receipt.

(3)	Once a new or changed SPS or RAP is approved by ERCOT pursuant to the document “SPS Review and Distribution Operations Support Procedure” and by all directly affected Entities, the facility owner shall ensure that equipment impacted by an SPS or RAP is modeled correctly.

(4)	ERCOT shall model approved SPSs and RAPs using a NOMCR and include the SPS and RAP  in the security analysis.  The NOMCR shall include a detailed description of the system conditions required to implement the SPS or RAP.  Execution of an SPS or RAP shall be included or assumed in the calculation of LMPs as well as the Network Operations Model.  ERCOT shall issue a Market Notice and post on the MIS Secure Area all approved SPSs and RAPs at least two Business Days before implementation, identifying the date of implementation.