2705 West Lake Drive

Taylor, Texas 76574

(512) 248-6800

HTTP://WWW.ERCOT.COM

Methodology for Incorporating Power Augmentation Costs into the Mitigated Offer Cap of Generation Resources

V0.03

June 28, 2013

Revision History

| Revision | Comments | Date | Author |
| --- | --- | --- | --- |
| v0.01 | First Draft | 6/21/2013 | Ino Gonzalez/ERCOT |
| v0.02 | Minor edits made at RCWG to clarify concepts | 6/28/2013 | RCWG |
| v0.03 | Minor edits to the tables for clarification | 7/8/2013 | ERCOT |

Table of Contents

Section 1 – Introduction	4

Section 2 – Statement of Purpose	4

Section 3 – General form of the Mitigated Offer Cap	4

Section 4 – Proposal for incorporating the power augment VOM costs into the MOC	4

Section 5 – Interim solution sample calculation	7

Section 6 – Long-term system change	9

# Section 1 – Introduction

The objective of this White Paper is to recommend a proposal for incorporating the power augmentation variable O&M (VOM) into the Mitigated Offer Cap (MOC).

# Section 2 – Statement of Purpose

To establish mitigated offer cap values for Resources that are high enough to provide a reasonable expectation of recovery of at least incremental cost when dispatched in Step 2 of SCED, and low enough to provide reasonable mitigation for the management of non-competitive transmission constraints.  In addition, current ERCOT systems only allow one single VOM value for the entire range of the MOC curve without consideration for variations in the variable O&M as power augmentation techniques are added to increase the output of the turbine.  Hence, this white paper introduces a methodology for incorporating power augmentation VOM into the MOC.

# Section 3 – General form of the Mitigated Offer Cap

Resources that are solving non-competitive solutions under Step 2 of SCED are mitigated based on the greater of the reference LMP at the appropriate Resource Node from Step 1 or the MOC.  In general, the MOC is calculated as:

The greater of A or B (based on verifiable costs)

- Generic Heat Rate x Fuel Price

- [Incremental Heat Rate (IHR) x Fuel Price + VOM]* W

Where

W = Factor as defined in Section 4.4.9.4.1 (e) Mitigated Offer Cap of the nodal Protocols.

# Section 4 – Proposal for incorporating the power augment VOM costs into the MOC

Resources using power augmentation techniques such as Peak or Duct Firing, Steam Injection and Air Cooled Condenser Fogging, may be subject to higher operations and maintenance costs, while these technologies are usually not part of the units’ normal operations.  These incremental costs (and possibly changes to the heat rates) cannot be incorporated into the “normal” operating range of the MOC due to current limitations in the ERCOT systems.  As a result, the following proposal offers a manual option to include the VOM into the MOC until a system change is made to allow variations in the variable O&M.

If the Resource has submitted an actual IHR (verifiable cost),

- MOC = [IHRi x FIP + VOM]* W

Where, i = 1...10

And FIP = Fuel Price

And VOM = constant value along the IHR curve

Including units of measurements

- MOC ($/MWh) = [IHRi (MMBtu/MWh) x FIP ($/MMBtu) + VOM ($/MWh)]* W

Including a VOM for power Augmentation

- MOC ($/MWh) = [IHRi (MMBtu/MWh) x FIP ($/MMBtu) + VOM ($/MWh) +  VOMP ($/MWh)]* W

Where,

VOM = average variable O&M for operations without power augmentation, and

VOMP= corresponding variable O&M value for power augmentation technique (assuming only one power augmentation online)

Re-arranging equation 3

- MOC ($/MWh) = [IHRi (MMBtu/MWh) + VOM ($/MWh)/ FIP ($/MMBtu)  +  VOMP ($/MWh)/ FIP ($/MMBtu)] * FIP ($/MMBtu) * W

Or (and omitting the units)

- MOC = [IHRi + VOM / FIP   +  VOMP / FIP ] * FIP * W

Note, the term VOMP / FIP is an implied heat rate (IMHR), or

- MOC = [IHRi + VOM / FIP   +  IMHR ] *FIP * W

And

- MOC = [(IHRi + IMHRj)*FIP + VOM]* W

Where,

IMHRj = VOMP/ FIPavg, and j = 10 (assuming power augmentation is added to the

last point of the IHR curve as shown in Table 1 below.

FIPavg = represents the average Fuel Index Price for the first two weeks of the month 	prior to the effective month.  For example, the FIPavg for August is based on the 	average FIP price for period July 1- July 15.

| Table 1: Summary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Point | MW | IHR | VOM | VOMPa | IMHRb | Final IHR | Final VOM |  |
| 1 | MW1 | IHR1 | VOM | Null | Null | IHR1 | VOM |  |
| 2 | MW2 | IHR2 |  |  |  | IHR2 |  |  |
| 3 | MW3 | IHR3 |  |  |  | IHR3 |  |  |
| 4 | MW4 | IHR4 |  |  |  | IHR4 |  |  |
| 5 | MW5 | IHR5 |  |  |  | IHR5 |  |  |
| 6 | MW6 | IHR6 |  |  |  | IHR6 |  |  |
| 7 | MW7 | IHR7 |  |  |  | IHR7 |  |  |
| 8 | MW8 | IHR8 |  |  |  | IHR8 |  |  |
| 9 | MW9 | IHR9 |  |  |  | IHR9 |  |  |
| 10 | MW10 | IHR10 |  | VOMP | IMHR | IHR10+IMHR |  |  |
| Power Augmentation operating range |  |  |  |  |  |  |  |  |
| a VOMP - Incremental variable cost above normal VOM |  |  |  |  |  |  |  |  |
| b IMHR = VOMP/FIPavg |  |  |  |  |  |  |  |  |

# Section 5 – Interim solution sample calculation

The following example provides an illustration of the manual calculation to incorporate the VOMP of a single power augmentation technique into the MOC.

Typical Power Augmentation Arrangement

Note:  Assume the power augmentation block is 10MW and constant IHR equal to the last point on the curve without power augmentation.

Example using Figure 1

- HSL = 120MW (base load)

- FIP = FIPavg = $4/MMBtu

- VOM = $3/MWh

- VOMP = $80/MWh

- W = 1.1

Using equation 7 above:

MOC ($/MWh) = [{IHR + IMHR}*FIP + VOM]* W

Where,

IMHR = VOMP/ FIPavg

| Table 2: Summary |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Point | MW | IHR | VOM | VOMPa | IMHRb | Final IHR | Final VOM | MOC |  |
| 1 | 30 | 8 | 3 |  |  | 8 | 3 | 38.5 |  |
| 2 | 40 | 8.2 |  |  |  | 8.2 | 3 | 39.38 |  |
| 3 | 50 | 8.4 |  |  |  | 8.4 | 3 | 40.26 |  |
| 4 | 60 | 8.6 |  |  |  | 8.6 | 3 | 41.14 |  |
| 5 | 70 | 8.8 |  |  |  | 8.8 | 3 | 42.02 |  |
| 6 | 80 | 9 |  |  |  | 9 | 3 | 42.9 |  |
| 7 | 90 | 9.2 |  |  |  | 9.2 | 3 | 43.78 |  |
| 8 | 100 | 9.4 |  |  |  | 9.4 | 3 | 44.66 |  |
| 9 | 110 | 9.6 |  |  |  | 9.6 | 3 | 45.54 |  |
| 10 | 120 | 9.6 |  | 80 | 20 | 29.6 | 3 | 133.54 |  |
| Power Augmentation operating range |  |  |  |  |  |  |  |  |  |
| a Incremental cost above normal VOM |  |  |  |  |  |  |  |  |  |
| b Excludes VOM for normal operations ($3/MWh) |  |  |  |  |  |  |  |  |  |
| FIPavg = $4/MMBtu; IMHR = 20($/MWh) / 4($/MMBtu) |  |  |  |  |  |  |  |  |  |

Note 1:  Every month ERCOT calculates a new value of IMHR and combines it with existing IHR.  The effective date for the new IMHR value is the first of every month to the end of month.

Note 2:  If the Incremental Heat Rate changes over the output range of power augmentation, an appropriate adjustment to the Final IHR will be made.

# Section 6 – Long-term system change

The long-term solution as described in Section 4 above involves making changes to ERCOT systems to allow multiple values for VOM.  With system changes, the MOC results in:

- MOC = [IHRi x FIP + VOMi ]* W

Where, i = 1...10

Assuming the power augmentation techniques are not used under normal conditions

Future system changes should include a monotonically increasing VOM curve